3GPP TS 32.746 V1.0.0 (2010-05)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Configuration Management (CM);

Signalling Transport Network (STN) Interface

Network Resource Model (NRM)

Integration Reference Point (IRP):

Solution Set (SS) definitions

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

©2010, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	Solution Set Definitions	7

Annex A (normative):	CORBA Solution Set	8

A.1	Architectural features	8

A.1.1	Syntax for Distinguished Names	8

A.1.2	Rules for NRM extensions	8

A.1.2.1	Allowed extensions	8

A.1.2.2	Extensions not allowed	8

A.2	Mapping	9

A.2.1	General mappings	9

A.2.2	Information Object Class (IOC) mapping	9

A.2.2.1	IOC MtpSignPoint	9

A.2.2.2	IOC SignLinkSetTp	9

A.2.2.3	IOC SignLinkTp	10

A.2.2.4	IOC SignRouteSetNePart	10

A.2.2.5	IOC SignRouteNePart	10

A.2.3	Information Object Class (IOC) Mapping	10

A.2.3.1	IOC M3UAEntity	11

A.2.3.2	IOC M3UALinkSetTp	11

A.2.3.3	IOC M3UALinkTp	11

A.2.3.4	IOC M3UARouteSetNePart	11

A.2.3.5	IOC M3UARouteNePart	12

A.2.3.6	Roles	12

A.3	Solution Set definitions	13

A.3.1	IDL definition structure	13

A.3.2	IDL specification "STNNetworkResourcesIRPSystem.idl"	14

A.3.3	IDL specification "STNNetworkResourcesIRPDefs.idl"	16

Annex B (normative):	XML Definitions	19

B.1	Architectural features	19

B.1.1	Syntax for Distinguished Names	19

B.2	Mapping	19

B.2.1	General mapping	19

B.2.2	Information Object Class (IOC) mapping	19

B.3	Solution Set definitions	20

B.3.1	XML definition structure	20

B.3.2	Graphical Representation	20

B.3.3	XML schema "stnNrm.xsd"	21

Annex C (informative):	Change history	26

# Foreword

- This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

- Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project: Technical Specification Group Services and System Aspects; Telecommunication management; Configuration Management (CM); as identified below:

32.741:	Configuration Management (CM); Signalling Transport Network (STN) interface Network Resource Model (NRM) Integration Reference Point (IRP): Requirements

32.742:	Configuration Management (CM); Signalling Transport Network (STN) interface Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)

32.746:	Configuration Management (CM); Signalling Transport Network (STN) interface Network Resource Model (NRM) Integration Reference Point (IRP): Solution Set (SS) definitions

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. CM actions have the objective to control and monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs.

CM actions may be requested as part of an implementation programme (e.g. additions and deletions), as part of an optimisation programme (e.g. modifications), and to maintain the overall Quality of Service (QoS). The CM actions are initiated either as single actions on single NEs of the 3G network, or as part of a complex procedure involving actions on many resources/objects in one or several NEs.

# 1	Scope

The present document is part of an Integration Reference Point (IRP) named Signalling Transport Network (STN) interface Network Resource Model (NRM) IRP, through which an IRPAgent can communicate configuration management information to one or several IRPManagers concerning STN resources. The STN NRM IRP comprises a set of specifications defining Requirements, a protocol neutral Information Service and one or more Solution Set(s).

- The present document specifies the Solution Sets for the STN NRM IRP.

This specification is related to 3GPP TS 32.742 [9] V10.0.X.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.612: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Information Service (IS)".

[3]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Solution Set (SS) definitions".

[4]	W3C REC-xml-20001006: "Extensible Markup Language (XML) 1.0 (Second Edition)".

[5]	W3C REC-xmlschema-0-20010502: "XML Schema Part 0: Primer".

[6]	W3C REC-xmlschema-1-20010502: "XML Schema Part 1: Structures".

[7]	W3C REC-xmlschema-2-20010502: "XML Schema Part 2: Datatypes".

[8]	W3C REC-xml-names-19990114: "Namespaces in XML".

[9]	3GPP TS 32.742: "Telecommunication management; Configuration Management (CM); Signalling Transport Network (STN) interface Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[10]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[11]	3GPP TS 32.306: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Solution Set (SS) definitions".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

XML file: file containing an XML document

XML document: composed of the succession of an optional XML declaration followed by a root XML element

NOTE:	See [4]; in the scope of the present document.

XML declaration: it specifies the version of XML being used

NOTE:	See [4].

XML element: has a type, is identified by a name, may have a set of XML attribute specifications and is either composed of the succession of an XML start-tag followed by the XML content of the XML element followed by an XML end-tag, or composed simply of an XML empty-element tag; each XML element may contain other XML elements

NOTE:	See [4].

empty XML element: having an empty XML content; an empty XML element still possibly has a set of XML attribute specifications; an empty XML element is either composed of the succession of an XML start-tag directly followed by an XML end-tag, or composed simply of an XML empty-element tag

NOTE:	See [4].

XML content (of an XML element): empty if the XML element is simply composed of an XML empty-element tag; otherwise the part, possibly empty, of the XML element between its XML start-tag and its XML end-tag

XML start-tag: the beginning of a non-empty XML element is marked by an XML start-tag containing the name and the set of XML attribute specifications of the XML element

NOTE:	See [4].

XML end-tag: the end of a non-empty XML element is marked by an XML end-tag containing the name of the XML element

NOTE:	See [4].

XML empty-element tag: composed simply of an empty-element tag containing the name and the set of XML attribute specifications of the XML element.

NOTE:	See [4].

XML attribute specification: has a name and a value

NOTE:	See [4].

DTD: defines structure and content constraints to be respected by an XML document to be valid with regard to this DTD

NOTE:	See [4].

XML schema: more powerful than a DTD, an XML schema defines structure and content constraints to be respected by an XML document to conform with this XML schema; through the use of XML namespaces several XML schemas can be used together by a single XML document; an XML schema is itself also an XML document that shall conform with the XML schema for XML schemas

NOTE:	See [5], [6] and [7].

XML namespace: enables qualifying element and attribute names used in XML documents by associating them with namespaces identified by different XML schemas

NOTE:	See [8], in the scope of the present document.

XML complex type: defined in an XML schema; cannot be directly used in an XML document; can be the concrete type or the derivation base type for an XML element type or for another XML complex type; ultimately defines constraints for an XML element on its XML attribute specifications and/or its XML content

NOTE:	See [5], [6] and [7].

XML element type: declared by an XML schema; can be directly used in an XML document; as the concrete type of an XML element, directly or indirectly defines constraints on its XML attribute specifications and/or its XML content; can also be the concrete type or the derivation base type for another XML element type

NOTE:	See [5], [6] and [7].

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CM	Configuration Management

CORBA	Common Object Request Broker Architecture

DN	Distinguished Name

DTD	Document Type Definition

IDL	Interface Definition Language

IRP	Integration Reference Point

IS	Information Service

MO	Managed Object

MOC	Managed Object Class

NRM	Network Resource Model

OMG	Object Management Group

SS	Solution Set

STN	Signalling Transport Network

UMTS	Universal Mobile Telecommunications System

UTRAN	Universal Terrestrial Radio Access Network

XML	eXtensible Markup Language

# 4	Solution Set Definitions

This specification defines the following 3GPP STN NRM IRP Solution Set Definitions:

- 3GPP STN NRM IRP CORBA SS (Annex A)

- 3GPP STN NRM IRP XML Definitions (Annex B)

Annex A (normative):
CORBA Solution Set

This annex contains the CORBA Solution Set for the IRP whose semantics is specified in STN NRM IRP: Information Service (TS 32.742 [9]).

# A.1	Architectural features

The overall architectural feature of STN Network Resources IRP is specified in 3GPP TS 32.742 [9]. 
This clause specifies features that are specific to the CORBA SS.

## A.1.1	Syntax for Distinguished Names

The syntax of a Distinguished Name is defined in 3GPP TS 32.300 [10].

## A.1.2	Rules for NRM extensions

This clause discusses how the models and IDL definitions provided in the present document can be extended for a particular implementation and still remain compliant with 3GPP SA5's specifications.

### A.1.2.1	Allowed extensions

Vendor-specific MOCs may be supported. The vendor-specific MOCs may support new types of attributes. 
The 3GPP SA5-specified notifications may be issued referring to the vendor-specific MOCs and vendor-specific attributes. New MOCs shall be distinguishable from 3GPP SA5 MOCs by name. 3GPP SA5-specified and vendor-specific attributes may be used in vendor-specific MOCs. Vendor-specific attribute names shall be distinguishable from existing attribute names.

NRM MOCs may be subclassed. Subclassed MOCs shall maintain the specified behaviour of the 3GPP SA5's superior classes.  They may add vendor-specific behaviour with vendor-specific attributes. When subclassing, naming attributes cannot be changed. The subclassed MOC shall support all attributes of its superior class. Vendor-specific attributes cannot be added to 3GPP SA5 NRM MOCs without subclassing.

When subclassing, the 3GPP SA5-specified containment rules and their specified cardinality shall still be followed. 
As an example, ManagementNode (or its subclasses) shall be contained under SubNetwork (or its subclasses).

Managed Object Instances may be instantiated as CORBA objects. This requires that the MOCs be represented in IDL. 3GPP SA5's NRM MOCs are not currently specified in IDL, but may be specified in IDL for instantiation or subclassing purposes. However, management information models should not require that IRPManagers access the instantiated managed objects other than through supported methods in the present document.

Extension rules related to notifications (Notification categories, Event Types, Extended Event Types etc.) are for further study.

### A.1.2.2	Extensions not allowed

The IDL specifications in the present document cannot be edited or altered. Any additional IDL specifications shall be specified in separate IDL files.

IDL interfaces (note: not MOCs) specified in the present document may not be subclassed or extended. New interfaces may be defined with vendor-specific methods.

# A.2	Mapping

## A.2.1	General mappings

Attributes modelling associations as defined in the NRM (here also called "reference attributes") are in this SS mapped to attributes. 
The names of the reference attributes in the NRM are mapped to the corresponding attribute names in the MOC. 
When the cardinality for an association is 0..1 or 1..1 the datatype for the reference attribute is defined as an MOReference. The value of an MO reference contains the distinguished name of the associated MO. 
When the cardinality for an association allows more than one referred MO, the reference attribute will be of type MOReferenceSet, which contains a sequence of MO references.

## A.2.2	Information Object Class (IOC) mapping

This SS supports reference attributes for relations other than containment relations between objects. Reference attributes are therefore introduced in each MOC where needed.

### A.2.2.1	IOC MtpSignPoint

Mapping from NRM IOC MtpSignPoint attributes to SS equivalent MOC MtpSignPoint attributes

| NRM Attributes of IOC MtpSignPoint in 32.742 [9] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| mtpSignPointId | mtpSignPointId | string | Read-Only, M |
| pointCode | pointCode | unsigned long | Read-Only, M |
| networkIndicator | networkIndicator | STNNetworkResourcesIRPSystem::AttributeTypes::NetworkIndicatorType | Read-Only, M |
| pointCodeLength | pointCodeLength | STNNetworkResourcesIRPSystem::AttributeTypes::PointCodeLengthType | Read-Only, M |
| spType | spType | STNNetworkResourcesIRPSystem::AttributeTypes::SPTypeType | Read-Only, M |
| userLabel | userLabel | string | Read-Write, M |

### A.2.2.2	IOC SignLinkSetTp

Mapping from NRM IOC SignLinkSetTp attributes to SS equivalent MOC SignLinkSetTp attributes

| NRM Attributes of IOC SignLinkSetTp in 32.742 [9] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| signLinkSetTpId | signLinkSetTpId | string | Read-Only, M |
| adjPc | adjPc | unsigned long | Read-Only, M |
| userLabel | userLabel | string | Read-Write, M |
| maxCapacityLS | maxCapacityLS | float | Read-Only, M |

### A.2.2.3	IOC SignLinkTp

Mapping from NRM IOC SignLinkTp attributes to SS equivalent MOC SignLinkTp attributes

| NRM Attributes of IOC SignLinkTp in 32.742 [9] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| signLinkTpId | signLinkTpId | string | Read-Only, M |
| slCode | slCode | unsigned long | Read-Only, M |
| slsCodeNormalList | slsCodeNormalList | STNNetworkResourcesIRPSystem::AttributeTypes::SLSListType | Read-Only, M |
| slsCodeCurrentList | slsCodeCurrentList | STNNetworkResourcesIRPSystem::AttributeTypes::SLSListType | Read-Only, M |
| linkTpStatus | linkTpStatus | STNNetworkResourcesIRPSystem::AttributeTypes::LinkStatusType | Read-Only, M |
| maxCapacitySL | maxCapacitySL | float | Read-Only, M |
| userLabel | userLabel | string | Read-Write, M |
| signLinkType | signLinkType | STNNetworkResourcesIRPSystem::AttributeTypes::SignLinkTypeType | Read-Only, M |

### A.2.2.4	IOC SignRouteSetNePart

Mapping from NRM IOC SignRouteSetNePart attributes to SS equivalent MOC SignRouteSetNePart attributes

| NRM Attributes of IOC SignRouteSetNePart in 32.742 [9] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| signRouteSetNePartId | signRouteSetNePartId | string | Read-Only, M |
| destinationPc | destinationPc | unsigned long | Read-Only, M |
| userLabel | userLabel | string | Read-Write, M |
| loadsharingInformationRouteSetNePart | loadsharingInformationRouteSetNePart | string | Read-Only, M |

### A.2.2.5	IOC SignRouteNePart

Mapping from NRM IOC SignRouteNePart attributes and association roles to SS equivalent MOC SignRouteNePart attributes

| NRM Attributes/association roles of IOC SignRouteNePart in 32.742 [9] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| signRouteNePartId | signRouteNePartId | string | Read-Only, M |
| signLinkSetTpPointer | signLinkSetTpPointer | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, M |
| fixedPriority | fixedPriority | unsigned long | Read-Only, M |
| userLabel | userLabel | string | Read-Write, M |

## A.2.3	Information Object Class (IOC) Mapping

This SS supports reference attributes for relations other than containment relations between objects. Reference attributes are therefore introduced in each MOC where needed.

### A.2.3.1	IOC M3UAEntity

Mapping from NRM IOC M3UAEntity attributes to SS equivalent MOC M3UAEntity attributes

| NRM Attributes of IOC m3UAEntity in 32.742 [9] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| m3UAEntityId | m3UAEntityId | string | Read-Only, M |
| m3UAEntityPointCode | m3UAEntityPointCode | unsigned long | Read-Only, M |
| m3UAEntityType | m3UAEntityType | STNNetworkResourcesIRPSystem::AttributeTypes::m3UAEntityTypeType | Read-Only, M |
| networkIndicator | networkIndicator | STNNetworkResourcesIRPSystem::AttributeTypes::networkIndicatorType | Read-Only, M |
| pointCodeLength | pointCodeLength | STNNetworkResourcesIRPSystem::AttributeTypes::PointCodeLengthType | Read-Only, M |

### A.2.3.2	IOC M3UALinkSetTp

Mapping from NRM IOC m3UALinkSetTp attributes to SS equivalent MOC m3UALinkSetTp attributes

| NRM Attributes of IOC m3UALinkSetTp in 32.742 [9] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| m3UALinkSetTPId | m3UALinkSetTPId | string | Read-Only, M |
| adjPc | adjPc | unsigned long | Read-Only, M |
| trafficMode | trafficMode | STNNetworkResourcesIRPSystem::AttributeTypes::trafficModeType | Read-Only, M |

### A.2.3.3	IOC M3UALinkTp

Mapping from NRM IOC m3UALinkTp attributes to SS equivalent MOC m3UALinkTp attributes

| NRM Attributes of IOC m3UALinkTp in 32.742 [9] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| m3UALinkTpId | m3UALinkTpId | string | Read-Only, M |
| m3UALinkTPState | m3UALinkTPState | STNNetworkResourcesIRPSystem::AttributeTypes::m3UALinkTPStateType | Read-Only, M |
| sCTPAssocLocalAddr | sCTPAssocLocalAddr | STNNetworkResourcesIRPSystem::AttributeTypes::sCTPAssocAddrType | Read-Only, M |
| sCTPAssocRemoteAddr | sCTPAssocRemoteAddr | STNNetworkResourcesIRPSystem::AttributeTypes::sCTPAssocAddrType | Read-Only, M |

### A.2.3.4	IOC M3UARouteSetNePart

Mapping from NRM IOC m3UARouteSetNePart attributes to SS equivalent MOC m3UARouteSetNePart attributes

| NRM Attributes of IOC m3UARouteSetNePart in 32.742 [9] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| m3UARouteSetNePartId | m3UARouteSetNePartId | string | Read-Only, M |
| destinationPc | destinationPc | unsigned long | Read-Only, M |
| m3UARouteNePartm3UALinkSetTP | m3UARouteNePartm3UALinkSetTP | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, M |

### A.2.3.5	IOC M3UARouteNePart

Mapping from NRM IOC m3UARouteNePart attributes to SS equivalent MOC m3UARouteNePart attributes

| NRM Attributes of IOC m3UARouteNePart in 32.742 [9] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| m3UARouteNePartId | m3UARouteNePartId | string | Read-Only, M |
| m3UALinkSetTPId | m3UALinkSetTPId | unsigned long | Read-Only, M |
| fixedPriority | fixedPriority | STNNetworkResourcesIRPSystem::AttributeTypes::fixedPriorityType | Read-Only, M |

### A.2.3.6	Roles

Roles of the relation ConnectedTo1

| Name | Definition |
| --- | --- |
| m3UARouteNePart-m3UALinkSetTP | This role (when present) represents M3UARouteNePart capability to identify the connected M3UALinkSetTP. When the role is present, the M3UARouteNePart- M3UALinkSetTP shall carry the M3UALinkSetTP DN. |

# A.3	Solution Set definitions

## A.3.1	IDL definition structure

Clause A.3.2 defines the types which are used by the STN NRM IRP.

Clause A.3.3 defines the MO classes for the STN NRM IRP.

## A.3.2	IDL specification "STNNetworkResourcesIRPSystem.idl"

// File: STNNetworkResourcesIRPSystem.idl

#ifndef _STN_Network_Resources_IRP_System_IDL_

#define _STN_Network_Resources_IRP_System_IDL_

#pragma prefix "3gppsa5.org"

module STNNetworkResourcesIRPSystem

{

/**

*  This module adds datatype definitions for types

*  used in the NRM which are not basic datatypes defined

*  already in CORBA.

*/

module AttributeTypes

{

enum NetworkIndicatorType

{

INTERNATIONAL,

SPARE,

NATIONAL,

NATIONAL_SPARE

};

enum PointCodeLengthType

{

BITS_24,

BITS_14

};

enum SPTypeType

{

SEP,

STP,

STEP

};

typedef unsigned long SLSType; // 0..15

typedef sequence<SLSType,16> SLSListType;

enum LinkStati

{

DEACTIVATED,

FAILED,

LOCAL_BLOCKED,

REMOTE_BLOCKED,

LOCAL_INHIBITED,

REMOTE_INHIBITED

};

typedef sequence <LinkStati,6> LinkStatusType;

enum SignLinkTypeType

{

ST_64K,

ST_2M

};

enum m3UAEntityType

{

M3UA_AS,

SG

};

enum m3UALinkTPStateType

{

UNESTABLISH,

ESTABLISHED,

INACTIVE,

ACTIVE

};

enum AddrType

{

IPV4,

IPV6

};

struct sCTPAssocAddrType

{

unsigned long portId;

AddrType addrType;

string IPaddr;

};

enum trafficModeType

{

OVERRIDE,

LOAD_SHARE,

BROADCAST

};

};

};

#endif // _STN_Network_Resources_IRP_SYSTEM_IDL_

## A.3.3	IDL specification "STNNetworkResourcesIRPDefs.idl"

// File: STNNetworkResourcesIRPDefs.idl

#ifndef _STN_Network_Resources_IRP_DEFS_IDL_

#define _STN_Network_Resources_IRP_DEFS_IDL_

#include "GenericNetworkResourcesNRMDefs.idl"

#pragma prefix "3gppsa5.org"

/**

* This module defines constants for each MO class name and

* the attribute names for each defined MO class.

*/

module STNNetworkResourcesIRPDefs

{

/**

*  Definitions for MO class MtpSignPoint

*/

interface MtpSignPoint: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "MtpSignPoint";

// Attribute Names

//

const string mtpSignPointId = "mtpSignPointId";

const string pointCode = "pointCode";

const string networkIndicator = "networkIndicator";

const string pointCodeLength = "pointCodeLength";

const string spType = "spType";

const string userLabel = "userLabel";

};

/**

*  Definitions for MO class SignLinkSetTp

*/

interface SignLinkSetTp: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "SignLinkSetTp";

// Attribute Names

//

const string signLinkSetTpId = "signLinkSetTpId";

const string adjPc = "adjPc";

const string userLabel = "userLabel";

const string maxCapacityLS = "maxCapacityLS";

};

/**

*  Definitions for MO class SignLinkTp

*/

interface SignLinkTp: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "SignLinkTp";

// Attribute Names

//

const string signLinkTpId = "signLinkTpId";

const string slCode = "slCode";

const string slsCodeNormalList = "slsCodeNormalList";

const string slsCodeCurrentList = "slsCodeCurrentList";

const string linkTpStatus = "linkTpStatus";

const string maxCapacitySL = "maxCapacitySL";

const string userLabel = "userLabel";

const string signLinkType = "signLinkType";

};

/**

*  Definitions for MO class SignRouteSetNePart

*/

interface SignRouteSetNePart: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "SignRouteSetNePart";

// Attribute Names

//

const string signRouteSetNePartId = "signRouteSetNePartId";

const string destinationPc = "destinationPc";

const string userLabel = "userLabel";

const string loadsharingInformationRouteSetNePart = "loadsharingInformationRouteSetNePart";

};

/**

*  Definitions for abstract MO class SignRouteNePart

*

*/

interface SignRouteNePart: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "SignRouteNePart";

// Attribute Names

//

const string signRouteNePartId = "signRouteNePartId";

const string signLinkSetTpPointer = "signLinkSetTpPointer";

const string fixedPriority = "fixedPriority";

const string userLabel = "userLabel";

};

/**

*  Definitions for MO class M3UAEntity

*

*/

interface M3UAEntity: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "M3UAEntity";

// Attribute Names

//

const string m3UAEntityId = "m3UAEntityId";

const string m3UAEntityPointCode = "m3UAEntityPointCode";

const string m3UAEntityType = "m3UAEntityType";

const string networkIndicator = "networkIndicator";

const string pointCodeLength = "pointCodeLength";

};

/**

*  Definitions for MO class M3UALinkSetTp

*/

interface M3UALinkSetTp: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "M3UALinkSetTp";

// Attribute Names

//

const string m3UALinkSetTPId = "m3UALinkSetTPId";

const string adjPc = "adjPc";

const string trafficMode = "trafficMode";

};

/**

*  Definitions for MO class M3UALinkTp

*/

interface M3UALinkTp: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "M3UALinkTp";

// Attribute Names

//

const string m3UALinkTpId = "m3UALinkTpId";

const string m3UALinkTPState = "m3UALinkTPState";

const string sCTPAssocLocalAddr = "sCTPAssocLocalAddr";

const string sCTPAssocRemoteAddr = "sCTPAssocRemoteAddr";

};

/**

*  Definitions for MO class M3UARouteSetNePart

*/

interface M3UARouteSetNePart: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "M3UARouteSetNePart";

// Attribute Names

const string m3UARouteSetNePartId = "m3UARouteSetNePartId";

const string destinationPc = "destinationPc";

const string m3UARouteNePartm3UALinkSetTP = "m3UARouteNePartm3UALinkSetTP";

};

/**

*  Definitions for abstract MO class M3UARouteNePart

*

*/

interface M3UARouteNePart: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "M3UARouteNePart";

// Attribute Names

//

const string m3UARouteNePartId = "m3UARouteNePartId";

const string m3UALinkSetTPId = "m3UALinkSetTPId";

const string fixedPriority = "fixedPriority";

};

};

#endif // _STN_Network_Resources_IRP_DEFS_IDL_

Annex B (normative):
XML Definitions

This annex contains the XML Definitions for the Generic NRM IRP as it applies to Itf-N, in accordance with STN NRM IRP Information Service (TS 32.742 [9]).

# B.1	Architectural features

The overall architectural feature of STN IRP is specified in 3GPP TS 32.742 [9]. 
This clause specifies features that are specific to the Schema definitions.

## B.1.1	Syntax for Distinguished Names

The syntax of a Distinguished Name is defined in 3GPP TS 32.300 [10].

# B.2	Mapping

### B.2.1	General mapping

An IOC maps to an XML element of the same name as the IOC's name in the IS. An IOC attribute maps to a sub-element of the corresponding IOC's XML element, and the name of this sub-element is the same as the attribute's name in the IS.

### B.2.2	Information Object Class (IOC) mapping

Not present in the current version of this specification.

# B.3	Solution Set definitions

## B.3.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by 3GPP TS 32.616 [3].

Annex B.3.3 defines the NRM-specific XML schema stnNrm.xsd for the STN Network Resources IRP NRM defined in 3GPP TS 32.742 [9].

XML schema stnNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in 3GPP TS 32.616 [3].

## B.3.2	Graphical Representation

Not present in the current version of this specification.

## B.3.3	XML schema "stnNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--
  3GPP TS 32.746 STN Network Resources IRP
  Bulk CM Configuration data file NRM-specific XML schema
  stnNrm.xsd
-->

<schema
  targetNamespace=
"http://www.3gpp.org/ftp/specs/archive/32_series/32.746#stnNrm"
  elementFormDefault="qualified"
  xmlns="http://www.w3.org/2001/XMLSchema"
  xmlns:xn=
"http://www.3gpp.org/ftp/specs/archive/32_series/32.626#genericNrm"
  xmlns:stn=
"http://www.3gpp.org/ftp/specs/archive/32_series/32.746#stnNrm"
>
  <import
    namespace=
"http://www.3gpp.org/ftp/specs/archive/32_series/32.626#genericNrm"
  />

  <!-- STN Network Resources IRP NRM attribute related XML types -->

  <simpleType name="networkIndicator">
    <restriction base="string">
      <enumeration value="International"/>
      <enumeration value="Spare"/>
      <enumeration value="National"/>
      <enumeration value="NationalSpare"/>
    </restriction>
  </simpleType>

  <simpleType name="pointCodeLength">
    <restriction base="string">
      <enumeration value="BITS_24"/>
      <enumeration value="BITS_14"/>
    </restriction>
  </simpleType>

  <simpleType name="spType">
    <restriction base="string">
      <enumeration value="SEP"/>
      <enumeration value="STP"/>
      <enumeration value="STEP"/>
    </restriction>
  </simpleType>

  <complexType name="slsCodeList">
    <sequence>
      <element name="slsCode" minOccurs="0" maxOccurs="16">
        <simpleType>
          <restriction base="integer">
            <minInclusive value="0"/>
            <maxInclusive value="15"/>
          </restriction>
        </simpleType>
      </element>
    </sequence>
  </complexType>

<simpleType name="linkTpStatusElementType">

<restriction base="string">

<enumeration value="deactivated"/>

<enumeration value="failed"/>

<enumeration value="localBlocked"/>

<enumeration value="remoteBlocked"/>

<enumeration value="localInhibited"/>

<enumeration value="remoteInhibited"/>

</restriction>

</simpleType>

<complexType name="linkTpStatusType">

<sequence minOccurs="0" maxOccurs="6">

<element name="linkTpStatusElement" type="stn:linkTpStatusElementType"/>

</sequence>

</complexType>

<simpleType name="signLinkType">
    <restriction base="string">
      <enumeration value="ST_64K"/>
      <enumeration value="ST_2M"/>
    </restriction>
  </simpleType>

<simpleType name="m3UAEntityTypeType">

<restriction base="string">

<enumeration value="M3UA_AS"/>

<enumeration value="SG"/>

</restriction>

</simpleType>

<simpleType name="m3UALinkTPStateType">

<restriction base="string">

<enumeration value="UNESTABLISH"/>

<enumeration value="ESTABLISHED"/>

<enumeration value="INACTIVE"/>

<enumeration value="ACTIVE"/>

</restriction>

</simpleType>

<simpleType name="IPAddrTypeType">

<restriction base="string">

<enumeration value="IPv4"/>

<enumeration value="IPv6"/>

</restriction>

</simpleType>

<complexType name="sCTPAssocAddrType">

<sequence minOccurs="0" maxOccurs="unbounded">

<element name="IPAddrType" type="stn:IPAddrTypeType"/>

<element name="IPAddr" type="string"/>

</sequence>

</complexType>

<simpleType name="trafficModeType">

<restriction base="string">

<enumeration value="Override"/>

<enumeration value="LoadShare"/>

<enumeration value="Broadcast"/>

</restriction>

</simpleType>

<!-- STN Network Resources IRP NRM class associated XML elements -->

  <element name="MtpSignPoint" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">
    <complexType>
      <complexContent>
        <extension base="xn:NrmClass">
          <sequence>
            <element name="attributes" minOccurs="0">
              <complexType>
                <all>
                  <element name="pointCode" type="integer" minOccurs="0"/>
                  <element name="networkIndicator" type="stn:networkIndicator" minOccurs="0"/>
                  <element name="pointCodeLength" type="stn:pointCodeLength" minOccurs="0"/>
                  <element name="spType" type="stn:spType" minOccurs="0"/>
                  <element name="userLabel" minOccurs="0"/>
                </all>
              </complexType>
            </element>
            <choice minOccurs="0" maxOccurs="unbounded">
              <element ref="stn:SignLinkSetTp"/>
              <element ref="stn:SignRouteSetNePart"/>
              <element ref="xn:VsDataContainer"/>
            </choice>
          </sequence>
        </extension>
      </complexContent>
    </complexType>
  </element>

  <element name="SignLinkSetTp">
    <complexType>
      <complexContent>
        <extension base="xn:NrmClass">
          <sequence>
            <element name="attributes" minOccurs="0">
              <complexType>
                <all>
                  <element name="adjPc" type="integer" minOccurs="0"/>
                  <element name="userLabel" minOccurs="0"/>
                  <element name="maxCapacityLS" type="integer" minOccurs="0"/>
                </all>
              </complexType>
            </element>
            <choice minOccurs="0" maxOccurs="unbounded">
              <element ref="stn:SignLinkTp"/>
              <element ref="xn:VsDataContainer"/>
            </choice>
          </sequence>
        </extension>
      </complexContent>
    </complexType>
  </element>

  <element name="SignLinkTp">
    <complexType>
      <complexContent>
        <extension base="xn:NrmClass">
          <sequence>
            <element name="attributes" minOccurs="0">
              <complexType>
                <all>
                  <element name="slCode" type="integer" minOccurs="0"/>
                  <element name="slsCodeNormalList" type="stn:slsCodeList" minOccurs="0"/>
                  <element name="slsCodeCurrentList" type="stn:slsCodeList" minOccurs="0"/>
                  <element name="linkTpStatus" type="stn:linkTpStatusType" minOccurs="0"/>
                  <element name="maxCapacitySL" type="integer" minOccurs="0"/>
                  <element name="userLabel" minOccurs="0"/>
                  <element name="signLinkType" type="stn:signLinkType" minOccurs="0"/>
                </all>
              </complexType>
            </element>
            <choice minOccurs="0" maxOccurs="unbounded">
              <element ref="xn:VsDataContainer"/>
            </choice>
          </sequence>
        </extension>
      </complexContent>
    </complexType>
  </element>

  <element name="SignRouteSetNePart">
    <complexType>
      <complexContent>
        <extension base="xn:NrmClass">
          <sequence>
            <element name="attributes" minOccurs="0">
              <complexType>
                <all>
                  <element name="destinationPc" type="integer" minOccurs="0"/>
                  <element name="userLabel" minOccurs="0"/>
                  <element name="loadsharingInformationRouteSetNePart" type="string" minOccurs="0"/>
                </all>
              </complexType>
            </element>
            <choice minOccurs="0" maxOccurs="unbounded">
              <element ref="stn:SignRouteNePart"/>
              <element ref="xn:VsDataContainer"/>
            </choice>
          </sequence>
        </extension>
      </complexContent>
    </complexType>
  </element>

  <element name="SignRouteNePart">
    <complexType>
      <complexContent>
        <extension base="xn:NrmClass">
          <sequence>
            <element name="attributes" minOccurs="0">
              <complexType>
                <all>
                  <element name="signLinkSetTpPointer" type="xn:dn" minOccurs="0"/>
                  <element name="fixedPriority" type="integer" minOccurs="0"/>
                  <element name="userLabel" minOccurs="0"/>
                </all>
              </complexType>
            </element>
            <choice minOccurs="0" maxOccurs="unbounded">
              <element ref="xn:VsDataContainer"/>
            </choice>
          </sequence>
        </extension>
      </complexContent>
    </complexType>
  </element>

<!-- M3UA Network Resources IRP NRM class associated XML elements -->

<element name="M3UAEntity" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<sequence>

<element name="m3UAEntityPointCode" type="unsignedLong" minOccurs="0"/>

<element name="m3UAEntityType" type="stn:m3UAEntityTypeType" minOccurs="0"/>

<element name="networkIndicator" type="stn:networkIndicator" minOccurs="0"/>

<element name="pointCodeLength" type="stn:pointCodeLength" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

</sequence>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="stn:M3UALinkSetTp"/>

<element ref="stn:M3UALinkTp"/>

<element ref="stn:M3UARouteSetNePart"/>

<element ref="stn:M3UARouteNePart"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="M3UALinkSetTp">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="adjPc" type="unsignedLong" minOccurs="0"/>

<element name="trafficMode" type="stn:trafficModeType" minOccurs="0"/>

<element name="userLabel" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="stn:M3UALinkTp"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="M3UALinkTp">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="m3UALinkTPState" type="stn:m3UALinkTPStateType" minOccurs="0"/>

<element name="sCTPAssocLocalAddr" type="stn:sCTPAssocAddrType" minOccurs="0"/>

<element name="sCTPAssocRemoteAddr" type="stn:sCTPAssocAddrType" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="M3UARouteSetNePart">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="destinationPc" type="unsignedLong" minOccurs="0"/>

<element name="m3UARouteNePartm3UALinkSetTP" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="stn:M3UARouteNePart"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="M3UARouteNePart">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="m3UALinkSetTPId" type="string" minOccurs="0"/>

<element name="fixedPriority" type="unsignedLong" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| 05-2010 | SA-48 | SP-100279 | -- | -- | Presentation to SA for information and approval | -- | -- | 1.0.0 |