3GPP TS 32.607 V9.1.0 (2010-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Configuration Management (CM);

Basic CM Integration Reference Point (IRP);

SOAP Solution Set (SS)

(Release 9)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, management, SOAP, architecture

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2010, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	IRP document version number string	6

5	Architectural features	7

5.1	General	7

5.2	Filter language	7

5.3	Syntax for Distinguished Names and Versions	7

6	Mapping	8

6.1	General mappings	8

6.2	Operation mapping	8

6.3	Operation parameter mapping	9

6.3.1	Operation getMoAttributes	9

6.3.1.1	Input parameters	9

6.3.1.2	Output parameters	9

6.3.1.3	Fault definition	10

6.3.2	Operation getContainment	11

6.3.2.1	Input parameters	11

6.3.2.2	Output parameters	11

6.3.2.3	Fault definition	11

6.3.3	Operation createMO	12

6.3.3.1	Input parameters	12

6.3.3.2	Output parameters	12

6.3.3.3	Fault definition	12

6.3.4	Operation deleteMO	13

6.3.4.1	Input parameters	13

6.3.4.2	Output parameters	13

6.3.4.3	Fault definition	13

6.3.5	Operation setMOAttributes	14

6.3.5.1	Input parameters	14

6.3.5.2	Output parameters	14

6.3.5.3	Fault definition	15

Annex A (normative):	WSDL specification	16

Annex B (informative):	Void	23

Annex C (informative):	Change history	24

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project: Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.601:	Configuration Management (CM); Basic CM Integration Reference Point (IRP); Requirements

32.602:	Configuration Management (CM); Basic CM Integration Reference Point (IRP); Information Service (IS)

32.603:	Configuration Management (CM); Basic CM Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)

32.607:	Configuration Management (CM); Basic CM Integration Reference Point (IRP); SOAP Solution Set (SS)

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. CM actions have the objective to control and monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs.

CM actions may be requested as part of an implementation programme (e.g. additions and deletions), as part of an optimisation programme (e.g. modifications), and to maintain the overall Quality of Service (QoS). The CM actions are initiated either as single actions on single NEs of the 3G network, or as part of a complex procedure involving actions on many resources/objects in one or several NEs.

# 1	Scope

The purpose of this Basic CM IRP: SOAP Solution Set is to define the mapping of the Basic CM IRP: IS (3GPP TS 32.602 [4]) to the protocol specific details necessary for implementation of this IRP in a SOAP/WSDL environment.

The present document defines NRM independent data types and methods.

This Solution Set specification is related to 3G TS 32.602 V9.0.X.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP) Information Service (IS)".

[5]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[6]	3GPP TS 32.172: "Telecommunication management; Subscription Management (SuM) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service"

[7]	3GPP TS 32.311: "Telecommunication management; Generic Integration Reference Point (IRP) management; Requirement".

[8]	3GPP TS 32.317: "Telecommunication management; Generic Integration Reference Point (IRP) management; SOAP solution set".

[9]	3GPP TS 32.667: "Telecommunication management; Configuration Management (CM); Kernel CM Integration Reference Point (IRP): SOAP solution set".

[10]	W3C SOAP 1.1 specification (http://www.w3.org/TR/2000/NOTE-SOAP-20000508/)

[11]	W3C WSDL 1.1 specification (http://www.w3.org/TR/2001/NOTE-wsdl-20010315)

[12]	W3C XPath 1.0 specification (http://www.w3.org/TR/1999/REC-xpath-19991116)

[13]	WS-I Basic Profile Version 1.1 (http://www.ws-i.org/Profiles/BasicProfile-1.1-2004-08-24.html)

[14]	W3C SOAP 1.2 specification (http://www.w3.org/TR/soap12-part1/)

# 3	Definitions and abbreviations

## 3.1	Definitions

For terms and definitions refer to 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.600 [3] and 3GPP TS 32.602 [4].

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

DN	Distinguished Name

IS	Information Service

IRP	Integration Reference Point

MO	Managed Object

MOC	Managed Object Class

NRM	Network Resource Model

OMG	Object Management Group

SS	Solution Set

WSDL	Web Service Description Language

WS-I 	Web Services Interoperability Organization

# 4	IRP document version number string

The IRP document version number (sometimes called "IRPVersion" or "SS version number") string is used to identify this specification.  The string is derived using a rule described in 3GPP TS 32.311 [7].

This string (or sequence of strings, if more than one version is supported) is returned in getBasicCmIRPVersion method.

# 5	Architectural features

## 5.1	General

The overall architectural feature of Basic Configuration Management IRP is specified in 3GPP TS 32.602 [4]. 
This clause specifies features that are specific to the SOAP SS.

The SOAP 1.1 specification [10] and WSDL 1.1 specification [11] are supported.

The SOAP 1.2 specification [14] is supported optionally.

This specification uses "document" style in the WSDL description.

This specification uses "literal" encoding style in the WSDL description.

This specification uses a number of namespace prefixes throughout that are listed in Table 5.1.

Table 5.1: Prefixes and Namespaces used in this specification

| Prefix | Namespace |
| --- | --- |
| http | http://schemas.xmlsoap.org/wsdl/http/ |
| soap | http://schemas.xmlsoap.org/wsdl/soap/ |
| SOAP-ENV | http://schemas.xmlsoap.org/soap/envelope/ |
| SOAP-ENC or soapenc | http://schemas.xmlsoap.org/soap/encoding/ |
| xs or xsd | http://www.w3.org/2001/XMLSchema |
| xsi | http://www.w3.org/2001/XMLSchema-instance |
| basicCMIRPSystem | http://www.3gpp.org/ftp/Specs/archive/32_series/32607/schema/32607-800/BasicCMIRPSystem |
| basicCMIRPData | http://www.3gpp.org/ftp/Specs/archive/32_series/32607/schema/32607-800/BasicCMIRPData |
| genericIRPSystem | http://www.3gpp.org/ftp/Specs/archive/32_series/32317/schema/32317-800/GenericIRPSystem |

The WSDL structure is like Figure 5.1:

Figure 5.1: BasicCM IRP SOAP Solution Set WSDL structure

## 5.2	Filter language

The filter language used in the SS is the XPath Language (see W3C XPath 1.0 specification [12]). IRPAgents may throw a FilterComplexityLimit fault when a given filter is too complex.

## 5.3	Syntax for Distinguished Names and Versions

The format of a Distinguished Name is defined in 3GPP TS 32.300 [5].

The version of this IRP is represented as a string (see also clause 4).

# 6	Mapping

## 6.1	General mappings

The IS parameter name managedObjectInstance is mapped into DN.

Attributes modelling associations as defined in the NRM (here also called "reference attributes") are in this SS mapped to attributes. The names of the reference attributes in the NRM are mapped to the corresponding attribute names in the MOC. When the cardinality for an association is 0..1 or 1..1 the datatype for the reference attribute is defined as a MOReference. The value of an MO reference contains the distinguished name of the associated MO. When the cardinality for an association allows more than one referred MO, the reference attribute will be of type MOReferenceSet, which contains a sequence of MO references.

If a reference attribute is changed, an AttributeValueChange notification (see 3GPP TS 32.667 [9]) is emitted.

## 6.2	Operation mapping

The Basic CM IRP: IS (see 3GPP TS 32.602 [4]) defines semantics of operation visible across the Basic Configuration Management IRP. Table 6.2 indicates mapping of these operations to their equivalents defined in this SS.

Table 6.2: Mapping from IS Operation to SS equivalents

| IS Operation (3GPP TS 32.602 [4]) | SS Operation | Qualifier |
| --- | --- | --- |
| getMoAttributes | getMOAttributes | M |
| getContainment | getContainment | O |
| cancelOperation (see note 1) | N/A | N/A |
| createMO | createMO | O |
| deleteMO | deleteMO | O |
| setMOAttributes | setMOAttributes | O |
| getIRPVersion (see note 2) | getIRPVersion | M |
| getOperationProfile (see note 2) | getOperationProfile | O |
| getNotificationProfile (see note 2) | getNotificationProfile | O |
| NOTE 1: This operation is NOT mapped because it’s useful for one-request-and-multiple-responses operations, which are not used in this Solution Set. NOTE 2: This operation is of IOC ManagedGenericIRP specified in [10]. The IOC BasicCmIRP of [4] inherits from it. |  |  |

## 6.3	Operation parameter mapping

The Basic CM IRP: IS (see 3GPP TS 32.602 [4]) defines semantics of parameters carried in operations across the Basic Configuration Management IRP.  The following tables show the mapping of these parameters, as per operation, to their equivalents defined in the present document.

### 6.3.1	Operation getMoAttributes

#### 6.3.1.1	Input parameters

Mapping from IS getMoAttributes input parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| invokeIdentifierIn | string invokeIdentifierIn | M |
| baseObjectInstance, scope, filter, attributeListIn | string queryXpathExp | M |

Here is the XML schema fragment of the getMOAttributes request:

<!-- getMOAttributes Request -->

<element name="getMOAttributes">

<complexType>

<sequence>

<element name="invokeIdentifierIn" type="string"/>

<element name="queryXpathExp" type="string"/>

</sequence>

</complexType>

</element>

#### 6.3.1.2	Output parameters

Mapping from IS getMoAttributes output parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| invokeIdentifierOut | string invokeIdentifierOut | M |
| managedObjectClass, managedObjectInstance, attributeListOut | basicCMIRPData:MOSequenceType moiListOut | M |
| status | basicCMIRPData:getMOAttributesFault | M |

The specific “attributeListOut” definition depends on the corresponding NRM XML definition.

Here is the XML schema fragment of the getMOAttributes response:

<!-- getMoAttributes Response -->

<element name="getMoAttributesResponse">

<complexType>

<sequence>

<element name="moiListOut" type="basicCMIRPData:MOSequenceType"/>

<element name="invokeIdentifierOut" type="string"/>

</sequence>

</complexType>

</element>

#### 6.3.1.3	Fault definition

<!-- getMoAttributes Fault -->

<element name="getMOAttributesFault">

<complexType>

<choice>

<element name="getMOAttributesFault" type="string"/>

<element name="resourceLimitationFault" type="string"/>

<element name="operationCancelledFault" type="string"/>

<element name="complexityLimitationFault" type="string"/>

<element ref="basicCMIRPData:InvalidParameterFault"/>

</choice>

</complexType>

</element>

### 6.3.2	Operation getContainment

#### 6.3.2.1	Input parameters

Mapping from IS getContainment input parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| invokeIdentifierIn | string invokeIdentifierIn | M |
| baseObjectInstance, scope | string queryXpathExp | M |

Here is the XML schema fragment of the getContainment request:

<!-- getContainment Request -->

<element name="getContainment">

<complexType>

<sequence>

<element name="invokeIdentifierIn" type="string"/>

<element name="queryXpathExp" type="string"/>

</sequence>

</complexType>

</element>

#### 6.3.2.2	Output parameters

Mapping from IS getContainment output parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| invokeIdentifierOut | string invokeIdentifierOut | M |
| containment | string topContainerLoc | M |
| status | basicCMIRPData:getContainmentFault | M |

Here is the XML schema fragment of the getContainment response:

<!-- getContainment Response -->

<element name="getContainmentResponse">

<complexType>

<sequence>

<element name="invokeIdentifierOut" type="string"/>

<element name="topContainerLoc" type="string"/>

<!--each element contains only id attribute and other MO it contains -->

<any minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</complexType>

</element>

#### 6.3.2.3	Fault definition

<!-- getContainment Fault -->

<element name="getContainmentFault">

<complexType>

<choice>

<element name="getContainmentFault" type="string"/>

<element name="resourceLimitationFault" type="string"/>

<element name="operationCancelledFault" type="string"/>

<element name="complexityLimitationFault" type="string"/>

<element ref="basicCMIRPData:OperationNotSupportedFault"/>

<element ref="basicCMIRPData:InvalidParameterFault"/>

</choice>

</complexType>

</element>

### 6.3.3	Operation createMO

#### 6.3.3.1	Input parameters

Mapping from IS createMO input parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| managedObjectClass, managedObjectInstance | string mOIElementLoc | M |
| referenceObjectInstance | string referenceObjectInstance | O |
| attributeListIn | basicCMIRPData:AnyMOType mO | M |

The specific “attributeListIn” definition depends on the corresponding NRM XML definition.

Here is the XML schema fragment of the createMO request:

<!-- createMO Request -->

<element name="createMO">

<complexType>

<sequence>

<element name="mOIElementLoc" type="string"/>

<element name="referenceObjectInstance" type="string"/>

<element name="mO" type="basicCMIRPData:AnyMOType"/>

</sequence>

</complexType>

</element>

#### 6.3.3.2	Output parameters

Mapping from IS createMO output parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| attributeListOut | basicCMIRPData:AnyMOType mO | M |
| status | basicCMIRPData:createMOFault | M |

The specific “attributeListOut” definition depends on the corresponding NRM XML definition.

Here is the XML schema fragment of the createMO response:

<!-- createMO Response -->

<element name="createMOResponse">

<complexType>

<sequence>

<element name="mO" type="basicCMIRPData:AnyMOType"/>

</sequence>

</complexType>

</element>

#### 6.3.3.3	Fault definition

<!-- createMO Fault -->

<element name="createMOFault">

<complexType>

<choice>

<element name="createMOFault" type="string"/>

<element name="objectClassSpecificationMissmatchedFault" type="string"/>

<element name="InvalidObjectInstanceFault" type="string"/>

<element name="createNotAllowedFault" type="string"/>

<element name="noSuchObjectClassFault" type="string"/>

<element name="classInstanceConflictFault" type="string"/>

<element name="noSuchAttributeFault" type="string"/>

<element name="invalidAttributeValueFault" type="string"/>

<element name="missingAttributeValueFault" type="string"/>

<element name="parentObjectDoesNotExistFault" type="string"/>

<element ref="basicCMIRPData:OperationNotSupportedFault"/>

<element ref="basicCMIRPData:InvalidParameterFault"/>

</choice>

</complexType>

</element>

### 6.3.4	Operation deleteMO

#### 6.3.4.1	Input parameters

Mapping from IS deleteMO input parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| baseObjectInstance, scope, filter | string queryXpathExp | M |

Here is the XML schema fragment of the deleteMO request:

<!-- deleteMO Request -->

<element name="deleteMO">

<complexType>

<sequence>

<element name="queryXpathExp" type="string"/>

</sequence>

</complexType>

</element>

#### 6.3.4.2	Output parameters

Mapping from IS deleteMO output parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| deletionList | basicCMIRPData:MOSequenceTypedeletionList | M |
| status | basicCMIRPData:deleteMOFault | M |

Here is the XML schema fragment of the deleteMO response:

<!-- deleteMO Response -->

<element name="deleteMOResponse">

<complexType>

<sequence>

<element name="deletionList" type="basicCMIRPData:MOSequenceType">

</element>

</sequence>

</complexType>

</element>

#### 6.3.4.3	Fault definition

<!-- deleteMO Fault -->

<element name="deleteMOFault">

<complexType>

<choice>

<element name="deleteMOFault" type="string"/>

<element name="invalidObjectInstanceFault" type="string"/>

<element name="deleteNotAllowedFault" type="string"/>

<element name="resourceLimitationFault" type="string"/>

<element name="complexityLimitationFault" type="string"/>

<element ref="basicCMIRPData:OperationNotSupportedFault"/>

<element ref="basicCMIRPData:InvalidParameterFault"/>

</choice>

</complexType>

</element>

### 6.3.5	Operation setMOAttributes

#### 6.3.5.1	Input parameters

Mapping from IS setMOAttributes input parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| baseObjectInstance, scope, filter, modificationList | stringqueryXpathExp | M |
| modificationList | basicCMIRPData:AttributeModificationSetType modificationList | M |

Here is the XML schema fragment of the setMOAttributes request:

<!-- setMOAttributes Request -->

<element name="setMOAttributes">

<complexType>

<sequence>

<element name="queryXpathExp" type="string"/>

<element name="modificationList" type="basicCMIRPData:AttributeModificationSetType"/>

</sequence>

</complexType>

</element>

<complexType name=”AttributeModificationSetType”>

<sequence>

<element name="AttributeModification" maxOccurs="unbounded">

<complexType>

<sequence>

<any/>

<element name="operator" type=”basicCMIRPData:ModifyOperatorType”/>

</sequence>

</complexType>

</element>

</sequence>

</complexType>

<simpleType name="ModifyOperatorType">

<restriction base="string">

<enumeration value="REPLACE"/>

<enumeration value="ADDValues"/>

<enumeration value="REMOVEValues"/>

<enumeration value="SETToDefault"/>

</restriction>

</simpleType>

#### 6.3.5.2	Output parameters

Mapping from IS setMOAttributes output parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| modificationListOut | basicCMIRPData:MOSequenceTypemodificationListOut | M |
| status | basicCMIRPData:setMOAttributesFault | M |

Here is the XML schema fragment of the setMOAttributes response:

<!-- setMOAttributes Response -->

<element name="setMOAttributesResponse">

<complexType>

<sequence>

<element name="modificationListOut" type="basicCMIRPData:MOSequenceType"/>

</sequence>

</complexType>

</element>

#### 6.3.5.3	Fault definition

<!-- setMOAttributes Fault -->

<element name="setMOAttributesFault">

<complexType>

<choice>

<element name="setMOAttributesFault" type="string"/>

<element name="modifyNotAllowedFault" type="string"/>

<element name="noSuchAttributeFault" type="string"/>

<element name="invalidAttributeValueFault" type="string"/>

<element name="missingAttributeValueFault" type="string"/>

<element name="resourceLimitationFault" type="string"/>

<element name="complexityLimitationFault" type="string"/>

<element ref="basicCMIRPData:OperationNotSupportedFault"/>

<element ref="basicCMIRPData:InvalidParameterFault"/>

</choice>

</complexType>

</element>

Annex A (normative):
WSDL specification

<?xml version="1.0" encoding="UTF-8"?>

<definitions xmlns="http://schemas.xmlsoap.org/wsdl/" xmlns:soap="http://schemas.xmlsoap.org/wsdl/soap/" xmlns:http="http://schemas.xmlsoap.org/wsdl/http/" xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns:soapenc="http://schemas.xmlsoap.org/soap/encoding/" xmlns:basicCMIRPSystem="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/BasicCMIRPSystem"

xmlns:basicCMIRPData="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/BasicCMIRPData"

xmlns:genericIRPSystem="http://www.3gpp.org/ftp/Specs/archive/32_series/32.317/schema/32317-800/GenericIRPSystem"

targetNamespace="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/BasicCMIRPSystem">

<import namespace="http://www.3gpp.org/ftp/Specs/archive/32_series/32.317/schema/32317-800/GenericIRPSystem" location="http://www.3gpp.org/ftp/Specs/archive/32_series/32.317/schema/32317-800-wsdl.zip"/>

<types>

<schema targetNamespace="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/BasicCMIRPData" xmlns="http://www.w3.org/2001/XMLSchema">

<!-- getMOAttributes Request -->

<element name="getMOAttributes">

<complexType>

<sequence>

<element name="invokeIdentifierIn" type="string"/>

<element name="queryXpathExp" type="string"/>

</sequence>

</complexType>

</element>

<!-- getMoAttributes Response -->

<element name="getMoAttributesResponse">

<complexType>

<sequence>

<element name="invokeIdentifierOut" type="string"/>

<element name="moiListOut" type="basicCMIRPData:MOSequenceType"/>

</sequence>

</complexType>

</element>

<complexType name="AnyMOType">

<sequence>

<!--MO instance location in XPath expression-->

<element name="moiLocation" type="string"/>

<!--each MO-->

<any/>

</sequence>

</complexType>

<complexType name="MOSequenceType">

<sequence>

<element name="mo" type="basicCMIRPData:AnyMOType" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</complexType>

<!-- getMoAttributes Fault -->

<element name="getMOAttributesFault">

<complexType>

<choice>

<element name="getMOAttributesFault" type="string"/>

<element name="resourceLimitationFault" type="string"/>

<element name="operationCancelledFault" type="string"/>

<element name="complexityLimitationFault" type="string"/>

<element ref="basicCMIRPData:InvalidParameterFault"/>

</choice>

</complexType>

</element>

<!-- getContainment Request -->

<element name="getContainment">

<complexType>

<sequence>

<element name="invokeIdentifierIn" type="string"/>

<element name="queryXpathExp" type="string"/>

</sequence>

</complexType>

</element>

<!-- getContainment Response -->

<element name="getContainmentResponse">

<complexType>

<sequence>

<!--top container element xpath location -->

<element name="invokeIdentifierOut" type="string"/>

<element name="topContainerLoc" type="string"/>

<!--each element contains only id attribute and other MO it contains -->

<any minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</complexType>

</element>

<!-- getContainment Fault -->

<element name="getContainmentFault">

<complexType>

<choice>

<element name="getContainmentFault" type="string"/>

<element name="resourceLimitationFault" type="string"/>

<element name="operationCancelledFault" type="string"/>

<element name="complexityLimitationFault" type="string"/>

<element ref="basicCMIRPData:OperationNotSupportedFault"/>

<element ref="basicCMIRPData:InvalidParameterFault"/>

</choice>

</complexType>

</element>

<!-- createMO Request -->

<element name="createMO">

<complexType>

<sequence>

<element name="mOIElementLoc" type="string"/>

<element name="referenceObjectInstance" type="string"/>

<element name="mO" type="basicCMIRPData:AnyMOType"/>

</sequence>

</complexType>

</element>

<!-- createMO Response -->

<element name="createMOResponse">

<complexType>

<sequence>

<element name="mO" type="basicCMIRPData:AnyMOType"/>

</sequence>

</complexType>

</element>

<!-- createMO Fault -->

<element name="createMOFault">

<complexType>

<choice>

<element name="createMOFault" type="string"/>

<element name="objectClassSpecificationMissmatchedFault" type="string"/>

<element name="InvalidObjectInstanceFault" type="string"/>

<element name="createNotAllowedFault" type="string"/>

<element name="noSuchObjectClassFault" type="string"/>

<element name="classInstanceConflictFault" type="string"/>

<element name="noSuchAttributeFault" type="string"/>

<element name="invalidAttributeValueFault" type="string"/>

<element name="missingAttributeValueFault" type="string"/>

<element name="parentObjectDoesNotExistFault" type="string"/>

<element ref="basicCMIRPData:OperationNotSupportedFault"/>

<element ref="basicCMIRPData:InvalidParameterFault"/>

</choice>

</complexType>

</element>

<!-- deleteMO Request -->

<element name="deleteMO">

<complexType>

<sequence>

<element name="queryXpathExp" type="string"/>

</sequence>

</complexType>

</element>

<!-- deleteMO Response -->

<element name="deleteMOResponse">

<complexType>

<sequence>

<element name="deletionList" type="basicCMIRPData:MOSequenceType">

</element>

</sequence>

</complexType>

</element>

<!-- deleteMO Fault -->

<element name="deleteMOFault">

<complexType>

<choice>

<element name="deleteMOFault" type="string"/>

<element name="invalidObjectInstanceFault" type="string"/>

<element name="deleteNotAllowedFault" type="string"/>

<element name="resourceLimitationFault" type="string"/>

<element name="complexityLimitationFault" type="string"/>

<element ref="basicCMIRPData:OperationNotSupportedFault"/>

<element ref="basicCMIRPData:InvalidParameterFault"/>

</choice>

</complexType>

</element>

<!-- setMOAttributes Request -->

<element name="setMOAttributes">

<complexType>

<sequence>

<element name="queryXpathExp" type="string"/>

<element name="modificationList" type="basicCMIRPData:AttributeModificationSetType"/>

</sequence>

</complexType>

</element>

<simpleType name="ModifyOperatorType">

<restriction base="string">

<enumeration value="REPLACE"/>

<enumeration value="ADDValues"/>

<enumeration value="REMOVEValues"/>

<enumeration value="SETToDefault"/>

</restriction>

</simpleType>

<complexType name="AttributeModificationSetType">

<sequence>

<element name="AttributeModification" maxOccurs="unbounded">

<complexType>

<sequence>

<any/>

<element name="operator" type="basicCMIRPData:ModifyOperatorType"/>

</sequence>

</complexType>

</element>

</sequence>

</complexType>

<!-- setMOAttributes Response -->

<element name="setMOAttributesResponse">

<complexType>

<sequence>

<element name="modificationListOut" type="basicCMIRPData:MOSequenceType"/>

</sequence>

</complexType>

</element>

<!-- setMOAttributes Fault -->

<element name="setMOAttributesFault">

<complexType>

<choice>

<element name="setMOAttributesFault" type="string"/>

<element name="modifyNotAllowedFault" type="string"/>

<element name="noSuchAttributeFault" type="string"/>

<element name="invalidAttributeValueFault" type="string"/>

<element name="missingAttributeValueFault" type="string"/>

<element name="resourceLimitationFault" type="string"/>

<element name="complexityLimitationFault" type="string"/>

<element ref="basicCMIRPData:OperationNotSupportedFault"/>

<element ref="basicCMIRPData:InvalidParameterFault"/>

</choice>

</complexType>

</element>

<element name="OperationNotSupportedFault" type="string"/>

<element name="InvalidParameterFault" type="string"/>

<simpleType name="VersionNumberType">

<restriction base="string"/>

</simpleType>

<complexType name="VersionNumberSetType">

<sequence>

<element name="versionNumber" type="basicCMIRPData:VersionNumberType" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="ParameterSetType">

<sequence>

<element name="parameterName" type="string" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="OperationType">

<sequence>

<element name="operationName" type="string"/>

<element name="parameterSet" type="basicCMIRPData:ParameterSetType"/>

</sequence>

</complexType>

<complexType name="OperationSetType">

<sequence>

<element name="operation" type="basicCMIRPData:OperationType" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="NotificationType">

<sequence>

<element name="notificationName" type="string"/>

<element name="parameterSet" type="basicCMIRPData:ParameterSetType"/>

</sequence>

</complexType>

<complexType name="NotificationSetType">

<sequence>

<element name="notification" type="basicCMIRPData:NotificationType" maxOccurs="unbounded"/>

</sequence>

</complexType>

</schema>

</types>

<message name="getMOAttributesRequest">

<part name="parameter" element="basicCMIRPData:getMOAttributes"/>

</message>

<message name="getMOAttributesResponse">

<part name="parameter" element="basicCMIRPData:getMoAttributesResponse"/>

</message>

<message name="getMOAttributesFault">

<part name="parameter" element="basicCMIRPData:getMOAttributesFault"/>

</message>

<message name="getContainmentRequest">

<part name="parameter" element="basicCMIRPData:getContainment"/>

</message>

<message name="getContainmentResponse">

<part name="parameter" element="basicCMIRPData:getContainmentResponse"/>

</message>

<message name="getContainmentFault">

<part name="parameter" element="basicCMIRPData:getContainmentFault"/>

</message>

<message name="createMORequest">

<part name="parameter" element="basicCMIRPData:createMO"/>

</message>

<message name="createMOResponse">

<part name="parameter" element="basicCMIRPData:createMOResponse"/>

</message>

<message name="createMOFault">

<part name="parameter" element="basicCMIRPData:createMOFault"/>

</message>

<message name="deleteMORequest">

<part name="parameter" element="basicCMIRPData:deleteMO"/>

</message>

<message name="deleteMOResponse">

<part name="parameter" element="basicCMIRPData:deleteMOResponse"/>

</message>

<message name="deleteMOFault">

<part name="parameter" element="basicCMIRPData:deleteMOFault"/>

</message>

<message name="setMOAttributesRequest">

<part name="parameter" element="basicCMIRPData:setMOAttributes"/>

</message>

<message name="setMOAttributesResponse">

<part name="parameter" element="basicCMIRPData:setMOAttributesResponse"/>

</message>

<message name="setMOAttributesFault">

<part name="parameter" element="basicCMIRPData:setMOAttributesFault"/>

</message>

<portType name="BasicCMIRP">

<operation name="getIRPVersion">

<input message="genericIRPSystem:getIRPVersionRequest"/>

<output message="genericIRPSystem:getIRPVersionResponse"/>

<fault name="getIRPVersionFault" message="genericIRPSystem:getIRPVersionFault"/>

</operation>

<operation name="getOperationProfile">

<input message="genericIRPSystem:getOperationProfileRequest"/>

<output message="genericIRPSystem:getOperationProfileResponse"/>

<fault name="getOperationProfileFault" message="genericIRPSystem:getOperationProfileFault"/>

</operation>

<operation name="getNotificationProfile">

<input message="genericIRPSystem:getNotificationProfileRequest"/>

<output message="genericIRPSystem:getNotificationProfileResponse"/>

<fault name="getNotificationProfileFault" message="genericIRPSystem:getNotificationProfileFault"/>

</operation>

<operation name="getMOAttributes">

<input message="basicCMIRPSystem:getMOAttributesRequest"/>

<output message="basicCMIRPSystem:getMOAttributesResponse"/>

<fault name="getMOAttributesFault" message="basicCMIRPSystem:getMOAttributesFault"/>

</operation>

<operation name="getContainment">

<input message="basicCMIRPSystem:getContainmentRequest"/>

<output message="basicCMIRPSystem:getContainmentResponse"/>

<fault name="getContainmentFault" message="basicCMIRPSystem:getContainmentFault"/>

</operation>

<operation name="createMO">

<input message="basicCMIRPSystem:createMORequest"/>

<output message="basicCMIRPSystem:createMOResponse"/>

<fault name="createMOFault" message="basicCMIRPSystem:createMOFault"/>

</operation>

<operation name="deleteMO">

<input message="basicCMIRPSystem:deleteMORequest"/>

<output message="basicCMIRPSystem:deleteMOResponse"/>

<fault name="deleteMOFault" message="basicCMIRPSystem:deleteMOFault"/>

</operation>

<operation name="setMOAttributes">

<input message="basicCMIRPSystem:setMOAttributesRequest"/>

<output message="basicCMIRPSystem:setMOAttributesResponse"/>

<fault name="setMOAttributesFault" message="basicCMIRPSystem:setMOAttributesFault"/>

</operation>

</portType>

<binding name="BasicCMIRP" type="basicCMIRPSystem:BasicCMIRP">

<soap:binding style="document" transport="http://schemas.xmlsoap.org/soap/http"/>

<operation name="getIRPVersion">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/getIRPVersion"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="getIRPVersionFault">

<soap:fault name="getIRPVersionFault" use="literal"/>

</fault>

</operation>

<operation name="getOperationProfile">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/getOperationProfile"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="getOperationProfileFault">

<soap:fault name="getOperationProfileFault" use="literal"/>

</fault>

</operation>

<operation name="getNotificationProfile">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/getNotificationProfile"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="getNotificationProfileFault">

<soap:fault name="getNotificationProfileFault" use="literal"/>

</fault>

</operation>

<operation name="getMOAttributes">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/getMOAttributes"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="getMOAttributesFault">

<soap:fault name="getMOAttributesFault" use="literal"/>

</fault>

</operation>

<operation name="getContainment">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/getContainment"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="getContainmentFault">

<soap:fault name="getContainmentFault" use="literal"/>

</fault>

</operation>

<operation name="createMO">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/createMO"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="createMOFault">

<soap:fault name="createMOFault" use="literal"/>

</fault>

</operation>

<operation name="deleteMO">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/deleteMO"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="deleteMOFault">

<soap:fault name="deleteMOFault" use="literal"/>

</fault>

</operation>

<operation name="setMOAttributes">

<soap:operation soapAction="http://www.3gpp.org/ftp/Specs/archive/32_series/32.607/schema/32607-800/setMOAttributes"/>

<input>

<soap:body use="literal"/>

</input>

<output>

<soap:body use="literal"/>

</output>

<fault name="setMOAttributesFault">

<soap:fault name="setMOAttributesFault" use="literal"/>

</fault>

</operation>

</binding>

<service name="BasicCMIRP">

<port name="BasicCMIRP" binding="basicCMIRPSystem:BasicCMIRP">

<soap:address location="To be defined."/>

</port>

</service>

</definitions>

Annex B (informative):
Void

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | R | Subject/Comment | Cat | Old | New |
| Dec 2005 | SA_30 | SP-050732 | -- | -- | Submitted to TSG SA#30 for Information | -- | 1.0.0 |  |
| Dec 2006 | SA_34 | SP-060742 | -- | -- | Submitted to TSG SA #34 for Approval | -- | 2.0.0 | 7.0.0 |
| Mar 2007 | -- | -- | -- | -- | Delete reference to the 32.604 CMIP SS. Reason: SA#35 endorsed the SA5 decision to not propagate the CMIP Solution Sets to Rel-7 (TS 32.3x4, TS 32.4x4, TS 32.6x4) | -- | 7.0.0 | 7.0.1 |
| Jun 2007 | SA_36 | -- | -- | -- | Introduction clean-up. TS layout cosmetics | -- | 7.0.1 | 7.0.2 |
| Jun 2007 | -- | -- | -- | -- | Changed TS Title. Reason: SOAP does not stand anymore for "Simple Object Access Protocol" | -- | 7.0.2 | 7.0.3 |
| Dec 2008 | SA_42 | SP-080845 | 0001 | - | Incorrect URI specified in TS 32.607 (both word document and corresponding wsdl file) | F | 7.0.3 | 7.10 |
| Dec 2008 | SA_42 | -- | -- | -- | Upgrade to Release 8 | -- | 7.1.0 | 8.0.0 |
| Dec 2009 | SA_46 | SP-090718 | 002 | -- | Increase the linked IS version by the CR for IS to clarify scope parameters for getMoAttributes operation | F | 8.0.0 | 8.1.0 |
| Dec 2009 | SA_46 | SP-090719 | 001 | -- | Discontinuation of the maintenance of WSDL/XML schema folders | F | 8.1.0 | 9.0.0 |
| Sep 2010 | SA_49 | SP-100488 | 004 | -- | Syntax errors in WSDL schema | A | 9.0.0 | 9.1.0 |