3GPP TSG-SA5 (Telecom Management)	S5-102912

Meeting SA5#74, 08 – 10 Nov 2010, Issy-les-Moulineaux, FRANCE	revision of S5-102827

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0161 |  rev | 2 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add the missing RecordType for GWMBMSRecord |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | MBMS_EPS |  |  |  |  |  |  | Date:  |  | 10/11/2010 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | -The asn.1 description for MBMS-GW-CDR is missing in PS Domain CDRs and the CTEID parameter of MBMS-GW-CDR is also missing under the PS DATA TYPES. -CDR TAG for GWMBMSRecord is missing. -RrecordType for GWMBMSRecord is missing form Generic ASN.1 definitions. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add asn.1 description for GWMBMSRecord and CTEID parameter under PS data types. Add CDR Tag description for GWMBMSRecord. Add RecordType description for GWMBMSRecord. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | There would be no criterion for encoding MBMS-GW-CDR |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.1, 5.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| Affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

### 5.2.1	Generic ASN.1 definitions

This subclause contains generic CDR syntax definitions, where the term "generic" implies that these constructs are applicable for more than one domain/service/subsystem. Examples of this are syntax definitions that are imported from non-charging 3GPP TSs, e.g. TS 29.002 [214].

GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallReferenceNumber, NumberOfForwarding

FROM MAP-CH-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CH-DataTypes (13) version6 (6) }

-- from TS 29.002 [214]

AddressString, BasicServiceCode, IMSI, IMEI, ISDN-AddressString, LCSClientExternalID, LCSClientInternalID

FROM MAP-CommonDataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18) version6 (6) }

-- from TS 29.002 [214]

DestinationRoutingAddress

FROM CAP-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) cap-datatypes (52) version1 (0) }

-- from TS 29.078 [217]

DefaultCallHandling, DefaultSMS-Handling, NotificationToMSUser, ServiceKey

FROM MAP-MS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version6 (6) }

-- from TS 29.002 [214]

Ext-GeographicalInformation, LCSClientType, LCS-Priority, LocationType

FROM MAP-LCS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25) version7 (7)}

-- from TS 29.002 [214]

PositionMethodFailure-Diagnostic, UnauthorizedLCSClient-Diagnostic

FROM MAP-ER-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-ER-DataTypes (17) version7 (7)}

--

-- from TS 29.002 [214]

--

BasicService

FROM Basic-Service-Elements	{ itu-t identified-organization (4) etsi (0)

196 basic-service-elements (8) }

--

-- from "Digital Subscriber Signalling System No. one (DSS1) protocol" ETS 300 196 [310]

--

ObjectInstance

FROM CMIP-1 {joint-iso-itu-t ms (9) cmip (1) version1 (1) protocol (3)}

--

-- from Rec. X.2ab[304]  Editor’s note:  clarify if this definition is still needed.  It appears that it ends in Nirvana.

ManagementExtension

FROM Attribute-ASN1Module {joint-iso-itu-t ms (9) smi (3) part2 (2) asn1Module (2) 1}

--

-- from Rec. X.721 [305]  Editor’s note:  clarify if this definition is still needed.

--

AE-title

FROM ACSE-1 {joint-iso-itu-t association-control (2) abstract-syntax (1) apdus (0) version (1) };

--

-- From Rec. X.2cd[306].  Note that the syntax of AE-title to be used is from

-- ITU-T Rec. X.227 / ISO 8650 corrigendum and not "ANY"

--  Editor’s note:  clarify if this definition is still needed.  It appears that it ends in Nirvana.

;

------------------------------------------------------------------------------

--

--  Generic Data Types

--

------------------------------------------------------------------------------

BCDDirectoryNumber		::= OCTET STRING

--

-- This type contains the binary coded decimal representation of

-- a directory number e.g. calling/called/connected/translated number.

-- The encoding of the octet string is in accordance with the

-- the elements "Calling party BCD number", "Called party BCD number"

-- and "Connected number" defined in TS 24.008 [208].

-- This encoding includes type of number and number plan information

-- together with a BCD encoded digit string.

-- It may also contain both a presentation and screening indicator

-- (octet 3a).

- For the avoidance of doubt, this field does not include

-- octets 1 and 2, the element name and length, as this would be

-- redundant.

--

CallDuration 			::= INTEGER

--

-- The call duration is counted in seconds.

-- For successful calls /sessions / PDP contexts, this is the chargeable duration.

-- For call attempts this is the call holding time.

--

Editor’s note:  the explanation above should be removed as proper definitions are required in the individual CDR parameter descriptions in TS 32.250[10] – TS 32.275 [35]

CalledNumber			::= BCDDirectoryNumber

RecordType 	::= INTEGER

{--

--	Record values 0..17 are CS specific.

--	The contents are defined in TS 32.250 [10]

moCallRecord		(0),

mtCallRecord		(1),

roamingRecord		(2),

incGatewayRecord	(3),

outGatewayRecord	(4),

transitCallRecord	(5),

moSMSRecord			(6),

mtSMSRecord			(7),

moSMSIWRecord		(8),

mtSMSGWRecord		(9),

ssActionRecord		(10),

hlrIntRecord		(11),

locUpdateHLRRecord	(12),

locUpdateVLRRecord	(13),

commonEquipRecord	(14),

moTraceRecord		(15),	--- used in earlier releases

mtTraceRecord		(16),	--- used in earlier releases

termCAMELRecord	    (17),

--

--	Record values 18..22 are GPRS specific.

--	The contents are defined in TS 32.251 [11]

--

sgsnPDPRecord		(18),

sgsnMMRecord		(20),

sgsnSMORecord		(21),

sgsnSMTRecord		(22),

--

--  Record values 23..25 are CS-LCS specific.

--	The contents are defined in TS 32.250 [10]

--

mtLCSRecord			(23),

moLCSRecord			(24),

niLCSRecord			(25),

--

--  Record values 26..28 are GPRS-LCS specific.

--	The contents are defined in TS 32.251 [11]

--

sgsnMtLCSRecord		(26),

sgsnMoLCSRecord		(27),

sgsnNiLCSRecord		(28),

--

--  Record values 30..62 are MMS specific.

--  The contents are defined in TS 32.270 [30]

--

mMO1SRecord			(30),

mMO4FRqRecord		(31),

mMO4FRsRecord		(32),

mMO4DRecord			(33),

mMO1DRecord			(34),

mMO4RRecord			(35),

mMO1RRecord			(36),

mMOMDRecord			(37),

mMR4FRecord			(38),

mMR1NRqRecord		(39),

mMR1NRsRecord		(40),

mMR1RtRecord		(41),

mMR1AFRecord		(42),

mMR4DRqRecord		(43),

mMR4DRsRecord		(44),

mMR1RRRecord		(45),

mMR4RRqRecord		(46),

mMR4RRsRecord		(47),

mMRMDRecord			(48),

mMFRecord			(49),

mMBx1SRecord		(50),

mMBx1VRecord		(51),

mMBx1URecord		(52),

mMBx1DRecord		(53),

mM7SRecord			(54),

mM7DRqRecord		(55),

mM7DRsRecord		(56),

mM7CRecord			(57),

mM7RRecord			(58),

mM7DRRqRecord		(59),

mM7DRRsRecord		(60),

mM7RRqRecord		(61),

mM7RRsRecord		(62),

--

--  Record values 63..69, 70, 82 are IMS specific.

--  The contents are defined in TS 32.260 [20]

--

sCSCFRecord			(63),

pCSCFRecord			(64),

iCSCFRecord			(65),

mRFCRecord			(66),

mGCFRecord			(67),

bGCFRecord			(68),

aSRecord			(69),

eCSCFRecord			(70),

iBCFRecord			(82),

--

--  Record values 71..75 are LCS specific.

--  The contents are defined in TS 32.271 [31]

--

lCSGMORecord		(71),

lCSRGMTRecord		(72),

lCSHGMTRecord		(73),

lCSVGMTRecord		(74),

lCSGNIRecord		(75),

--

--  Record values 76..79,86 are MBMS specific.

--  The contents are defined in TS 32.251 [11]

--  Record values 76,77 and 86 are MBMS bearer context specific

--

sgsnMBMSRecord		(76),

ggsnMBMSRecord		(77),

gwMBMSRecord        (86),

--

--  And TS 32.273 [33]

--  Record values 78 and 79 are MBMS service specific

--  and defined in TS 32.273 [33]

--

sUBBMSCRecord		(78),

cONTENTBMSCRecord	(79),

--

--  Record Values 80..81 are PoC specific.

--  The contents are defined in TS 32.272 [32]

--

pPFRecord			(80),

cPFRecord			(81),

--  Record values 84..85 are EPC specific.

--  The contents are defined in TS 32.251 [11]

--

sGWRecord			(84),

pGWRecord			(85),

--

--  Record Value 83 is MMTel specific.

--  The contents are defined in TS 32.275 [35]

--

mMTelRecord			(83)

}

CallingNumber	::= BCDDirectoryNumber

CallReference			::= INTEGER

CellId	::= OCTET STRING (SIZE(2))

--

-- Coded according to TS 24.008 [208]

--

ChargeIndicator			::= INTEGER

{

noCharge			(0),

charge				(1)

}

Diagnostics						::= CHOICE

{

gsm0408Cause				[0] INTEGER,

-- See TS 24.008 [208]

gsm0902MapErrorValue		[1] INTEGER,

-- Note: The value to be stored here corresponds to

-- the local values defined in the MAP-Errors and

-- MAP-DialogueInformation modules, for full details

-- see TS 29.002 [214].

itu-tQ767Cause				[2] INTEGER,

-- See Q.767 [309]

networkSpecificCause		[3] ManagementExtension,

-- To be defined by network operator

manufacturerSpecificCause	[4] ManagementExtension,

-- To be defined by manufacturer

positionMethodFailureCause	[5] PositionMethodFailure-Diagnostic,

-- see TS 29.002 [214]

unauthorizedLCSClientCause	[6] UnauthorizedLCSClient-Diagnostic

-- see TS 29.002 [214]

}

IPAddress	::= CHOICE

{

iPBinaryAddress	 IPBinaryAddress,

iPTextRepresentedAddress	 IPTextRepresentedAddress

}

IPBinaryAddress	::= CHOICE

{

iPBinV4Address			[0] OCTET STRING (SIZE(4)),

iPBinV6Address			[1] OCTET STRING (SIZE(16))

}

IPTextRepresentedAddress	::= CHOICE

{	--

-- IP address in the familiar "dot" notation

--

iPTextV4Address			[2] IA5String (SIZE(7..15)),

iPTextV6Address			[3] IA5String (SIZE(15..45))

}

LCSCause				::= OCTET STRING (SIZE(1))

--

-- See LCS Cause Value, TS 49.031 [227]

--

LCSClientIdentity 		::= SEQUENCE

{

lcsClientExternalID	[0] LCSClientExternalID OPTIONAL,

lcsClientDialedByMS	[1] AddressString OPTIONAL,

lcsClientInternalID	[2] LCSClientInternalID OPTIONAL

}

LCSQoSInfo				::= OCTET STRING (SIZE(4))

--

-- See LCS QoS IE, TS 49.031 [227]

--

LevelOfCAMELService		::= BIT STRING

{

basic						(0),

callDurationSupervision		(1),

onlineCharging				(2)

}

LocalSequenceNumber ::= INTEGER (0..4294967295)

--

-- Sequence number of the record in this node

-- 0.. 4294967295 is equivalent to 0..2**32-1, unsigned integer in four octets

--

LocationAreaAndCell		::= SEQUENCE

{

locationAreaCode	[0] LocationAreaCode,

cellId				[1] CellId,

mCC-MNC				[2] MCC-MNC OPTIONAL

}

LocationAreaCode		::= OCTET STRING (SIZE(2))

--

-- See TS 24.008 [208]

--

MCC-MNC		::= OCTET STRING (SIZE(3))

--

-- See TS 24.008 [208]

--

ManagementExtensions	::= SET OF ManagementExtension

MessageReference		::= OCTET STRING

MscNo					::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

MSISDN					::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

MSTimeZone	::= OCTET STRING (SIZE (2))

--

-- 1.Octet: Time Zone and 2. Octet: Daylight saving time, see TS 29.060 [215]

--

NodeAddress ::= CHOICE

{

iPAddress 	[0] IPAddress,

domainName	[1] GraphicString

}

PositioningData	::= OCTET STRING (SIZE(1..33))

--

-- See Positioning Data IE (octet 3..n), TS 49.031 [227]

--

RecordingEntity 		::= AddressString

ServiceContextID		::= UTF8String

ServiceSpecificInfo  ::=  SEQUENCE

{

serviceSpecificData		[0] GraphicString OPTIONAL, 
	serviceSpecificType		[1] INTEGER OPTIONAL

}

SMSResult					::= Diagnostics

SmsTpDestinationNumber ::= OCTET STRING

--

-- This type contains the binary coded decimal representation of

-- the SMS address field the encoding of the octet string is in

-- accordance with the definition of address fields in TS 23.040 [201].

-- This encoding includes type of number and numbering plan indication

-- together with the address value range.

--

SubscriptionID	::= SET

{

subscriptionIDType	[0]	SubscriptionIDType,

subscriptionIDData	[1]	UTF8String

}

SubscriptionIDType	::= ENUMERATED

{

eND-USER-E164		(0),

eND-USER-IMSI		(1),

eND-USER-SIP-URI	(2),

eND-USER-NAI		(3),

eND-USER-PRIVATE	(4)

}

SystemType	::= ENUMERATED

{	--

--  "unknown" is not to be used in PS domain.

--

unknown				(0),

iuUTRAN				(1),

gERAN				(2)

}

TimeStamp					::= OCTET STRING (SIZE(9))

--

-- The contents of this field are a compact form of the UTCTime format

-- containing local time plus an offset to universal time. Binary coded

-- decimal encoding is employed for the digits to reduce the storage and

-- transmission overhead

-- e.g. YYMMDDhhmmssShhmm

-- where

-- YY 	= 	Year 00 to 99		BCD encoded

-- MM 	= 	Month 01 to 12 		BCD encoded

-- DD	=	Day 01 to 31		BCD encoded

-- hh	=	hour 00 to 23		BCD encoded

-- mm	=	minute 00 to 59		BCD encoded

-- ss	=	second 00 to 59		BCD encoded

-- S	=	Sign 0 = "+", "-"	ASCII encoded

-- hh	=	hour 00 to 23		BCD encoded

-- mm	=	minute 00 to 59		BCD encoded

--

END

Next Changes

### 5.2.2.2	PS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the GPRS and EPC CDR types defined in TS 32.251 [11].

GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration, CalledNumber, RecordType, CallingNumber, CallReferenceNumber, CellId, DefaultSMS-Handling, Diagnostics, Ext-GeographicalInformation, IMSI, IMEI, IPAddress, ISDN-AddressString, LCSCause, LCSClientExternalID, LCSClientIdentity, LCSClientInternalID, LCSClientType, LCS-Priority, LCSQoSInfo, LevelOfCAMELService, LocalSequenceNumber, LocationAreaAndCell, LocationAreaCode, LocationType, ManagementExtensions, MessageReference, MSISDN, NotificationToMSUser, PositioningData, RecordingEntity, ServiceKey, ServiceSpecificInfo, SMSResult, SmsTpDestinationNumber, SubscriptionID, TimeStamp, AddressString, MSTimeZone

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

DefaultGPRS-Handling, RAIdentity

FROM MAP-MS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version6 (6)}

--

-- from TS 29.002 [214]

--

LocationMethod

FROM SS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2) version7 (7)}

--

-- from TS 24.080 [209]

--

MBMS2G3GIndicator, FileRepairSupported, MBMSServiceType, MBMSUserServiceType, RequiredMBMSBearerCapabilities, MBMSSessionIdentity, TMGI, MBMSInformation

FROM MBMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) mbmsChargingDataTypes (8) asn1Module (0) version1 (0)}

Editor’s note:  consider moving the above 2 items also into the generic module in order to avoid again copying from external sources.

;

------------------------------------------------------------------------------

--

--  GPRS RECORDS

--

------------------------------------------------------------------------------

GPRSRecord	::= CHOICE

--

-- Record values 20, 22..27 are specific

-- Record values 76,77,86 are MBMS specific

-- Record values 78..79 are EPC specific

--

{

sgsnPDPRecord			[20] SGSNPDPRecord,

sgsnMMRecord			[22] SGSNMMRecord,

sgsnSMORecord			[23] SGSNSMORecord,

sgsnSMTRecord			[24] SGSNSMTRecord,

sgsnLCTRecord			[25] SGSNLCTRecord,

sgsnLCORecord			[26] SGSNLCORecord,

sgsnLCNRecord			[27] SGSNLCNRecord,

--

sgsnMBMSRecord			[76] SGSNMBMSRecord,

ggsnMBMSRecord			[77] GGSNMBMSRecord

gwMBMSRecord			[86] GWMBMSRecord,

sGWRecord				[78] SGWRecord,

pGWRecord				[79] PGWRecord

}

SGWRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI,

s-GWAddress					[4] GSNAddress,

chargingID					[5] ChargingID,

servingNodeAddress			[6] SEQUENCE OF GSNAddress,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servingNodePLMNIdentifier	[27] PLMN-Id OPTIONAL,

servedIMEISV				[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

mSTimeZone 					[31] MSTimeZone OPTIONAL,

userLocationInformation		[32] OCTET STRING OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

servingNodeType				[35] SEQUENCE OF ServingNodeType,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionID				[40] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 	[41] NULL OPTIONAL,

userCSGInformation			[42] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL

}

PGWRecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI,

p-GWAddress						[4] GSNAddress,

chargingID						[5] ChargingID,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext				[25] NULL OPTIONAL,

externalChargingID				[26] OCTET STRING OPTIONAL,

servinggNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEISV					[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

cAMELChargingInformation		[33] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionID					[41] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 		[42] NULL OPTIONAL,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

3GPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 			[45] PDPAddress OPTIONAL

}

SGSNMMRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

sgsnAddress					[3] GSNAddress OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

routingArea					[5] RoutingAreaCode OPTIONAL,

locationAreaCode			[6] LocationAreaCode OPTIONAL,

cellIdentifier				[7] CellId OPTIONAL,

changeLocation				[8] SEQUENCE OF ChangeLocation OPTIONAL,

recordOpeningTime			[9] TimeStamp,

duration					[10] CallDuration OPTIONAL,

sgsnChange					[11] SGSNChange OPTIONAL,

causeForRecClosing			[12] CauseForRecClosing,

diagnostics					[13] Diagnostics OPTIONAL,

recordSequenceNumber		[14] INTEGER OPTIONAL,

nodeID						[15] NodeID OPTIONAL,

recordExtensions			[16] ManagementExtensions OPTIONAL,

localSequenceNumber			[17] LocalSequenceNumber OPTIONAL,

servedMSISDN				[18] MSISDN OPTIONAL,

chargingCharacteristics		[19] ChargingCharacteristics,

cAMELInformationMM 			[20] CAMELInformationMM OPTIONAL,

rATType						[21] RATType OPTIONAL,

chChSelectionMode			[22] ChChSelectionMode OPTIONAL,

cellPLMNId					[23] PLMN-Id OPTIONAL

}

SGSNPDPRecord 	::= SET

{

recordType					[0] RecordType,

networkInitiation			[1] NetworkInitiatedPDPContext OPTIONAL,

servedIMSI					[3] IMSI,

servedIMEI					[4] IMEI OPTIONAL,

sgsnAddress					[5] GSNAddress OPTIONAL,

msNetworkCapability			[6] MSNetworkCapability OPTIONAL,

routingArea					[7] RoutingAreaCode OPTIONAL,

locationAreaCode			[8] LocationAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

chargingID					[10] ChargingID,

ggsnAddressUsed				[11] GSNAddress,

accessPointNameNI			[12] AccessPointNameNI OPTIONAL,

pdpType						[13] PDPType OPTIONAL,

servedPDPAddress			[14] PDPAddress OPTIONAL,

listOfTrafficVolumes		[15] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[16] TimeStamp,

duration					[17] CallDuration,

sgsnChange					[18] SGSNChange OPTIONAL,

causeForRecClosing			[19] CauseForRecClosing,

diagnostics					[20] Diagnostics OPTIONAL,

recordSequenceNumber		[21] INTEGER OPTIONAL,

nodeID						[22] NodeID OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL,

localSequenceNumber			[24] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[25] APNSelectionMode OPTIONAL,

accessPointNameOI			[26] AccessPointNameOI OPTIONAL,

servedMSISDN				[27] MSISDN OPTIONAL,

chargingCharacteristics		[28] ChargingCharacteristics,

rATType						[29] RATType OPTIONAL,

cAMELInformationPDP  		[30] CAMELInformationPDP OPTIONAL,

rNCUnsentDownlinkVolume		[31] DataVolumeGPRS OPTIONAL,

chChSelectionMode			[32] ChChSelectionMode OPTIONAL,

dynamicAddressFlag			[33] DynamicAddressFlag OPTIONAL,

iMSIunauthenticatedFlag 	[34] NULL OPTIONAL,

userCSGInformation			[35] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[36] PDPAddress OPTIONAL

}

SGSNSMORecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

serviceCentre				[5] AddressString OPTIONAL,

recordingEntity				[6] RecordingEntity OPTIONAL,

locationArea				[7] LocationAreaCode OPTIONAL,

routingArea					[8] RoutingAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

messageReference			[10] MessageReference,

eventTimeStamp				[11] TimeStamp,

smsResult					[12] SMSResult OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

nodeID						[14] NodeID OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		[16] ChargingCharacteristics,

rATType						[17] RATType OPTIONAL,

destinationNumber			[18] SmsTpDestinationNumber OPTIONAL,

cAMELInformationSMS			[19] CAMELInformationSMS OPTIONAL,

chChSelectionMode			[20] ChChSelectionMode OPTIONAL

}

SGSNSMTRecord	::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN			[3] MSISDN OPTIONAL,

msNetworkCapability		[4] MSNetworkCapability OPTIONAL,

serviceCentre			[5] AddressString OPTIONAL,

recordingEntity			[6] RecordingEntity OPTIONAL,

locationArea			[7] LocationAreaCode OPTIONAL,

routingArea				[8] RoutingAreaCode OPTIONAL,

cellIdentifier			[9] CellId OPTIONAL,

eventTimeStamp			[10] TimeStamp,

smsResult				[11] SMSResult OPTIONAL,

recordExtensions		[12] ManagementExtensions OPTIONAL,

nodeID					[13] NodeID OPTIONAL,

localSequenceNumber		[14] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[15] ChargingCharacteristics,

rATType					[16] RATType OPTIONAL,

chChSelectionMode		[17] ChChSelectionMode OPTIONAL,

cAMELInformationSMS		[18] CAMELInformationSMS OPTIONAL

}

SGSNMTLCSRecord			::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType,

lcsClientIdentity		[3] LCSClientIdentity,

servedIMSI				[4] IMSI,

servedMSISDN			[5] MSISDN OPTIONAL,

sgsnAddress				[6] GSNAddress OPTIONAL,

locationType			[7] LocationType,

lcsQos					[8] LCSQoSInfo OPTIONAL,

lcsPriority				[9] LCS-Priority OPTIONAL,

mlcNumber				[10] ISDN-AddressString,

eventTimeStamp			[11] TimeStamp,

measurementDuration		[12] CallDuration OPTIONAL,

notificationToMSUser	[13] NotificationToMSUser OPTIONAL,

privacyOverride			[14] NULL OPTIONAL,

location				[15] LocationAreaAndCell OPTIONAL,

routingArea				[16] RoutingAreaCode OPTIONAL,

locationEstimate		[17] Ext-GeographicalInformation OPTIONAL,

positioningData			[18] PositioningData OPTIONAL,

lcsCause				[19] LCSCause OPTIONAL,

diagnostics				[20] Diagnostics OPTIONAL,

nodeID					[21] NodeID OPTIONAL,

localSequenceNumber		[22] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[23] ChargingCharacteristics,

chChSelectionMode		[24] ChChSelectionMode OPTIONAL,

rATType					[25] RATType OPTIONAL,

recordExtensions		[26] ManagementExtensions OPTIONAL,

causeForRecClosing		[27] CauseForRecClosing

}

SGSNMOLCSRecord			::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI,

servedMSISDN			[5] MSISDN OPTIONAL,

sgsnAddress				[6] GSNAddress OPTIONAL,

locationMethod			[7] LocationMethod,

lcsQos					[8] LCSQoSInfo OPTIONAL,

lcsPriority				[9] LCS-Priority OPTIONAL,

mlcNumber				[10] ISDN-AddressString OPTIONAL,

eventTimeStamp			[11] TimeStamp,

measurementDuration		[12] CallDuration OPTIONAL,

location				[13] LocationAreaAndCell OPTIONAL,

routingArea				[14] RoutingAreaCode OPTIONAL,

locationEstimate		[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause				[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

nodeID					[19] NodeID OPTIONAL,

localSequenceNumber	    [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[21] ChargingCharacteristics,

chChSelectionMode		[22] ChChSelectionMode OPTIONAL,

rATType					[23] RATType OPTIONAL,

recordExtensions		[24] ManagementExtensions OPTIONAL,

causeForRecClosing		[25] CauseForRecClosing

}

SGSNNILCSRecord			::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI OPTIONAL,

servedMSISDN			[5] MSISDN OPTIONAL,

sgsnAddress				[6] GSNAddress OPTIONAL,

servedIMEI				[7] IMEI OPTIONAL,

lcsQos					[8] LCSQoSInfo OPTIONAL,

lcsPriority				[9] LCS-Priority OPTIONAL,

mlcNumber				[10] ISDN-AddressString OPTIONAL,

eventTimeStamp			[11] TimeStamp,

measurementDuration		[12] CallDuration OPTIONAL,

location				[13] LocationAreaAndCell OPTIONAL,

routingArea			    [14] RoutingAreaCode OPTIONAL,

locationEstimate		[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause				[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

nodeID					[19] NodeID OPTIONAL,

localSequenceNumber	    [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[21] ChargingCharacteristics,

chChSelectionMode		[22] ChChSelectionMode OPTIONAL,

rATType					[23] RATType OPTIONAL,

recordExtensions		[24] ManagementExtensions OPTIONAL,

causeForRecClosing		[25] CauseForRecClosing

}

SGSNMBMSRecord 	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofRAs					[3] SEQUENCE OF RAIdentity OPTIONAL,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

sgsnPLMNIdentifier			[15] PLMN-Id OPTIONAL,

numberofReceivingUE			[16] Integer OPTIONAL,

mbmsInformation				[17] MBMSInformation OPTIONAL

}

GGSNMBMSRecord 	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

mbmsInformation				[15] MBMSInformation OPTIONAL

}

GWMBMSRecord 	::= SET

{

recordType					[0] RecordType,

mbmsGWAddress				[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

pdpPDNType					[5] PDPType OPTIONAL,

servedPDPPDNAddress			[6] PDPAddress OPTIONAL,

listOfTrafficVolumes		[7] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[8] TimeStamp,

duration					[9] CallDuration,

causeForRecClosing			[10] CauseForRecClosing,

diagnostics					[11] Diagnostics OPTIONAL,

recordSequenceNumber		[12] INTEGER OPTIONAL,

nodeID						[13] NodeID OPTIONAL,

recordExtensions			[14] ManagementExtensions OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

mbmsInformation				[16] MBMSInformation OPTIONAL,

commonTeid   				[17] CTEID OPTIONAL,

iPMulticastSourceAddress	[18] PDPAddress OPTIONAL

}

------------------------------------------------------------------------------

--

--  PS DATA TYPES

--

------------------------------------------------------------------------------

AccessPointNameNI	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of APN in  dot representation.

-- For example, if the complete APN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- NI is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR..

--

AccessPointNameOI	::= IA5String (SIZE(1..37))

--

-- Operator Identifier part of APN in dot representation.

-- In the 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs' example, the OI portion is 'mnc022.mcc111.gprs'

-- and is presented in this form in the CDR.

--

AFChargingIdentifier ::= OCTECT STRING

--

-- see AF-Charging-Identifier AVP as defined in TS 29.214[221]

--

AFRecordInformation	::= SEQUENCE

{

aFChargingIdentifier		[1] AFChargingIdentifier,

flows						[2] Flows OPTIONAL

}

APNSelectionMode::= ENUMERATED

--

-- See Information Elements TS 29.060 [215], TS 29.274 [223] or TS 29.275 [224]

--

{

mSorNetworkProvidedSubscriptionVerified				(0),

mSProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

CAMELAccessPointNameNI	::= AccessPointNameNI

CAMELAccessPointNameOI	::= AccessPointNameOI

CAMELInformationMM		::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2]	ServiceKey OPTIONAL,

defaultTransactionHandling		[3]	DefaultGPRS-Handling OPTIONAL,

numberOfDPEncountered  			[4] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[5] LevelOfCAMELService OPTIONAL,

freeFormatData					[6] FreeFormatData OPTIONAL,

fFDAppendIndicator				[7]	FFDAppendIndicator OPTIONAL

}

CAMELInformationPDP	::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2]	ServiceKey OPTIONAL,

defaultTransactionHandling		[3]	DefaultGPRS-Handling OPTIONAL,

cAMELAccessPointNameNI			[4] CAMELAccessPointNameNI OPTIONAL,

cAMELAccessPointNameOI			[5] CAMELAccessPointNameOI OPTIONAL,

numberOfDPEncountered			[6] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[7] LevelOfCAMELService OPTIONAL,

freeFormatData					[8]	FreeFormatData OPTIONAL,

fFDAppendIndicator				[9]	FFDAppendIndicator OPTIONAL

}

CAMELInformationSMS		::= SET

{

sCFAddress								[1] SCFAddress OPTIONAL,

serviceKey								[2]	ServiceKey OPTIONAL,

defaultSMSHandling						[3]	DefaultSMS-Handling OPTIONAL,

cAMELCallingPartyNumber					[4] CallingNumber OPTIONAL,

cAMELDestinationSubscriberNumber		[5] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress						[6] AddressString OPTIONAL,

freeFormatData							[7]	FreeFormatData OPTIONAL,

smsReferenceNumber						[8] CallReferenceNumber	OPTIONAL

}

CauseForRecClosing	::= INTEGER

--

-- In PGW-CDR and SGW-CDR the value servingNodeChange is used for partial record

-- generation due to Serving Node Address list Overflow

-- In SGSN servingNodeChange indicates the SGSN change

--

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [214]

--

-- cause codes 0 to 15 are defined 'CauseForTerm' (cause for termination)

--

{

normalRelease					(0),

abnormalRelease					(4),

cAMELInitCallRelease			(5),

volumeLimit						(16),

timeLimit						(17),

servingNodeChange				(18),

maxChangeCond					(19),

managementIntervention			(20),

intraSGSNIntersystemChange		(21),

rATChange						(22),

mSTimeZoneChange				(23),

sGSNPLMNIDChange 				(24),

unauthorizedRequestingNetwork	(52),

unauthorizedLCSClient			(53),

positionMethodFailure			(54),

unknownOrUnreachableLCSClient	(58),

listofDownstreamNodeChange		(59)

}

ChangeCondition	::= ENUMERATED

{

qoSChange								(0),

tariffTime								(1),

recordClosure							(2),

cGI-SAICHange							(6), 	-- bearer modification. “CGI-SAI Change”

rAIChange								(7), 	-- bearer modification. “RAI Change”

dT-Establishment						(8),

dT-Removal								(9),

eCGIChange								(10), 	-- bearer modification. “ECGI Change”

tAIChange								(11), 	-- bearer modification. “TAI Change”

userLocationChange						(12)	-- bearer modification. “User Location Change”

}

ChangeOfCharCondition	::= SEQUENCE

--

-- qosRequested and qosNegotiated are used in S-CDR only

-- ePCQoSInformation used in SGW-CDR only

--

{

qosRequested				[1] QoSInformation OPTIONAL,

qosNegotiated				[2] QoSInformation OPTIONAL,

dataVolumeGPRSUplink		[3] DataVolumeGPRS OPTIONAL,

dataVolumeGPRSDownlink		[4] DataVolumeGPRS OPTIONAL,

changeCondition				[5] ChangeCondition,

changeTime					[6] TimeStamp,

userLocationInformation		[8] OCTET STRING OPTIONAL,

ePCQoSInformation			[9] EPCQoSInformation OPTIONAL

}

ChangeOfMBMSCondition	::= SEQUENCE

--

-- Used in MBMS record

--

{

qosRequested				[1] QoSInformation OPTIONAL,

qosNegotiated				[2] QoSInformation OPTIONAL,

dataVolumeMBMSUplink		[3] DataVolumeMBMS OPTIONAL,

dataVolumeMBMSDownlink		[4] DataVolumeMBMS,

changeCondition				[5] ChangeCondition,

changeTime					[6] TimeStamp,

failureHandlingContinue		[7] FailureHandlingContinue OPTIONAL

}

ChangeOfServiceCondition	::= SEQUENCE

--

-- Used for Flow based Charging service data container

--

{

ratingGroup 					[1] RatingGroupId,

chargingRuleBaseName			[2] ChargingRuleBaseName OPTIONAL,

resultCode						[3] ResultCode OPTIONAL,

localSequenceNumber				[4] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage				[5] TimeStamp OPTIONAL,

timeOfLastUsage					[6] TimeStamp OPTIONAL,

timeUsage 						[7] CallDuration OPTIONAL,

serviceConditionChange			[8] ServiceConditionChange,

qoSInformationNeg				[9] EPCQoSInformation OPTIONAL,

servingNodeAddress 				[10] GSNAddress OPTIONAL,

datavolumeFBCUplink				[12] DataVolumeGPRS OPTIONAL,

datavolumeFBCDownlink			[13] DataVolumeGPRS OPTIONAL,

timeOfReport					[14] TimeStamp,

failureHandlingContinue			[16] FailureHandlingContinue OPTIONAL,

serviceIdentifier				[17] ServiceIdentifier OPTIONAL,

pSFurnishChargingInformation	[18] PSFurnishChargingInformation OPTIONAL,

aFRecordInformation				[19] SEQUENCE OF AFRecordInformation OPTIONAL,

userLocationInformation			[20] OCTER STRING OPTIONAL,

eventBasedChargingInformation	[21] EventBasedChargingInformation OPTIONAL,

timeQuotaMechanism				[22] TimeQuotaMechanism OPTIONAL,

serviceSpecificInfo				[23] SEQUENCE OF ServiceSpecificInfo OPTIONAL,

3GPP2UserLocationInformation	[24] OCTET STRING OPTIONAL

}

ChangeLocation	::= SEQUENCE

--

-- used in SGSNMMRecord only

--

{

locationAreaCode		[0] LocationAreaCode,

routingAreaCode			[1] RoutingAreaCode,

cellId					[2] CellId OPTIONAL,

changeTime				[3] TimeStamp,

mCC-MNC					[4] PLMN-Id OPTIONAL

}

ChargingCharacteristics	::= OCTET STRING (SIZE(2))

ChargingID	::= INTEGER (0..4294967295)

--

-- Generated in P-GW, part of IP CAN bearer

-- 0..4294967295 is equivalent to 0..2**32-1

--

ChargingRuleBaseName ::= IA5String (SIZE(1..16))

--

-- identifier for the group of charging rules

-- see Charging-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

ChChSelectionMode		::= ENUMERATED

{

servingNodeSupplied			(0),	-- For S-GW/P-GW

subscriptionSpecific		(1),	-- For SGSN only

aPNSpecific					(2),	-- For SGSN only

homeDefault					(3),	-- For SGSN, S-GW and P-GW

roamingDefault				(4),	-- For SGSN, S-GW and P-GW

visitingDefault				(5)		-- For SGSN, S-GW and P-GW

}

CSGAccessMode ::= ENUMERATED

{

ClosedMode  (0),

HybridMode  (1)

}

CSGId	::= OCTET STRING (SIZE(4))

--

-- Defined in 23.003[200]. Coded according to TS 29.060[215] for GTP, and in TS 29.274 [223] for 	-- eGTP.

-- 24.008 [208]

--

CTEID	::=	OCTET STRING (SIZE(4))

--

-- Defined in 32.251[11] for MBMS-GW-CDR. Common Tunnel Endpoint Identifier of MBMS GW for user --

-- plane, defined in TS23.246 [207].

--

DataVolumeGPRS	::= INTEGER

--

-- The volume of data transferred in octets.

--

DynamicAddressFlag	::= BOOLEAN

EPCQoSInformation	::= SEQUENCE

--

-- See TS 29.212 [220] for more information

--

{

qCI						[1] INTEGER,

maxRequestedBandwithUL	[2] INTEGER OPTIONAL,

maxRequestedBandwithDL	[3] INTEGER OPTIONAL,

guaranteedBitrateUL		[4] INTEGER OPTIONAL,

guaranteedBitrateDL		[5] INTEGER OPTIONAL,

aRP						[6] INTEGER OPTIONAL

}

ETSIAddress	::= AddressString

--

-- First octet for nature of address, and numbering plan indicator (3 for X.121)

-- Other octets TBCD, See TS 29.002 [214]

--

EventBasedChargingInformation  ::= SEQUENCE

{

numberOfEvents		[1] INTEGER,

eventTimeStamps  	[2] SEQUENCE OF TimeStamp OPTIONAL

}

FailureHandlingContinue  ::= BOOLEAN

--

-- This parameter is included when the failure handling procedure has been executed and new

-- containers are opened. This parameter shall be included in the first and subsequent

-- containers opened after the failure handling execution.

--

FFDAppendIndicator	::= BOOLEAN

Flows  ::= 	SEQUENCE

--

-- See Flows AVP as defined in TS 29.214 [221]

--

{

mediaComponentNumber	[1] INTEGER,

flowNumber  			[2] SEQUENCE OF INTEGER OPTIONAL

}

FreeFormatData	::=	OCTET STRING (SIZE(1..160))

--

-- Free formatted data as sent in the FurnishChargingInformationGPRS

-- see TS 29.078 [217]

--

GSNAddress	::= IPAddress

MSNetworkCapability	::= OCTET STRING (SIZE(1..8))

--

-- see TS 24.008 [208]

--

NetworkInitiatedPDPContext	::= BOOLEAN

--

-- Set to true if PDP context was initiated from network side

--

NodeID	::= IA5String (SIZE(1..20))

NumberOfDPEncountered  ::= INTEGER

PDPAddress	::= CHOICE

{

iPAddress				[0] IPAddress,

eTSIAddress			   [1] ETSIAddress

}

PDPType		::= OCTET STRING (SIZE(2))

--

-- OCTET 1: PDP Type Organization

-- OCTET 2: PDP/PDN Type Number

-- See TS 29.060 [215] for GTP, TS 29.274 [223] clause 8.14 for eGTP and TS 29.275 [224] for PMIP

--

PLMN-Id		::= OCTET STRING (SIZE (3))

--

--	This is a 1:1 copy from the Routing Area Identity (RAI) IE specified in TS 29.060 [215]

--  as follows:

--	OCTET 1 of PLMN-Id = OCTET 2 of RAI

--	OCTET 2 of PLMN-Id = OCTET 3 of RAI

--	OCTET 3 of PLMN-Id = OCTET 4 of RAI

--

PSFurnishChargingInformation  ::= SEQUENCE

{

pSFreeFormatData		[1] FreeFormatData,

pSFFDAppendIndicator  	[2] FFDAppendIndicator OPTIONAL

}

QoSInformation	::= OCTET STRING (SIZE (4..255))

--

-- This  octet string

-- is a 1:1 copy of the contents (i.e. starting with octet 5) of the "Bearer Quality of

-- Service" information element specified in TS 29.274 [223].

--

RatingGroupId 	::= INTEGER

--

-- IP service flow identity (DCCA), range of 4 byte (0...4294967259)

-- see Rating-Group AVP as used in TS 32.299 [50]

--

RATType		::= INTEGER (0..255)

--

-- Ihis integer is 1:1 copy of the RAT type value as defined in TS 29.060 [215] for GTP,

-- TS 29.274 [223] for eGTP and TS 29.275 [224] for PMIP.

--

ResultCode 	::= INTEGER

--

-- charging protocol return value, range of 4 byte (0...4294967259)

-- see Result-Code AVP as used in 32.299 [40]

--

RoutingAreaCode	::= OCTET STRING (SIZE(1))

--

-- See TS 24.008 [208]

--

ServiceConditionChange	::= BIT STRING

{

qoSChange 								 (0),	-- bearer modification

sGSNChange 								 (1), 	-- bearer modification

sGSNPLMNIDChange 						 (2), 	-- bearer modification

tariffTimeSwitch 						 (3), 	-- tariff time change

pDPContextRelease 						 (4), 	-- bearer release

rATChange 								 (5),	-- bearer modification

serviceIdledOut 						 (6),	-- IP flow idle out, DCCA QHT expiry

reserved 								 (7),	-- old: QCTexpiry is no report event

configurationChange 					 (8),	-- configuration change

serviceStop 							 (9), 	-- IP flow termination.From ”Service Stop” in

-- Change-Condition” AVP

dCCATimeThresholdReached 				(10),	-- DCCA quota reauthorization

dCCAVolumeThresholdReached 				(11), 	-- DCCA quota reauthorization

dCCAServiceSpecificUnitThresholdReached	(12), 	-- DCCA quota reauthorization

dCCATimeExhausted 						(13), 	-- DCCA quota reauthorization

dCCAVolumeExhausted 					(14), 	-- DCCA quota reauthorization

dCCAValidityTimeout 					(15), 	-- DCCA quota validity time (QVT expiry)

reserved1								(16),	-- reserved due to no use case,

-- old: return Requested is covered by (17),(18)

dCCAReauthorisationRequest 				(17), 	-- DCCA quota reauthorization request by OCS

dCCAContinueOngoingSession 				(18), 	-- DCCA failure handling (CCFH),

-- continue IP flow

dCCARetryAndTerminateOngoingSession		(19), 	-- DCCA failure handling (CCFH),

-- terminate IP flow after DCCA retry

dCCATerminateOngoingSession 			(20), 	-- DCCA failure handling,

-- terminate IP flow

cGI-SAIChange							(21), 	-- bearer modification. “CGI-SAI Change”

rAIChange								(22), 	-- bearer modification. “RAI Change”

dCCAServiceSpecificUnitExhausted		(23), 	-- DCCA quota reauthorization

recordClosure							(24),	-- PGW-CDR closure

timeLimit								(25),	-- intermediate recording. From “Service Data 													--	Time Limit” Change-Condition AVP value

volumeLimit								(26),	-- intermediate recording.From “Service Data 														--	Volume Limit” Change-Condition AVP value

serviceSpecificUnitLimit				(27),	-- intermediate recording

envelopeClosure 						(28),

eCGIChange								(29), 	-- bearer modification. “ECGI Change”

tAIChange								(30), 	-- bearer modification. “TAI Change”

userLocationChange						(31)	-- bearer modification. “User Location Change”

}

--

-- Trigger and cause values for IP flow level recording are defined for support of independent

-- online and offline charging and also for tight interworking between online and offline charging.

-- Unused bits will always be zero.

-- Some of the values are non-exclusive (e.g. bearer modification reasons).

--

SCFAddress	::= AddressString

--

-- See TS 29.002 [214]

--

ServiceIdentifier	::= INTEGER (0..4294967295)

--

-- The service identifier is used to identify the service or the service component

-- the service data flow relates to. See Service-Identifier AVP as defined in TS 29.212 [220]

--

ServingNodeType	::= ENUMERATED

{

sGSN		(0),

pMIPSGW		(1),

gTPSGW		(2),

ePDG		(3),

hSGW		(4),

mME			(5)

}

SGSNChange	::= BOOLEAN

--

-- present if first record after inter SGSN routing area update in new SGSN

--

SGWChange	::= BOOLEAN

--

-- present if first record after inter S-GW change

--

TimeQuotaMechanism	::= SEQUENCE

{

timeQuotaType					[1] TimeQuotaType,

baseTimeInterval				[2] Integer

}

TimeQuotaType	::= ENUMERATED

{

dISCRETETIMEPERIOD			(0),

cONTINUOUSTIMEPERIOD		(1)

}

UserCSGInformation	::= SEQUENCE

{

cSGId					[0] CSGId,

cSGAccessMode			[1] CSGAccessMode,

cSGMembershipIndication [2] NULL OPTIONAL

}

END

End of changes