3GPP TSG-SA5 (Telecom Management)	S5-111824

SA5#77, 9-13 May 2011; Shenzhen, P.R. China 	revision of S5-11abcd

| CR-Form-v9.8 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.409 | CR | 0027 | rev | - | Current version: | 10.3.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| cr |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | CR R11 32.409 Add answered session traffic related measurements |  |  |  |  |  |  |  |  |  |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM-ePM-IMS |  |  |  |  |  |  | Date: |  | 13/05/2011 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | Answered session traffic could reflect the session traffic which is an important aspect of network load.It also reflects the occupancy of session traffic in relation to system resource. It may help operators to decide whether the network expansion or network configuration adjustment is needed. The Answered session traffic related measurements are missing in current TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add answered session traffic related measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.X(new), 4.1.2.Y(new), 4.1.2.Z(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... rev ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... rev ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... rev ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

4.1.2.X	Answered session traffic

- This measurement provides the answered session traffic at the P-CSCF or S-CSCF.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for all successful session establishment between the transmission by the P-CSCF or S-CSCF of a SIP_200_OK response message corresponding to the initial SIP_INVITE messages (see 3GPP TS 23.228 [7]) and the receipt of corresponding release message "BYE" (see 3GPP TS 23.228 [7]) over a given granularity period. The final result should be calculated to ERLANG. The accumulator shall be reinitialised at the beginning of each granularity period.

- A real value. (Unit: ERLANG)

- SC.AnsTraf.

- PCSCFFunction.

SCSCFFunction.

- Valid for packet switched traffic.

- IMS.

4.1.2.Y	Answered originating session traffic

- This measurement provides the answered originating session traffic at the P-CSCF or S-CSCF.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for all successful session establishment between the transmission by the originating P-CSCF or S-CSCF of a SIP_200_OK response message corresponding to the initial SIP_INVITE messages (see 3GPP TS 23.228 [7]) and the receipt of corresponding release message "BYE" (see 3GPP TS 23.228 [7]) over a given granularity period. The final result should be calculated to ERLANG. The accumulator shall be reinitialised at the beginning of each granularity period.

- A real value. (Unit: ERLANG)

- SC.AnsTrafOrig.

- PCSCFFunction.

SCSCFFunction.

- Valid for packet switched traffic.

- IMS.

4.1.2.Z	Answered terminating session traffic

- This measurement provides the answered terminating session traffic at the P-CSCF or S-CSCF.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for all successful session establishment between the transmission by the terminating P-CSCF or S-CSCF of a SIP_200_OK response message corresponding to the initial SIP_INVITE messages (see 3GPP TS 23.228 [7]) and the receipt of corresponding release message "BYE" (see 3GPP TS 23.228 [7]) over a given granularity period. The final result should be calculated to ERLANG. The accumulator shall be reinitialised at the beginning of each granularity period.

- A real value. (Unit: ERLANG)

- SC.AnsTrafTerm.

- PCSCFFunction.

SCSCFFunction.

- Valid for packet switched traffic.

Next change

A.X Use case of Answered session traffic related measurements at CSCF

Answered session traffic which is an important aspect of network load could reflect the call traffic. It also reflects occupancy of session traffic in relation to system resource and may help operators to decide whether the network capacity expansion or network configuration adjustment is needed.

End of change