3GPP TS 32.652 V10.2.0 (2011-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Configuration Management (CM);

GERAN network resources Integration Reference Point (IRP); Network Resource Model (NRM)

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, UMTS, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2011, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	8

4	System overview	9

4.1	Void	9

4.2	Compliance rules	9

5	Modelling approach	9

6	Information Object Classes	9

6.1	Imported information entities and local labels	9

6.2	Class diagram	10

6.2.1	Attributes and relationships	10

6.2.2	Inheritance	12

6.3	Information object class definitions	12

6.3.1	BssFunction	12

6.3.1.1	Definition	12

6.3.1.2	Attributes	12

6.3.2	BtsSiteMgr	12

6.3.2.1	Definition	12

6.3.2.2	Attributes	13

6.3.3	GsmCell	13

6.3.3.1	Definition	13

6.3.3.2	Attributes	13

6.3.3.3	Attribute constraints	13

6.3.4	GsmRelation	13

6.3.4.1	Definition	13

6.3.4.2	Attributes	14

6.3.4.3	Attribute constraints	14

6.3.5	ExternalGsmCell	14

6.3.5.1	Definition	14

6.3.5.2	Attributes	15

6.3.5.3	Attribute constraints	15

6.3.6	ExternalBssFunction	15

6.3.6.1	Definition	15

6.3.6.2	Attributes	15

6.3.6.3	Attribute constraints	15

6.4	Information relationship definitions	15

6.4.1	ExternalGsmNeighbourCellRelation (M)	15

6.4.1.1	Definition	15

6.4.1.2	Roles	15

6.4.1.3	Constraints	16

6.4.2	GsmNeighbourCellRelation (M)	17

6.4.2.1	Definition	17

6.4.2.2	Roles	17

6.4.2.3	Constraints	17

6.5	Information attribute definitions	18

6.5.1	Definition and legal values	18

6.5.2	Constraints	19

6.6	Common Notifications	19

6.7	Void	20

Annex A (informative):	Change history	21

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.651:	Configuration Management (CM); GERAN network resources Integration Reference Point (IRP): Requirements

32.652:	Configuration Management (CM); GERAN network resources Integration Reference Point (IRP): Network Resource Model (NRM)

32.653:	Configuration Management (CM); GERAN network resources Integration Reference Point (IRP): Common Object Request Broker Architecture (CORBA) Solution Set (SS)

32.655:	Configuration Management (CM); GERAN network resources Integration Reference Point (IRP): Bulk CM eXtensible Markup Language (XML) file format definition

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. CM actions have the objective to control and monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs.

CM actions may be requested as part of an implementation programme (e.g. additions and deletions), as part of an optimisation programme (e.g. modifications), and to maintain the overall Quality of Service (QoS). The CM actions are initiated either as single actions on single NEs of the 3G network, or as part of a complex procedure involving actions on many resources/objects in one or several NEs.

# 1	Scope

The present document specifies the GERAN network resource information that can be communicated between an IRPAgent and one or several IRPManagers for network management purposes.

This document specifies the semantics and behaviour of information object class attributes and relations visible across the reference point in a protocol and technology neutral way.  It does not define their syntax and encoding.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[4]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control (RRC) protocol".

[5]	3GPP TS 45.008: "Radio subsystem link control".

[6]	3GPP TS 45.002: "Multiplexing and multiple access on the radio path".

[7]	ITU-T Recommendation X.710 (1991): "Common Management Information Service Definition for CCITT Applications".

[8]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[9]	3GPP TS 32.672: "Telecommunication management; Configuration Management (CM); State Management Integration Reference Point (IRP): Information Service (IS)".

[10]	Void.

[11]	3GPP TS 32.111-2: "Telecommunication management; Fault Management (FM); Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[12]	Void

[13]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[14]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[15]	Void.

[16]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[17]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP) Information Service (IS)".

[18]	3GPP TS 32.612: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Information Service (IS)".

[19]	3GPP TS 32.642: " Telecommunication management; Configuration Management (CM); UTRAN network resources Integration Reference Point (IRP); Network Resource Model (NRM) ".

[20]	3GPP TS 23.002: "Network architecture".

[21]	3GPP TS 32.792: "Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply. For terms and definitions not found here, please refer to 3GPP TS 32.101 [1], 3GPP TS 32.102 [2] and 3GPP TS 32.600 [14].

Association: In general it is used to model relationships between Managed Objects. Associations can be implemented in several ways, such as:

- name bindings,

- reference attributes, and

- association objects.

This IRP stipulates that containment associations shall be expressed through name bindings, but it does not stipulate the implementation for other types of associations as a general rule. These are specified as separate entities in the object models (UML diagrams).

Managed Element (ME): An instance of the IOC ManagedElement, defined in 3GPP TS 32.622 [16].

Managed Object (MO): In the context of the present document, a Managed Object (MO) is a software object that encapsulates the manageable characteristics and behaviour of a particular Network Resource. The MO is instance of a class defined in a MIM/NRM. This class, called Information Object Class (IOC) has attributes that provide information used to characterize the objects that belong to the class (the term "attribute" is taken from TMN and corresponds to a "property" according to CIM). Furthermore, the IOC can have operations that represent the behaviour relevant for that class (the term "operation" is taken from TMN and corresponds to a "method" according to CIM). The IOC may support the emission of notifications that provide information about an event occurrence within a network resource.

Management Information Base (MIB): A MIB is an instance of an NRM and has some values on the defined attributes and associations specific for that instance. In the context of the present document, an MIB consists of:

- a Name space (describing the MO containment hierarchy in the MIB through Distinguished Names),

- a number of IOCs with their attributes and

- a number of Associations between these IOCs. Also note that TMN (ITU-T Recommendation X.710 [7]) defines a concept of a Management Information Tree (also known as a Naming Tree) that corresponds to the name space (containment hierarchy) portion of this MIB definition. Figure 3.1 depicts the relationships between a Name space and a number of participating MOs (the shown association is of a non-containment type).

Figure 3.1: Relationships between a Name space and a number of participating MOs

Management Information Model (MIM): Also referred to as NRM – see the definition below.

Name space: A name space is a collection of names. The IRP name convention (see 3GPP TS 32.300 [13]) restricts the name space to a hierarchical containment structure, including its simplest form - the one-level, flat name space. 
All Managed Objects in a MIB shall be included in the corresponding name space and the MIB/name space shall only support a strict hierarchical containment structure (with one root object). A Managed Object that contains another is said to be the superior (parent); the contained Managed Object is referred to as the subordinate (child). The parent of all MOs in a single name space is called a Local Root. The ultimate parent of all MOs of all managed systems is called the Global Root.

Network Resource Model (NRM): A model representing the actual managed telecommunications network resources that a System is providing through the subject IRP. An NRM identifies and describes the IOCs, their associations, attributes and operations. The NRM is also referred to as "MIM" (see above), which originates from the ITU-T TMN.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CIM	Common Information Model

DN	Distinguished Name (see 3GPP TS 32.300 [13])

EM	Element Manager

FM	Fault Management

GERAN	GSM-EDGE Radio Access Network

GPRS	General Packet Radio System

IOC	Information Object Class

IRP	Integration Reference Point

ME	Managed Element

MIB	Management Information Base

MIM	Management Information Model

MO	Managed Object

NE	Network Element

NR	Network Resource

NRM	Network Resource Model

RDN	Relative Distinguished Name (see 3GPP TS 32.300 [13])

RNC	Radio Network Controller

SS	Solution Set

TMN	Telecommunications Management Network

UML	Unified Modelling Language

XML	eXtensible Mark-up Language

# 4	System overview

## 4.1	Void

## 4.2	Compliance rules

The following defines the meaning of Mandatory and Optional IOC attributes and associations between IOCs, in Solution Sets to the IRP defined by the present document:

- The IRPManager shall support all mandatory attributes/associations. The IRPManager shall be prepared to receive information related to mandatory as well as optional attributes/associations without failure; however the IRPManager does not have to support handling of the optional attributes/associations.

- The IRPAgent shall support all mandatory attributes/associations. It may support optional attributes/associations.

An IRPAgent that incorporates vendor-specific extensions shall support normal communication with a 3GPP SA5-compliant IRPManager with respect to all Mandatory and Optional information object classes, attributes, associations, operations, parameters and notifications without requiring the IRPManager to have any knowledge of the extensions.

Given that

- rules for vendor-specific extensions remain to be fully specified, and

- many scenarios under which IRPManager and IRPAgent interwork may exist,

it is recognised that the IRPManager, even though it is not required to have knowledge of vendor-specific extensions, may be required to be implemented with an awareness that extensions can exist and behave accordingly.

# 5	Modelling approach

The modelling approach adopted and used in this IRP is described in clause 5 of Generic Network Resources IRP: NRM 3GPP TS 32.622 [16].

# 6	Information Object Classes

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 32.622 [16], information object class, ManagedElement | ManagedElement |
| 3GPP TS 32.622 [16], information object class, ManagedFunction | ManagedFunction |
| 3GPP TS 32.622 [16], information object class, SubNetwork | SubNetwork |
| 3GPP TS 32.622 [16], information object class, Top | Top |
| 3GPP TS 32.622 [16], information object class, VsDataContainer | VsDataContainer |
| 3GPP TS 32.642 [19], information object class, ExternalUtranGenericCell | ExternalUtranGenericCell |
| 3GPP TS 32.642 [19], information object class, UtranGenericCell | UtranGenericCell |
| 3GPP TS 32.642 [19], information object class, UtranRelation | UtranRelation |

## 6.2	Class diagram

### 6.2.1	Attributes and relationships

This clause depicts the set of IOCs that encapsulate information relevant for this service. This clause provides the overview of all information object classes in UML.  Subsequent clauses provide more detailed specification of various aspects of these information object classes.

The figures below show the containment/naming hierarchy and the associations of the GERAN NRM.

NOTE 1:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios.

NOTE 2:	TheUtranGenericCell, ExternalUtranGenericCell, GsmRelation and UtranRelation can be contained under IOCs defined in other NRMs.

NOTE 3:	The ExternalBssFunction is used in the Core Network NRM.

Figure 6.1: GERAN NRM Containment/Naming and Association diagram

Each Managed Object is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [13] that expresses its containment hierarchy. As an example, the DN of an IOC representing a cell could have a format like:

SubNetwork =Sweden,MeContext =MEC-Gbg-1, ManagedElement =RNC-Gbg-1, BssFunction=BSS1.

NOTE 1:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios.

NOTE 2:	Each instance of the VsDataContainer shall only be contained under one IOC. The VsDataContainer can be contained under IOCs defined in other NRMs.

Figure 6.2: GERAN NRM Containment/Naming and Association diagram

The VsDataContainer is only used for the Bulk CM IRP.

### 6.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 6.3 shows the inheritance hierarchy for the GERAN NRM.

Figure 6.3: GERAN NRM Inheritance Hierarchy

## 6.3	Information object class definitions

### 6.3.1	BssFunction

#### 6.3.1.1	Definition

This IOC represents BSS functionality. For more information about the BSS, see Ref 3GPP TS  23.002 [20].

#### 6.3.1.2	Attributes

Table 6.1: Attributes of BssFunction

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |

### 6.3.2	BtsSiteMgr

#### 6.3.2.1	Definition

This IOC contains site specific information for a BTS site.

#### 6.3.2.2	Attributes

Table 6.3a: Attributes of BtsSiteMgr

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| latitude | O | M | M |
| longitude | O | M | M |

Table 6.3b: Additional attributes of BtsSiteMgr for the support of the State Management IRP

| Attribute Name | Support Qualifier | READ | WRITE |
| --- | --- | --- | --- |
| operationalState | O | M | — |
| NOTE: No state propagation shall be implied. |  |  |  |

### 6.3.3	GsmCell

#### 6.3.3.1	Definition

This IOC represents the GSM radio cell. The applicability of instantiation of this class is depending on the ME type. It may only be instantiated under ME of type BSC.

#### 6.3.3.2	Attributes

Table 6.5: Attributes of GsmCell

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| cellIdentity | M | M | M |
| cellAllocation | M | M | M |
| ncc | M | M | M |
| bcc | M | M | M |
| lac | M | M | M |
| mcc | M | M | M |
| mnc | M | M | M |
| rac | CM | M | M |
| racc | CM | M | M |
| tsc | CM | M | M |
| rxLevAccessMin | M | M | M |
| msTxPwrMaxCCH | M | M | M |
| rfHoppingEnabled | M | M | M |
| hoppingSequenceList | M | M | M |
| plmnPermitted | M | M | M |

#### 6.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| rac CM support qualifier | GPRS is supported in the cell. |
| racc CM support qualifier | GPRS is supported in the cell. |
| tsc CM support qualifier | RET, TMA etc. are not modelled according to Ref 3GPP TS 32.792 [21]. |

### 6.3.4	GsmRelation

#### 6.3.4.1	Definition

This IOC contains radio network related parameters for the relation to the GsmCell or ExternalGsmCell managed object. Note: In handover relation terms, the cell containing the GsmRelation object is the source cell for the handover. The cell referred to in the GsmRelation object is the target cell for the handover. This defines a one-way handover relation where the direction is from source cell to target cell.

#### 6.3.4.2	Attributes

Table 6.7: Attributes of GsmRelation

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| adjacentCell | M | M | M |
| bcchFrequency | CM | M | - |
| ncc | CM | M | - |
| bcc | CM | M | - |
| lac | CM | M | - |

#### 6.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| bcchFrequency CM support qualifier | The EM does not guarantee consistency between the cell definition and what is broadcasted on system information and RET, TMA etc. are not modelled according to Ref 3GPP TS 32.792 [21]. |
| ncc CM support qualifier | The EM does not guarantee consistency between the cell definition and what is broadcasted on system information. |
| bcc CM support qualifier | The EM does not guarantee consistency between the cell definition and what is broadcasted on system information. |
| lac CM support qualifier | The EM does not guarantee consistency between the cell definition and what is broadcasted on system information. |

### 6.3.5	ExternalGsmCell

#### 6.3.5.1	Definition

This IOC represents a radio cell controlled by another IRPAgent. This IOC has necessary attributes for inter-system handover.  It contains a subset of the attributes of related IOCs controlled by another IRPAgent. To maintain the consistency between the attribute values of these two IOCs is outside the scope of this document.

#### 6.3.5.2	Attributes

Table 6.9: Attributes of ExternalGsmCell

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| cellIdentity | M | M | M |
| bcchFrequency | M | M | M |
| ncc | M | M | M |
| bcc | M | M | M |
| lac | M | M | M |
| mcc | M | M | M |
| mnc | M | M | M |
| rac | CM | M | M |
| racc | CM | M | M |

#### 6.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| rac CM support qualifier | GPRS is supported in the cell. |
| racc CM support qualifier | GPRS is supported in the cell. |

### 6.3.6	ExternalBssFunction

#### 6.3.6.1	Definition

This IOC represents a BssFunction controlled by another IRPAgent. It contains a subset of the attributes of related IOCs controlled by another IRPAgent. To maintain the consistency between the attribute values of these two IOCs is outside the scope of the present document.

#### 6.3.6.2	Attributes

Table 6.11: Attributes of ExternalBssFunction

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |

#### 6.3.6.3	Attribute constraints

None.

## 6.4	Information relationship definitions

### 6.4.1	ExternalGsmNeighbourCellRelation (M)

#### 6.4.1.1	Definition

This represents a unidirectional relation from GsmRelation to the ExternalGsmCell. The role of the relation shall be mapped to a reference attribute, named adjacentCell, of the IOC.

#### 6.4.1.2	Roles

Table 6.11: Roles of the relation ExternalGsmNeighbourCellRelation

| Name | Definition |
| --- | --- |
| gsmRelation-externalGsmNeighbourCell | This role (when present) represents GsmRelation capability to identify one ExternalGsmCell. When this role is present, the GsmRelation.adjacentCell shall contain one ExternalGsmCell DN. |

#### 6.4.1.3	Constraints

| Name | Definition |
| --- | --- |
| external_neighbour_cell_relation | The role gsmRelation-externalGsmNeighbourCell is only present if the target cell is managed by another IRPAgent than the serving cell. |

### 6.4.2	GsmNeighbourCellRelation (M)

#### 6.4.2.1	Definition

This represents the unidirectional relation from the GsmRelation to GsmCell. The role of the relation shall be mapped to a reference attribute, named adjacentCell, of the IOC.

#### 6.4.2.2	Roles

Table 6.12: Roles of the relation GsmNeighbourCellRelation

| Name | Definition |
| --- | --- |
| gsmRelation-gsmNeighbourCell | This role (when present) represents GsmRelation capability to identify one GsmCell. When this role is present, the GsmRelation.adjacentCell shall contain one GsmRelation DN. |

#### 6.4.2.3	Constraints

This role (for a particular GsmRelation) shall be present if the ExternalGsmNeighbourCellRelation of this particular GsmRelation is absent.  This role shall be absent if the ExternalGsmNeighbourCellRelation of this particular GsmRelation is present.

| Name | Definition |
| --- | --- |
| internal_neighbour_cell_relation | The role gsmRelation-gsmNeighbourCell is only present if the target cell is managed by the same IRPAgent as the serving cell. |

## 6.5	Information attribute definitions

### 6.5.1	Definition and legal values

Table 6.13 defines the attributes that are present in several information object classes of the present document.

Table 6.13: Attributes

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| adjacentCell | Pointer to GSM cell or external GSM cell. Distinguished Name of the corresponding object. |  |
| bcc | IOCs GsmCell and ExternalGsmCell: Base station colour code, BCC (part of BSIC). Ref 3GPP TS 44.018 [4]. IOC GsmRelation: Base station colour code, BCC (part of BSIC. Ref 3GPP TS 44.018 [4]) for another GSM cell or the external GSM cell, that is broadcast in System Information in the Cell. | See Ref 3GPP TS 44.018 [4]. |
| bcchFrequency | IOC ExternalGsmCell: This attribute contains the absolute radio frequency channel number of the BCCH channel of the GSM cell. IOC GsmRelation: This attribute contains the absolute radio frequency channel number of the BCCH channel of another GSM cell or the external GSM cell, that is broadcast in System Information in the Cell. |  |
| cellAllocation | This attribute defines the set of radio frequencies allocated and available to a cell, the first element sets the BCCH frequency, Ref 3GPP TS 44.018 [4]. | See Ref 3GPP TS 44.018 [4]. |
| cellIdentity | Cell Identity (Ref 3GPP TS 24.008 [3]). | See Ref 3GPP TS 24.008 [3]. |
| hoppingSequenceList | A list of hopping sequence, which is a structure including MA(see 44.018[4]) and HSN(see 45.002[6]). | See Ref 3GPP TS 44.018 [4]. |
| lac | IOCs GsmCell and ExternalGsmCell: Location Area Code, LAC . Ref 3GPP TS 24.008 [3]. IOC GsmRelation: Location Area Code, LAC (Ref 3GPP TS 24.008 [3]) for another GSM cell or the external GSM cell, that is broadcast in System Information in the Cell. | See Ref 3GPP TS 24.008 [3]. |
| latitude | Used for geographical positioning of the sitemanager |  |
| longitude | Used for geographical positioning of the sitemanager |  |
| mcc | Mobile Country Code, MCC (part of the PLMN Id, Ref. 3GPP TS 23.003 [8]). | See Ref 3GPP TS 24.008 [3]. |
| mnc | Mobile Network Code, MNC (part of the PLMN Id, Ref. 3GPP TS 23.003 [8]). | See Ref 3GPP TS 24.008 [3]. |
| msTxPwrMaxCCH | Maximum Transmission Power for a Mobile Station on a CCH. Attribute description Ref 3GPP TS 45.008 [5] (MS_TXPWR_MAX_CCH) | See Ref 3GPP TS 45.008 [5]. |
| ncc | IOCs GsmCell and ExternalGsmCell: Network Colour Code, NCC (part of BSIC). Ref 3GPP TS 44.018 [4]. IOC GsmRelation: Network Colour Code, NCC (part of BSIC. Ref 3GPP TS 44.018 [4]) for another GSM cell or the external GSM cell, that is broadcast in System Information in the Cell. | See Ref 3GPP TS 44.018 [4]. |
| plmnPermitted | Network Colour Code Permitted. Attribute description reference 3GPP TS 45.008 [5] (NCC_PERMITTED) | See Ref 3GPP TS 45.008 [5]. |
| rac | Routing Area Code, RAC. Ref 3GPP TS 44.018 [4]. | See Ref 3GPP TS 44.018 [4]. |
| racc | Routing Area Colour Code, RACC. Ref 3GPP TS 44.018 [4]. | See Ref 3GPP TS 44.018 [4]. |
| rfHoppingEnabled | Indicates if frequency hopping is enabled. | Boolean values 0 represents “disabled”, 1 represents “enabled”. |
| rxLevAccessMin | Minimum Access Level. Attribute description Ref 3GPP TS 45.008 [5] (RXLEV_ACCESS_MIN) | See Ref 3GPP TS 45.008 [5]. |
| tsc | Training Sequence Code, an attribute of the class channel in Ref 3GPP TS 44.018 [4] | See Ref 3GPP TS 44.018 [4]. |

### 6.5.2	Constraints

| Name | Definition |
| --- | --- |
| - | - |

## 6.6	Common Notifications

The following notifications apply to the following IOCs:

- BssFunction;

- BtsSiteMgr;

- GsmCell.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAttributeValueChange | O |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |

The following notifications apply to the following IOCs:

- GsmRelation;

- ExternalGsmCell;

- ExternalBssFunction.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |

## 6.7	Void

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Jun 2001 | SA_12 | SP-010283 | -- | -- | Approved at TSG SA #12 and placed under Change Control |  | 2.0.0 | 4.0.0 |
| Sep 2001 | SA_13 | SP-010477 | 0001 | -- | Addition of mcc and mnc in the object model of GERAN | F | 4.0.0 | 4.1.0 |
| Dec 2001 | SA_14 | SP-010650 | 0002 | -- | Correction of references | F | 4.1.0 | 4.2.0 |
| Jun 2002 | SA_16 | SP-020305 | 0003 | -- | Addition of the attributes mcc and mnc in the object model of GERAN | F | 4.2.0 | 4.3.0 |
| Jun 2002 | SA_16 | SP-020305 | 0004 | -- | Correction of attribute descriptions in the Managed Object Class (MOC) GsmRelation of 32.652 (GERAN network resources IRP: NRM) | F | 4.2.0 | 4.3.0 |
| Jun 2002 | SA_16 | SP-020304 | 0005 | -- | Correction of supported IRP in system context | F | 4.2.0 | 4.3.0 |
| Sep 2002 | SA_17 | SP-020494 | 0006 | -- | UML corrections | F | 4.3.0 | 4.4.0 |
| Sep 2002 | SA_17 | SP-020496 | 0007 | -- | Add State Management | B | 4.4.0 | 5.0.0 |
| Dec 2002 | -- | -- | -- | -- | Cosmetics | -- | 5.0.0 | 5.0.1 |
| Jun 2003 | SA_20 | SP-030282 | 0010 | -- | Include notification tables | A | 5.0.1 | 5.1.0 |
| Jun 2003 | SA_20 | SP-030282 | 0012 | -- | Correction of UML diagram vsDataContainer Containment/Naming and Association in GERAN NRM | A | 5.0.1 | 5.1.0 |
| Jun 2003 | SA_20 | SP-030283 | 0014 | -- | Deletion of GERAN attribute relationType | A | 5.0.1 | 5.1.0 |
| Sep 2003 | SA_21 | SP-030418 | 0015 | -- | Inclusion of External BSS Function in GERAN NRM - Alignment with 32.632 | F | 5.1.0 | 5.2.0 |
| Dec 2003 | SA_22 | SP-030641 | 0016 | -- | Add missing notification notifyPotentialFaultyAlarmlist | F | 5.2.0 | 5.3.0 |
| Dec 2003 | SA_22 | SP-030643 | 0017 | -- | VsDataContainer Containment UML - Now covered by 32.622 | F | 5.2.0 | 5.3.0 |
| Sep 2004 | SA_25 | SP-040584 | 0018 |  | Add support for the state change notification in GERAN network resources IRP NRM | B | 5.3.0 | 6.0.0 |
| Mar 2005 | -- | -- | -- | -- | Title corrected (changed Release 5 to Release 6) | -- | 6.0.0 | 6.0.1 |
| Jun 2005 | SA_28 | SP-050298 | 0019 | -- | Correction of System Context and Compliance rules | F | 6.0.1 | 6.1.0 |
| Dec 2005 | SA_30 | SP-050713 | 0020 | -- | Correct relationships for External IOCs | F | 6.1.0 | 6.2.0 |
| Dec 2005 | SA_30 | SP-050723 | 0021 | -- | Apply IS Template and the UML Repertoire - Align with 32.151 and 32.152 | F | 6.1.0 | 6.2.0 |
| Jan 2006 | -- | -- | -- | -- | Editorial correction to change history table. | -- | 6.2.0 | 6.2.1 |
| Jun 2007 | SA_36 | -- | -- | -- | Automatic upgrade to Rel-7 (no CR) at freeze of Rel-7. Deleted reference to CMIP SS, discontinued from R7 onwards. | -- | 6.2.1 | 7.0.0 |
| Dec 2008 | SA_42 | -- | -- | -- | Upgrade to Release 8 | -- | 7.0.0 | 8.0.0 |
| Dec 2009 | SA_46 | SP-090719 | 0022 | -- | Correction of imported IOC UtranCell and ExternalUtranCell from 32.642 | F | 8.0.0 | 9.0.0 |
| Jan 2009 | -- | -- | -- | -- | Removal of track changes | -- | 9.0.0 | 9.0.1 |
| Sep 2010 | SA_49 | SP-100488 | 0024 | -- | Corrections of attribute hoppingSequenceNumber in GsmCell IOC | F | 9.0.1 | 9.1.0 |
| Sep 2010 | SA_49 | SP-100489 | 0023 | -- | Add attribute rfHoppingEnabled in GsmCell IOC | B | 9.1.0 | 10.0.0 |
| Mar 2011 | SA-51 | SP-110095 | 0025 | -- | Methodology clean-up of Configuration Management; GERAN network resources IRP; Network Resource Model | F | 10.0.0 | 10.1.0 |
| Mar 2011 | SA-51 | SP-110096 | 0026 | -- | Correcting the support qualifier of attributes tsc and bcchFrequency | F | 10.0.0 | 10.1.0 |
| May 2011 | SA-52 | SP-110285 | 0028 | - | Correct attribute constraints | F | 10.1.0 | 10.2.0 |