# 3GPP TSG SA WG5 (Telecom Management) Meeting #88	S5-130629

# 15-19 April 2013 Qingdao (China);	revision of S5-130abc

# Source:	China Mobile

# Title:	Add attributes to PdgFunction in 28.602

# Document for:	Approval

Agenda Item:	6.4.1

1	Decision/action requested

SA5 is asked to discuss new added attributes and converged into TS 28.602.

2	References

3	Rationale

This pCR is targeted to add additional attributes of PdgFunction to TS 28.602.

4	Detailed proposal

Text Proposal for TS 28.602

1st Modified Section

6.3	Information object class definitions

6.3.2	PdgFunction

6.3.2.1	Definition

This IOC represents PDG  functionality. For more information about the PDG, see 3GPP TS 23.234[5].

6.3.2.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| iPConfigInfo | M | M | - |
| proceduralStatus(Note) | O | - | - |
| Note: This proceduralStatus is not settable or readable via any Interface IRP except conveyed by notifyStateChange notifications. |  |  |  |

End of 1st Modified Section