# 3GPP TSG SA WG5 (Telecom Management) Meeting #92	S5-132194 revision of S5-132049

# 11-15 November 2013; San Francisco (USA

# Source:	NEC

# Title:	Interoperability issues of Distributed MLB implementation in Multi-vendor HetNet deployments

# Document for:	Approval

Agenda Item:	6.9.2 Study on Enhancements of OAM aspects of Distributed Mobility Load Balancing (MLB) SON function

# 1	Decision/action requested

The group is asked to agree to the proposed text proposals

# 2	References

[1]	S5-131443,  New WID Enhancements of OAM aspects of Distributed Load Balancing.

[2]	S5-131658, Insights into possible interoperability issues of Distributed MLB implementation in Multi-vendor HetNet deployments; NEC.

[3]	S5-132193; SI TR Skeleton

# 3	Rationale & Background

The study item: Enhancements of OAM aspects of Distributed Mobility Load Balancing SON function was approved in SA Plenary #61 [1]. This contribution is a follow up from the previous analysis and discussion paper presented and reviewed in SA5#91 in Shenzhen [2]. The conclusion from the discussion in Shenzhen was that part of the analysis text and recommended practices presented in [2] could be adopted in suitable format for inclusion in the study item TR [3]. This contribution presents some text proposals for TR. The prime focus of these text proposals is to highlight number of possible interoperability observations specifically associated with D-MLB operations in multi-vendor HetNet deployments. The focus of the present analysis is specifically on the inter-eNBs interface load-exchange procedure.

# 4	Text Proposals

# ---Text Proposal (1) ---

# 4.2.x		Use Cases related to multi-vendor HetNet

# 4.2.x.x			Background

The objective of Mobility Load Balancing (MLB) is to distribute cell load evenly among adjacent cells or to transfer part of the traffic from congested cells to other cells so that radio resources remain highly optimised. In MLB, this is done by self-optimization of the mobility parameters.  Handover and cell selection parameters can be tuned in order to cope with the unequal traffic load and to minimize the number of handovers and redirections needed to achieve the load balancing.  SON procedures for MLB must be implemented locally at the eNB level and communicate over X2 interface to overcome processing delays and enable a fast adaptation to changing conditions. The 3GPP TS 32.522 [x1] specifies the location of SON decision algorithm of the load balancing at the eNB.

4.2.x.x		Interoperability issues

One of the most likely deployments where the MLB is expected to play a vital role is in the HetNet or multi-layered network deployments. HetNets are adopted primarily for improving capacity and coverage in areas with unequal user distribution. Typically small cells are deployed to provide extra capacity in areas with dense user demand while macro-cells are used to provide coverage in the remaining areas.

A key area that requires operator’s attention in such heterogeneous networks is the inter-layer coordination for efficient radio resources assignment, in a deployment scenario where different layers of base stations are provided by different suppliers (see figure x).

Figure x: System architecture – Distributed MLB in multi-vendor HetNet.

The architecture in figure x assumes that the Load Balancing SON function is located at EM or eNB level for both Macro and Small cell vendors. Normally in this case each vendor would have implemented standardised 3GPP X 2 interfaces [x2] and will be able to support most of the X2 exchanges for inter-working purposes. However, possible interoperability issue may arise from the fact that vendors are free to run any load balancing algorithm at any timescale, with any load metrics. Without coordination, the load balancing action might conflict between vendors.

Hence, in general the main challenges for multi-vendor interoperability over X2 in this deployment scenario are:

- Alignment of supported 3GPP optional signaling,

- Alignment of exchanged parameters value meaning,

- Alignment of timing for function monitoring and reporting.

It should be highlighted here that in a single vendor environement load balancing entities could be easily aligned, hence most of the above interoperability issues and challenges can be avoided.

An insight analysis of the details of possible interoperability challenges associated with load balancing SON mechanism in multi-vendor HetNets deployments is provided below:

### Interface related issues/observations: Load Exchange

The X2AP provides a range of functions including Load Management. This function is used by eNBs to indicate resource status, overload and traffic load to each other.

An eNB should support the following X2AP elementary procedures/messages for exchange of load information:

- X2AP Resource Status Reporting Initiation

  - RESOURCE STATUS REQUEST

  - RESOURCE STATUS RESPONSE

  - RESOURCE STATUS FAILURE

- X2AP Resource Status Reporting

  - RESOURCE STATUS UPDATE

3GPP TS 36.423 (X2AP) [x2] defines the RESOURCE STATUS UPDATE message containing the following measurement information elements (IEs) for reporting of cell load (as highlighted in Table 1 below). This message is sent by one eNB to neighbouring eNB to report the results of the requested load-related measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) |  | YES | reject |
| eNB2 Measurement ID | M |  | INTEGER (1..4095,...) |  | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 to maxCellineNB |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  |  |  |
| >>Hardware Load Indicator | O |  | 9.2.34 |  |  |  |
| >>S1 TNL Load Indicator | O |  | 9.2.35 |  |  |  |
| >>Radio Resource Status | O |  | 9.2.37 |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.44 |  | YES | ignore |
| >>ABS Status | O |  | 9.2.58 |  | YES | ignore |

Table x: X2AP RESOURCE STATUS UPDATE

Issue#1

The type of load information can be requested by the source eNB, via the Report Characteristics IE in the X2AP message RESOURCE STATUS REQUEST. If the target eNB is capable of providing the requested type of load information, it shall initiate the corresponding measurement and send RESOURCE STATUS RESPONSE. Otherwise, it shall send RESOURCE STATUS FAILURE, or alternatively, in case of partial failure, the target eNB may send RESOURCE STATUS RESPONSE with a specific failure cause.

A possible failure of Resource Status Reporting Initiation can happen due to non-supported load information by the target eNB.

Issue#2

- Hardware Load Indicator and S1 TNL Load Indicator can take 4 values (low, mid, high, overload). The definition of HW/S1 TNL Load is not standardised, neither is how to map a measured HW/S1 TNL Load to the HW/S1 TNL Load Indicator value.

- Radio Resource Status (PRB usage) is carried in the Radio Resource Status IE. The PRB usage definition is standardised in TS 36.314 [x3], therefore no interoperability problems associated with this load measurement type are expected.

- Composite Available Capacity (scaled at 0 to 100) can be used for carrying any combined, operator-specific load metric and calculation formula for composite available capacity can vary between vendors.

Issue#3

The metric used by SON Load Balancing can be calculated differently by each vendor. It is typically derived from a combination of Load Measurements from X2 and internal measurements.

--- End of Text Proposal (1) ----

---Text Proposal (2) ---

# 2		References

[x1]	3GPP TS 32.522; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

[x2]	3GPP TS 36.423 V11.3.0, X2 application protocol (X2AP)

[x3]	3GPP TS36.314; Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 - Measurements

--- End of Text Proposal (2) ---