3GPP TS 32.274 V12.2.0 (2013-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Charging management;

Short Message Service (SMS) charging

(Release 12)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Charging, SMS,

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2013, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	Architecture considerations	8

4.1	High level SMS architecture	8

4.2	SMS offline charging architecture	8

4.3	SMS online charging architecture	8

5	SMS charging principles and scenarios	9

5.1	SMS charging principles	9

5.1.1	General principles	9

5.1.2	Segmentation and concatenation	9

5.1.3	Triggers for generation of charging information	9

5.2	SMS offline charging scenarios	11

5.2.1	Basic principles	11

5.2.2	Rf message flows	11

5.2.2.1	SMS Submission to SMS-SC	11

5.2.2.2	SMS Delivery from SMS-SC	13

5.2.2.3	Delivery Report	14

5.2.2.4	Device Triggering using T4	14

5.2.2.4.1	SMS submission to SMS-SC for Device Triggering	14

5.2.2.4.2	SMS Delivery from SMS-SC for Device Triggering	15

5.2.2.4.3	SMS Device Triggering Delivery Report	15

5.2.2.5	Offline charging error cases - Diameter procedures	15

5.2.3	CDR generation	16

5.2.3.1	Triggers for xxx-CDR charging information collection	16

5.2.3.2	Triggers for xxx-CDR Charging Information Addition	16

5.2.3.3	Triggers for xxx-CDR closure	16

5.2.4	Ga record transfer flows	16

5.2.5	Bsm CDR file transfer	16

5.3	SMS online charging scenarios	17

5.3.1	Basic principles	17

5.3.2	Ro message flows	17

5.3.2.1	Simple Submission	17

5.3.2.2	Enhanced Submission	18

5.3.2.3	Delivery Report	18

5.3.2.4	Origination retry	18

5.3.2.5	Termination charge	20

5.3.2.6	Termination charge retry	22

5.3.2.7	Unsuccessful transaction	24

5.3.2.8	IMS/SMS Interworking Messages Charging	26

5.3.2.9	Simple Submission with SM service request	28

5.3.3	Credit Control related	29

5.3.3.1	Triggers for stopping for an SMS Credit Control session	29

5.3.3.2	Triggers for providing interim information for a SMS Credit Control session	29

6	Definition of charging information	30

6.1	Data description for SMS offline charging	30

6.1.1	Rf message contents	30

6.1.1.1	Summary of Offline Charging Message Formats	30

6.1.1.2	Structure for the Offline Charging  Message Formats	30

6.1.1.2.1	Charging Data-Request Message	30

6.1.1.2.2	Charging Data Response Message	31

6.1.2	Ga message contents	31

6.1.3	CDR description on the Bsm interface	31

6.2	Data description for SMS online charging	32

6.2.1	Ro message contents	32

6.2.1.0	General	32

6.2.1.1	Summary of Message Formats	33

6.2.1.2	Structure for the Credit Control Message Formats	33

6.2.1.2.1	Debit/Reserve Units Request Message	33

6.2.1.2.2	Debit / Reserve Units Response Message	34

6.3	SMS charging specific parameters	35

6.3.1	Definition of the SMS charging information	35

6.3.1.1	SMS charging information assignment for Service-Information	35

6.3.1.2	Definition of the SMS Information	36

6.3.2	Formal parameter description	37

6.3.2.1	SMS charging information for CDRs	37

6.3.2.2	SMS charging information for charging events	37

Annex A (informative):	Change history	38

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is part of a series of documents that specify charging functionality and charging management in GSM/UMTS and EPS networks. The GSM/UMTS and EPS core network charging architecture and principles are specified in TS 32.240 [2], which provides an umbrella for other charging management TSs that specify:

- the content of the CDRs per domain / subsystem / service (offline charging);

- the content of real-time charging messages per domain / subsystem / service (online charging);

- the functionality of online and offline charging for those domains / subsystems / services;

- the interfaces that are used in the charging framework to transfer the charging information (i.e. CDRs or charging events).

The complete document structure for these TSs is defined in TS 32.240 [2].

The present document specifies the Offline and Online Charging description for the Short Message Service (SMS), based on the functional description in TS 23.040 [7] and on the SMS over IP in TS 23.204 [8]. The present document does not replace existing offline SMS charging functionality defined for Circuit Switched in TS 32.250 [9] and for Packet Switched in TS 32.251 [10], and therefore is in addition to those specificationst. This charging description includes the offline and online charging architecture and scenarios specific to SMS, as well as the mapping of the common 3GPP charging architecture specified in TS 32.240 [1] onto SMS. It further specifies the structure and content of the CDRs for offline charging, and the charging events for online charging. The present document is related to other 3GPP charging TSs as follows:

- The common 3GPP charging architecture is specified in TS 32.240 [2];

- The parameters, abstract syntax and encoding rules for the CDRs are specified in TS 32.298 [3];

- A transaction based mechanism for the transfer of CDRs within the network is specified in TS 32.295 [6];

- The file based mechanism used to transfer the CDRs from the network to the operator’s billing domain (e.g. the billing system or a mediation device) is specified in TS 32.297 [5];

- The 3GPP Diameter application that is used for SMS offline and online charging is specified inTS 32.299 [4].

Furthermore, requirements that govern the charging work are specified in TS 22.115 [102].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging 	architecture and principles".

[3]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[4]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[5]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[6]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[7]	3GPP TS 23.040: "Technical realization of Short Message Service (SMS)"

[8]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2"

[9]	3GPP TS 32.250: "Telecommunication management; Charging management; Circuit Switched (CS) domain charging".

[10]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[11]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

[12]	IETF RFC 4006: "Diameter Credit-Control Application"

[13]	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

[14]	3GPP TS 23.038: "Alphabets and language-specific information".

[15]	3GPP TS 32.260:  "Telecommunication management; Charging management; IP Multimedia Services (IMS) charging".

[16]	3GPP TS 22.142: " Value Added Services (VAS) for Short Message Service (SMS) requirements "

[17]	3GPP TS 23.682: " Architecture enhancements to facilitate communications with packet data networks and applications"

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 32.240 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] or TS 32.240[2].

SMS node: An SMS node, in this specification, refers to either an SMS router, IP-SM-GW, SMS-SC or a combination of these nodes.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 32.240 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] or TS 32.240 [2].

# 4	Architecture considerations

## 4.1	High level SMS architecture

The high level SMS architecture is as defined in 3GPP TS 23.040 [7].Only the SMS Router, SMS-SC and the IP-SM-GW are within the scope of this specification. The details for the other nodes in the SMS architecture are defined within 3GPP TS 32.250 [9] and 3GPP TS 32.251 [10].

## 4.2	SMS offline charging architecture

The architecture for SMS offline charging is described in figure 4.2.

Figure 4.2: SMS Offline charging architecture

Details on the interfaces and functions can be found in TS 32.240 [2] for the general architecture components, The Rf interface is described in clause 6.1.1, Ga in clause 6.1.2, and Bsm in clause 6.1.3 of this document.

## 4.3	SMS online charging architecture

For online charging, the relevant SMS nodes utilise the Ro interface and application towards the OCS as specified in TS 32.299 [4]. The Ro reference point covers all online charging functionality required for SMS.

The SMS online charging architecture is depicted in figure 4.3.

Figure 4.3: SMS online charging architecture

Details on the interfaces and functions can be found in TS 32.240 [2] for the general architecture components, TS 32.296 [11] for the OCS, and 32.299 [4] for the Ro application.

# 5	SMS charging principles and scenarios

## 5.1	SMS charging principles

### 5.1.1	General principles

The Short Message Service comprises 5 main operational scenarios:

-	Person to Person: The message is sent by a UE as originator and received by a UE as destination.

-	Person to Application: The message is sent by a UE as originator and received by a third party application as destination.

-	Application to Person: The message is sent by a third party application as originator and received by a UE as destination.

-	Application to Application: The message is sent by a third party application as originator and received by another third party application as destination.

-	Device Triggering : the message is sent on behalf of an application as originator and received by a device as destination .

In addition SMS Nodes may apply services such as value added services specified in 3GPP TS 22.142 [16], services defined in industry standard protocols for SM submission from applications in a fixed network (protocols such as SMPP, UCP/EMI, OIS, CIMD, etc.)  or vendor specific services as endorsed by 3GPP TS 23.040 [7].As such, the SMS node collects charging information such as:

-	the destination and source addresses applied for an SM;

-	an indication of origination or termination handling;

-	identification of the node(s) and connection(s) involved in the SM transaction;

-	SM validity period;

-	in scenarios involving an application / VASP, the charging information describes the identification of the application / VASP;

-	requested SM service type.

### 5.1.2	Segmentation and concatenation

Information about concatenated messages should be sent to the charging systems in order to apply the appropriate charging models. The charging system may be required to be stateful to process information about segmented messages.

### 5.1.3	Triggers for generation of charging information

The following service level events shall, based on operator configuration, trigger the generation of charging information:

-	Simple submission – based on reception at the SMS node.

-	Enhanced submission – based on completion of the transaction handling at the SMS node.

-	Origination retry – based on the enhanced submission where the initial handling fails and a redelivery attempt is initiated.

-	Delivery report

-	Termination – Application to Person scenario only.

-	Termination retry – Application to Person scenario only – reattempt delivery of an SM to a terminating entity;

-	SM service request.

Depending on the charging model applied, a "refund" may be necessary for unsuccessful delivery.

See clause 5.2 and 5.3 for detailed procedures associated with the triggers above for Offline Charging and Online Charging respectively.

## 5.2	SMS offline charging scenarios

### 5.2.1	Basic principles

SMS offline charging functionality is based on SMS Nodes reporting chargeable events associated with SM transactions.

The SMS offline charging applies to the SMS-SC.

SMS offline charging uses the Diameter Offline Charging as specified in 3GPP TS 32.299 [4]. Event based charging applies, with reporting achieved by sending Charging Data Request [event] to the CDF.

SMS transactions are collected independently by the SMS-SC, on completion handling at SMS-SC (enhanced submission), for:

-	UE originated: SM submitted to the SMS-SC from a UE

-	Application originated: SM submitted to the SMS-SC from a Third Party Application

-	UE terminated: SM transferred to the UE from SMS-SC

-	Application terminated: SM transferred to a Third Party Application from SMS-SC

In addition to the different service level events described in chapter 5.1.3, SMS transactions associated to “Device Triggering using T4” as specified in TS 23.682 [17] are chargeable events defined for offline charging:

-	SMS submitted to the SMS-SC from MTC-IWF for Device Triggering.

-	SMS transferred from the SMS-SC towards the UE for Device Triggering.

-	Device Triggering Delivery report transferred from the SMS-SC to the MTC-IWF.

### 5.2.2	Rf message flows

The different scenarios below focus on the different message exchanges from/to the SMS-SC and the corresponding message flows between the SMS-SC and the CDF.

The sequence of messages exchanged between the SMS-SC and the other Nodes are described with generic names, (i.e SMS submit, SMS deliver) to reflect SMS reception or sending by/from the SMS-SC, independently from the protocol conveying the SMS.

#### 5.2.2.1	SMS Submission to SMS-SC

The following figure describes the scenario where UE or a third party application originates SMS-MO destined to a recipient UE:

Figure 5.2.2.1-1: Offline charging - SMS submission to SMS-SC

1)	The SMS-SC receives a “SMS Submit” incoming message originated by a UE or a third party application.

2)	The SMS-SC returns “SMS Submit Answer” with appropriate result associated to the reception of the Short Message: successfully received by SMS-SC or failed due to error at SMS-SC.

3)	The SMS-SC triggers a Charging Data  Charging Data Request with  Operation Type  indicating EVENT_RECORD to record successful or unsuccessful reception of the SM, with originator identified as UE or as a third party application, depending on the scenario.

NOTE:	In the scenario where a third party application is originator, sending application identification to the CDF will allow to apply accurate charging model of Termination scenario, i.e recipient UE to be charged for the delivered SM, instead of originator or both parties.

4)	The CDF creates a SC-SMO CDR and acknowledges the reception of the data.

#### 5.2.2.2	SMS Delivery from SMS-SC

The following figure describes the scenario where SMS-SC originates SM transfer towards the receiving party.

Figure 5.2.2.2-1: Offline charging SMS Transfer from SMS-SC

1)	The SMS-SC forwards “SMS Deliver” message towards the receiving party, as a first attempt or due to internal trigger for a retry delivery of a previously failed and stored SM.

2)  The SMS-SC receives “SMS Deliver Answer” message as the delivery success or failure of the SM transfer attempt.

3)	The SMS-SC triggers an Charging Data Request with Operation Type indicating EVENT_RECORD to record successful or unsuccessful result of SM delivery.

4)	The CDF creates a SC-SMT CDR and acknowledges the reception of the data.

#### 5.2.2.3	Delivery Report

Delivery Report or Status Report (SC informing the originating UE of the delivery outcome of a previously submitted short message) issued by the SMS-SC uses the same procedures as the “SMS Delivery from the SMS-SC” described within clause 5.2.2.2, as it is contained within a new SM.

#### 5.2.2.4	Device Triggering using T4

##### 5.2.2.4.1	SMS submission to SMS-SC for Device Triggering

The following figure describes the scenario where the MTC-IWF submits a request to SMS-SC for SM transfer towards the UE for Device Triggering purpose.

Figure 5.2.2.4.1-1: Offline charging - SMS submission to SMS-SC for Device Triggering

1)	The SMS-SC receives an incoming “Device Trigger Request” from an MTC-IWF over T4, destined to a UE recipient.

2)	The SMS-SC returns “Device Trigger Answer” with appropriate result associated to the reception of the trigger request: successfully received by SMS-SC or failed due to error at SMS-SC.

3)	The SMS-SC triggers a Charging Data Request with  Operation Type indicating EVENT_RECORD to record successful or unsuccessful reception of the SM from the MTC-IWF, with originator identified as MTC-IWF.

4)	The CDF creates a SC-SMO CDR and acknowledges the reception of the data.

##### 5.2.2.4.2	SMS Delivery from SMS-SC for Device Triggering

The scenario where SMS-SC originates the SMS Device Triggering transfer towards the UE is the same as “SMS Delivery from SMS-SC” described in 5.2.2.2, except the  Charging Data Request with  Operation Type indicating EVENT_RECORD includes a value for  “Device Triggering indication”.

##### 5.2.2.4.3	SMS Device Triggering Delivery Report

The following figure describes the scenario where the SMS-SC reports Delivery Report of Device Trigger to the MTC-IWF.

Figure 5.2.2.4.3-1: Offline charging - SMS Device Triggering Delivery Report

1)	The SMS-SC receives an incoming “Deliver Answer” as outcome from a previous Device Trigger transferred to a UE.

2)	The SMS-SC issues a “Delivery Report Request” towards the MTC-IWF, to report the success or failure of delivering the device trigger to the UE.

3)	The SMS-SC triggers a Charging Data Request with Operation Type indicating EVENT_RECORD to record successful or unsuccessful delivering of the device trigger to the UE.

4)	The CDF creates a SC-SMT CDR and acknowledges the reception of the data

#### 5.2.2.5	Offline charging error cases - Diameter procedures

The Offline Charging error cases in Diameter (Accounting) related procedures associated to Charging Data Request /Response, from SMS Node as network element are specified in TS 32.299 [4] chapter 6.1.3.

### 5.2.3	CDR generation

#### 5.2.3.1	Triggers for xxx-CDR charging information collection

Editor's Note: For Further Study.

#### 5.2.3.2	Triggers for xxx-CDR Charging Information Addition

Editor's Note: For Further Study.

#### 5.2.3.3	Triggers for xxx-CDR closure

Editor's Note: For Further Study.

### 5.2.4	Ga record transfer flows

Editor's Note: For Further Study.

### 5.2.5	Bsm CDR file transfer

Editor's Note: For Further Study.

## 5.3	SMS online charging scenarios

### 5.3.1	Basic principles

SMS online charging uses the Credit Control application as specified in 3GPP TS 32.299 [4].

SMS charging may use the Immediate Event Charging (IEC) principle or the Event Charging with Unit Reservation (ECUR) principle as specified in 3GPP TS 32.299 [4]. The chargeable events for subscriber charging are associated with SM transactions.

An implementation may use either IEC or ECUR for charging events based on operator configuration.

The units used for quota shall be service specific and based on an SM.

The selection of the OCS is implementation specific as there is no guaranteed means of providing the OCS address to the CTF.

In addition, SMS charging may use the Refund Account principle when the operation has not been successfully completed after an Immediate Event Charging (IEC).

NOTE:	For SMSIP, the IP-SM-GW may receive information relevant for online charging through signalling in IMS.

### 5.3.2	Ro message flows

#### 5.3.2.1	Simple Submission

This clause contains message flows for the different operation models IEC (figure 5.3.2.1-1) and ECUR (figure 5.3.2.1-2).

Figure 5.3.2.1-1: Online charging in simple submission for IEC

1)	Depending on which SMS mechanism (i.e. SMS or SMSIP) is in operation, the SMS node receives an incoming SM-Submit or a MAP-Forward-SM.

2)	The SMS node triggers a Debit Units Request message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Debit Units Response message to the SMS node.

5)	If authorised, the SMS node continues the SM processing as appropriate for the origination procedures.

Figure 5.3.2.1-2: Online charging in simple submission for ECUR

1)	Depending on which SMS mechanism (i.e. SMS or SMSIP) is in operation, the SMS node receives an incoming SM-Submit or a MAP-Forward-SM.

2)	The SMS node triggers a Reserve Units Request (Initial) message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Reserve Units Response message to the SMS node.

5)	If authorised, the SMS node continues the SM processing as appropriate for the origination procedures.

6)	The SM transaction is successfully acknowledged.

7)	The SMS node triggers a Reserve Units Request (Terminate) message to the OCS reporting the successful event transaction.

8)	The OCS performs the appropriate credit processing based on the received request.

9)	The OCS responds with a Reserve Units Response message to the SMS node..

#### 5.3.2.2	Enhanced Submission

The enhanced submission procedures are similar to the simple submission procedures using ECUR. However, the trigger for Reserve Units Request (Terminate) may be based on unsuccessful handling e.g. negative acknowledgement and with or without successful storage of the message for future redelivery attempts. See failure scenarios defined in clause 5.3.2.7.

#### 5.3.2.3	Delivery Report

The origination of delivery reports use the same procedures as the simple submission procedures as described within clause 5.3.2.1. The delivery report itself is contained within a new SM.

#### 5.3.2.4	Origination retry

This clause contains message flows for the different operation models IEC (figure 5.3.2.4-1) and ECUR (figure 5.3.2.4-2) for redelivery attempts in the origination direction.

Figure 5.3.2.4-1: Online charging in origination redelivery attempt for IEC

1)	An SMS node internal trigger occurs to attempt a redelivery of a previously failed and stored SM.

2)	The SMS node triggers a Debit Units Request message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Debit Units Response message to the SMS node.

5)	If authorised, the SMS node continues the SM processing as appropriate for the origination procedures.

Figure 5.3.2.4-2: Online charging in origination redelivery attempt for ECUR

1)	An SMS node internal trigger occurs to attempt a redelivery of a previously failed and stored SM.

2)	The SMS node triggers a Reserve Units Request (Initial) message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Reserve Units Response message to the SMS node.

5)	If authorised, the SMS node continues the SM processing as appropriate for the origination or termination procedures.

6)	The SM transaction is successfully acknowledged.

7)	The SMS node triggers a Reserve Units Request (Terminate) message to the OCS reporting the successful event transaction.

8)	The OCS performs the appropriate credit processing based on the received request.

9)	The OCS responds with a Reserve Units Response message to the SMS node.

#### 5.3.2.5	Termination charge

Figure 5.3.2.5-1: Online charging in termination for IEC

1)	The SMS node receives an incoming SM-SUBMIT from an application destined for a mobile recipient.

NOTE:	This scenario differs from simple submission charging as described in subclause 5.3.2.1 in that typically the mobile recipient (instead of originator or both parties) will be charged for such a short message.

2)	The SMS node triggers a Debit Units Request message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Debit Units Response message to the SMS node.

5)	If authorised, the SMS node continues the SM processing as appropriate for the termination procedures.

Figure 5.3.2.5-2: Online charging in termination for ECUR

1)	The SMS node receives an incoming SM-SUBMIT from an application destined for a mobile recipient.

NOTE:	This scenario differs from simple submission charging as described in subclause 5.3.2.1 in that typically the mobile recipient (instead of originator or both parties) will be charged for such a short message.

2)	The SMS node triggers a Reserve Units Request (Initial) message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Reserve Units Response message to the SMS node.

5)	If authorised, the SMS node continues the SM processing as appropriate for the termination procedures.

6)	The SM transaction is successfully acknowledged.

7)	The SMS node triggers a Reserve Units Request (Terminate) message to the OCS reporting the successful event transaction.

8)	The OCS performs the appropriate credit processing based on the received request.

9)	The OCS responds with a Reserve Units Response message to the SMS node.

#### 5.3.2.6	Termination charge retry

Figure 5.3.2.6-1: Online charging in termination redelivery attempt for IEC

1)	An SMS node internal trigger occurs to attempt a redelivery of a previously failed and stored SM.

2)	The SMS node triggers a Debit Units Request message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Debit Units Response message to the SMS node.

5)	If authorised, the SMS node continues the SM processing as appropriate for the termination procedures.

Figure 5.3.2.6-2: Online charging in termination redelivery attempt for ECUR

1)	An SMS node internal trigger occurs to attempt a redelivery of a previously failed and stored SM.

2)	The SMS node triggers a Reserve Units Request (Initial) message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Reserve Units Response message to the SMS node.

5)	If authorised, the SMS node continues the SM processing as appropriate for the termination procedures.

6)	The SM transaction is successfully acknowledged.

7)	The SMS node triggers a Reserve Units Request (Terminate) message to the OCS reporting the successful event transaction.

8)	The OCS performs the appropriate credit processing based on the received request.

9)	The OCS responds with a Reserve Units Response message to the SMS node.

#### 5.3.2.7	Unsuccessful transaction

Unsuccessful transaction after IEC

The following flow in figure 5.3.2.7-1 only applies where a refund action is required for unsuccessful delivery.

Figure 5.3.2.7-1: Unsuccessful transaction after IEC

1)	The SMS node receives a trigger to attempt delivery of an SM. This may be for origination, termination or redelivery attempt.

2)	The SMS node triggers a Debit Units Request message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Debit Units Response message to the SMS node.

5)	If authorised, the SMS node continues the SM processing as appropriate for origination or termination procedures.

6)	The SM transaction is acknowledged as an unsuccessful transaction (either via explicit signalling or an internal trigger).

7)	The SMS node triggers a Debit Units Request (Refund Account) message to the OCS.

8)	The OCS performs the appropriate refund processing based on the received request.

9)	The OCS responds with a Debit Units Response (Refund Account) message to the SMS node.

Unsuccessful transaction in ECUR

Figure 5.3.2.7-2: Unsuccessful transaction for ECUR

1)	The SMS node receives a trigger occurs to attempt delivery of an SM. This may be for origination, termination or redelivery attempt.

2)	The SMS node triggers a Reserve Units Request (Initial) message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Reserve Units Response message to the SMS node.

5)	If authorised, the SMS node continues the SM processing as appropriate for the origination or termination procedures.

6)	The SM transaction is acknowledged as an unsuccessful transaction (either via explicit signalling or an internal trigger).

7)	The SMS node triggers a Reserve Units Request (Terminate) message to the OCS reporting the used unit for the service to zero. This characterizes the unsuccessful event transaction.

8)	The OCS performs the appropriate credit processing based on the received request.

9)	The OCS responds with a Reserve Units Response message to the SMS node.

#### 5.3.2.8	IMS/SMS Interworking Messages Charging

This clause contains message flows for the different operation models IEC (figure 5.3.2.8-1) and ECUR (figure 5.3.2.8-2) for IMS/SMS Interworking messages in the origination direction.

Figure 5.3.2.8-1: Online charging in origination IMS/SMS Interworking Messages for IEC

1)	The IP-SM-GW receives an incoming SIP MESSAGE.

2) 	The IP-SM-GW parses the SIP MESSAGE.

3) 	The IP-SM-GW triggers a Debit Units Request message to the OCS.

4)	The OCS performs the appropriate credit processing based on the received Debit Units Request.

5)	The OCS responds with a Debit Units Response message to the IP-SM-GW.

6)	IP-SM-GW converts the SIP MESSAGE to 1 or n (n>=1) short messages.

7) 	If authorised, the IP-SM-GW forwards the segmented short messages.

Figure 5.3.2.8-2: Online charging in origination IMS/SMS Interworking Messages for ECUR

1)	The IP-SM-GW receives an incoming SIP MESSAGE.

2) 	The IP-SM-GW parses the SIP MESSAGE.

3)	The IP-SM-GW triggers a Reserve Units Request (Initial) message to the OCS.

4)	The OCS performs the appropriate credit processing based on the received Reserve Units Request.

5)	The OCS responds with a Reserve Units Response message to the IP-SM-GW.

6)	IP-SM-GW converts the SIP MESSAGE to 1 or n (n>=1) short messages.

7)	If authorised, the IP-SM-GW forwards segmented short messages.

8)	All the short messages transactions are successfully acknowledged.

9)	The IP-SM-GW triggers a Reserve Units Request (Terminate) message to the OCS reporting the successful event transaction.

10)	The OCS performs the appropriate credit processing based on the received request.

11)	The OCS responds with a Reserve Units Response message to the IP-SM-GW.

#### 5.3.2.9	Simple Submission with SM service request

This clause contains message flows for operation models IEC (figure 5.3.2.9-1) where application of a SM service is subject to charging independent from the SM submission.

Editors Note:	Simple SM submission with SM service request for operation model ECUR is FFS.

Figure 5.3.2.9-1: Online charging in simple submission with SM service request for IEC

1)	Depending on which SMS mechanism (i.e. SMS or SMSIP) is in operation, the SMS node receives an incoming SM-Submit or a MAP-Forward-SM which includes a SM service request (such as forwarding or SM copy).

2)	The SMS node triggers a Debit Units Request message to the OCS for the SM submission.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Debit Units Response message for the SM submission to the SMS node.

5)	If normal SM processing is authorized in step 4, the SMS Node analyzes the SM and detects that a SM service must be applied that is subject to charging.

6)	If  a SM service subject to charging is detected in step 5, the SMS node triggers an additional  Debit Units Request message to the OCS for the requested SM service.

7)	The OCS performs the appropriate credit processing based on the received request.

8)	The OCS responds with a Debit Units Response message for the requested SM service to the SMS node.

9)	If authorised in step 7, the SMS node applies the requested SM service.

NOTE 1:	Depending on the nature of the requested SM service, "service application" may involve creating additional messages (for example in case of a SM copy service). This is deemed part of step 9 and not otherwise shown in this diagram.

10)	If authorised in step 4, the SMS node continues the SM processing as appropriate for the origination procedures.

NOTE 2:	Authorization of SM processing is independent of the authorization for application of a SM service. I.e. If authorization is for SM processing is granted in step 4 but authorization for SM service is refused in step 8 SM processing appropriate for the originating service continues without applying the requested SM service.

### 5.3.3	Credit Control related

#### 5.3.3.1	Triggers for stopping for an SMS Credit Control session

Used in ECUR only, a Debit / Reserve Units Request message to terminate the credit control session is sent to OCS when:

-	Validity time for granted quota expires

-	Granted quota runs out (i.e. a successful event has occurred)

-	Abort-Session-Request is received from the OCS

The expiration of the validity time for quota does not require the SMS procedures to be terminated. The CTF shall be configurable as to whether on expiration of validity time, the service should be aborted or not; i.e. whether the stored message should be deleted and no further (re-)delivery attempt should be made.

#### 5.3.3.2	Triggers for providing interim information for a SMS Credit Control session

The provision of interim information for credit control is not used in this release of the specification, due to the use of IEC and ECUR.

# 6	Definition of charging information

## 6.1	Data description for SMS offline charging

### 6.1.1	Rf message contents

#### 6.1.1.1	Summary of Offline Charging Message Formats

The SMS Node generates accounting information that can be transferred from the CTF to the CDF. For this purpose, SMS offline charging utilises the Charging Data Transfer Operation that is specified in the 3GPP accounting application described in TS 32.299 [50].

The Charging Data Transfer operation employs the Charging Data Request and Charging Data Response messages. The following table describes the use of these messages for offline charging.

- Table 6.1.1.1: Offline Charging Messages Reference Table

| Command-Name | Source | Destination |
| --- | --- | --- |
| Charging Data Request | CTF | CDF |
| Charging Data Response | CDF | CTF |

#### 6.1.1.2	Structure for the Offline Charging  Message Formats

##### 6.1.1.2.1	Charging Data-Request Message

The following table illustrates the basic structure of a Charging Data-Request message as used for SMS offline charging.

- Table 6.1.1.1.2.1: Charging Data Request Message Contents

| Field | Category | Description |
| --- | --- | --- |
| Session Identifier | M | Described in 32.299 [4] |
| Originator Host | M | Described in 32.299 [4] |
| Originator Domain | M | Described in 32.299 [4] |
| Destination Domain | M | Described in 32.299 [4] |
| Operation Type | M | This field contains event type transfer ( immediate event based charging). |
| Operation Number | M | Described in 32.299 [4] |
| Operation Identifier | OM | Described in 32.299 [4] |
| User Name | OC | This field contains the identification of the source node. |
| Origination Timestamp | OC | Described in 32.299 [4] |
| Proxy Information | OC | Described in 32.299 [4] |
| Route Information | OC | Described in 32.299 [4] |
| Operation Token | OM | This field contains the service context, i.e. SMS charging. |
| Service Information | OM | This field holds the 3GPP specific SMS parameter, described in 6.3. |

##### 6.1.1.2.2	Charging Data Response Message

The following table illustrates the basic structure of a Charging Data Response message as used for SMS offline charging.

- Table 6.1.1.2.2: Charging Data Response Message Contents

| Field | Category | Description |
| --- | --- | --- |
| Session Identifier | M | Described in 32.299 [4] |
| Operation Result | M | Described in 32.299 [4] |
| Originator Host | M | Described in 32.299 [4] |
| Originator Domain | M | Described in 32.299 [4] |
| Operation Type | M | This field contains event type transfer ( immediate event based charging). |
| Operation Number | M | Described in 32.299 [4] |
| Operation Identifier | OM | Described in 32.299 [4] |
| User Name | OC | This field contains the identification of the source node. |
| Destination Host | OC | Described in 32.299 [4] |
| Error-Reporting-Host | OC | Described in 32.299 [4] |
| Origination Timestamp | OC | Described in 32.299 [4] |
| Proxy Information | OC | Described in 32.299 [4] |

### 6.1.2	Ga message contents

Editor's Note: For Further Study.

### 6.1.3	CDR description on the Bsm interface

Editor's Note: For Further Study.

## 6.2	Data description for SMS online charging

### 6.2.1	Ro message contents

#### 6.2.1.0	General

The SMS node generates Debit / Reserve Units information that can be transferred from the CTF to the OCF. For this purpose, SMS online charging utilises the Debit Units and Reserve Units procedure that is specified in the 3GPP Debit / Reserve Units operation in 3GPP TS 32.299 [4].

The SMS node generates refund information that can be transferred from the CTF to the OCF. For this purpose, it uses REFUND procedure defined in IETF RFC 4006 [12] with extended AVPs.

The Debit / Reserve Units procedure employs the Debit / Reserve Units Request and Debit / Reserve Units Response messages.

The Refund Account procedure employs the Debit Units Request (Refund Account) request and response messages.

Table 6.2.1 describes the use of these messages for SMS online charging.

- Table 6.2.1: SMS Online Charging Messages contents

| Command-Name | Source | Destination |  |
| --- | --- | --- | --- |
| Debit / Reserve Units Request | CTF | OCS |  |
| Debit / Reserve Units Response | OCS | CTF |  |

This clause 6.2.1 describes the different fields used in the credit control messages.

Detailed descriptions of the fields are provided in 3GPP TS 32.299 [4].

#### 6.2.1.1	Summary of Message Formats

#### 6.2.1.2	Structure for the Credit Control Message Formats

##### 6.2.1.2.1	Debit/Reserve Units Request Message

Table 6.2.1.2.1 illustrates the basic structure of a Debit / Reserve Units Request message from SMS node as used for SMS online charging.

- Table 6.2.1.2.1: Debit / Reserve Units Request Message Contents for SMS

| Field | Category | Description |
| --- | --- | --- |
| Session Identifier | M | This field identifies the operation session. |
| Originator Host | M | This field contains the identification of the source point of the operation. |
| Originator Domain | M | This field contains the realm of the operation originator. |
| Destination Domain | M | This field contains the realm of the operation destination. |
| Operation Identifier | M | This field is a unique operation identifier. |
| Operation Token | M | This field contains the service context, i.e. SMS charging. |
| Operation Type | M | This field defines the transfer type: event for immediate event based charging and initial, terminate for ECUR based charging. |
| Operation Number | M | This field contains the sequence number of the transferred messages. |
| Destination Host | OC | This field contains the identification of the destination point of the operation. |
| User Name | OC | This field contains the identification of the source node. |
| Origination State | OC | Used for ECUR only. |
| Origination Timestamp | OC | This field contains the time when the operation is requested. |
| Subscriber Identifier | OM | This field contains the identification of the subscriber (i.e. MSISDN) that uses the requested service. |
| Termination Cause | OC | This field contains information about the cause for termination of a credit control session. Used for terminating credit control sessions in ECUR only. |
| Requested-Action | OC | This field contains the requested action, used for IEC only. |
| Multiple Operation | OM | This field indicate the occurrence of multiple operations. Used for ECUR only |
| Multiple Unit Operation | OC | This field contains the parameter for the quota management. Used for ECUR only |
| Subscriber Equipment Number | OC | This field contains the identification of the user equipment used to access service. Included if information is made available to the node. |
| Proxy Information | OC | This field contains the parameter of the proxy. |
| Route Information | OC | This field contains the parameter of the route. |
| Service Information | OM | This field holds the SMS specific parameter and is described in clause 6.3. |

##### 6.2.1.2.2	Debit / Reserve Units Response Message

Table 6.2.1.2.2 illustrates the basic structure of a Debit / Reserve Units Response message as used for SMS charging. This message is always used by the OCS as specified below, independent of the receiving SMS node and the operation type that is being replied to.

- Table 6.2.1.2.2: Debit / Reserve Units Response Message Contents for SMS

| Field | Category | Description |
| --- | --- | --- |
| Session Identifier | M | This field identifies the operation session. |
| Operation Result | M | This field identifies the result of the operation. |
| Originator Host | M | This field contains the identification of the source point of the operation. |
| Originator Domain | M | This field contains the realm of the operation originator. |
| Operation Identifier | M | This field is a unique operation identifier. |
| Operation Type | M | This field defines the transfer type: event for event based charging and start, interim, stop for session based charging. |
| Operation Number | M | This field contains the sequence number of the transferred messages. |
| Operation Failover | - | Not used for SMS in 3GPP. |
| Multiple Unit Operation | OC | This field contains the parameter for the quota management. Used in IEC for refund purpose and in ECUR. |
| Operation Failure Action | OC | This field defines the resulting operation at the SMS node if a failure has occurred at the OCS for ECUR. |
| Operation Event Failure Action | OC | This field defines the resulting operation at the SMS node if a failure has occurred at the OCS for IEC. |
| Redirection Host | OC |  |
| Redirection Host Usage | OC |  |
| Redirection Cache Time | OC |  |
| Proxy Information | OC | This field contains the parameter of the proxy. |
| Route Information | OC | This field contains the parameter of the route. |
| Failed parameter | OC | This field contains missing and/or unsupported parameter that caused the failure. |
| Service Information | OC | This field contains SMS specific information. |

Editor's Note: The mechanism to carry refund information is For Future Study.

## 6.3	SMS charging specific parameters

### 6.3.1	Definition of the SMS charging information

#### 6.3.1.1	SMS charging information assignment for Service-Information

The components in the Service Information that are use for SMS charging can be found in table 6.3.1.1.

Table 6.3.1.1: Service Information used for SMS Charging

| Field | Category | Description |
| --- | --- | --- |
| Service Information | OM | This is a structured field and holds the 3GPP specific parameter as defined in TS 32.299 [50]. For SMS Charging the SMS-Information and selected parameters of MMS Information,PS-Information and IMS information are used. |
| SMS Information | OM | This is a structured field and holds the SMS specific parameters. The details are defined in table 6.3.1.2. |
| MMS Information | OM | This is a structure field and the following parameters are specific to SMS. The complete structure is defined in 3GPP TS 32.270 [13] |
| Originator Address | OM | This field holds the address of the originator of the SM. This will typically be an E.164 number or a shortcode. Multiple addresses may be carried if additional information is available, e.g. IMSI and E.164 number. |
| Submission Time | OC | This field holds the timestamp of when the submitted SM arrived at the originating SMS Node. The information to populate this field is obtained from the TP-Service-Center-Time-Stamp (TP-SCTS) as defined in 3GPP TS 23.040 [7]. If a refund or retransmission is required, the timestamp carries the timestamp associated with the original submitted SM. |
| Priority | OC | This field holds any priority information associated with an SM. Applicable to terminating procedures only. Priority handling is defined in 3GPP TS 23.040 [13]. The value “low” is not applicable. |
| Message Id | OM | This field carries the identity used to identify an SM in the SMS node associated with entity that submitted it. The information to populate this field is obtained from the TP-Message-Reference (TP-MR) as defined in 3GPP TS 23.040 [7]. |
| Message Size | OM | This field carries the length of the user data part of the SM. The information to populate this field is obtained from the TP-User-Data-Length (TP-UDL) as defined in 3GPP TS 23.040 [7] |
| Message Class | OM | Used as defined in 3GPP TS 32.270 [13]. It is implementation dependent the value selected for a specific transaction. |
| Delivery Report Requested | OC | This field indicates whether a delivery report is requested by the SM originator. The information to populate this field is obtained from the TP-Status-Report-Request (TP-SRR) as defined in 3GPP TS 23.040 [7] |
| PS Information | OC | This is a structured field and the following parameters are specific to SMS. The complete structure is defined in TS 32.251 [11]. |
| PDP Address | OC | This field holds the IP address used by the subscriber for the SMS transaction. Included if the SMS node is the IP-SM-GW. |
| 3GPP User Location Info | OC | This field holds the information about the location of the subscriber during the SMS transaction. |
| 3GPP RAT Type | OC | This field holds information about the radio access technology used for the SMS transaction. |
| MS Time Zone | OC | This field indicates the offset between universal time and local time in steps of 15 minutes of where the MS currently resides. |
| IMS Information | OC | This is a structured field and the following parameters are specific to SMS. The complete structure is defined in TS 32.260 [15]. |
| User Session Id | OC | This field holds the session identifier. For a SIP session the Session-ID contains the SIP Call ID. |
| Number Portability routing information | OC | This field includes information on number portability after DNS/ENUM request from S-CSCF in the calling user’s home network. |
| Carrier Select routing information | OC | This field includes information on carrier select after DNS/ENUM request from S-CSCF in the calling user’s home network. |

#### 6.3.1.2	Definition of the SMS Information

The components in the SMS Information that are used for SMS charging can be found in table 6.3.1.2.

Table 6.3.1.2: SMS Information used for SMS Charging

| Field | Category | Description |
| --- | --- | --- |
| SMS Node | OM | Identifies the SMS Node as IP-SM-GW or SMS Router or a combined IP-SM-GW / SMS Router or as SMS-SC. |
| SM Client Address | OM | This field holds the address of the SMS node to which the charging system is connected to. This may be the same as the SMSC Address field. |
| Originator SCCP Address | OC | This field holds the SCCP calling address used to receive the SM at the SMS node. Only present if SMSIP is not used for the inward connection. |
| Originator Received Address | OC | This field holds the original, unmodified address of the originator of the SM, as received by the SMS node, in case address manipulation (such as number plan corrections) have been applied in the SMS node. This will typically be an E.164 number or a shortcode. Multiple addresses may be carried if additional information is available, e.g. IMSI and E.164 number. |
| Recipient Info | Oc | This field holds recipient information for the SM. Each occurrence of this field denotes a different recipient. Multiple occurrences of this field are allowed in case - multiple recipients are associated with the charged event and - all other charging information is identical for all recipients. In case the SM contains a Delivery Report, as described in subclause 5.3.2.3, this field identifies the recipient of this Delivery Report. This recipient information must correspond to the originator information of the message that triggered this Delivery Report. (Note 2) |
| Recipient Address | Oc | This field holds the address of the recipient of the SM. This will typically be an E.164 number or a shortcode. Multiple addresses may be carried if additional information is available, e.g. shortcode or IMSI and E.164 number. |
| Recipient Received Address | OC | This field holds the original, unmodified address of the recipient of the SM, as received by the SMS node, in case address manipulation (such as number plan corrections) have been applied in the SMS node. This will typically be an E.164 number or a shortcode. Multiple addresses may be carried if additional information is available, e.g. shortcode or IMSI and E.164 number. |
| Recipient SCCP Address | OC | This field holds the SCCP called address used by the SMS node to onward deliver the SM. Only present if SMSIP is not used for the outward connection. |
| SM Destination Interface | OM | This is a structured field containing information describing the interface on which the SM is to be delivered (i.e. the next hop) In case the charging event is for person to application messaging or for application to application messaging (see subclause 5.1.1) this field holds the identification of the application. (See also Note 3) |
| SM Protocol Id | OC | This field holds the TP-PROTOCOL-ID (TP-PID) as defined in 3GPP TS 23.040 [7]. This field relates to the recipient when charging MT SMS messages as specified in 32.240 [1]. |
| SMSC Address | OM | This field holds the address of the SMSC to which the originating or terminating SM is directed to. |
| SM Data Coding Scheme | OM | This field holds the data coding scheme used within the SM. The information to populate this field is obtained from TP-DCS header. |
| SM Message Type | OM | This field identifies the message that triggered the generation of charging information. |
| SM Originator Interface | OM | This is a structured field containing information describing the interface on which the SM was received by the SMS node (i.e. the previous hop) In case the charging event is for application to person messaging or for application to application messaging (see subclause 5.1.1) this field holds the identification of the application. (See also Note 3) |
| SM Protocol Id | OC | This field holds the TP-PROTOCOL-ID (TP-PID) as defined in 3GPP TS 23.040 [7] This field relates to the originator when charging MO SMS messages as specified in 32.240 [1]. |
| SM Reply Path Requested | OC | This field carries an indication of whether a reply SM to an original SM shall follow the same path as identified by the TP-Reply-Path (TP-RP) flag. |
| SM User Data Header | OC | This field carries the user data header extracted from the user data of the SM. The user data header (TP-UDH) is specified in 3GPP TS 23.040 [7] |
| SM Status | OC | This field holds the information from the TP-Status field in a Status-Report TPDU. This information is only applicable to delivery report charging procedures or where ECUR is employed. |
| SM Discharge Time | OC | This field holds the time associated with the event being reported in the SM Status field. This information is only applicable to delivery report charging procedures. |
| Number of Messages Sent | OC | Indicates the number of SMSs sent by the IMS application if applicable. |
| SM Service Type | OC | This field indicates the type of SM service that caused the charging interaction. It is only applicable for SM supplementary service procedures. |

NOTE 1:	The case of multi-destinations of SMS refers to SMS and Internet Electronic Mail interworking as specified in subsclause 3.8 of TS 23.040 [7]

NOTE 2:	Implementations vary as to the originator address that is presented to an end user for a Delivery Report. Typically the originator address either identifies the SMS node that generated the Delivery Report or the originator address of a Delivery Report identifies the recipient of the original message that triggered this Report. It is expected that the charging event contains the information presented to the end user.

NOTE 3: There is a distinction between short numbers (as conveyed in originator and/or recipient address fields) and the identification of SM applications (as carried in SM Originator Interface and/or SM Destination Interface). Short numbers are used by end users to address a service of an applications. Multiple short numbers may map to one application capable of multiple services. The identification of an application is how an application is know to the operator.

### 6.3.2	Formal parameter description

#### 6.3.2.1	SMS charging information for CDRs

Editor’s Note:	For Future Study.

#### 6.3.2.2	SMS charging information for charging events

Editor’s Note:	For Future Study.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Sep 2007 | SP-37 | SP-070620 | -- | -- | Submitted to TSG SA#37 for Information | -- | 1.0.0 |  |
| Dec 2007 | SP-38 | SP-070748 | -- | -- | Submitted to TSG SA#38 for Approval | -- | 2.0.0 | 8.0.0 |
| Dec 2007 | -- | -- | -- | -- | editHelp: the figures are now visible in normal view | -- | 8.0.0 | 8.0.1 |
| Mar 2008 | SP-39 | SP-080074 | 0001 | -- | Correction on Service Information for SMS Charging | F | 8.0.1 | 8.1.0 |
| Jun 2008 | SP-40 | SP-080330 | 0002 | -- | Support of Number Portability and Carrier Selection in SMS Online Charging | B | 8.1.0 | 8.2.0 |
| Sep 2008 | SP-41 | SP-081216 | 0003 | -- | Multiple SMS destination – Alignment with TS 23.040 | C | 8.2.0 | 8.3.0 |
| Sep 2008 | SP-41 | SP-081224 | 0004 | -- | Introduce Online Charging from SMS-SC into 3GPP TS 32.274 | B | 8.2.0 | 8.3.0 |
| Dec 2008 | SP-42 | SP-080706 | 0005 | -- | Correction on Multiple Unit Operation category | F | 8.3.0 | 8.4.0 |
| Dec 2008 | SP-42 | SP-080706 | 0006 | -- | Add SMS-SC as SMS node type | B | 8.3.0 | 8.4.0 |
| Dec 2008 | SP-42 | SP-080706 | 0007 | -- | Additional Address Info for SMS charging | B | 8.3.0 | 8.4.0 |
| Dec 2008 | SP-42 | SP-080706 | 0008 | -- | Add charging of SMS services to 32.274 | B | 8.3.0 | 8.4.0 |
| Mar 2009 | SP-43 | SP-090045 | 0009 | -- | SMS IE structure alignment | F | 8.4.0 | 8.5.0 |
| Jun 2009 | SP-44 | SP-090293 | 0010 | -- | Clarification of "Termination charge" and application addressing | F | 8.5.0 | 8.6.0 |
| Sep 2009 | SP-45 | SP-090536 | 0011 | -- | IE usage for Delivery Reports and Application Identifiers | F | 8.6.0 | 8.7.0 |
| Dec 2009 | - | - | - | - | Update to Rel-9 version (MCC) | - | 8.7.0 | 9.0.0 |
| Mar 2011 | - | - | - | - | Update to Rel-10 version (MCC) | - | 9.0.0 | 10.0.0 |
| Sep-2012 | SP-57 | SP-120575 | 0014 | - | Addition of MS Timezone for NetLoc | B | 10.0.0 | 11.0.0 |
| Mar 2013 | SP-59 | SP-130055 | 0017 | 1 | Introduction of SMS-SC Offline Charging Architecture | B | 11.0.0 | 12.0.0 |
| Mar 2013 |  |  |  |  | Editorial changes on CDR tables (MCC) |  | 12.0.0 | 12.0.1 |
| Jun-2013 | SP-60 | SP-130274 | 0018 | 1 | Introduction SMS Offline charging principles and flows | B | 12.0.1 | 12.1.0 |
| Dec-2013 | SP-62 | SP-130628 | 0020 | 1 | Introduce Charging Data Request/Response in flow description | B | 12.1.0 | 12.2.0 |
|  |  |  | 0021 | - | Introduction of offline charging Diameter error cases | B |  |  |
|  |  |  | 0022 | 1 | Introduction of Rf message content | B |  |  |