3GPP TS 28.752 V11.1.0 (2014-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Subscription Management (SuM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)

(Release 11)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Subscription Management, NRM, IRP, Converged Management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2014, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	7

Introduction	7

1	Scope	8

2	References	8

3	Definitions and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	9

4	Model	10

4.1	Imported information entities and local labels	10

4.2	Class diagram	10

4.2.1	Relationships	10

4.2.2	Inheritance	14

4.3	Class definitions	17

4.3.1	SuMService	17

4.3.1.1	Definition	17

4.3.1.2	Attributes	17

4.3.1.3	Attribute constraints	17

4.3.1.4	Notifications	17

4.3.2	CSService	17

4.3.2.1	Definition	17

4.3.2.2	Attributes	18

4.3.2.3	Attribute constraints	19

4.3.2.4	Notifications	19

4.3.3	GPRSService	19

4.3.3.1	Definition	19

4.3.3.2	Attributes	20

4.3.3.3	Attribute constraints	21

4.3.3.4	Notifications	21

4.3.4	IMSService	22

4.3.4.1	Definition	22

4.3.4.2	Attributes	22

4.3.4.3	Attribute constraints	22

4.3.4.4	Notifications	22

4.3.5	SuMSubscribedService	23

4.3.5.1	Definition	23

4.3.5.2	Attributes	23

4.3.5.3	Attribute constraints	23

4.3.5.4	Notifications	23

4.3.6	CSSubscribedService	23

4.3.6.1	Definition	23

4.3.6.2	Attributes	24

4.3.6.3	Attribute constraints	25

4.3.6.4	Notifications	25

4.3.7	GPRSSubscribedService	25

4.3.7.1	Definition	25

4.3.7.2	Attributes	26

4.3.7.3	Attribute constraints	27

4.3.7.4	Notifications	27

4.3.8	IMSSubscribedService	27

4.3.8.1	Definition	27

4.3.8.2	Attributes	28

4.3.8.3	Attribute constraints	28

4.3.8.4	Notifications	28

4.3.9	SuMSubscriberProfile	28

4.3.9.1	Definition	28

4.3.9.2	Attributes	29

4.3.9.3	Attribute constraints	29

4.3.9.4	Notifications	29

4.3.10	SuMServiceProfile	29

4.3.10.1	Definition	29

4.3.10.2	Attributes	29

4.3.10.3	Attribute constraints	29

4.3.10.4	Notifications	29

4.3.11	CSServiceProfile	29

4.3.11.1	Definition	29

4.3.11.2	Attributes	30

4.3.11.3	Attribute constraints	32

4.3.11.4	Notifications	32

4.3.12	GPRSSServiceProfile	32

4.3.12.1	Definition	32

4.3.12.2	Attributes	33

4.3.12.3	Attribute constraints	33

4.3.12.4	Notifications	34

4.3.13	IMSServiceProfile	34

4.3.13.1	Definition	34

4.3.13.2	Attributes	34

4.3.13.3	Attribute constraints	34

4.3.13.4	Notifications	35

4.3.14	SubscriptionFunction	35

4.3.14.1	Definition	35

4.3.14.2	Attributes	35

4.3.14.3	Attribute constraints	35

4.3.14.4	Notifications	35

4.3.15	SuMCSSupplService	35

4.3.15.1	Definition	35

4.3.15.2	Attributes	36

4.3.15.3	Attribute constraints	36

4.3.15.4	Notifications	36

4.3.16	SuMGPRSSupplService	37

4.3.16.1	Definition	37

4.3.16.2	Attributes	37

4.3.16.3	Attribute constraints	37

4.3.16.4	Notifications	37

4.3.17	SuMUser	37

4.3.17.1	Definition	37

4.3.17.2	Attributes	37

4.3.17.3	Attribute constraints	37

4.3.17.4	Notifications	38

4.3.18	SuMCredentials	38

4.3.18.1	Definition	38

4.3.18.2	Attributes	38

4.3.18.3	Attribute constraints	38

4.3.18.4	Notifications	38

4.3.19	IMSPublicIdentification	39

4.3.19.1	Definition	39

4.3.19.2	Attributes	39

4.3.19.3	Attribute constraints	39

4.3.19.4	Notifications	39

4.3.20	 MobileUserData	39

4.3.20.1	Definition	39

4.3.20.2	Attributes	39

4.3.20.3	Attribute constraints	39

4.3.20.4	Notifications	39

4.3.21	SuMServiceParameter	40

4.3.21.1	Definition	40

4.3.21.2	Attributes	40

4.3.21.3	Attribute constraints	40

4.3.21.4	Notifications	40

4.3.22	SuMIdentity	40

4.3.22.1	Definition	40

4.3.22.2	Attributes	40

4.3.22.3	Attribute constraints	40

4.3.22.4	Notifications	40

4.3.23	ServerCapabilities	40

4.3.23.1	Definition	40

4.3.23.2	Attributes	41

4.3.23.3	Attribute constraints	41

4.3.23.4	Notifications	41

4.3.24	EPSService	41

4.3.24.1	Definition	41

4.3.24.2	Attributes	42

4.3.24.3	Attribute constraints	43

4.3.24.4	Notifications	43

4.3.25	EPSSubscribedService	43

4.3.25.1	Definition	43

4.3.25.2	Attributes	44

4.3.25.3	Attribute constraints	44

4.3.25.4	Notifications	45

4.3.26	EPSServiceProfile	45

4.3.26.1	Definition	45

4.3.26.2	Attributes	45

4.3.26.3	Attribute constraints	45

4.3.26.4	Notifications	46

4.3.27	PSCommonServiceProfile	46

4.3.27.1	Definition	46

4.3.27.2	Attributes	46

4.3.27.3	Attribute constraints	46

4.3.27.4	Notifications	46

4.3.28	IWANParameters	47

4.3.28.1	Definition	47

4.3.28.2	Attributes	47

4.3.28.3	Attribute constraints	47

4.3.28.4	Notifications	47

4.3.29	OptHandover3GPP2Parameters	48

4.3.29.1	Definition	48

4.3.29.2	Attributes	48

4.3.29.3	Attribute constraints	48

4.3.29.4	Notifications	48

4.3.30	Non3GPPAccessParameters	48

4.3.30.1	Definition	48

4.3.30.2	Attributes	48

4.3.30.3	Attribute constraints	48

4.3.30.4	Notifications	48

4.3.31	Common3GPPAccessServiceProfile	49

4.3.31.1	Definition	49

4.3.31.2	Attributes	49

4.3.31.3	Attribute constraints	49

4.3.31.4	Notifications	49

4.4	Attribute definitions	49

4.4.1	Attribute properties	49

4.4.2	Constraints	71

4.5	Common notifications	71

Annex A (informative):	Change history	72

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The interface Itf-N, defined in 3GPP TS 32.102 [2], is built up by a number of Integration Reference Points (IRPs) and a related Name Convention, which realise the functional capabilities over this interface. The basic structure of the IRPs is defined in 3GPP TS 32.101 [1] and 3GPP TS 32.102 [2].

The present document is part of a TS-family covering the 3rd Generation Partnership Project: Technical Specification Group Services and System Aspects; Telecommunication management, as identified below:

TS 32.140:	Subscription Management (SuM) requirements.

TS 32.141:	Subscription Management (SuM) architecture.

TS 28.751: 	Subscription Management (SuM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements.

TS 28.752: 	Subscription Management (SuM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS).

TS 28.753:	Subscription Management (SuM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Sets (SS) definitions.

# 1	Scope

The present document defines the Network Resource Model (NRM) for the SuM IRP which can be communicated between an IRPAgent and IRPManagers for subscription management purposes, including in the context of converged networks.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.140: "Telecommunication management; Subscription Management (SuM) Requirements".

[4]	3GPP TS 32.141: "Telecommunication management; Subscription Management (SuM) Architecture."

[5]	3GPP TS 28.751: "Telecommunication management; Subscription Management (SuM) Network Resource Model (NRM) Integration Reference Point (IRP): Requirements".

[6]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[7]	3GPP TS 23.008: "Organization of subscriber data".

[8]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx interfaces; Signalling flows and message contents".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply. For terms and definitions not found here, please refer to 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.140 [3] and TS 32.141 [4].

Subscription Profile: see 3GPP TS 32.140 [3].

Subscription Profile Component: see 3GPP TS 32.140 [3].

Subscription Profile Component Definition: specifies a Subscription Profile Component.

Subscriber Profile: see 3GPP TS 32.140 [3].

User: see 3GPP TS 32.140 [3].

## 3.2	Abbreviations

For the purposes of the present document the following and the abbreviations in 32.101 [1], 32.102 [2], 32.140 [3] and 32.141 [4] apply.

EM	Element Manager

IOC	Information Object Class

IRP	Integration Reference Point

MOC	Managed Object Class

NE	Network Element

SuM	Subscription Management

# 4	Model

## 4.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 28.622 [6], information object class, ManagedFunction | ManagedFunction |
| 28.622 [6], information object class, SubNetwork | SubNetwork |
| 28.622 [6], information object class, Top | Top |

## 4.2	Class diagram

### 4.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this IRP. This clause provides an overview of the relationships between relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

Figure 1a: SuM NRM naming diagram 1

Figure 1b: SuM NRM Containment and Association diagram 1

Figure 1c: SuM NRM Containment and Association diagram 2

Figure 1d: SuM Supplementary Service NRM Aggregation diagram 1

Figure 1e: SuM Supplementary Service NRM Aggregation diagram 2

Figure 1f: SuM Supplementary Service NRM Aggregation diagram 3

Figure 1g: SuM IMS Service Containment and Association diagram 1

Figure 1h: Service Profile NRM Association diagram

### 4.2.2	Inheritance

This subclause depicts the inheritance relationships.

- Figure 2a: SuM NRM Inheritance Hierarchy 1

- Figure 2b: SuM NRM Inheritance Hierarchy 2

- Figure 2c: SuM NRM Inheritance Hierarchy 3

- Figure 2d: SuM NRM Inheritance Hierarchy 4

- Figure 2e: SuM NRM Inheritance Hierarchy 5

Figure 2f: SuM NRM Inheritance Hierarchy 8

Figure 2g: SuM NRM Inheritance Hierarchy 9

## 4.3	Class definitions

### 4.3.1	SuMService

#### 4.3.1.1	Definition

This is an abstract IOC provided for sub-classing only. It provides attribute(s) that are common to Services IOCs.

#### 4.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| theContract | M | M | O | - | - |

#### 4.3.1.3	Attribute constraints

None.

#### 4.3.1.4	Notifications

None.

### 4.3.2	CSService

#### 4.3.2.1	Definition

This class represents a CS service (see 3GPP TS 23.008 [7]) provided by a service provider

This class contains the valid parameters that define the CS service from subscription point of view.

#### 4.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkAccessMode | M | M | M | - | - |
| basicMSISDNIndicator | O | M | M | - | - |
| mSISDNAlertIndicator | O | M | M | - | - |
| subscriptionRestriction | O | M | M | - | - |
| rSZIlists | O | M | M | - | - |
| lSAIdentity | O | M | M | - | - |
| lSAPriority | O | M | M | - | - |
| lSAPreferentialAccessIndicator | O | M | M | - | - |
| lSAActiveModeSupportIndicator | O | M | M | - | - |
| lSAOnlyAccessIndicator | O | M | M | - | - |
| lSAActiveModeIndicator | O | M | M | - | - |
| vPLMNIdentifier | O | M | M | - | - |
| accessRestrictionData | O | M | M | - | - |
| privacyExceptionList | O | M | M | - | - |
| gMLCNumbers | O | M | M | - | - |
| mOLRList | O | M | M | - | - |
| serviceTypes | O | M | M | - | - |
| mobileStationCategory | O | M | M | - | - |
| lMUIdentifier | O | M | M | - | - |
| subscriberStatus | O | M | M | - | - |
| gsmSCFaddresslistForODB | M | M | M | - | - |
| provisionOfBearerService | M | M | M | - | - |
| provisionOfTeleservice | M | M | M | - | - |
| bCAllocation | O | M | M | - | - |
| barringOfOutgoingCalls | O | M | M | - | - |
| barringOfIncomingCalls | O | M | M | - | - |
| barringOfRoaming | O | M | M | - | - |
| barringOfPremiumRateCalls | O | M | M | - | - |
| barringOfSupplementaryService Management | O | M | M | - | - |
| barringOfRegistrationOfCall Forwarding | O | M | M | - | - |
| barringOfInvocationOfCall Transfer | O | M | M | - | - |
| operatorDeterminedBarring PLMNspecificData | O | M | M | - | - |
| vGCSGroupMembershipList | O | M | M | - | - |
| vBSGroupMembershipList | O | M | M | - | - |
| broadcastCallInitiation AllowedList | O | M | M | - | - |
| originatingCAMELSubscription InformationOCSI | O | M | M | - | - |
| terminatingCAMELSubscription InformationTCSI | O | M | M | - | - |
| vMSCTerminatingCAMEL SubscriptionInformationVTCSI | O | M | M | - | - |
| locationInformation SubscriberStateInformation | O | M | M | - | - |
| uSSDCAMELSubscription InformationUCSI | O | M | M | - | - |
| sSInvocationNotificationSSCSI | O | M | M | - | - |
| translationInformation FlagTIFCSI | O | M | M | - | - |
| dialledServiceCAMEL SubscriptionInformationDCSI | O | M | M | - | - |
| uSSDGeneralCAMELService InformationUGCSI | O | M | M | - | - |
| shortMessageServiceCAMEL SubscriptionInformationMOSMSCSI | O | M | M | - | - |
| shortMessageServiceCAMEL SubscriptionInformationMTSMSCSI | O | M | M | - | - |
| mTSMSCSIVLRNegotiatedCAMEL CapabilityHandling | O | M | M | - | - |
| gsmSCFaddressforCSI | O | M | M | - | - |
| iSTAlertTimer | O | M | M | - | - |
| cSAllocationRetentionpriority | O | M | M | - | - |
| iCSIndicator | O | M | M | - | - |
| Attribute related to role |  |  |  |  |  |
| theCSSupplServiceForCS | O | M | O | - | - |

#### 4.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.2.4	Notifications

None.

### 4.3.3	GPRSService

#### 4.3.3.1	Definition

This class represents a GPRS service (see 3GPP TS 23.008 [7]) provided by a service provider.

This class contains the valid parameters that define the GPRS service from subscription point of view.

#### 4.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkAccessMode | M | M | M | - | - |
| basicMSISDNIndicator | O | M | M | - | - |
| mSISDNAlertIndicator | O | M | M | - | - |
| rSZIlists | O | M | M | - | - |
| lSAIdentity | O | M | M | - | - |
| lSAPriority | O | M | M | - | - |
| lSAPreferentialAccessIndicator | O | M | M | - | - |
| lSAActiveModeSupportIndicator | O | M | M | - | - |
| lSAOnlyAccessIndicator | O | M | M | - | - |
| lSAActiveModeIndicator | O | M | M | - | - |
| vPLMNIdentifier | O | M | M | - | - |
| accessRestrictionData | O | M | M | - | - |
| privacyExceptionList | O | M | M | - | - |
| gMLCNumbers | O | M | M | - | - |
| mOLRList | O | M | M | - | - |
| serviceTypes | O | M | M | - | - |
| gGSNNumber | M | M | M | - | - |
| provisionofTeleservice | O | M | M | - | - |
| transferofSMoption | M | M | M | - | - |
| subscriberStatus | M | M | M | - | - |
| barringofPacketOrientedServices | O | M | M | - | - |
| pDPType | O | M | M | - | - |
| pDPAddress | O | M | M | - | - |
| accessPointName | O | M | M | - | - |
| vPLMNAddressAllowed | O | M | M | - | - |
| qualityofServiceSubscribed | O | M | M | - | - |
| pDPContextChargingCharacteristics | O | M | M | - | - |
| gPRSCAMELSubscriptionInformationGPRSCSI | O | M | M | - | - |
| mOShortMessageServiceCAMELSubscriptionInformationMOSMSCSI | O | M | M | - | - |
| mTShortMessageServiceCAMELSubscriptionInformationMTSMSCSI | O | M | M | - | - |
| mOSMSCSISGSNNegotiatedCAMELCapabilityHandling | O | M | M | - | - |
| mTSMSCSISGSNNegotiatedCAMELCapabilityHandling | O | M | M | - | - |
| mobilityManagementforGPRSeventnotificationMGCSI | O | M | M | - | - |
| mGCSINegotiatedCAMELCapabilityHandling | O | M | M | - | - |
| gsmSCFaddressforCSI | O | M | M | - | - |
| subscribedChargingCharacteristics | O | M | M | - | - |
| closedSubscriberGroupInformation | O | M | M | - | - |
| gSMSCFaddresslistForODB | O | M | M | - | - |
| barringOfOutgoingCalls | O | M | M | - | - |
| barringOfRoaming | O | M | M | - | - |
| operatorDeterminedBarringPLMNspecificData | O | M | M | - | - |
| sTNSR | O | M | M | - | - |
| iCSIndicator | O | M | M | - | - |
| listofauthorizedvisitednetworkidentifiers | O | M | M | - | - |
| wLANUERemoteIPaddress | O | M | M | - | - |
| staticWLANUERemoteIPAddressList | O | M | M | - | - |
| w-APNBarringTypeList | O | M | M | - | - |
| w-APNChargingDataList | O | M | M | - | - |
| accessDependenceFlag | O | M | M | - | - |
| chargingCharacteristics | O | M | M | - | - |
| primaryOCSChargingFunctionName | O | M | M | - | - |
| secondaryOCSChargingFunctionName | O | M | M | - | - |
| primaryChargingCollectionFunctionName | O | M | M | - | - |
| secondaryChargingCollectionFunctionName | O | M | M | - | - |
| w-APNIdentifierList | O | M | M | - | - |
| maxNumberAccessesList | O | M | M | - | - |
| maxSubscribedBandwidth | O | M | M | - | - |
| subscribed3GPPWLANQoSProfile | O | M | M | - | - |
| Attribute related to role |  |  |  |  |  |
| theGPRSSupplServiceForGPRS | O | M | O | - | - |

#### 4.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.3.4	Notifications

None.

### 4.3.4	IMSService

#### 4.3.4.1	Definition

This class represents a IMS service (see 3GPP TS 23.008 [7]) provided by a service provider

This class contains the valid parameters that define the IMS service from subscription point of view.

#### 4.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| listofauthorizedvisitednetworkidentifiers | M | M | M | - | - |
| registrationStatus | M | M | M | - | - |
| sCSCFName | O | M | - | - | - |
| diameterClientAddressofSCSCF | O | M | - | - | - |
| subscribedMediaProfileIdentifier | O | M | M | - | - |
| initialFilterCriteriaList | O | M | M | - | - |
| applicationServerInformation | O | M | M | - | - |
| serviceIndicationList | O | M | - | - | - |
| primaryEventChargingFunctionName | O | M | M | - | - |
| secondaryEventChargingFunctionName | O | M | M | - | - |
| primaryChargingCollectionFunctionName | M | M | M | - | - |
| secondaryChargingCollectionFunctionName | O | M | M | - | - |
| gsmSCFaddressforIMCSI | O | M | M | - | - |
| oIMCSI | O | M | M | - | - |
| vTIMCSI | O | M | M | - | - |
| dIMCS | O | M | M | - | - |
| gAAServiceType | O | M | M | - | - |
| gAAServiceIdentifier | O | M | M | - | - |
| gBAUserSecuritySettings | O | M | M | - | - |
| userSecuritySetting | O | M | M | - | - |
| userPublicIdentity | O | M | M | - | - |
| gAAAuthorizationflag | O | M | M | - | - |
| keyLifetime | O | M | M | - | - |
| uICCSecurityType | O | M | M | - | - |
| nAFGroupIdentity | O | M | M | - | - |
| Attribute related to role |  |  |  |  |  |
| theServerCapabilities | O | M | - | - | - |

#### 4.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.4.4	Notifications

None.

### 4.3.5	SuMSubscribedService

#### 4.3.5.1	Definition

This is an abstract IOC provided for sub-classing only. It provides attribute(s) that are common to Subscribed Services IOCs.

#### 4.3.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| theService | M | M | O | - | - |
| theOffer | M | M | O | - | - |

#### 4.3.5.3	Attribute constraints

None.

#### 4.3.5.4	Notifications

None.

### 4.3.6	CSSubscribedService

#### 4.3.6.1	Definition

This class represents subscription to a CS service (see 3GPP TS 23.008 [7]) for a subscriber.  The entire set of CS subscribed services owned by a subscriber constitutes the subscriber’s Subscribed Services Profile for CS services.

This class contains the agreed (between the Subscriber and the Service Provider) parameter settings for a CS service that a Subscriber subscribes to.

#### 4.3.6.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkAccessMode | M | M | M | - | - |
| basicMSISDNIndicator | O | M | M | - | - |
| mSISDNAlertIndicator | O | M | M | - | - |
| subscriptionRestriction | O | M | M | - | - |
| rSZIlists | O | M | M | - | - |
| lSAIdentity | O | M | M | - | - |
| lSAPriority | O | M | M | - | - |
| lSAPreferentialAccessIndicator | O | M | M | - | - |
| lSAActiveModeSupportIndicator | O | M | M | - | - |
| lSAOnlyAccessIndicator | O | M | M | - | - |
| lSAActiveModeIndicator | O | M | M | - | - |
| vPLMNIdentifier | O | M | M | - | - |
| accessRestrictionData | O | M | M | - | - |
| privacyExceptionList | O | M | M | - | - |
| gMLCNumbers | O | M | M | - | - |
| mOLRList | O | M | M | - | - |
| serviceTypes | O | M | M | - | - |
| mobileStationCategory | O | M | M | - | - |
| lMUIdentifier | O | M | M | - | - |
| subscriberStatus | O | M | M | - | - |
| gsmSCFaddresslistForODB | M | M | M | - | - |
| provisionOfBearerService | M | M | M | - | - |
| provisionOfTeleservice | M | M | M | - | - |
| bCAllocation | O | M | M | - | - |
| barringOfOutgoingCalls | O | M | M | - | - |
| barringOfIncomingCalls | O | M | M | - | - |
| barringOfRoaming | O | M | M | - | - |
| barringOfPremiumRateCalls | O | M | M | - | - |
| barringOfSupplementaryService Management | O | M | M | - | - |
| barringOfRegistrationOfCall Forwarding | O | M | M | - | - |
| barringOfInvocationOfCall Transfer | O | M | M | - | - |
| operatorDeterminedBarring PLMNspecificData | O | M | M | - | - |
| vGCSGroupMembershipList | O | M | M | - | - |
| vBSGroupMembershipList | O | M | M | - | - |
| broadcastCallInitiation AllowedList | O | M | M | - | - |
| originatingCAMELSubscription InformationOCSI | O | M | M | - | - |
| terminatingCAMELSubscription InformationTCSI | O | M | M | - | - |
| vMSCTerminatingCAMEL Subscription InformationVTCSI | O | M | M | - | - |
| locationInformation/ SubscriberStateInformation | O | M | M | - | - |
| uSSDCAMELSubscription InformationUCSI | O | M | M | - | - |
| sSInvocationNotification SSCSI | O | M | M | - | - |
| translationInformationFlag TIFCSI | O | M | M | - | - |
| dialledServiceCAMEL SubscriptionInformationDCSI | O | M | M | - | - |
| uSSDGeneralCAMELService InformationUGCSI | O | M | M | - | - |
| shortMessageServiceCAMEL SubscriptionInformationMOSMSCSI | O | M | M | - | - |
| shortMessageServiceCAMEL SubscriptionInformationMTSMSCSI | O | M | M | - | - |
| mTSMSCSIVLRNegotiatedCAMEL CapabilityHandling | O | M | M | - | - |
| gsmSCFaddressforCSI | O | M | M | - | - |
| iSTAlertTimer | O | M | M | - | - |
| cSAllocationRetentionpriority | O | M | M | - | - |
| iCSIndicator | O | M | M | - | - |
| Attribute related to role |  |  |  |  |  |
| theCSSupplServiceForCSSS | O | M | O | - | - |

#### 4.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.6.4	Notifications

None.

### 4.3.7	GPRSSubscribedService

#### 4.3.7.1	Definition

This class represents subscription to a GPRS service (see 3GPP TS 23.008 [7]) for a subscriber.  The entire set of GPRS subscribed services owned by a subscriber constitutes the subscriber’s Subscribed Services Profile for GPRS services.

#### 4.3.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkAccessMode | M | M | M | - | - |
| basicMSISDNIndicator | O | M | M | - | - |
| mSISDNAlertIndicator | O | M | M | - | - |
| rSZIlists | O | M | M | - | - |
| lSAIdentity | O | M | M | - | - |
| lSAPriority | O | M | M | - | - |
| lSAPreferentialAccessIndicator | O | M | M | - | - |
| lSAActiveModeSupportIndicator | O | M | M | - | - |
| lSAOnlyAccessIndicator | O | M | M | - | - |
| lSAActiveModeIndicator | O | M | M | - | - |
| vPLMNIdentifier | O | M | M | - | - |
| accessRestrictionData | O | M | M | - | - |
| privacyExceptionList | O | M | M | - | - |
| gMLCNumbers | O | M | M | - | - |
| mOLRList | O | M | M | - | - |
| serviceTypes | O | M | M | - | - |
| gGSNNumber | M | M | M | - | - |
| provisionofTeleservice | O | M | M | - | - |
| transferofSMoption | M | M | M | - | - |
| subscriberStatus | M | M | M | - | - |
| barringofPacketOrientedServices | O | M | M | - | - |
| pDPType | O | M | M | - | - |
| pDPAddress | O | M | M | - | - |
| accessPointName | O | M | M | - | - |
| vPLMNAddressAllowed | O | M | M | - | - |
| qualityofServiceSubscribed | O | M | M | - | - |
| pDPContextCharging Characteristics | O | M | M | - | - |
| gPRSCAMELSubscription InformationGPRSCSI | O | M | M | - | - |
| mOShortMessageService CAMELSubscription InformationMOSMSCSI | O | M | M | - | - |
| mTShortMessageService CAMELSubscription InformationMTSMSCSI | O | M | M | - | - |
| mOSMSCSISGSNNegotiated CAMELCapabilityHandling | O | M | M | - | - |
| mTSMSCSISGSNNegotiated CAMELCapabilityHandling | O | M | M | - | - |
| mobilityManagementforGPRSevent notificationMGCSI | O | M | M | - | - |
| mGCSINegotiatedCAMEL CapabilityHandling | O | M | M | - | - |
| gsmSCFaddressforCSI | O | M | M | - | - |
| subscribedCharging Characteristics | O | M | M | - | - |
| closedSubscriberGroup Information | O | M | M | - | - |
| gSMSCFaddresslistForODB | O | M | M | - | - |
| barringOfOutgoingCalls | O | M | M | - | - |
| barringOfRoaming | O | M | M | - | - |
| operatorDeterminedBarring PLMNspecificData | O | M | M | - | - |
| sTNSR | O | M | M | - | - |
| iCSIndicator | O | M | M | - | - |
| listofauthorizedvisitednetwork identifiers | O | M | M | - | - |
| wLANUERemoteIPaddress | O | M | M | - | - |
| staticWLANUERemoteIP AddressList | O | M | M | - | - |
| w-APNBarringTypeList | O | M | M | - | - |
| w-APNChargingDataList | O | M | M | - | - |
| accessDependenceFlag | O | M | M | - | - |
| chargingCharacteristics | O | M | M | - | - |
| primaryOCSCharging FunctionName | O | M | M | - | - |
| secondaryOCSCharging FunctionName | O | M | M | - | - |
| primaryChargingCollection FunctionName | O | M | M | - | - |
| secondaryChargingCollection FunctionName | O | M | M | - | - |
| w-APNIdentifierList | O | M | M | - | - |
| maxNumberAccessesList | O | M | M | - | - |
| maxSubscribedBandwidth | O | M | M | - | - |
| subscribed3GPPWLANQoSProfile | O | M | M | - | - |
| Attribute related to role |  |  |  |  |  |
| theGPRSSupplServiceforGPRSSS | O | M | O | - | - |

#### 4.3.7.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.7.4	Notifications

None.

### 4.3.8	IMSSubscribedService

#### 4.3.8.1	Definition

This class represents subscription to an IMS (see 3GPP TS 23.008 [7]) service for a subscriber.  The entire set of IMS subscribed services owned by a subscriber constitutes the subscriber’s Subscribed Services profile for IMS services.

This class contains the agreed (between the Subscriber and the Service Provider) parameter settings for an IMS service that a Subscriber subscribes to.

#### 4.3.8.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| listofauthorizedvisitednetworkidentifiers | M | M | M | - | - |
| registrationStatus | M | M | M | - | - |
| sCSCFName | O | M | - | - | - |
| diameterClientAddressofSCSCF | O | M | - | - | - |
| subscribedMediaProfileIdentifier | O | M | M | - | - |
| initialFilterCriteriaList | O | M | M | - | - |
| applicationServerInformation | O | M | M | - | - |
| serviceIndicationList | O | M | - | - | - |
| primaryEventChargingFunctionName | O | M | M | - | - |
| secondaryEventChargingFunctionName | O | M | M | - | - |
| primaryChargingCollectionFunctionName | M | M | M | - | - |
| secondaryChargingCollectionFunctionName | O | M | M | - | - |
| gsmSCFaddressforIMCSI | O | M | M | - | - |
| oIMCSI | O | M | M | - | - |
| vTIMCSI | O | M | M | - | - |
| dIMCS | O | M | M | - | - |
| gAAServiceType | O | M | M | - | - |
| gAAServiceIdentifier | O | M | M | - | - |
| gBAUserSecuritySettings | O | M | M | - | - |
| userSecuritySetting | O | M | M | - | - |
| userPublicIdentity | O | M | M | - | - |
| gAAAuthorizationflag | O | M | M | - | - |
| keyLifetime | O | M | M | - | - |
| uICCSecurityType | O | M | M | - | - |
| nAFGroupIdentity | O | M | M | - | - |
| Attribute related to role |  |  |  |  |  |
| theServerCapabilities | O | M | - | - | - |

#### 4.3.8.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.8.4	Notifications

None.

### 4.3.9	SuMSubscriberProfile

#### 4.3.9.1	Definition

This class represents the Subscriber Profile.

This class also contains the reference to its aggregated Subscribed Service as well as the users for a subscriber.

#### 4.3.9.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| suMSubscriberProfileInfo | M | M | O | - | - |
| Attribute related to role |  |  |  |  |  |
| theUser | M | M | O | - | - |
| theSubscribedService | M | M | O | - | - |
| theMobileUserData | M | M | O | - | - |

#### 4.3.9.3	Attribute constraints

None.

#### 4.3.9.4	Notifications

None.

### 4.3.10	SuMServiceProfile

#### 4.3.10.1	Definition

This is an abstract IOC provided for sub-classing only. It provides attribute(s) that are common to Service Profiles IOCs.

#### 4.3.10.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| theSpecification | M | M | O | - | - |

#### 4.3.10.3	Attribute constraints

None.

#### 4.3.10.4	Notifications

None.

### 4.3.11	CSServiceProfile

#### 4.3.11.1	Definition

This class represents for a subscribed CS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user.

#### 4.3.11.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| basicMSISDNIndicator | O | M | M | - | - |
| mSISDNAlertIndicator | O | M | M | - | - |
| vLRNumber | M | M | - | - | - |
| mSCNumber | M | M | - | - | - |
| subscriptionRestriction | O | M | M | - | - |
| mSCAreaRestrictedFlag | M | M | - | - | - |
| oDBinducedBarringData | O | M | - | - | - |
| roamingRestrictionDueTo UnsupportedFeature | M | M | - | - | - |
| lSAIdentity | O | M | M | - | - |
| lSAPriority | O | M | M | - | - |
| lSAPreferentialAccessIndicator | O | M | M | - | - |
| lSAActiveModeSupportIndicator | O | M | M | - | - |
| lSAOnlyAccessIndicator | O | M | M | - | - |
| lSAActiveModeIndicator | O | M | M | - | - |
| vPLMNIdentifier | O | M | M | - | - |
| ageIndicator | O | M | - | - | - |
| lMSI | M | M | - | - | - |
| mobileStationCategory | M | M | M | - | - |
| lMUIdentifier | O | M | M | - | - |
| mNRRMSC | M | M | - | - | - |
| notificationToCSEFlagForODB | O | M | - | - | - |
| gsmSCFaddresslistForODB | O | M | M | - | - |
| messagesWaitingData | O | M | - | - | - |
| mobileStationNotReachableFlag | O | M | - | - | - |
| memoryCapacityExceededFlag | M | M | - | - | - |
| traceReference | O | M | - | - | - |
| traceType | O | M | - | - | - |
| operationsSystemsIdentity | O | M | - | - | - |
| hLRTraceType | O | M | - | - | - |
| mAPErrorOnTrace | O | M | - | - | - |
| traceActivatedInVLR | O | M | - | - | - |
| provisionOfBearerService | M | M | M | - | - |
| provisionOfTeleservice | M | M | M | - | - |
| bCAllocation | O | M | M | - | - |
| checkSSIndicator | M | M | - | - | - |
| mSPurgedForNonGPRSFlag | M | M | - | - | - |
| barringOfIncomingCalls | O | M | M | - | - |
| barringOfPremiumRateCalls | O | M | M | - | - |
| barringOfSupplementaryService Management | O | M | M | - | - |
| barringOfRegistrationOfCall Forwarding | O | M | M | - | - |
| barringOfInvocationOfCallTransfer | O | M | M | - | - |
| vGCSGroupMembershipList | O | M | M | - | - |
| vBSGroupMembershipList | O | M | M | - | - |
| broadcastCallInitiationAllowedList | O | M | M | - | - |
| originatingCAMELSubscription InformationOCSI | O | M | M | - | - |
| terminatingCAMELSubscription InformationTCSI | O | M | M | - | - |
| vMSCTerminatingCAMEL SubscriptionInformationVTCSI | O | M | M | - | - |
| locationInformationSubscriber StateInformation | O | M | M | - | - |
| uSSDCAMELSubscription InformationUCSI | O | M | M | - | - |
| sSInvocationNotificationSSCSI | O | M | M | - | - |
| translationInformationFlagTIFCSI | O | M | M | - | - |
| dialledServiceCAMELSubscription InformationDCSI | O | M | M | - | - |
| uSSDGeneralCAMELService InformationUGCSI | O | M | M | - | - |
| oCSINegotiatedCAMEL CapabilityHandling | O | M | - | - | - |
| sSCSINegotiatedCAMEL CapabilityHandling | O | M | - | - | - |
| vTCSINegotiatedCAMEL CapabilityHandling | O | M | - | - | - |
| shortMessageServiceCAMEL SubscriptionInformationMOSMSCSI | O | M | M | - | - |
| shortMessageServiceCAMEL SubscriptionInformationMTSMSCSI | O | M | M | - | - |
| mOSMSCSIVLRNegotiatedCAMEL CapabilityHandling | O | M | - | - | - |
| mTSMSCSIVLRNegotiatedCAMEL CapabilityHandling | O | M | M | - | - |
| mCSINegotiatedCAMEL CapabilityHandling | O | M | - | - | - |
| vLRSupportedCAMELPhases | O | M | - | - | - |
| gsmSCFaddressforCSI | O | M | M | - | - |
| vLROfferedCAMEL4CSIs | O | M | - | - | - |
| iSTAlertTimer | O | M | M | - | - |
| cSAllocationRetentionpriority | O | M | M | - | - |
| Attribute related to role |  |  |  |  |  |
| theCSSupplServiceForCSSP | O | M | O | - | - |

#### 4.3.11.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.11.4	Notifications

None.

### 4.3.12	GPRSSServiceProfile

#### 4.3.12.1	Definition

This class represents for a subscribed GPRS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user.

#### 4.3.12.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| basicMSISDNIndicator | O | M | M | - | - |
| mSISDNAlertIndicator | O | M | M | - | - |
| vLRNumber | M | M | - | - | - |
| lSAIdentity | O | M | M | - | - |
| lSAPriority | O | M | M | - | - |
| lSAPreferentialAccessIndicator | O | M | M | - | - |
| lSAActiveModeSupportIndicator | O | M | M | - | - |
| lSAOnlyAccessIndicator | O | M | M | - | - |
| lSAActiveModeIndicator | O | M | M | - | - |
| vPLMNIdentifier | O | M | M | - | - |
| ageIndicator | O | M | - | - | - |
| sGSNNumber | M | M | - | - | - |
| gGSNNumber | M | M | M | - | - |
| mNRG | M | M | - | - | - |
| provisionofTeleservice | O | M | M | - | - |
| transferofSMoption | M | M | M | - | - |
| mSpurgedforGPRSflag | M | M | - | - | - |
| pDPType | O | M | M | - | - |
| pDPAddress | O | M | M | - | - |
| gGSNlist | M | M | - | - | - |
| qualityofServiceSubscribed | O | M | M | - | - |
| pDPContextIdentifier | O | M | - | - | - |
| pDPContextChargingCharacteristics | O | M | M | - | - |
| gPRSCAMELSubscriptionInformation GPRSCSI | O | M | M | - | - |
| mOShortMessageServiceCAMEL SubscriptionInformationMOSMSCSI | O | M | M | - | - |
| mTShortMessageServiceCAMEL SubscriptionInformationMTSMSCSI | O | M | M | - | - |
| mOSMSCSISGSNNegotiatedCAMEL CapabilityHandling | O | M | M | - | - |
| mTSMSCSISGSNNegotiatedCAMEL CapabilityHandling | O | M | M | - | - |
| mobilityManagementforGPRS eventnotificationMGCSI | O | M | M | - | - |
| mGCSINegotiatedCAMEL CapabilityHandling | O | M | M | - | - |
| gPRSCSINegotiatedCAMEL CapabilityHandling | O | M | - | - | - |
| sGSNSupportedCAMELPhases | O | M | - | - | - |
| sGSNOfferedCAMEL4CSIs | O | M | - | - | - |
| gsmSCFaddressforCSI | O | M | M | - | - |
| gSMSCFaddresslistForODB | O | M | M | - | - |
| iPSMGWNumber | O | M | - | - | - |
| mNRR-SGSN | O | M | - | - | - |
| notificationToCSEFlagForODB | O | M | - | - | - |
| Attribute related to role |  |  |  |  |  |
| theGPRSSupplServiceForGPRSSP | O | M | O | - | - |

#### 4.3.12.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.12.4	Notifications

None.

### 4.3.13	IMSServiceProfile

#### 4.3.13.1	Definition

This class represents for a subscribed IMS (see 3GPP TS 23.008 [7]) service the service preferences and settings chosen for a user.

This class contains the reference to its aggregated IMS Public Identifications.

#### 4.3.13.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| listofauthorizedvisitednetworkidentifiers | M | M | M | - | - |
| registrationStatus | M | M | M | - | - |
| sCSCFName | O | M | - | - | - |
| diameterClientAddressofSCSCF | O | M | - | - | - |
| subscribedMediaProfileIdentifier | O | M | M | - | - |
| initialFilterCriteriaList | O | M | M | - | - |
| applicationServerInformation | O | M | M | - | - |
| serviceIndicationList | O | M | - | - | - |
| primaryEventChargingFunctionName | O | M | M | - | - |
| secondaryEventChargingFunctionName | O | M | M | - | - |
| primaryChargingCollectionFunctionName | M | M | M | - | - |
| secondaryChargingCollectionFunctionName | O | M | M | - | - |
| gsmSCFaddressforIMCSI | O | M | M | - | - |
| iMSSFaddressforIMCSI | O | M | - | - | - |
| oIMCSI | O | M | M | - | - |
| vTIMCSI | O | M | M | - | - |
| dIMCS | O | M | M | - | - |
| gAAServiceType | O | M | M | - | - |
| gAAServiceIdentifier | O | M | M | - | - |
| gBAUserSecuritySettings | O | M | M | - | - |
| userSecuritySetting | O | M | M | - | - |
| userPublicIdentity | O | M | M | - | - |
| gAAAuthorizationflag | O | M | M | - | - |
| keyLifetime | O | M | M | - | - |
| uICCSecurityType | O | M | M | - | - |
| nAFGroupIdentity | O | M | M | - | - |
| thePublicId | M | M | O | - | - |
| Attribute related to role |  |  |  |  |  |
| theServerCapabilities | O | M | - | - | - |

#### 4.3.13.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.13.4	Notifications

None.

### 4.3.14	SubscriptionFunction

#### 4.3.14.1	Definition

This class represents subscription functionality.

All managed elements related to the subscription of services have this functionality.

This class inherits from ManagedFunction (defined in 32.622 [6]).

This class contains the reference to its aggregated Users.

This class contains the reference to its aggregated Subscriber Profiles.

#### 4.3.14.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| theUser | M | M | O | - | - |
| theSubscriberProfile | M | M | O | - | - |

#### 4.3.14.3	Attribute constraints

None.

#### 4.3.14.4	Notifications

None.

### 4.3.15	SuMCSSupplService

#### 4.3.15.1	Definition

This class represents a Supplementary Service for a CS service (see 3GPP TS 23.008 [7]) provided by a service provider

This class contains the valid parameters that define the CS supplementary service from subscription point of view.

#### 4.3.15.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sS-Code | M | M | M | - | - |
| provisioningState | M | M | M | - | - |
| activationState | M | M | M | - | - |
| registrationState | O | M | M | - | - |
| overrideCategory | O | M | M | - | - |
| cLIRestrictionOption | O | M | M | - | - |
| forwarded-toNumber | O | M | M | - | - |
| forwarded-toSubaddress | O | M | M | - | - |
| noReplyConditionTimer | O | M | M | - | - |
| maximumEntitledPriority | O | M | M | - | - |
| defaultPriority | O | M | M | - | - |
| nbrUser | O | M | M | - | - |
| nbrSB | O | M | M | - | - |

#### 4.3.15.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.15.4	Notifications

None.

### 4.3.16	SuMGPRSSupplService

#### 4.3.16.1	Definition

This class represents a Supplementary Service for a GPRS service (see 3GPP TS 23.008 [7]) provided by a service provider

This class contains the valid parameters that define the GPRS supplementary service from subscription point of view.

#### 4.3.16.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sS-Code | M | M | M | - | - |
| provisioningState | M | M | M | - | - |
| activationState | M | M | M | - | - |

#### 4.3.16.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.16.4	Notifications

None.

### 4.3.17	SuMUser

#### 4.3.17.1	Definition

This class represents a User.

This class contains the valid parameters that define the User from subscription point of view.

#### 4.3.17.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| theMobileUserData | M | M | O | - | - |
| theSubscriber | O | M | O | - | - |
| theCredentials | M | M | O | - | - |
| theServiceProfile | M | M | O | - | - |
| theIdentity | O | M | O | - | - |

#### 4.3.17.3	Attribute constraints

None.

#### 4.3.17.4	Notifications

None.

### 4.3.18	SuMCredentials

#### 4.3.18.1	Definition

This class represents a User’s credentials.

This class contains the valid parameters that define the credentials from subscription point of view.

#### 4.3.18.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| iMSPrivateUserIdentity | M | M | M | - | - |
| Attribute related to role |  |  |  |  |  |
| thePublicId | M | M | O | - | - |
| theServiceProfile | M | M | O | - | - |

#### 4.3.18.3	Attribute constraints

| Name | Definition |
| --- | --- |
| theServiceProfile | - If theServiceProfile points to at least one instance of IMSServiceProfile, thePublicId of the same object instance of SuMCredentials may not point to any instance of IMSPublicIdentification. All instances of this relationship from the same object instance of SuMCredentials shall be to object instances of IMSServiceProfile that are i) name-contained by the same object instance of SuMUser as name-contains the SuMCredentials object instance, and ii) associated (through the relationship to SuMSubscribedService name-contained by SuMSubscriberProfile) to a single object instance of SuMSubscriberProfile. |
| thePublicId | - If thePublicId points to at least one instance of IMSPublicIdentification, theServiceProfile of the same object instance of SuMCredentials may not point to any instance of IMSServiceProfile. All instances of this relationship from the same object instance of SuMCredentials shall be to object instances of IMSPublicIdentification that are name-contained by object instances of IMSServiceProfile that are i) name-contained by the same object instance of SuMUser as name-contains the SuMCredentials object instance, and ii) associated (through the relationship to SuMSubscribedService name-contained by SuMSubscriberProfile) to a single object instance of SuMSubscriberProfile. |

#### 4.3.18.4	Notifications

None.

### 4.3.19	IMSPublicIdentification

#### 4.3.19.1	Definition

This class represents the IMS Public Identification (see 3GPP TS 23.008 [7] and 3GPP TS 29.228 [8]).

This class contains the valid parameters that define the IMS Public Identification from subscription point of view.

#### 4.3.19.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| publicUserIdentity | M | M | M | - | - |
| barringIndication | M | M | M | - | - |

#### 4.3.19.3	Attribute constraints

None.

#### 4.3.19.4	Notifications

None.

### 4.3.20		MobileUserData

#### 4.3.20.1	Definition

This class represents user data of a mobile User Equipment.

This class contains the valid parameters that define the user data of the mobile User Equipment from subscription point of view.

#### 4.3.20.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| theServiceProfile | M | M | O | - | - |
| iMSI | M | M | M | - | - |
| mSISDN | M | M | M | - | - |
| multinumberingMSISDNs | M | M | M | - | - |
| iMEISV | O | M | - | - | - |

#### 4.3.20.3	Attribute constraints

None.

#### 4.3.20.4	Notifications

None.

### 4.3.21	SuMServiceParameter

#### 4.3.21.1	Definition

This class represents a structured Service Parameter defined for a service and is an abstract class provided for sub-classing only.

This class shall be used as a base when having the requirement to provision common shared data referred to by specific SuMSubscribedService or SuMServiceProfile instances.

#### 4.3.21.2	Attributes

None.

#### 4.3.21.3	Attribute constraints

None.

#### 4.3.21.4	Notifications

None.

### 4.3.22	SuMIdentity

#### 4.3.22.1	Definition

This class represents a user related identity, regardless of any type service to be provisioned and is an abstract class provided for sub-classing only.

This class allows an identity to be used for multiple services.

#### 4.3.22.2	Attributes

None.

#### 4.3.22.3	Attribute constraints

None.

#### 4.3.22.4	Notifications

None.

### 4.3.23	ServerCapabilities

#### 4.3.23.1	Definition

This class is used to provision common shared ServerCapabilities data referred by specific SuMSubscribedService or SuMServiceProfile instances.

This class contains information to assist the I-CSCF in the selection of an S-CSCF for an IMS subscriber or a PSI user.

#### 4.3.23.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serverCapabilities | M | M | M | - | - |

#### 4.3.23.3	Attribute constraints

None.

#### 4.3.23.4	Notifications

None.

### 4.3.24	EPSService

#### 4.3.24.1	Definition

This class represents an EPS service (see 3GPP TS 23.008 [7]) provided by a service provider.  This class contains the valid parameters that define the EPS service from a subscription point of view.

#### 4.3.24.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| accessPointName | O | M | M | - | - |
| access Restriction Data | O | M | M | - | - |
| aPNAMBR | M | M | M | - | - |
| barringofPacketOrientedServices | O | M | M | - | - |
| contextIdentifier | M | M | M | - | - |
| operatorDeterminedBarringPLMNspecificData | O | M | M | - | - |
| pDNAddress | O | M | M | - | - |
| rSZILists | O | M | M | - | - |
| sTNSR | O | M | M | - | - |
| subscribedChargingCharacteristics | O | M | M | - | - |
| subscriberStatus | O | M | M | - | - |
| networkAccessMode | M | M | M | - | - |
| barringOfOutgoingCalls | O | M | M | - | - |
| barringOfRoaming | O | M | M | - | - |
| privacyExceptionList | O | M | M | - | - |
| gMLCNumbers | O | M | M | - | - |
| mOLRList | O | M | M | - | - |
| serviceTypes | O | M | M | - | - |
| iCSIndicator | O | M | M | - | - |
| aPNOIReplacement | O | M | M | - | - |
| subscribedUEAMBR | M | M | M | - | - |
| aPNConfigurationProfile | M | M | M | - | - |
| rFSP | O | M | M | - | - |
| vPLMNAddressAllowed-EPS | M | M | M | - | - |
| pDNGWIdentity | O | M | M | - | - |
| ePSPDNConnectionChargingCharacteristics | O | M | M | - | - |
| pDNGWAllocationType | M | M | M | - | - |
| aPNLevelAPNOIReplacement | O | M | M | - | - |
| closedSubscriberGroupInformation | O | M | M | - | - |
| listofauthorizedvisitednetworkidentifiers | O | M | M | - | - |
| staticWLANUERemoteIPAddressList | O | M | M | - | - |
| w-APNBarringTypeList | O | M | M | - | - |
| w-APNChargingDataList | O | M | M | - | - |
| wLANUERemoteIPAddress | O | M | M | - | - |
| accessDependenceFlag | O | M | M | - | - |
| chargingCharacteristics | O | M | M | - | - |
| primaryOCSChargingFunctionName | O | M | M | - | - |
| secondaryOCSChargingFunctionName | O | M | M | - | - |
| primaryChargingCollectionFunctionName | O | M | M | - | - |
| secondaryChargingCollectionFunctionName | O | M | M | - | - |
| w-APNIdentifierList | O | M | M | - | - |
| maxNumberAccessesList | O | M | M | - | - |
| maxSubscribedBandwidth | O | M | M | - | - |
| subscribed3GPPWLANQoSProfile | O | M | M | - | - |
| permanentUserIdentity | O | M | M | - | - |
| mIPSubscriberProfile | O | M | M | - | - |
| accessRestrictionData | O | M | M | - | - |

#### 4.3.24.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.24.4	Notifications

None.

### 4.3.25	EPSSubscribedService

#### 4.3.25.1	Definition

This class represents subscription to an EPS service (see 3GPP TS 23.008 [7]) for a subscriber.  The entire set of EPS subscribed services owned by a subscriber constitutes the subscriber’s Subscribed Services Profile for EPS services.

This class contains the agreed (between the Subscriber and the Service Provider) parameter settings for an EPS service that a Subscriber subscribes to.

#### 4.3.25.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| accessPointName | O | M | M | - | - |
| access Restriction Data | O | M | M | - | - |
| aPNAMBR | M | M | M | - | - |
| barringofPacketOrientedServices | O | M | M | - | - |
| contextIdentifier | M | M | M | - | - |
| operatorDeterminedBarringPLMNspecificData | O | M | M | - | - |
| pDNAddress | O | M | M | - | - |
| rSZILists | O | M | M | - | - |
| sTNSR | O | M | M | - | - |
| subscribedChargingCharacteristics | O | M | M | - | - |
| subscriberStatus | O | M | M | - | - |
| networkAccessMode | M | M | M | - | - |
| barringOfOutgoingCalls | O | M | M | - | - |
| barringOfRoaming | O | M | M | - | - |
| privacyExceptionList | O | M | M | - | - |
| gMLCNumbers | O | M | M | - | - |
| mOLRList | O | M | M | - | - |
| serviceTypes | O | M | M | - | - |
| iCSIndicator | O | M | M | - | - |
| aPNOIReplacement | O | M | M | - | - |
| subscribedUEAMBR | M | M | M | - | - |
| aPNConfigurationProfile | M | M | M | - | - |
| rFSP | O | M | M | - | - |
| vPLMNAddressAllowed-EPS | M | M | M | - | - |
| pDNGWIdentity | O | M | M | - | - |
| ePSPDNConnectionChargingCharacteristics | O | M | M | - | - |
| pDNGWAllocationType | M | M | M | - | - |
| aPNLevelAPNOIReplacement | O | M | M | - | - |
| closedSubscriberGroupInformation | O | M | M | - | - |
| Listofauthorizedvisitednetworkidentifiers | O | M | M | - | - |
| wLANUERemoteIPaddress | O | M | M | - | - |
| staticWLANUERemoteIPAddressList | O | M | M | - | - |
| w-APNBarringTypeList | O | M | M | - | - |
| w-APNChargingDataList | O | M | M | - | - |
| accessDependenceFlag | O | M | M | - | - |
| chargingCharacteristics | O | M | M | - | - |
| primaryOCSChargingFunctionName | O | M | M | - | - |
| secondaryOCSChargingFunctionName | O | M | M | - | - |
| primaryChargingCollectionFunctionName | O | M | M | - | - |
| secondaryChargingCollectionFunctionName | O | M | M | - | - |
| w-APNIdentifierList | O | M | M | - | - |
| maxNumberAccessesList | O | M | M | - | - |
| maxSubscribedBandwidth | O | M | M | - | - |
| subscribed3GPPWLANQoSProfile | O | M | M | - | - |
| permanentUserIdentity | O | M | M | - | - |
| mIPSubscriberProfile | O | M | M | - | - |
| accessRestrictionData | O | M | M | - | - |

#### 4.3.25.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.25.4	Notifications

None.

### 4.3.26	EPSServiceProfile

#### 4.3.26.1	Definition

This class represents for a subscribed EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user.

#### 4.3.26.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aPNAMBR | M | M | M | - | - |
| contextIdentifier | M | M | M | - | - |
| pDNAddress | O | M | M | - | - |
| aPNOIReplacement | O | M | M | - | - |
| subscribedUEAMBR | M | M | M | - | - |
| aPNConfigurationProfile | M | M | M | - | - |
| rFSP | O | M | M | - | - |
| pDNGWIdentity | O | M | M | - | - |
| ePSPDNConnectionChargingCharacteristics | O | M | M | - | - |
| vPLMNAddressAllowed-EPS | M | M | M | - | - |
| pDNGWAllocationType | M | M | M | - | - |
| aPNLevelAPNOIReplacement | O | M | M | - | - |
| roamingRestrictionDueToUnsupportedFeatureMME | M | M | - | - | - |
| roamingRestrictionDueToUnsupportedFeatureSGSN | M | M | - | - | - |
| mSPurgedForEPSFlag | M | M | - | - | - |
| uRRPMME | O | M | - | - | - |
| mMEName | M | M | - | - | - |
| sGSNName | M | M | - | - | - |
| iWFNumber | O | M | - | - | - |
| sGSNAreaRestrictedFlag | M | M | - | - | - |
| Attribute related to role |  |  |  |  |  |
| theOptHandover3GPP2Par | M | M | O | - | - |
| theNon3GPPAccessPar | M | M | O | - | - |

#### 4.3.26.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.26.4	Notifications

None.

### 4.3.27	PSCommonServiceProfile

#### 4.3.27.1	Definition

This is an abstract IOC provided for sub-classing only. It provides attribute(s) that are common to PS Service Profile IOCs (Regarding GPRS or EPS service, please see 3GPP TS 23.008 [7]).

#### 4.3.27.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| roamingRestrictedintheSGSNduetounsupportedfeature | M | M | - | - | - |
| barringofPacketOrientedServices | O | M | M | - | - |
| iPAddressOfTraceCollectionEntity | O | M | - | - | - |
| accessPointName | O | M | M | - | - |
| subscribedChargingCharacteristics | O | M | M | - | - |
| sTNSR | O | M | M | - | - |
| closedSubscriberGroupInformation | O | M | M | - | - |
| theIWANPar | M | M | O | - | - |

#### 4.3.27.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.27.4	Notifications

None.

### 4.3.28	IWANParameters

#### 4.3.28.1	Definition

This class represents for a subscribed GPRS or EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user that are relevant to IWAN services.

#### 4.3.28.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| w-APN | O | M | - | - | - |
| listofauthorizedvisitednetworkidentifiers | O | M | M | - | - |
| 3GPPAAAServerName | O | M | - | - | - |
| 3GPPAAAProxyName | O | M | - | - | - |
| wLANUERemoteIPaddress | O | M | M | - | - |
| staticWLANUERemoteIPAddressList | O | M | M | - | - |
| w-APNBarringTypeList | O | M | M | - | - |
| w-APNChargingDataList | O | M | M | - | - |
| accessDependenceFlag | O | M | M | - | - |
| i-WLANAccessType | O | M | - | - | - |
| wLANIdentifierList | O | M | M | - | - |
| maxNumberAccessesList | O | M | M | - | - |
| maxSubscribedBandwidth | O | M | M | - | - |
| subscribed3GPPWLANQoSProfile | O | M | M | - | - |
| chargingCharacteristics | M | M | M | - | - |
| primaryOCSChargingFunctionName | O | M | M | - | - |
| secondaryOCSChargingFunctionName | O | M | M | - | - |
| primaryChargingCollectionFunctionName | O | M | M | - | - |
| secondaryChargingCollectionFunctionName | O | M | M | - | - |
| userStatus | M | M | - | - | - |

#### 4.3.28.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.28.4	Notifications

None.

### 4.3.29	OptHandover3GPP2Parameters

#### 4.3.29.1	Definition

This class represents for a subscribed EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user that are relevant to the optimization of handover to a 3GPP2 network.

#### 4.3.29.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| rATType | O | M | - | - | - |

#### 4.3.29.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.29.4	Notifications

None.

### 4.3.30	Non3GPPAccessParameters

#### 4.3.30.1	Definition

This class represents for a subscribed EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user that are relevant to use of a non-3GPP access network.

#### 4.3.30.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| rATType | O | M | - | - | - |
| visitedNetworkIdentifier | O | M | - | - | - |
| mIPSubscriberProfile | M | M | M | - | - |
| permanentUserIdentity | M | M | M | - | - |

#### 4.3.30.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.30.4	Notifications

None.

### 4.3.31	Common3GPPAccessServiceProfile

#### 4.3.31.1	Definition

This is an abstract IOC provided for sub-classing only. It provides attribute(s) that are common to CS/PS Service Profile IOCs (Regarding CS, GPRS or EPS service, please see 3GPP TS 23.008 [7]).

#### 4.3.31.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| rSZIlists | O | M | M | - | - |
| accessRestrictionData | O | M | M | - | - |
| subscriberStatus | O | M | M | - | - |
| barringOfOutgoingCalls | O | M | M | - | - |
| barringOfRoaming | O | M | M | - | - |
| operatorDeterminedBarringPLMNspecificData | O | M | M | - | - |
| traceReference2 | O | M | - | - | - |
| traceDepth | O | M | - | - | - |
| listOfNETypestoTrace | O | M | - | - | - |
| triggeringEvents | O | M | - | - | - |
| listOfInterfacesToTrace | O | M | - | - | - |
| privacyExceptionList | O | M | M | - | - |
| gMLCNumbers | O | M | M | - | - |
| iCSIndicator | O | M | M | - | - |
| mOLRList | O | M | M | - | - |
| serviceTypes | O | M | M | - | - |
| networkAccessMode | M | M | M | - | - |

#### 4.3.31.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All optional attributes | The NE supports the parameter (see TS 23.008 [7]). |

#### 4.3.31.4	Notifications

None.

## 4.4	Attribute definitions

### 4.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed values | Properties |
| --- | --- | --- |
| suMSubscriberProfileInfo | A description of (information for) the Subscriber Profile. It is with free structure and format. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| iMSI | 23.008 clause 2.1.1.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networkAccessMode | 23.008 clause 2.1.1.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mSISDN | 23.008 clause 2.1.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| multinumberingMSISDNs | 23.008 clause 2.1.3 It is a sequence of two items: - An MSISDN number (string) - A multinumberingMSISDNIndicator which can be one of two choices: - “BASIC” - “ALERT” allowedValues: N/A | type: <<dataType>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| basicMSISDNIndicator | 23.008 clause 2.1.3.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mSISDNAlertIndicator | 23.008 clause 2.1.3.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lMSI | 23.008 clause 2.1.8 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mobileStationCategory | 23.008 clause 2.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lMUIdentifier | 23.008 clause 2.2.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| iMEISV | 23.008 clause 2.2.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| subscriptionRestriction | 23.008 clause 2.4.10 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rSZIlists | 23.008 clause 2.4.11.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mSCAreaRestrictedFlag | 23.008 clause 2.4.12 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sGSNarearestrictedflag | 23.008 clause 2.4.14 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| oDBinducedBarringData | 23.008 clause 2.4.15.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| roamingRestrictionDueToUnsupportedFeature | 23.008 clause 2.4.15.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| roamingRestrictedintheSGSNduetounsupportedfeature | 23.008 clause 2.4.15.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lSAIdentity | 23.008 clause 2.4.17.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lSAPriority | 23.008 clause 2.4.17.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lSAPreferentialAccessIndicator | 23.008 clause 2.4.17.2A allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lSAActiveModeSupportIndicator | 23.008 clause 2.4.17.2B allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lSAOnlyAccessIndicator | 23.008 clause 2.4.17.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lSAActiveModeIndicator | 23.008 clause 2.4.17.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vPLMNIdentifier | 23.008 clause 2.4.17.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| accessRestrictionData | 23.008 clause 2.4.18 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vLRNumber | 23.008 clause 2.4.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mSCNumber | 23.008 clause 2.4.6 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sGSNNumber | 23.008 clause 2.4.8.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gGSNNumber | 23.008 clause 2.4.8.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| provisionOfBearerService | 23.008 clause 2.5.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| provisionOfTeleservice | 23.008 clause 2.5.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bCAllocation | 23.008 clause 2.5.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| transferofSMoption | 23.008 clause 2.5.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mNRG | 23.008 clause 2.7.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| checkSSIndicator | 23.008 clause 2.7.4.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mSPurgedForNonGPRSFlag | 23.008 clause 2.7.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mSpurgedforGPRSflag | 23.008 clause 2.7.6 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mNRRMSC | 23.008 clause 2.7.7 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| subscriberStatus | 23.008 clause 2.8.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| barringOfOutgoingCalls | 23.008 clause 2.8.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| barringOfIncomingCalls | 23.008 clause 2.8.2.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| barringOfRoaming | 23.008 clause 2.8.2.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| barringOfPremiumRateCalls | 23.008 clause 2.8.2.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| barringOfSupplementaryServiceManagement | 23.008 clause 2.8.2.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| barringOfRegistrationOfCallForwarding | 23.008 clause 2.8.2.6 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| barringOfInvocationOfCallTransfer | 23.008 clause 2.8.2.7 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| barringofPacketOrientedServices | 23.008 clause 2.8.2.8 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| operatorDeterminedBarringPLMNspecificData | 23.008 clause 2.8.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notificationToCSEFlagForODB | 23.008 clause 2.8.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gsmSCFaddresslistForODB | 23.008 clause 2.8.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| messagesWaitingData | 23.008 clause 2.10.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mobileStationNotReachableFlag | 23.008 clause 2.10.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| memoryCapacityExceededFlag | 23.008 clause 2.10.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traceReference | 23.008 clause 2.11.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traceType | 23.008 clause 2.11.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| operationsSystemsIdentity | 23.008 clause 2.11.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| hLRTraceType | 23.008 clause 2.11.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mAPErrorOnTrace | 23.008 clause 2.11.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traceActivatedInVLR | 23.008 clause 2.11.6 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traceActivatedinSGSN | 23.008 clause 2.11.7 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vGCSGroupMembershipList | 23.008 clause 2.12.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vBSGroupMembershipList | 23.008 clause 2.12.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| broadcastCallInitiationAllowedList | 23.008 clause 2.12.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pDPType | 23.008 clause 2.13.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gGSNlist | 23.008 clause 2.13.11 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qualityofServiceSubscribed | 23.008 clause 2.13.12 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pDPAddress | 23.008 clause 2.13.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pDPContextIdentifier | 23.008 clause 2.13.24 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pDPContextChargingCharacteristics | 23.008 clause 2.13.25 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| accessPointName | 23.008 clause 2.13.6 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vPLMNAddressAllowed | 23.008 clause 2.13.8 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vPLMNAddressAllowed-EPS | 23.008 clause 2.13.40 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| originatingCAMELSubscriptionInformationOCSI | 23.008 clause 2.14.1.1/3.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gPRSCAMELSubscriptionInformationGPRSCSI | 23.008 clause 2.14.1.10/2.14.4.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dialledServiceCAMELSubscriptionInformationDCSI | 23.008 clause 2.14.1.11/3.7 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mobilityManagementforGPRSeventnotificationMGCSI | 23.008 clause 2.14.1.12/2.14.4.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| terminatingCAMELSubscriptionInformationTCSI | 23.008 clause 2.14.1.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vMSCTerminatingCAMELSubscriptionInformationVTCSI | 23.008 clause 2.14.1.2/3.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| locationInformationSubscriberStateInformation | 23.008 clause 2.14.1.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uSSDCAMELSubscriptionInformationUCSI | 23.008 clause 2.14.1.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sSInvocationNotificationSSCSI | 23.008 clause 2.14.1.5/3.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| translationInformationFlagTIFCSI | 23.008 clause 2.14.1.6/3.6 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| shortMessageServiceCAMELSubscriptionInformationMOSMSCSI | 23.008 clause 2.14.1.8/2.14.3.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mOShortMessageServiceCAMELSubscriptionInformationMOSMSCSI | 23.008 clause 2.14.1.8/2.14.4.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| shortMessageServiceCAMELSubscriptionInformationMTSMSCSI | 23.008 clause 2.14.1.9/2.14.3.6 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mTShortMessageServiceCAMELSubscriptionInformationMTSMSCSI | 23.008 clause 2.14.1.9/2.14.4.2. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gPRSCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mGCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mOSMSCSISGSNNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mOSMSCSIVLRNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mTSMSCSISGSNNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mTSMSCSIVLRNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| oCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sSCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vTCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SGSNOfferedCAMEL4CSIs | 23.008 clause 2.14.2.2A allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vLROfferedCAMEL4CSIs | 23.008 clause 2.14.2.2A allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sGSNSupportedCAMELPhases | 23.008 clause 2.14.2.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vLRSupportedCAMELPhases | 23.008 clause 2.14.2.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gsmSCFaddressforCSI | 23.008 clause 2.14.2.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uSSDGeneralCAMELServiceInformationUGCSI | 23.008 clause 2.14.2.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| iSTAlertTimer | 23.008 clause 2.15.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| privacyExceptionList | 23.008 clause 2.16.1.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gMLCNumbers | 23.008 clause 2.16.1.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mOLRList | 23.008 clause 2.16.1.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceTypes | 23.008 clause 2.16.1.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ageIndicator | 23.008 clause 2.17.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cSAllocationRetentionpriority | 23.008 clause 2.18.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| subscribedChargingCharacteristics | 23.008 clause 2.19.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| iMSPrivateUserIdentity | 23.008 clause 3.1.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| publicUserIdentity | 23.008 clause 3.1.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| barringIndication | 23.008 clause 3.1.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| listofauthorizedvisitednetworkidentifiers | 23.008 clause 3.1.4 23.008 clause 3B.1.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| registrationStatus | 23.008 clause 3.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sCSCFName | 23.008 clause 3.2.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| diameterClientAddressofSCSCF | 23.008 clause 3.2.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serverCapabilities | 23.008 clause 3.4.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| initialFilterCriteriaList | This attribute is a list which contains multiple items of initialFilterCriteria (Ref. 23.008 clause 3.5.2) It is a set of initialFilterCriteria (string) allowedValues: N/A | type: <<datatype>> multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| applicationServerInformation | 23.008 clause 3.5.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceIndicationList | This attribute is a list which contains multiple items of serviceIndication (Ref. 23.008 clause 3.5.4) It is a set of serviceIndication (string) allowedValues: N/A | type: <<datatype>> multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| subscribedMediaProfileIdentifier | 23.008 clause 3.6.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| primaryEventChargingFunctionName | 23.008 clause 3.7.1 23.008 clause 3B.7.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| secondaryEventChargingFunctionName | 23.008 clause 3.7.2 23.008 clause 3B.7.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| primaryChargingCollectionFunctionName | 23.008 clause 3.7.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| secondaryChargingCollectionFunctionName | 23.008 clause 3.7.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gsmSCFaddressforIMCSI | 23.008 clause 3.8.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| iMSSFaddressforIMCSI | 23.008 clause 3.8.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| oIMCSI | 23.008 clause 3.8.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vTIMCSI | 23.008 clause 3.8.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dIMCS | 23.008 clause 3.8.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gAAServiceType | 23.008 clause 3A.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gAAServiceIdentifier | 23.008 clause 3A.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gBAUserSecuritySettings | 23.008 clause 3A.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| userSecuritySetting | 23.008 clause 3A.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| userPublicIdentity | 23.008 clause 3A.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gAAAuthorizationflag | 23.008 clause 3A.6 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| keyLifetime | 23.008 clause 3A.8 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uICCSecuritySetting | 23.008 clause 3A.9 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nAFGroupIdentity | 23.008 clause 3A.11 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| w-APN | 23.008 clause 3B.1.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| 3GPPAAAServerName | 23.008 clause 3B.1.6 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| wLANUERemoteIPaddress | 23.008 clause 3B.1.10 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| staticWLANUERemoteIPAddressList | 23.008 clause 3B.5.1.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| w-APNBarringTypeList | 23.008 clause 3B.5.1.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| w-APNChargingDataList | 23.008 clause 3B.5.1.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| accessDependenceFlag | 23.008 clause 3B.5.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| i-WLANAccessType | 23.008 clause 3B.5.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| chargingCharacteristics | 23.008 clause 3B.7.1.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| primaryOCSChargingFunctionName | 23.008 clause 3B.7.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| secondaryOCSChargingFunctionName | 23.008 clause 3B.7.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sS-Code | 23.008 clause 2.6 29.002 clause 7.6.4.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| provisioningState | 23.008 clause 2.6 29.002 clause 7.6.4.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| activationState | 23.008 clause 2.6 29.002 clause 7.6.4.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| registrationState | 23.008 clause 2.6 29.002 clause 7.6.4.2 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| overrideCategory | 23.008 clause 2.6 29.002 clause 7.6.4.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cLIRestrictionOption | 23.008 clause 2.6 29.002 clause 7.6.4.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| forwarded-toNumber | 23.008 clause 2.6 29.002 clause 7.6.2.22 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| forwarded-toSubaddress | 23.008 clause 2.6 29.002 clause 7.6.2.23 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| noReplyConditionTimer | 23.008 clause 2.6 29.002 clause 7.6.4.7 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maximumEntitledPriority | 23.008 clause 2.6 29.002 clause 7.6.4.41 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| defaultPriority | 23.008 clause 2.6 29.002 clause 7.6.4.41 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nbrUser | 23.008 clause 2.6 29.002 clause 7.6.4.47 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nbrSB | 23.008 clause 2.6 29.002 clause 7.6.4.47 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TraceReference2 | 23.008 Clause 2.11.9 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TraceDepth | 23.008 Clause 2.11.10 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ListofNeTypestoTrace | 23.008 Clause 2.11.11 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TriggeringEvents | 23.008 Clause 2.11.12 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ListofInterfacestoTrace | 23.008 Clause 2.11.13 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| iCSIndicator | 23.008 Clause 2.20.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| iCSIndicator | 23.008 Clause 2.20.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sTNSR | 23.008 Clause 2.21.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| 3GPPAAAProxyName | 23.008 Clause 3B.1.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| w-APNIdentifierList | 23.008 Clause 3B.5.1.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxNumberAccessesList | 23.008 Clause 3B.5.1.5 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxSubscribedBandwidth | 23.008 Clause 3B.6.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| subscribed3GPPWLANQoSProfile | 23.008 Clause 3B.6.4 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| userStatus | 23.008 Clause 3B.2.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rATType | 23.008 Clause 2.13.75 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| iPAddressOfTraceCollectionEntity | 23.008 Clause 2.11.14 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aPNConfigurationProfile | 23.008 Clause 2.13.31 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pDNAddress | 23.008 Clause 2.13.39 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| permanentUserIdentity | 23.008 Clause 2.13.79 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| visitedNetworkIdentifier | 23.008 Clause 2.13.82 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mIPSubscriberProfile | 23.008 Clause 2.13.86 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aPNAMBR | 23.008 Clause 2.13.32 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| contextIdentifier | 23.008 Clause 2.13.38 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aPNOIReplacement | 23.008 Clause 2.13.29 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| subscribedUEAMBR | 23.008 Clause 2.13.30 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rFSP | 23.008 Clause 2.13.33 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pDNGWIdentity | 23.008 Clause 2.13.41 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ePSPDNConnectionChargingCharacteristics | 23.008 Clause 2.13.71 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pDNGWAllocationType | 23.008 Clause 2.13.105 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aPNLevelAPNOIReplacement | 23.008 Clause 2.13.109 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| closedSubscriberGroupInformation | 23.008 Clause 2.4.22 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| roamingRestrictionDueToUnsupportedFeatureMME | 23.008 Clause 2.4.15.3a allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mSPurgedForEPSFlag | 23.008 Clause 2.7.6A allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uRRPMME | 23.008 Clause 2.7.9.1 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mMEName | 23.008 Clause 2.13.26 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sGSNName | 23.008 Clause 2.13.100 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| iWFNumber | 23.008 Clause 2.4.8.3 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Attribute related to role |  |  |
| theContract | Models the relationship to a subscribed service. The relationship is between a service and a subscription to the same type of service (for example between CSService and CSSubscribedService). This attribute contains a list of the DN(s) of the related SuMSubscribedService instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theOffer | Models the relationship to a service. The relationship is between a service and a subscription to the same type of service (for example between CSService and CSSubscribedService). This attribute contains the DN of the related SuMService instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| theService | Models the relationship to a service profile. The relationship is between a subscribed service and a service profile of the same type of service (for example between CSSubscribedService and CSServiceProfile). This attribute contains a list of the DN(s) of the related SuMServiceProfile instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theSpecification | Models the relationship to a subscribed service. The relationship is between a subscribed service and a service profile of the same type of service (for example between CSSubscribedService and CSServiceProfile). This attribute contains the DN of the related SuMSubscribedService instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thePublicId | Models the relationship from a user’s credentials to the IMS Public Identification. This attribute contains a list of the DN(s) of the related IMSPublicIdentification instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theMobileUserData | Models the relationship from a user to the user data of a mobile User Equipment. This attribute contains a list of the DN(s) of the related MobileUserData instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theServiceProfile | Models the relationship from the user data of a mobile User Equipment to a service profile or from a user’s credentials to an IMS service profile. This attribute contains a list of the DN(s) of the related SuMServiceProfile derived instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theSubscriber | Models the relationship from a user to a subscriber. This attribute contains a list of the DN(s) of the related SuMSubscriberProfile instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theUser | Models the relationship from a subscriber profile or a Subscription Function to a user. This attribute contains a list of the DN(s) of the related SuMUser instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theCSSupplServiceForCS | Models the relationship from a CS service to a CS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMCSSupplService instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theGPRSSupplServiceForGPRS | Models the relationship from a GPRS service to a GPRS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMGPRSSupplService instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theCSSupplServiceForCSSS | Models the relationship from a CS subscribed service to a CS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMCSSupplService instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theGPRSSupplServiceForGPRSSS | Models the relationship from a GPRS subscribed service to a GPRS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMGPRSSupplService instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theCSSupplServiceForCSSP | Models the relationship from a CS service profile to a CS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMCSSupplService instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theGPRSSupplServiceForGPRSSP | Models the relationship from a GPRS service profile to a GPRS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMGPRSSupplService instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theSubscribedService | Models the relationship from a subscriber profile to a subscribed service. This attribute contains a list of the DN(s) of the aggregated SuMSubscribedService derived instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| thePublicId | Models the relationship from an IMS service profile to a public identification. This attribute contains a list of the DN(s) of the aggregated IMSPublicIdentification instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theSubscriberProfile | Models the relationship from a Subscription Function to a subscriber profile. This attribute contains a list of the DN(s) of the aggregated SuMSubscriberProfile instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theCredentials | Models the relationship from a user to a credential. This attribute contains a list of the DN(s) of the aggregated SuMCredentials instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theServiceProfile | Models the relationship from a user to a service profile. This attribute contains a list of the DN(s) of the aggregated SuMServiceProfile instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theIdentity | Models the relationship from a user to an identity. This attribute contains a list of DN(s) of the related SuMIdentity instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theServerCapabilities | Models the relationship from a SuM IOC object instance (E.g., IMSService, IMSSubscribedService, IMSServiceProfile) to one or more Server Capabilities object instance(s). allowedValues: N/A | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| theIWANPar | Models the relationship between the common packet access service parameters and the I-WAN service parameters. This attribute contains the DN of the related IWANParameters instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| theOptHandover3GPP2Par | Models the relationship between the EPS service profile when designed for optimal handover to a 3GPP2 access network. This attribute contains the DN of the related OptHandover3GPP2Parameters instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| theNon3GPPAccessPar | Models the relationship between the EPS service profile and another set of service parameters needed to when access is via a non 3GPP network. This attribute contains the DN of the related Non3GPPAccessParameters instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |

### 4.4.2	Constraints

- None.

## 4.5	Common notifications

- None.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| 2013-06 |  |  |  |  | Editorial changes (MCC): removal of blank page on page 2 |  | 11.0.0 | 11.0.1 |
| 2014-06 | SA#64 | SP-140359 | 001 | - | remove the feature support statements | F | 11.0.1 | 11.1.0 |