3GPP TS 28.611 V12.1.0 (2014-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Evolved Packet Core (EPC) and non-3GPP access

interworking system Network Resource Model (NRM)

Integration Reference Point (IRP);

Requirements

(Release 12)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

OAM, WLAN, interworking, E-UTRAN, EPC

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2014, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	5

3.1	Definitions	5

3.2	Abbreviations	6

4	Concepts and background	6

5	Requirements	6

Annex A (informative):	 Change history	7

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

Ready for Converged Management

NOTE:	This specification is part of a set that has been developed for converged management solutions.

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects, Telecommunication management; as identified below:

TS 28.611:	"Evolved Packet Core (EPC) and non-3GPP access interworking system Network Resource Model (NRM) Integration Reference Point (IRP); Requirements"

TS 28.612:	"Evolved Packet Core (EPC) and non-3GPP access interworking system Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

TS 28.616:	"Evolved Packet Core (EPC) and non-3GPP access interworking system Network Resource Model (NRM) Integration Reference Point (IRP);  Solution Set (SS) definitions"

# 1	Scope

The present document describes the Network Resource Model (NRM) Integration Reference Point (IRP) requirements for the EPC and WLAN interworking system according to the structure defined in 3GPP TS 23.402 [8] (e.g. ePDG, 3GPP AAA, etc).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	-	For a specific reference, subsequent revisions do not apply.

-	-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[5]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions".

[6]	 Void

[7]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[8]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Integration Reference Point (IRP): See 3GPP TS 32.150 [5].

Information Service (IS): See 3GPP TS 32.150 [5].

Solution Set (SS): See 3GPP TS 32.150 [5].

IRP Solution Set: See 3GPP TS 32.101 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AAA	Access, Authentication and Authorisation

ePDG		evolved Packet Data Gateway

IRP	Integration Reference Point

IOC	Information Object Class

NRM	Network Resource Model

# Concepts and background

The architectures of enhancement for non-3GPP access (i.e. WLAN) to 3GPP system are defined in 3GPP TS 23.402 [8]. 
In TS 23.402, three kinds of architectures are defined, using S2a, S2b and S2c interface separately. 
The architectures of S2a and S2c don't introduce new network elements in 3GPP system, 
while the architecture S2b introduces a new network element to support the interworking:

-	evolved Packet Data Gateway (ePDG) (clause 4.3.4 of 3GPP TS 23.402 [8]).

The above interworking related network element should be managed by the 3GPP OAM, and the corresponding NRM IRP needs to be defined.

Based on the above characteristics, this specification defines the evolved 3GPP interworking WLAN NRM IRP requirements.

# 5	Requirements

The following general and high-level requirements apply for the present IRP:

a)	IRP-related requirements in 3GPP TS 32.101 [2].

b)	IRP-related requirements in 3GPP TS 32.102 [3].

c)	IRP-related requirements in 3GPP TS 32.600 [4].

In addition to the above, the following more specific requirements apply:

REQ-EPC_WLAN-CON-001
The Network Resource Model defined by this IRP shall contain ePDG specific IOCs and related definitions. 
This requirement should be satisfied when the S2b architecture is implemented.

REQ-EPC_WLAN-CON-002
The Network Resource Model defined by this IRP shall contain 3GPP AAA specific IOCs and related definitions.

Annex A (informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| Oct 2011 | SA5-79 |  | -- | -- | Initial version for TS skeleton | --- | 0.0.1 |
| Feb 2012 | SA5-81 |  |  |  | Added NRM IRP requirements | 0.0.1 | 0.1.0 |
| Aug 2013 |  |  |  |  | Revise the document title and release information | 0.1.0 | 0.1.1 |
| Sep 2013 | SA-61 | SP-130448 |  |  | Presented for information | 0.1.1 | 1.0.0 |
| Sep 2013 | - | - |  |  | MCC clean-up | 1.0.0 | 1.0.1 |
| Nov 2013 |  |  |  |  | Remove editor notes | 1.0.1 | 1.1.1 |
| Dec 2013 | SA-62 | SP-130633 |  |  | Presented for approval | 1.1.1 | 2.0.0 |
| Dec 2013 |  |  |  |  | Version after approval | 2.0.0 | 12.0.0 |
| Oct 2014 | SA-65 | SP-140560 | 004 | - | Remove unused reference | 12.0.0 | 12.1.0 |