ES 203 228 V<0.0.3> (<2013-12>)

<Environmental Engineering (EE); 
Assessment of Mobile Network Energy Efficiency >

[<GSM/UMTS/LTE spec>]

[Part element for endorsement]

<

ETSI SPECIFICATION

Reference

<EEPS DES00005>

Keywords

<energy efficiency, radio access networks, UMTS, GSM, LTE, base stations>

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

Individual copies of the present document can be downloaded from:
http://www.etsi.org

The present document may be made available in more than one electronic version or in print. In any case of existing or perceived difference in contents between such versions, the reference version is the Portable Document Format (PDF). In case of dispute, the reference shall be the printing on ETSI printers of the PDF version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, please send your comment to one of the following services:
http://portal.etsi.org/chaircor/ETSI_support.asp

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM and LTE™ are Trade Marks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
GSM® and the GSM logo are Trade Marks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	5

Foreword	5

Introduction	5

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	7

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Network under test definition	10

4.1 	Introduction	10

4.2 	Test parameter categorization	11

4.3 	Network classification	11

4.3.1	Demography	12

4.3.2	Topography	12

4.3.3	Climate	12

4.3.4	Optional classification classes	13

5	Metrics for energy efficiency assessment	14

5.1 	Energy Consumption metrics	14

5.2 	Performance metrics	14

5.3  	Energy efficiency metrics	14

6	Measurement of energy efficiency	16

6.1 	Time duration of the measurement	16

6.2	Measurement procedures	16

6.2.1	Measurement of Energy Consumption	16

6.2.2	Measurement of capacity (data)	16

6.2.3	Determination of coverage area	17

6.2.3.1 Geographic coverage area	17

6.2.3.2 Coverage ratio / effective coverage area	17

7	Extrapolation for global networks	17

8	Assessment report	17

History	22

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://ipr.etsi.org).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

# Introduction

The items included in the following table are stable in this version.

| Item | Section in the ES |
| --- | --- |
| Scope of the ES | Clause 1 |
| Metrics and methods applied to sub-networks and extrapolation for larger networks; demography, topography and climate classes | Clause 1 and Clause 4 |
| Metric based on EE as a ratio between DV and EC (i.e. EEMN,D) | Clause 5 formula (3) |
| Metric of EC in MN | Clause 5 formula (1) |
| Time duration of the Test | Clause 6.1 |
| Introduction of Reporting Tables (content of the Tables to be checked and updated in next drafts) | Clause 8 |

(Note: this table will be removed once the whole ES will be declared stable).

# 1	Scope

The present document is aimed to define the topology and level of analysis to assess the energy efficiency of mobile networks. Within the scope of the document there is the radio access part of the mobile networks, and namely there are radio base stations, backhauling systems, radio controllers and other infrastructure radio site equipment. The covered technology are GSM, UMTS, LTE (including LTE-A). In particular the document  defines metrics for network Energy Efficiency and methods for assessing (and measuring) Energy Efficiency in operational networks. The purpose of the specification is to allow comparisons of the networks energy efficiency.

The present document deals with both homogeneous and heterogeneous “network” considering  a network whose size and scale could be defined by topologic, geographic or demographic boundaries. For networks defined by topologic boundaries, the smallest network covered by this document consists of a control node (whenever applicable), its supported access nodes as well as the related network elements.  Networks could be defined by geographic boundaries, such as city-wide, national or continental networks and could be defined by demographic boundaries, such as urban or rural networks.

One goal is to introduce so-called “partial” networks where to measure energy efficiency in a standardized way. The specification extends the measurements to wider so-called “global” networks energy efficiency estimation (i.e. the network in a geographic area, the network in a whole country, the network of an operator,…).

Terminal (end-user) equipment is outside the scope of this document and is not considered in the energy efficiency measurement.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at http://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	ETSI TS 102 706: "Environmental Engineering (EE); Measurement Method for Energy Efficiency of Wireless Access Network Equipment".

[2]	ETSI TS 125 104: "Universal Mobile Telecommunications System (UMTS); Base Station (BS) radio transmission and reception (FDD) (3GPP TS 25.104)".

[3]	ETSI TS 136 104: "LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception (3GPP TS 36.104)".

[4]	3GPP TS 36.314: “Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 – Measurements”

[5]	3GPP TS 36.420: “Evolved Universal Terrestrial Radio Access (E-UTRA); X2 general aspects and principles”

[6]	3GPP TS 32.425: “Performance measurements Evolved Universal Terrestrial Radio Access Network” v12.0.0

[7]	3GPP TS 32.412: “Integration Reference Point (IRP): Information Service (IS)” v11.1.0

[8]	3GPP TS 23.203: “Policy and charging control architecture” v12.2.0

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	ETSI TR 103117: "Environmental Engineering (EE); Principles for Mobile Network level energy efficiency".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

Backhaul equipment: equipment used to connect base stations to the core network, or to other BSs (like X2 in LTE)

Integrated BS: BS architecture in which all BS element are located close to each other for example in one or two cabinets

NOTE:	The integrated BS architecture may include Tower Mount Amplifier (TMA) close to antenna.

Distributed RBS: BS architecture which contains radio heads (RRH) close to antenna element and a central element connecting BS to network infrastructure

energy efficiency (EE): relation between the useful output and energy/power consumption

Mobile Network: set of equipment  from the radio access network or sub-network that are relevant for the assessment of energy efficiency

Mobile Network Coverage Energy Efficiency : ratio between the area covered by the BS in the MN under investigation and the energy consumption  when assessed during the same time frame.

Mobile Network Data Energy Efficiency : ratio between the performance indicator  and the energy consumption  when assessed during the same time frame.

Mobile Network Energy Consumption : overall energy consumption of equipment included in the MN under investigation.

Mobile Network Performance Delivered: performance indicator of the MN under investigation, defined as the datavolume handled by the equipment of the mobile network under investigation during the time frame of the energy consumption assessment.

Mobile Network Energy Efficiency: EE of Mobile Networks

power consumption: power consumed by a device to achieve an intended application performance

Energy saving feature: feature which contributes to decreasing the energy consumption compared to the case when the feature is not implemented

Base Station (BS): generic term used for network component which serves one cell or more cells and interfaces the user terminal (through air interface) and a radio access network infrastructure, limited to the wide area base stations and medium range base stations defined by 3GPP

telecommunication network: network operated under a license granted by a national telecommunications authority, which provides telecommunications between Network Termination Points (NTPs)

radio access network: generic term used for a telecommunications network in which the access to the network (connection between user terminal and network) is implemented without the use of wires and that is part GERAN, UTRAN or E-UTRA defined by 3GPP

Remote Radio Head: cf. Distributed BS.

## 3.2	Symbols

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3GPP	3G (mobile) Partnership Project

BS	Base Station

BH	Backhaul

BHEC	BH Energy Consumption

CoA	Covered Area

DL	DownLink

DV	Data Volume

EDGE	Enhanced Datarate GSM Evolution

E-UTRA	Evolved UMTS Terrestrial Radio Access Network

eNB	E-UTRA BS

GERAN	GSM/EDGE Radio Access Network

GSM	Global System for Mobile communication

GSMA	GSM Association

HSDPA	High Speed Downlink Packet Access

HSPA	High Speed Packet Access

HW	HardWare

ICT 	Information Communications Tecnology

IP	Internet Protocol

ITU	International Telecommunications Union

KPI	Key Performance Indicator

LTE	Long Term Evolution

MDT	Minimization of Drive Tests

MN	Mobile Network

O&M	Operation & Maintenance

PDF	Probability Distribution Function

QoE	Quality of Experience (end-user)

QoS	Quality of Services

RAN	Radio Access Network

RAT	Radio Access Technology

RNC	Radio Network Controller

RRH	Remote Radio Head

RX	Receiver

SW	SoftWare

TCP	Transmission Control Protocol (ACK, SYN and FIN are signalling in the TCP session)

TX	Transmitter

UE	User Equipment

UL	UpLink

UTRAN	UMTS Terrestrial Radio Access Network

X2	Interface allowing to interconnect eNBs with each other

WCDMA	Wideband Code Division Multiple Access

# 4	Network under test definition

## 4.1 	Introduction

The Mobile Network (MN) under investigation shall include all the equipment that are necessary to run a radio access network or sub-network. Equipment to be included in the Mobile Network under investigation:

- Base Stations (see [2,3])

  - Wide area BS

  - Medium range BS

  - Local Area BS

  - Home BS

NOTE: Home BS (and WiFi access points) are not dealt with in this standard version, being possibly considered for future versions

- Radio site equipment

- Backhaul equipment required to interconnect the BS used in the assessment with the core network

- Radio controller (RC)

Power consumption and energy efficiency measurements of individual mobile network elements are described in several standards (for example TS 102 706 for radio base stations). This standard describes energy consumption and network efficiency measurements in operational networks.

As a complete and detailed energy consumption measurement of the complete network of a country or operator is in most cases impossible or economically not viable, the total network is split into a small number of networks with limited size (“sub-network”).

These sub-networks are defined to represent some specific characterisitcs like:

- Capacity limited networks representing urban and dense urban networks

- Sub-urban networds with high requirements for coverage and capacity

- Rural networks, which are usually coverage limited

Size and scale of the sub-networks are defined by topologic, geographic or demographic boundaries. For networks defined by topologic boundaries, the smallest network covered by this document consists of a Radio controller (whenever applicable), its supported access nodes as well as the related network elements.  Networks could be defined by geographic boundaries, such as city-wide, national or continental networks and could be defined by demographic boundaries, such as urban or rural networks.

The tests defined in this standard should provide the basis to estimate energy efficiency for large networks of one operator or within an entire country, as depicted in figure 1. The sub-networks analysed in the complete network might consist of macro-only base stations or heterogeneous networks, depending on the actual implementation (Figure 2).

Figure 1: Network efficiency - Measurement of representative samples within a life network and relation to TS 102 706

Figure 2: Sub-network for test purposes. Homogenous or heterogenous network structures are considered

## 4.2 	Test parameter categorization

The metrics for the energy efficiency assessment requires the definition and collection of a range of parameters and variable. These are separated into two categories:

- Parameters and variables required to calculate the network efficiency KPIs

- Parameters needed to allow network evaluation and comparison

The first category describes a set of network variables as described in Clause 5 (energy consumption, delivered bits, coverage) to be used to calculate the efficiency KPIs

The second category includes parameters which are not directly required in the KPI calculation. These parameters describe the network environment, like geographical conditions, population density, coverage area, targeted data rates, etc.) and are used to interpolate from the measured sub-network to a larger network as described in chapter 7. These parameters also help to identify reasons for different efficiency results of different networks.

| Category | Parameter | Remarks |
| --- | --- | --- |
| 1 | ECMN | Measured network energy consumption |
| 1 | Coverage area | As defined in 6.3.2.1 |
| … |  |  |
| 2 | Coverage ratio | As defined in 6.3.2.1 |
| 2 | Demography | Population density as defined in 4.3.1 |
| 2 | Topography | As defined in 4.3.2 |
| … |  |  |

## 4.3 	Network classification

To allow an interpolation from the measured sub-networks to a complete network, the test areas shall be classified into different categories as defined by Network classification

The environmental classes are: demography, topography and climate

### 4.3.1	Demography

For the test purpose defined in this standard the mobile network shall be split in domains depending on the population density. The following population density values per  domain categories shall be used (cf. for example [i.1]):

Table 1: sub-network demography class

| Demography Class | Typical population density (inhabitants/km²) | Population range (inhabitants/km²) |
| --- | --- | --- |
| Dense Urban (DU) | 20.000 | > 10.000 |
| Urban (U) | 2.000 | 1.000 – 10.000 |
| Sub-urban (SU) | 300 | 200 – 1.000 |
| Rural (RU) | 30 | 20 – 200 |
| Unpopulated | 0 | < 20 |

### 4.3.2	Topography

| Proposed system |  | Examples |
| --- | --- | --- |
| ETSI class | Median Slope |  |
| 1 Flat (FAO 1-3) | 0 - 5% | Danmark, Netherlands |
| 2 Rolling (FAO 4-6) | >5 - 30% | France, Italy |
| 3 Mountainous (FAO 7-8) | >30% | Norway, Switzerland |

Information from median slope gradient class world map (FAO) (http://www.fao.org/nr/land/sols/harmonized-world-soil-database/terrain-data/fr/)

### 4.3.3	Climate

The sub-network selection shall be made according to climate criteria as well. In particular the following climate zones are identified.

The above reported climate classification is based on FAO Koeppen classification (http://www.fao.org/sd/eidirect/climate/EIsp0002.htm).

The indication based on the five main classes A to E is mandatory; the subclasses are optionally indicated.

### 4.3.4	Optional classification classes

In order to properly select the sub-networks for the test terrain type for rural areas, operators penetration ratio and data traffic types are to be considered.

| Symbol (TBD) | Operator penetration class | Range TBC |
| --- | --- | --- |
| DP | Dominant Penetration | > 30% penetration |
| NDP | Non-Dominant Penetration | < 30% penetration |
| MP | Minor Penetration | < 10% penetration |

| Symbol | Traffic Class | Spec TBC |
| --- | --- | --- |
| CS | CS dominating | > 50% of data volume is CS |
| PSS | PS –small packages dominating | > 50% of data volume is PS, >80% of packages are small (< TBD B) |
| PSL | PS- large packages dominating | > 50% of data volume is PS, < 80% of packages are small (<TBD B) |

# 5	Metrics for energy efficiency assessment

The following metrics shall be used to assess the mobile network energy efficiency.

## 5.1 	Energy Consumption metrics

The Mobile Network Energy Consumption (ECMN) is the sum of the energy consumption of equipment included in the MN under investigation (cf. Clause 4). The network energy consumption is measured  according to the assessment process defined in Clause 6 such that individual metrics are provided per RAT and per operator.

| ECMN=i,kECBSi,k+iECSIi+jECBHj+lECRCl | (1) |
| --- | --- |

where

EC is Energy Consumption, BS refers to the Base Stations in the MN under measurement, BH is the backhauling providing connection to the BSs in the MN under measurement, SI is the site infrastructure (Rectifier, battery losses, climate equipment, TMA, tower illumination etc), RC is the control node(s), including all infrastructure of the RC site. i is an index spanning over the number of sites, j an index spanning over the number of  BH links to the i sites, k is the index spanning over the number of BSs in the i-th site and  l is the index spanning over the the control nodes of the MN.

ECMN shall be measured in Wh over the period of measurement T (see Clause 6).

Note: If the control node(s) support a larger MN than what is assessed, only a proportional share of RNC EC is considered, proportional to the RNC share of traffic that belongs to MN assessed.

## 5.2 	Performance metrics

The Mobile Network performance delivered (DVMN) is the performance indicator of the MN under investigation (cf. Clause 4) relevant for energy efficiency assessment.  DVMN is defined as the data volume handled with a defined quality of service by the equipment of the mobile network under investigation during the time frame T of the energy consumption assessment. The assessment process defined in Clause 6 shall be used.

| DVMN=i,kDVBSi,k | (2) |
| --- | --- |

where DV, measured in bit, is the performance delivered in terms of data volume in the network over the measurement period T (see Clause 6). i and k are like in formula (1).

DVMN can be estimated by referring to standard counters defined in [6] and [7], multiplying by the measurement duration T. These counters account also for QoS being reported in QoS Class Identifier (QCI) basis (see [8]).

Note:

  - DVMN includes data volumes for DL and UL

  - BH, supervision and control data volumes are not considered

## 5.3  	Energy efficiency metrics

Mobile Network data Energy Efficiency (EEMN,D) is the ratio between the performance indicator (DVMN) and the energy consumption (ECMN) when assessed during the same time frame. EEMN is expressed in bit/Wh.

| EEMN,D=DVMNECMN | (3) |
| --- | --- |

Mobile Network coverage Energy Efficiency (EEMN,C) is the ratio between the area covered by the BS in the MN under investigation (cf. Clause 4) and the energy consumption when assessed during one year. EEMN,C is  expressed in m²/Wh. EEMN,C is mainly used to complement EEMN,D for MNs handling low data volumes, in particular in rural or deep rural areas. The area covered shall be assessed using  rules (i.e. derived from geographic data or propagation models) defined in Clause 6.

| EEMN,C=coverage areaECMN | (4) |
| --- | --- |

Therefore, the mobile network efficiency shall be defined by two metrics:

- Capacity (data) efficiency EEMN,D, which is defined as the capacity vs. energy consumption provided within the different sub-network categories

- Coverage efficiency EEMN,C, which is defined as the covered area vs. energy consumption provided within the different sub-network categories

# 6	Measurement of energy efficiency

The measurement of the EEMN  in the MN under investigation shall be based on the separate measurement of the performance (in terms of capacity and coverage) and energy, according to the metrics defined in Clause 5.

## 6.1 	Time duration of the measurement

The time duration of the measurement, denoted as T, shall be one of the alternatives:

- Weekly measurement: T equal to 7 days

- Monthly measurement: T equal to 30 days

- Yearly measurement: T equal to 365 days

The minimum duration is therefore one week, monthly and yearly measurements are extensions of the basic week test.

## 6.2	Measurement procedures

### 6.2.1	Measurement of Energy Consumption

The Energy Consumption of the MN can be measured by means of metering information provided by utility suppliers or by mobile network integrated measurement systems (ES 202 336 series). Moreover, sensors can be used to measure site and equipment energy consumption.

The ECMN is based on site granularity and includes therefore all the equipment that are in the MN sites (including the network controllers whenever applicable). The ECMN shall be differentiated per operator providing service to the MN; in case of shared infrastructure the ECMN of the shared sites shall be computed per each operator sharing those sites in a proportional ratio. In case of separate metering per operators the respective part of the ECMN shall be assigned to each operator.

The ECMN shall be based on a per RAT estimation. If in the site there are BS of different RATs the ECMN shall be derived proportionally.

The list of equipment operating in the MN sites under investigation shall be reported in the assessment report, including cooling, power conversion, etc

The reporting frequency of the ECMN should be aligned with the utility provider settings and mobile network performance assessment settings. It shall be reported in the assessment report.

Note: as soon as a mobile network integrated measurement systems will be available, it should be used instead of the utility provided ECMN, allowing a more precise estimation of the consumption per RAT and per operator.

### 6.2.2	Measurement of capacity (data)

The DVMN shall be measured using network counters for data volume related to the aggregated traffic in the set of BS considered in the MN under test.

The data volume is considered as the overall amount of data transferred to and from the users present in the MN under test. Data volume shall be measured in an aggregated way per each RAT present in the MN and shall be measured referring to counters derived from vendors O&M systems.

In order to include reliability in the measurement the aggregated data volume shall be provided together with the 95 percentile of the cumulative distribution, for each RAT in the MN.

Note: As soon as the MDT related measurements in [4] (3GPP TS 36.314) is available the data volume may be measured according to the specification given therein (especially referring to clause 4.1.8 in [4]).

### 6.2.3	Determination of coverage area

The coverage area under test is initially defined by an area served of a cluster of radio base stations as defined in chapter 4.

Coverage area is subject to network planning and intended services delivered within a certain area. These parameters vary according to an operators strategy and might therefore differ from operator to operator but also within the network of one operator for different geographical area.

The coverage are shall be described by two distinct methods:

- Calculated coverage area derived from network design, planned service and geographical data

- Measured network coverage based on UE reporting

#### 6.2.3.1 Geographic coverage area

The geographic network coverage area is the total two dimensional area intentionally covered by the selected sub-network. This area is defined by an operators defined network service plans (minimum granted data rate / typical data rate, etc.) for the selected test area. The area is based on base station power, propagation conditions in the selected area and accepted outage criteria.

#### 6.2.3.2 Coverage ratio / effective coverage area

The actual coverage area might differ from the originally planned coverage area (coverage holes within the considered area). The coverage ratio is a measure to estimate the actually covered fraction of the planned total coverage area. User equipment reports like signal strength, failed call attempts shall be used to more accurately determine the coverage fraction. Drive tests and similar additional measurement campaigns should be avoided.

The coverage ratio / effective coverage area shall be provided for network efficiency result evaluations. It is linked to network quality and has to be defined in relation to the quality of service (QoS) definitions.

# 7	Extrapolation for global networks

A measure in each of the domains listed in Clause 4 shall be performed.

# 8	Assessment report

The results of the assessments shall be reported accurately, clearly, unambiguously and objectively, and in accordance with any specific instructions in the required method(s).

The report consists of a set of tables (Table 2, Table 3 and Table 4).

Table 2 reports the details of the Network Area under test, representing a sub-network where the measurements are conducted.

For each site reported in Table 2 the details are to be included in Table 3. Table 4 reports the measurements results for each site.

Table 2: Report of Network Area under test.

| Network Area under test |  |  |
| --- | --- | --- |
| Environmental class [Dense Urban, Urban, Suburban, Rural, Sparse] |  |  |
| Area definition [by Demography, by Geography, by Topology] |  |  |
|  | Number of inhabitants in the area [estimate] |  |
|  | Area dimensions [estimate, km2] |  |
|  | Number of sites in the Area [same BSC/RNC/Gateway?] |  |
| Type of sites in the Area |  |  |
|  | Number of “Macro” sites |  |
|  | Number of “Micro” sites |  |
|  | Number of other sites/equipment (picocells, relay nodes, etc) |  |
| Sites categorization |  |  |
|  | Number of sites in an Operator local exchange premise |  |
|  | Number of sites in buildings |  |
|  | Number of sites in a shelter |  |
|  | Number of any other sites |  |
| Multi-operator sites |  |  |
|  | Number of “single Operator” sites |  |
|  | Number of co-located multi-operators sites |  |
|  | Number of sites in “Network Sharing” mode |  |
| Multi-technology sites |  |  |
|  | Number of 2G only sites |  |
|  | Number of 3G only sites |  |
|  | Number of LTE only sites |  |
|  | Number of 2G+3G sites |  |
|  | Other options [indicate] |  |
| Backhauling information |  |  |
|  | Predominant type of backhauling [wireless, fiber, copper…] |  |
|  | Number of backhauling links per type |  |

Table 3: Report of sites under test.

| Site(s) under test in the Area (one table per site type to be measured in the Area) |  |  |
| --- | --- | --- |
| Type of site |  |  |
|  | Site “layer” [Macro, micro, other] In case of Macro, indicate number of sectors and carriers per sector |  |
|  | Site “technology” [2G, 3G, 2G+3G, LTE only, 2G+3G+LTE,other] |  |
|  | Site “operators” [single operator, co-location, network sharing, other] |  |
| Site and equipment age - Initial commission date of the site - Commission date of the current equipment in the site |  |  |
| Site infrastructure |  |  |
|  | Site location [local exchange premise, building, shelter, other] |  |
|  | Site composition |  |
|  | Air conditioners |  |
|  | Rectifiers/ batteries |  |
|  | Fixed network equipment consumption |  |
|  | Other |  |
|  | Estimated percentage of infrastructure consumption in the site |  |
| Energy Efficiency Enhancement methods affecting the site equipment during the test |  |  |
| Estimated percentage of presence of this site type in the Area |  |  |

Table 4: Report of Site measurement.

| Site measurement |  |  |
| --- | --- | --- |
| Measurement duration |  |  |
|  | Time duration of the measurement [T] |  |
|  | Measurement start date and time |  |
|  | Measurement finish date and time |  |
| Temperature class and average temperature during the test |  |  |
| Energy consumption in the site |  |  |
|  | Method of measurement [energy bills/counters, sensors, equipment information, other] |  |
|  | Estimated percentage of infrastructure consumption measured in the site |  |
|  | Measured energy consumption [kWh] |  |
|  | Week energy consumption [per week data/graph] |  |
|  | Month energy consumption [if T allows] |  |
|  | Year energy consumption [if T allows] |  |
| Traffic offered in the site |  |  |
|  | Method of measurement [operational counters, backhauling data, MDT, other] |  |
|  | Measured traffic volume [Gb, Tb..?] |  |
|  | Week traffic [per week data/graph] |  |
|  | Month traffic [if T allows] |  |
|  | Year traffic [if T allows] |  |
|  | Expected traffic threshold to evaluate availability [to be specified] |  |
| Energy efficiency [to be computed as a ratio of energy consumption and traffic] |  |  |
|  | Measured Energy Efficiency [bit/Wh?] |  |
|  | Week Energy Efficiency [per week data/graph] |  |
|  | Month Energy Efficiency [if T allows] |  |
|  | Year Energy Efficiency [if T allows] |  |

Further guidelines on the test report can be found in clause 5.10 of ISO/IEC 17025 [5].

# History

| Document history |  |  |
| --- | --- | --- |
| <Version> | <Date> | <Milestone> |
|  | January 2013 | First draft with table of contents and scope |
| V0.0.1 | June 2013 | First draft released |
| V0.0.2 | November 2013 | Second draft released (reporting tables, enhanced Clause 4) |
| V0.0.3 | December 2013 | Third draft version (minor improvements) |