3GPP TSG-SA WG6 Meeting #4	 S6-150319

Fukuoka, Japan May 25-29 2015

Title:	MCPTT Shared UE Scenario Flows Related to Temporary XDM Resources

Agenda Item:	6.4

Source:	Kodiak Networks

Contact:	Tom Hiller, thiller@kodiakptt.com,

Abstract: In order for the MCPTT Client and AS to function correctly, MCPTT users must have various MCPTT XDM documents. The contribution proposes MCPTT message flow scenarios associated with the creation of temporary XDM documents for a temporary user with a Shared MCPTT UE. The scenario does not seem to fit into existing candidate solutions procedures in section 5. The scenario follows ideas outlined in section “3.1 OMA Applications and XML Documents” of contribution S6-150058.  Note: Furthermore, one might also expect to see “temporary XDM document” delete scenarios corresponding to a temporary user signing-on with their own credentials or resetting the shared device at the end of a shift, along with yet other Shared UE scenario flows.

# 5.x.1 MCPTT Shared UE

## 5.x.1.1 XDM Resource Creation and Initialization for Temporary User

  - A temporary MCPTT user does not have XDM documents that persistently exist, as is the case of users that sign-on to the MCPTT service via their own credentials. However, for MCPTT procedures to be able to function, temporary XDMS resources must be created with default configuration data on behalf of the temporary user.

Prerequisites:

  - A temporary user has been authenticated based on a default identity stored in the Shared UE.

  - The group XDMS to which the temporary user will be added already exists and the MCPTT AS has a document change subscription to that group document.

- Summary:

  - Steps 1&2: MCPTT AS creates MCPTT XDM document for temporary user 1.  The MCPTT XDM document is initialized with default data.

  - Steps 3&4: MCPTT AS creates MCPTT Policy XDM document for temporary user 1. The MCPTT Policy XDM document is initialized with default data.

  - Step 5&6: MCPTT AS adds temporary user 1 to the group XDM document.

  - Step 7&8: The MCPTT AS subscribes to document changes to newly created MCPTT XDMS of the temporary user.

  - Steps 9&10: The MCPTT AS receives notification associated with subscription to document changes in MCPTT XDM document.

  - Steps 11&12: The MCPTT AS subscribes to document changes newly created MCPTT Policy XDMS of the temporary user.

  - Steps 13&14: The MCPTT AS receives notification associated with subscription to document changes in MCPTT Policy XDM document.

  - Step 15&16: MCPTT Client 1 creates a subscription to document changes to the newly created MCPTT XDM document.

  - Steps 17&18: MCPTT Client 1 receives notification associated with subscription to document changes in temporary MCPTT XDM document.