3GPP TSG-SA WG6 Meeting #8	S6-151354

Anaheim, USA, 16th – 20th November 2015	(revision of S6-151xxx)

Title:	Adding missing tables to Annexes B and C – data applicable to both on- and off-network

Agenda Item:	7.1.1

Source:	Vodafone

Contact:	peter.dawes@vodafone.com

Abstract: Annex B/C currently contain 30 configuration items, but not all Stage 1 configuration requirements are met. This p-CR adds the missing configuration data to Annex B/C.

This p-CR covers only items that apply to both on- and off-network. Other p-CRs are needed to cover on-network only and off-network only.

Annex B (normative):
MCPTT related data

## B.1	Data classification

Tables B.2-1, B2-2, B.3-1, B3-2, B4-1 and B.4-2 summarise the pieces of data with the reference to the introduction requirements from 3GPP TS 22.179 [2] for on-network use, related respectively to MCPTT UEs, MCPTT users and MCPTT groups. Tables BX-1 relate to the requirements valid both for on-network and for off-network (clause 5 of 3GPP TS 22.179 [2]) and tables BX-2 relate to the requirements valid for on-network (clause 6 of 3GPP TS 22.179 [2]). The various columns in the tables have the following meanings:

-	Reference: Is the reference of the corresponding requirement in 3GPP TS 22.179 [2], leading to the introduction of a given piece of data. Only one reference if given for each piece of data, even if several requirements in 3GPP TS 22.179 [2] mandates the same data or a slight variant of it.

-	Definition: A short definition of the semantics of the corresponding piece of data.

-	UE provisioned: The piece of data has to be known by the UE for the fulfilment of the corresponding stage 1 requirement. The rate of update in the UE may be low or moderate, meaning that a quasi-real time notification of a change of state to the UE is not required.

-	UE notified: The piece of data has also to be known by the UE, but a quasi-real time notification is required.

NOTE:	UE notification is assumed to use SIP signalling.

-	Network managed/enforced: This piece of data has to be known by the network. It is either been known only by the network, or it may also be known by the UE but its use is controlled and/or enforced by the network.

Editor's note: it is FFS if another SIP reference point is needed.

Editor's note: Following tables are incomplete.

## B.2	MCPTT UE related data

Table B.2-1 summarises the data related to UE. The only information which is assumed to be promptly notified to the MCPTT UE is its disabling/enabling status.

Table B.2-1: MCPTT UE related data (both on network and off network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.5.2-005 | Max number of simultaneous group calls (N4) | X |  | X (CSC-4) |
| 5.5.2-006 | Max number of transmissions (N5) in a group | X |  | X(CSC-4) |
| 5.5.2-007 | Max number of private calls(N10) | X |  | X(CSC-4) |

Table B.2-2: MCPTT UE related data (on network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified () | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 6.13.4-001 | Enabling/disabling of UE |  | X | X(CSC-4) |

## B.3	MCPTT user related data

Table B.3-1 summarises the data related to an MCPTT user.

Table B.3-1: MCPTT user related data (both on network and off network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.5.2-008 | Max number of simultaneously received group calls (N6) | X |  | X(CSC-2) |
| 5.5.2-009 | Max number of transmissions (N7) | X |  | X CSC-2) |
| 5.6.5-001 | Authorised to make private call with manual commencement | X |  | X(CSC-4) |
| 5.6.5-003 | User which can be called in private call | X |  | X(CSC-4) |
| 5.6.5-002 | Authorised to make private call with automatic commencement | X |  | X(CSC-4) |
| 5.6.5-004 | Max private call duration | X |  | X(CSC-4) |
| 5.6.5-005 | Max hang time for private calls | X |  | X(CSC-4) |
| 5.6.5-006 | Restriction in notification of call failure reason for private call | X |  | X(CSC-4) |
| 5.7.2.2.2-002 | Authorisation for imminent in- peril cancelation | X |  | X(CSC-4) |
| 5.7.2.3.1-001 | Authorisation to make an emergency private call | X |  | X(CSC-4) |
| 5.7.2.3.2-001 | Authorisation to cancel emergency priority in a private emergency call | X |  | X(CSC-4) |
| 5.7.2.4.1-002 | Authorisation to activate emergency alert | X |  | X(CSC-4) |

Table B.3-2: MCPTT user related data

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |

## B.4	Group related data

Tables B.4-1 and B4-2 summarise the data related to an MCPTT group.

Table B.4-1: MCPTT group related data (both on network and off network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.7.2.1.2-002 | Timeout value for the cancellation of an in progress emergency for a group call | ? |  | X(CSC-2) |
| 5.7.2.2.2-004 | Timeout value for the cancellation of an in progress imminent-peril group call | ? |  | X(CSC-2) |

Table B.4-2: MCPTT group related data (on network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 6.2.4-003 6.4.9-002 | Group call Hang timer |  |  | X(CSC-4) |

Table B.4-3: MCPTT related data (off network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-7.4-002] [R-7.4-003] | Group call Hang timer |  |  | X(CSC-4) |

## B.5	Service related data

Tables B.5-1, B.5-2 and B.5-3 summarise the data related to an instance of an MCPTT service.

Table B.5-1: Service related data (both on-network and off-network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.7.2.3.2-002 | Timeout value for the cancellation of an in progress emergency for a private call | X |  | X(CSC-2) |

Table B.5-2: Service related data (on-network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
|  | Hang timer for private calls |  |  |  |

Table B.5-3: Service related data (off-network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-7.4-002] [R-7.4-003] | Hang timer for private calls in off-network |  |  |  |

Annex C (informative):
Local UE settings for MCPTT

## C.1	Local UE settings for MCPTT

Table C.1-1 details local UE settings for MCPTT that represent important functionalities that may be required by MCPTT UEs, however these functionalities do not require central configuration like the parameters in annex B, so these are detailed in this annex for information to UE vendors interested in producing MCPTT UEs. The various columns in the tables have the following meanings:

-	Reference: Is the reference of the corresponding requirement in 3GPP TS 22.179 [2].

-	Definition: A short definition of the local UE setting.

Table C.1-1: Local UE settings for MCPTT

| Reference | Definition |
| --- | --- |
| 5.1.1-003 | Call reception signalling method |
| 5.1.1-004 | Disabling of call reception signalling |
| 5.8-003 | Configuration of display of User ID (on/off) |
| 5.8-003 | Configuration of display of aliases associated with the User ID (on/off) |
| 5.8-003 | Configuration of display of Selected MCPTT Group (on/off) |
| 5.8-003 | Configuration of display of Mission Critical Organization name (on/off) |
| 6.8.7.4.1-003 | Configuration of the notification of an emergency alert |

# Additional tables (to be re-located in spec.)

Table B.X-n: Group/status information

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.1.3-001] | who can be a member of specific MCPTT Groups, so that those MCPTT Groups shall be inaccessible to other users, including dispatchers or supervisors. |  |  | X (CSC-4) |

Table B.X-n: Membership/affiliation

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.1.5-004] | limit the total number (N2) of MCPTT Groups that an MCPTT User can be affiliated to simultaneously. | X |  | X (CSC-4) |

Table B.X-n: Group Call administration

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.1.6-001] | maximum duration for MCPTT Group Calls for MCPTT Users within MCPTT administrator authority. | X |  | X (CSC-4) |

Table B.X-n: Group-Broadcast Group (e.g., announcement group)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.2.2-001] | Levels of group hierarchy for Group-Broadcast Groups (B1) |  |  | X |

Table B.X-n: User-Broadcast Group (e.g., System Call)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.2.3-001] | Levels of user hierarchy for User-Broadcast Groups (B2) |  |  |  |

Table B.X-n: MCPTT Emergency Group Call

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.7.2.1.1-013] | MCPTT Group (i.e., user’s selected group or dedicated MCPTT Emergency Group) that is used for the MCPTT Emergency Group Call by an MCPTT User. |  |  |  |
| [R-5.7.2.1.2-005] | Authorize an MCPTT User to cancel in-progress Emergencies. |  |  |  |
| [R-5.7.2.2.1-009] | MCPTT Group (i.e., user’s selected group or dedicated imminent peril group) that is used for the Imminent Peril communications |  |  |  |

Table B.X-n: MCPTT Emergency Alert

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.7.2.4.1-005] | configure a geographical area for an MCPTT Emergency Alert to send a notification to MCPTT Users of the MCPTT User entering the MCPTT Emergency State, independent of the MCPTT Group Membership. |  |  |  |
| [R-5.7.2.4.1-005] | Authorized to configure a geographical area for an MCPTT Emergency Alert to send a notification to MCPTT Users of the MCPTT User entering the MCPTT Emergency State |  |  |  |
| [R-5.7.2.4.1-008] | Group (i.e., user’s selected group or dedicated MCPTT Emergency Group) or MCPTT User that is used for the MCPTT Emergency communications. |  |  |  |
| [R-5.7.2.4.1-010] [R-5.7.2.4.1-011] | MCPTT User configures, subject to MCPTT Service policy, how they are notified of an incoming MCPTT Emergency Alert (e.g., visual, audio). |  |  |  |
| [R-5.7.2.4.1-012] | MCPTT Group (i.e., user’s selected group or dedicated MCPTT Emergency Group) or MCPTT User (e.g., dispatcher) that is used for the MCPTT Emergency Alert by an MCPTT User. |  |  |  |

Table B.X-n: User ID

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.8-001] | creation and deletion of aliases for an MCPTT User and its associated user profiles by authorized parties. |  |  |  |
| [R-5.8-002] | Minimum length (N3) of an alphanumeric identifier (i.e., alias) assigned by an MCPTT Administrator. |  |  |  |

Table B.X-n: Security

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.13-012] | pre-provisioning and over-the-air provisioning of cryptographic keys |  |  |  |