3GPP TSG-SA WG6 Meeting #19	S6-171159

Dubrovnik, Croatia, 9th – 13th October 2017	(revision of S6-17xxxx)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0079 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional model for MCPTT interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCSMI |  |  |  |  |  |  | Date: |  | 2017-09-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Inclusion of gateway MCPTT server in functional model to permit interconnection, where gateway MCPTT server provides topology hiding at the application plane from interconnected MCPTT systems, and acts as a application proxy for the interconnected MCPTT system in a controlling or participating role in calls. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of gateway MCPTT server and new reference point MCPTT-10 to functional model. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interconnection to partner MCPTT system in different trust domain is not satisfied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | SIP-4 reference point specified in 23.280 CR0080 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.3.1	On-network functional model

Figure 7.3.1-1 shows the functional model for the application plane of the MCPTT service.

Figure 7.3.1-1: Functional model for application plane of the MCPTT service

In the model shown in figure 7.3.1-1, the following apply:

-	The MCPTT server is an instantiation of a MC service server in accordance with 3GPP TS 23.280 [16].

-	The MCPTT server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [9].

-	MCPTT-9 carries multicast floor control signalling between the floor control server of the MCPTT server and the floor participant of the MCPTT UE.

-	MCPTT-4 carries unicast floor control signalling between the floor control server of the MCPTT server and the floor participant of the MCPTT UE.

-	MCPTT-7 carries unicast media between the media distribution function of the MCPTT server and the media mixer of the MCPTT UE.

-	MCPTT-8 carries multicast media from the media distribution function of the MCPTT server to the media mixer of the MCPTT UE.

Figure 7.3.1-2 shows the relationships between the reference points of the application plane and the signalling plane.

Figure 7.3.1-2: Relationships between reference points of MCPTT application and signalling control planes

* * * Next Change * * * *

##### 7.4.2.3.x	Gateway MCPTT server

The gateway MCPTT server provides support for MCPTT interconnection services with a partner MCPTT system in a different trust domain, and is based on the MCPTT server described in subclause 7.4.2.3.2 of the present document.

The gateway MCPTT server functional entity does not act as a GCS AS described in 3GPP TS 23.468 [9] and does not control bearers for calls in the MCPTT system.

The gateway MCPTT server is responsible for relaying call control and floor control signalling messages, and media between MCPTT servers within the MCPTT system and the interconnected MCPTT system.

The gateway MCPTT server supports the controlling and participating role on behalf of the interconnected MCPTT system. If the interconnected MCPTT system has the controlling role in an MCPTT call, the gateway MCPTT server acts in this role within its MCPTT system on behalf of the connected MCPTT system. If an MCPTT server within MCPTT system of the gateway MCPTT server has the controlling role within an MCPTT call, the gateway MCPTT server takes a participating role on behalf of the interconnected MCPTT system.

The gateway MCPTT server does not act as the primary MCPTT server for MCPTT users, or as group host MCPTT server for MCPTT groups. However when acting in a controlling or participating role in MCPTT calls on behalf of the interconnected MCPTT system, it acts as if it were the primary MCPTT server for users receiving service in the interconnected MCPTT system and as if it were the group host MCPTT server for groups hosted in the interconnected MCPTT system.

* * * Next Change * * * *

#### 7.5.2.x	Reference point MCPTT-10 (between the gateway MCPTT server and the gateway MCPTT server in a different MCPTT system)

The MCPTT-10 reference point, which exists between the gateway MCPTT server and the gateway MCPTT server in an interconnected MCPTT system for MCPTT application signalling for establishing MCPTT sessions, shall use the SIP-4 reference point for transport and routing of signalling. Floor control signalling and media are also transferred using the MCPTT-10 reference point between interconnected MCPTT systems.

* * * End Changes * * * *