3GPP TSG-SA WG6 Meeting #19	S6-171605

Dubrovnik, Croatia, 9th – 13th October 2017	(revision of S6-171391, 171156)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0080 | rev | 2 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Signalling plane functional model for interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCSMI |  |  |  |  |  |  | Date: |  | 2017-11-20 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. Addition of reference points to permit interconnection to another MC service system. 2. Correction of common service core entities and reference points to add HTTP client function, as some CSC entities can be clients of others (e.g. key management server and group management server). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Signalling plane functional model figure replaced with figure adding new SIP and HTTP reference points to interconnected systems, and these reference points described in text. HTTP client function added to common services core, and HTTP-1 client to proxy interface added as correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Signalling plane will not support interconnection of MC systems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | SIP-4 reference point used in 23.379 CR0079 and 23.281 CR0080 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.3.1	On-network functional model

Each MC service can be represented by an application plane functional model. The functional model across MC services may be similar but is described by the individual functional entities and reference points that belong to that MC service. Within the application plane for an MC service there is a common set of functions and reference points. The common set is shared across services. This common set of functions and reference points is known as the common services core.

Figure 7.3.1-1 shows the functional model for the application plane for an MC system.

Figure 7.3.1-1: Functional model for application plane for an MC system

The common services core functions and reference points shown in figure 7.3.1-1 are shared across each MC service. The description of the functions and reference points specific to an MC service is contained in the corresponding MC service TS.

In the model shown in figure 7.3.1-1, the following apply:

-	A specific MC service server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [18].

Figure 7.3.1-2 shows the functional model for the signalling control plane.

Figure 7.3.1-2: Functional model for signalling control plane

Figure 7.3.1-3 shows the relationships between the reference points of the application plane of an MC service server and the signalling plane.

Figure 7.3.1-3: Relationships between reference points of MC service application plane and signalling control planes

NOTE 1:	Application plane reference point CSC-7 makes use of SIP-2 reference point when the group management servers are connected by a single SIP core. Where they are joined by more than one SIP core, CSC-7 also makes use of the SIP-3 reference point.

NOTE 2:	For simplicity, the HTTP proxy, which provides the interconnection between HTTP-1, HTTP-2 and HTTP-3 reference points, is not shown in figure 7.3.1-3.

NOTE 3:	CSC-5, CSC-9, and CSC-15 make use of SIP-1 and SIP-2 reference points. For simplicity, this mapping relationship is not shown in figure 7.3.1-3.

* * * Next Change * * * *

#### 7.5.3.4a	Reference point SIP-4 (between the SIP core and SIP core in another MC system)

The SIP-4 reference point is based on the SIP-3 reference point, and exists to establish SIP sessions in support of MC service to a SIP core in another MC system in another trust domain where interconnection is used.

Editor's note: If no functional difference is found between SIP-4 and SIP-3, SIP-4 can be deleted.

* * * Next Change * * * *

#### 7.5.3.7a	Reference point HTTP-4 (between the CSC entities and HTTP edge proxy)

The HTTP-4 reference point is used for HTTP transactions between entities in the common services core and the equivalent entities in another MC system in another trust domain where interconnection is used. The HTTP-4 reference point is based on the HTTP-3 reference point.

Editor's note: If no functional difference is found between HTTP-4 and HTTP-3, HTTP-4 can be removed.

* * * End Changes * * * *