3GPP TSG-SA6 Meeting #23	S6-180610

# Newport Beach, CA, USA,16th – 20th April 2018	 revision of S6-180513

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0001 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Flow name update from MCPTT call end to MCPTT private call end |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Hytera Communications Corp., Sepura plc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15, MCCI |  |  |  |  |  | Date: |  |  | 2018-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA6 agreed pCR S6-180459 to 3GPP TS 23.379 which included renaming the MCPTT call end request / response messages to MCPTT private call end request / response. This change needs to be propagated to TS 23.283 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Message name updated in the relevant information flows and descriptions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mismatch between 3GPP TS 23.379 and 3GPP TS 23.283 leading to possible confusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.4.1, 10.4.4.2, 10.4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.4.4.1	MCPTT client initiated

The procedure describes the case where an MCPTT client requests release of an ongoing MCPTT private call (with or without floor control) that was established in either of the two commencement modes (manual or automatic). This subclause is based upon the subclauses for MCPTT private call release in 3GPP TS 23.379 [7], subclauses 10.7.2.2.3.1 and 10.7.2.3.3.

Procedures in figure 10.4.4.1-1 are the basic signalling control plane procedures for the MCPTT client initiating the release of an ongoing interworked private call.

Pre-conditions:

1.	The MCPTT user on the MCPTT client is already registered for receiving MCPTT service and is involved in a private call with an LMR user via the IWF with or without floor control and established either in manual or automatic commencement mode, as described in subclause 10.4.2 and subclause 10.4.3.

Figure 10.4.4.1-1: Private call release – client initiated

1.	The user at the MCPTT client would like to release an ongoing interworked private call with the LMR user.

2.	The MCPTT client sends an MCPTT private call end request towards the MCPTT server (via SIP core) for tearing down the private call with the other client.

3.	The MCPTT server sends the corresponding IWF call end request towards the IWF, addressed to the MCPTT client ID specified in the original MCPTT private call end request.

NOTE:	The LMR user is also notified about the release of the private call. How the LMR user is notified is outside the scope of the present document.

4.	The IWF acknowledges the IWF call end request with an IWF call end response sent towards the MCPTT server.

5.	After receiving the MCPTT private call end request acknowledgement from the IWF, the MCPTT server generates an acknowledgement for the MCPTT client's MCPTT private call end request.

6.	The MCPTT client and the IWF release all the media plane resources used for the private call. Further, if the private call was established with floor control, floor control resources are released and the MCPTT client cannot make further requests for floor control or send media.

* * * Next Change * * * *

#### 10.4.4.2	MCPTT server initiated

The procedure describes the case where an MCPTT server terminates an ongoing interworked private call (with or without floor control) that was established in either of the two commencement modes (manual or automatic). The conditions causing the MCPTT server to terminate the call could include expiry of the MCPTT administrator configured maximum duration for MCPTT private calls or expiry of the maximum time permitted for an MCPTT private call without transmission/reception. This subclause is based upon the subclauses for MCPTT private call release in 3GPP TS 23.379 [7], subclauses 10.7.2.2.3.2 and 10.7.2.3.3.

Procedures in figure 10.4.4.2-1 are the basic signalling control plane procedures for the MCPTT server initiating termination of an ongoing interworked private call.

Pre-conditions:

1.	The MCPTT user on the MCPTT client is already registered for receiving MCPTT service and is involved in a private call with an LMR user via the IWF with or without floor control and established either in manual or automatic commencement mode, as described in subclause 10.4.2 and subclause 10.4.3.

Figure 10.4.4.2-1: End private call – server initiated

1.	Upon conditions to terminate call e.g., MCPTT administrator configured maximum duration for MCPTT private calls expiry or time out due to MCPTT private call without transmission/reception, the MCPTT server decides to initiate termination of an ongoing interworking private call between the MCPTT client and the LMR user.

2a.	The MCPTT server sends an MCPTT private call end request towards the MCPTT client (via SIP core) for tearing down the private call.

2b.	The MCPTT server sends a corresponding IWF call end request towards the MCPTT client identity associated with the LMR user

3.	The MCPTT user at the MCPTT client is notified about the termination of the private call.

NOTE:	The LMR user is also notified about the termination of the private call. How the LMR user is notified is outside the scope of the present document.

4.	The MCPTT client and the IWF acknowledge the request.

5.	The MCPTT client and the IWF release all the media plane resources used for the private call. Further, if the private call was established with floor control, floor control resources are released and the MCPTT client cannot make further requests for floor control or send media.

* * * Next Change * * * *

#### 10.4.4.3	LMR user initiated

The procedure describes the case where either an LMR user or the LMR system is requesting to release an ongoing interworked private call (with or without floor control) and the call established in either of the two commencement modes (manual or automatic). This subclause is based upon the subclauses for MCPTT private call release in 3GPP TS 23.379 [7], subclauses 10.7.2.2.3.1 and 10.7.2.3.3.

Procedures in figure 10.4.4.3-1 are the basic signalling control plane procedures for the LMR user, via the IWF, initiating the release of an ongoing interworked private call.

Pre-conditions:

1.	The MCPTT user on the MCPTT client is already registered for receiving MCPTT service and is involved in a private call with an LMR user via the IWF with or without floor control and established either in manual or automatic commencement mode, as described in subclause 10.4.2 and subclause 10.4.3.

Figure 10.4.4.3-1: Private call release – IWF initiated

1.	The LMR system would like to release an ongoing interworked private call with the MCPTT user.

2.	The IWF sends an IWF call end request towards the MCPTT server for tearing down the private call with the MCPTT client.

3.	The MCPTT server sends the corresponding MCPTT private call end request towards the MCPTT client specified in the original IWF call end request.

4.	The MCPTT user is notified about the release of the private call.

5.	The MCPTT client acknowledges the MCPTT private call end request.

6.	After receiving the MCPTT private call end request acknowledgement from the MCPTT client, the MCPTT server generates an acknowledgement for the IWF's IWF call end request.

7.	The MCPTT client and the IWF release all the media plane resources used for the private call. Further, if the private call was established with floor control, floor control resources are released and the MCPTT client cannot make further requests for floor control or send media.