3GPP TSG-SA WG6 Meeting #26	S6-181357

Vilnius, Lithuania, 15th – 19th October 2018	(revision of S6-18xxxx)

Source:	Motorola Solutions

Title:	pCR Scenario for logging

Spec:	3GPP TR 23.784 v020

Agenda item:	10.5

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

1. Introduction

This pCR adds a generic scenario for logging to TR 23.784.

2. Reason for Change

TR 23.784 contains a set of scenarios for discreet listening. It is not necessary to reproduce scenarios for logging for every possible service, but a single scenario can be introduced so that the differences between discreet listening and logging can be understood.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.784 v020.

* * * First Change * * * *

## 4.x	Scenario x:	Logging scenarios

### 4.x.1	General

According to the requirements for logging in 3GPP TS 22.280 [2] and 3GPP TS 22.281 [3] which are also reproduced in Annex A of the present document, mechanisms are to be provided to log both media and metadata for call related and call unrelated activity of the target MC service users and MC service groups.

The call related scenarios can be considered to be the same as those described in scenarios 1 to 6 related to discreet listening, except that the recipient of the information is a logging client or interface instead of the authorized user.