3GPP TSG-SA WG6 Meeting #28	S6-190257

Kochi, India, 21st – 25th January 2019	(revision of S6-190118)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0023 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCPTT ID in interworking floor control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCCI |  |  |  |  |  |  | Date: |  | 2019-01-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Floor control messages between IWF and MCPTT server need to include the MCPTT ID of the relevant user, otherwise the receipient server is not able to distinguish which party is indicated by the sending server in situations such as a race condition, or where the first requests are sent before receiving a response to the first request or where a queue position request or information is sent for a particular user. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add MCPTT ID in seven relevant floor control messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Floor control will not operate correctly in interworking in all cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.2.3, 10.5.2.4, 10.5.2.6, 10.5.2.9, 10.5.2.11, 10.5.2.13, 10.5.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.5.2.3	IWF floor granted

Table 10.5.2.3-1 describes the information flow IWF floor granted, from the IWF to the MCPTT floor control server and from the MCPTT floor control server to the IWF, which is used to indicate that a request for floor is granted and media transfer is possible.

Table 10.5.2.3-1: IWF floor granted

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Granted party identity |
| Duration | M | The time for which the granted party is allowed to transmit |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgement from the floor participant is required |

* * * Second Change * * * *

#### 10.5.2.4	IWF floor rejected

Table 10.5.2.4-1 describes the information flow IWF floor rejected, from the IWF to the MCPTT floor control server and from the MCPTT floor control server to the IWF, which is used to indicate that a request for the floor is rejected.

Table 10.5.2.4-1: IWF floor rejected

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Rejected party identity |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Rejection cause | O | Indicates the cause for floor rejection |
| Acknowledgement required | O | Indicates if acknowledgement from the floor participant is required |

* * * Third Change * * * *

#### 10.5.2.6	IWF floor request cancel response

Table 10.5.2.6-1 describes the information flow IWF floor request cancel response, from the IWF to the MCPTT floor control server and from the MCPTT floor control server to the IWF, which is used to indicate the response for the floor request cancellation.

Table 10.5.2.6-1: IWF floor request cancel response

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of party that initiated the cancellation request |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgement from the floor participant is required |

* * * Fourth Change * * * *

#### 10.5.2.9	IWF floor release

Table 10.5.2.9-1 describes the information flow IWF floor release, from the IWF to the MCPTT floor control server and from the MCPTT floor control server to the IWF, which is used to indicate the media transfer is completed and the floor is released.

Table 10.5.2.9-1: IWF floor release

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of party that released the floor |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

* * * Fifth Change * * * *

#### 10.5.2.11	IWF floor revoked

Table 10.5.2.11-1 describes the information flow IWF floor revoked, from the IWF to the MCPTT floor control server and from the MCPTT floor control server to the IWF, which is used to indicate the floor is revoked from its current holder (the floor participant who was granted the floor).

Table 10.5.2.11-1: IWF floor revoked

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Revoked party identity |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgement from the floor participant is required |

* * * Sixth Change * * * *

#### 10.5.2.13	IWF queue position request

Table 10.5.2.13-1 describes the information flow IWF queue position request, from the IWF to the MCPTT floor control server and from the MCPTT floor control server to the IWF, which is used to request the position in the floor request queue. The MCPTT server and the MCPTT client that support queuing of the floor control requests shall support this information flow.

Table 10.5.2.13-1: IWF queue position request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of party whose floor position is requested |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

* * * Seventh Change * * * *

#### 10.5.2.14	IWF queue position info

Table 10.5.2.14-1 describes the information flow IWF queue position info, from the IWF to the MCPTT floor control server and from the MCPTT floor control server to the IWF, which is used to indicate the floor request is queued and the queue position to the floor requesting UE.

Table 10.5.2.14-1: IWF queue position info

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of party whose floor position is provided |
| Queue position info | M | Position of the queued floor request in the queue |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgement from the floor participant is required |