3GPP TSG-SA WG6 Meeting #35	S6-200332

- Hyderabad, India, 13th - 17th Jan 2020											(revision of S6-200314)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0246 | rev | 5 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add announced call redirection for MCPTT private calls |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Kontron Transportation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  |  | Date: |  | 2020-01-16 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The existing redirection does not meet all the operational requirements for rail usage. Therefore this contribution adds the option of announced redirection for private calls to fully support requirements [R-5.6.3-014] and [R-6.7.4-015]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add add new procedure for MCPTT private call announced redirection. Change name from transfer to redirection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Important functionality for railway users is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.7.2.1.1, 10.7.5.2, 10.7.6.2.1, 10.7.6.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.7.5.2	Procedures

* * * End of Change * * * *

* * * Next Change * * * *

##### 10.7.2.1.1	MCPTT private call request (MCPTT client to MCPTT server)

Table 10.7.2.1.1-1 describes the information flow MCPTT private call request from the MCPTT client to the MCPTT server.

Table 10.7.2.1.1-1: MCPTT private call request (MCPTT client to MCPTT server) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID (see NOTE) | O | The MCPTT ID of the called party |
| Functional alias (see NOTE) | O | The functional alias of the called party |
| Use floor control indication | M | This element indicates whether floor control will be used for the private call. |
| SDP offer | O | Media parameters of MCPTT client. |
| Requested commencement mode | O | An indication that is included if the user is requesting a particular commencement mode |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| Location information | O | Location of the calling party |
| Requested priority | O | Application priority level requested for this call |
| Redirection indicator | O | Indicates that the MCPTT private call request is a result of a call redirection.(true/false) |
| NOTE: At least one identity must be present. |  |  |

* * * End of Change * * * *

* * * Next Change * * * *

##### 10.7.6.2.1	MCPTT private call unannounced redirection

The procedure for MCPTT private call unannounced redirection covers the case where an MCPTT client requests an ongoing MCPTT private call (with or without floor control) to be redirected to another MCPTT user without prior announcement.

Figure 10.7.6.2.1-1 below illustrates the procedure for MCPTT private call unannounced redirection.

Pre-conditions:

1.	MCPTT client 2 is authorized to use call redirection.

2.	MCPTT client 1 is authorized to make private calls to client 2.

3.	MCPTT client 2 is authorized to redirect private calls to MCPTT client 3.

4.	MCPTT client 1 and MCPTT client 3 are in the same security domain.

Figure 10.7.6.2.1-1: MCPTT private call unannounced redirection

1.	MCPTT client 1 initiates an MCPTT private call to MCPTT client 2 using the normal MCPTT call establishment procedures  (10.7.2.2). The user at MCPTT client 1 can talk with the user at MCPTT client 2.

2.	Now the MCPTT user at MCPTT client 2 decides to perform a call redirection.

3.	The MCPTT client 2 sends a MCPTT call redirectionrequest to the MCPTT server.

4.	The MCPTT server verifies that MCPTT client 2 is authorized toredirect the MCPTT private call to MCPTT client 3.

NOTE 1:	The authorization check in step 4 verifies that the MCPTT user at MCPTT client 2 is allowed to redirectprivate calls to the MCPTT user at MCPTT client 3.

5.	The MCPTT server sends an MCPTT private call redirectionresponse back to MCPTT client 2 indicating the outcome of the verification.

6.	If authorized, the MCPTT server performs a server initiated private call release of the call between MCPTT client 1 and MCPTT client 2 (10.7.2.2.3.2).

7.	The MCPTT server sends a MCPTT call redirectionrequest towards the MCPTT client 1.

8.	The user at MCPTT client 1 is notified that a call redirectionis in progress.

9.	MCPTT client 1 sends a MCPTT  call redirectionresponse back to the MCPTT server.

10.	MCPTT client 1 sends a MCPTT private call request towards the MCPTT server that includes a call redirectionindication set to true.

Editor's note: Checking if MCPTT client 3 is in the whitelist of MCPTT client 1 is FFS.

11.	The MCPTT server verifies that client 1 is authorized to perform the MCPTT private call as a result of the MCPTT private call redirectionrequest based on the fact that the redirection indication is present and set to true in the MCPTT private call request.

NOTE 2:	For call redirectionthe MCPTT server does not check if the initial originating MCPTT user at MCPTT client 1 is authorized to make an MCPTT private call to the final target MCPTT user at MCPTT client 3.

12.	The MCPTT server sends a MCPTT call request to MCPTT client 3.

13.	The user at MCPTT client 3 is notified about the incoming call.

14.	MCPTT client 3 sends an MCPTT private call response back to the MCPTT server.

15.	The MCPTT server forwards the MCPTT private call response towards client 1.

16.	The media plane for communication between client 1 and 3 is established.

##### 10.7.6.2.2	MCPTT private call announced redirection

The procedure for MCPTT private call announced redirection covers the case where an MCPTT client requests an ongoing MCPTT private call (with or without floor control) to be redirected to another MCPTT user with prior announcement.

Figure 10.7.6.2.2-1 below illustrates the procedure for MCPTT private call announced redirection.

Pre-conditions:

1.	MCPTT client 2 is authorized to use call redirection.

2.	MCPTT client 1 is authorized to make private calls to client 2.

3.	MCPTT client 2 is authorized to make private calls to client 3.

4.	MCPTT client 2 is authorized to redirect private calls to MCPTT client 3.

5.	MCPTT client 2 supports simultaneous sessions for MCPTT calls (10.8).

6.	MCPTT client 1 and MCPTT client 3 are in the same security domain.

Figure 10.7.6.2.2-1: MCPTT private call announced redirection

1.	MCPTT client 1 initiates an MCPTT private call to MCPTT client 2 using the normal MCPTT call establishment procedures  (10.7.2.2). The user at MCPTT client 1 can talk with the user at MCPTT client 2. The user at MCPTT client 2 decides to redirect the call.

2.	The MCPTT user at MCPTT client 2 puts the call locally on hold.

3.	MCPTT client 2 initiates an MCPTT private call to MCPTT client 3 using the normal MCPTT call establishment procedures  (10.7.2.2).

4.	The user at MCPTT client 2 can talk with the user at MCPTT client 3 and announce the call redirection.

5.	The MCPTT client 2 releases the MCPTT private call with  MCPTT client 3 using the normal MCPTT call release procedure  (10.7.2.2.3.1).

6.	Optionally the MCPTT user at MCPTT client 2 puts the call with client 1 locally off hold and confirms that the call will be redirected.

7.	Now the MCPTT user at MCPTT client 2 decides to perform a call redirection.

8.	The MCPTT client 2 sends a MCPTT call redirection request to the MCPTT server.

9.	The MCPTT server verifies that MCPTT client 2 is authorized to redirect the MCPTT private call to MCPTT client 3.

NOTE 1:	The authorization check in step 9 verifies that the MCPTT user at MCPTT client 2 is allowed to redirect private calls to the MCPTT user at MCPTT client 3.

10.	The MCPTT server sends an MCPTT private call redirection response back to MCPTT client 2 indicating the outcome of the verification.

11.	If authorized, the MCPTT server performs a server initiated private call release of the call between MCPTT client 1 and MCPTT client 2 (10.7.2.2.3.2).

12.	The MCPTT server sends a MCPTT call redirection request towards the MCPTT client 1.

13.	The user at MCPTT client 1 is notified that a call redirection is in progress.

14.	MCPTT client 1 sends a MCPTT call redirection response back to the MCPTT server.

15.	MCPTT client 1 sends a MCPTT private call request towards the MCPTT server that includes a call redirection indication set to true.

Editor's note: Checking if MCPTT client 3 is in the whitelist of MCPTT client 1 is FFS.

16.	The MCPTT server verifies that client 1 is authorized to perform the MCPTT private call as a result of the MCPTT private call redirection request.

NOTE 2:	For call redirection the MCPTT server does not check if the initial originating MCPTT user at MCPTT client 1 is authorized to make an MCPTT private call to the final target MCPTT user at MCPTT client 3 based on the fact that the redirection indication is present and set to true in the MCPTT private call request.

17.	The MCPTT server sends a MCPTT call request to MCPTT client 3.

18.	The user at MCPTT client 3 is notified about the incoming call.

19.	MCPTT client 3 sends an MCPTT private call response back to the MCPTT server.

20.	The MCPTT server forwards the MCPTT private call response towards client 1.

21.	The media plane for communication between client 1 and 3 is established.

* * * End of Changes * * * *