3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201745

e-meeting, 12th – 20th October 2020	(revision of S6-20xxxx)

Source:	Tencent

Title:	Pseudo-CR on key issue x: UAS Identification management

Spec:	3GPP TR 23.755

Agenda item:	8.5

Document for:	Approval

Contact:	shuaiizhao@tencent.com

1. Introduction

A UAS may connect to a USS/UTM using a 3GPP network, wifi, internet, or other means of networking approach. Whichever way the connection happens, a USS/UTM may only use one identifier for communicating with the UAS. In North America, the FAA has nearly adopted ASTM [19]’s remote ID proposal. The current IETF DRIP WG [18] is making great progress to make such a UAS remote ID feasible in either a wireless or wireless network environment. In 3GPP’s perspective, such RID is called a CAA-level UAV ID.

Based on TR 23.754 architecture’s assumption:

- “A UAV can be replaced by another from a UAS.”

- “A UAV Controller can be removed from a UAS and replaced with another UAV Controller or a TPAE.”

- “The USS/UTM accesses 3GPP services (e.g. location services) for a UAV corresponding to the CAA-level UAV Identity by using the 3GPP UAV Identity (i.e. the GPSI).”

Two IDs may be associated with one UAS, namely a CAA-Level ID (ex: IETF DRIP RID) and 3GPP UE ID (ex GPSI). Each id may be used separately or as a combination for the UAS tracking and management purpose, depending on the presence of each ID.

The current SA6 UAS architecture enables direct communication between the UAS application layer and USS/UTM using SEAL [9].  Once a 3GPP network connection has been successfully made, the subsequent communication between the UAS application layer and USS/UTM may solely rely on the ID(s) assigned either by the 3GPP network or a USS/UTM or third party UAS ID registries or a combination of both.  Regulations regarding the usage of CAA-level UAV ID may be found in the FAA [17] and [18].

Therefore, there is a need to work on UAS ID management within the UAS application layer based on the situations mentioned in SA2’s architecture assumption.

Editor’s notes: the assignment of either CAA-Level ID or 3GPP UE ID is outside of SA6’s scope.

2. Reason for Change

New KI

3. Conclusions

This proposal identifies a new KI relating to UAS ID management in the application layer.

4. Proposal

It is proposed to agree on the following changes to 3GPP TR 23.755

* * * First Change * * * *

## 5.x 	Key issue #X:  UAS Identification management

Once a 3GPP network connection has been successfully made, the subsequent communication between the UAS application layer and USS/UTM may solely rely on the ID(s) assigned either by the 3GPP network or a USS/UTM or third party UAS ID registries or a combination of both. Base on TR 23.754 architecture’s assumption, components of a UAS may be changed/updated. Therefore, a UAS ID may also be updated along the process.

The current SA6 UAS architecture enables direct communication between the UAS application layer and USS/UTM using SEAL [9]. Therefore, SEAL may need to be able to support the UAS ID(s) update to provide a seamless service to the UAS application layer.

Editor’s notes: the assignment of either CAA-Level ID or 3GPP UE ID is outside of SA6’s scope.

Hence, it is required to study the following:

- Whether and how the UAE layer can manage the association between 3GPP UE ID and CAA-level UAV ID.

- Whether and how SEAL can support dynamic UAS ID(s) update.

- Whether and how SEAL can be enhanced to support communication between UAS and USS/UTM using either IDs or a combination of both.