3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201746r01

e-meeting, 12th Oct – 20th Oct 2020

Source:	vivo, OPPO, Xiaomi

Title:	Clarification to Edge Configuration Server discovery

Spec:	3GPP TS 23.558 v1.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Lyu Huazhang <Huazhang.lv at vivo.com>

1. Introduction

There are several mechanisms to provision ECS address information to the UE, including provision it by MNO through 5GC procedure.

However, upon reception of ECS address information received from the 5GC, in order to delivery it to EEC, the OS of the UE needs enhancement and requires a new UE capability.

2. Reason for Change

This is unlike the user plane data (application layer date) can be delivered via existing OS to application client (transparent transmission). Existing UE doesn’t support the capability. Each UE’s OS is specific designed and the explicit ECS information need the OS enhancement to support the interworking between the OS and edge computing client.

It is not guaranteed all the UE with EEC support this capability.

Edge computing service is provided by MNO or 3rd parties, the service doesn’t know whether the UE supports this capability and MNO or 3rd party also can’t guarantee all of the UE owns this capability.

Hence, the ECS address information provisioned by MNO through 5GC procedure is not feasible for the UE without the capability for OS enhancement. If the UE without the capability for OS enhancement, the other ways which has no impact to UE to provide ECS address information to EEC shall be applied.

3. Conclusions

Adding an explaination to further describe that it may be possible to provision the ECS address information by MNO through 5GC procedure to EEC only if the UE OS have the capability to deliver ECS address information to EEC on the UE. If UE doesn’t have this capability, the other ways which has no impact to UE to provide ECS address information to EEC shall be applied.

4. Proposal

ECS address information can be provisioned by MNO through 5GC procedure only if the UE OS have the capability.Add an explaination: It may be possible to provision the ECS address information to the EEC from the 5GC only if the UE OS have the capability to deliver ECS address information to EEC on the UE.If UE doesn’t have this capability, the other ways which has no impact to UE to provide ECS address information to EEC shall be applied.

- * * * * First change * * * *

#### 8.3.2.2	Edge Configuration Server discovery

ECS address information can be pre-configured with the EEC, configured by an edge-aware Application Client, configured by the user, provisioned by MNO through 5GC procedure only if the UE OS have the capability, or derived from HPLMN identifier for non-roaming scenario or from VPLMN identifier for roaming scenario.

Editor’s Note: 5GC provision of ECS configuration information to the UE is in scope of SA2.

NOTE: How the ECS address information is configured to the EEC by the AC, user, or pre-configuration is out of scope of the present specification.

If the ECS configuration information is provided by 5GC, the EEC shall use the information for the initial provisioning request. Otherwise, the EEC shall use pre-configured ECS address for the initial provisioning if ECS configuration information is preconfigured with the EEC.

It may be possible to provision the ECS address information to the EEC from the 5GC only if the UE OS have the capability to deliver ECS address information to EEC on the UE. If UE doesn’t have this capability, the other ways which has no impact to UE to provide ECS address information to EEC shall be applied.

Editor’s Note: It is FFS whether ECS address can be derived from HPLMN or VPLMN identifier when no ECS address information is available.

- * * * * End of changes * * * *