3GPP TSG-SA WG6 Meeting #39-e	S6-201466

e-meeting, 31st August – 8th September 2020	(revision of S6-20xxxx)

Source:	Samsung Electronics

Title:	Pseudo-CR on Consistency - User plane path management API

Spec:	3GPP TS 23.558 v0.4.0

Agenda item:	7.5

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution updates the user plane path management API to make it consistent with the approach of defining individual service operations. It also removes normative text from the general clause, aligns the structure of clause 8.6.3, fixes the information flow name in the notify operation description and update the APIs accordingly.

2. Reason for Change

Current version defines the API with subscribe, subscription update and unsubscribe operations conflated in a single procedure and clause 8.6.3 requires multiple editorial fixes.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.4.0.

* * * First Change * * * *

### 8.6.3	User plane path management events

#### 8.6.3.1	General

The Edge Enabler Server exposes user plane path management event notifications of an UE to an Edge Application Server (e.g. in order to trigger the application context relocation). User plane path management event notifications API exposed by the Edge Enabler Server may rely on the NEF northbound API for monitoring event of user plane path management event.

The availability of the user plane path management event notifications may change due to UE mobility between 5GC and EPC. If an Edge Enabler Server re-exposes the northbound API for user plane path management notifications of 3GPP Core Network to Edge Application Server(s) for a UE, the Edge Enabler Server monitors the availability of the northbound API for UE(s) served by the Edge Application Server (e.g. by utilizing Nnef_APISupportCapability as in 3GPP TS 23.502 [3]) and provides the availability information to the Edge Application Server. If CAPIF is supported, the EES determines if the user plane path management event API is available and able to be exposed to the Edge Application Server for a UE via the Availability of service APIs event notifications provided by the CAPIF core function as in 3GPP TS 23.222 [6].

#### 8.6.3.2	Procedures

##### 8.6.3.2.1	Subscribe

Figure 8.6.3.2.1-1 illustrates the subscribe operation between the Edge Application Server and the Edge Enabler Server for user plane path management event notifications.

Figure 8.6.3.2.1-1: User plane path management event API: Subscribe operation

1.	The Edge Application Server sends user plane path management event subscribe request for tracking the UE's user plane path change continuously. The Edge Application Server shall include UE Identifier and

may include Subscription Type (Early and/or Late notification defined in clause 5.6.7 of 3GPP TS 23.501 [2]) and/or Indication of EAS Acknowledgement.

2.	The Edge Enabler Server checks if the Edge Application Server is authorized for this operation. If authorized, Edge Enabler Server checks if there exists a subscription with the 3GPP core network for the user plane path management event notifications corresponding to the UE information obtained in step 1 as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], which may be triggered by other Edge Application Server for the same UE. The Edge Enabler Server checks the availability of the user plane path management event service for the UE.

a.	if a subscription with 3GPP core network does not exist, then the Edge Enabler Server subscribes with the 3GPP core network (PCF, NEF or SCEF+NEF) for the user plane path management event notifications of the UE as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3] If the Edge Application Server provides Subscription Type and/or Indication of EES Acknowledgement, the Edge Enabler Server include the type of subscription the indication of "AF acknowledgement to be expected" as information on AF subscription to corresponding SMF events within the AF Request;

b.	if a subscription with 3GPP core network exists, then the Edge Enabler Server uses the locally cached user plane path management event notification information of the UE to respond to the Edge Application Server.

The Edge Enabler Server stores the subscription related to the Edge Application Server.

Editor's note: It is FFS how EES determines if User plane path management event API is available for a UE.

3.	If Edge Application Server is authorized, the Edge Enabler Server responds with user plane path management event subscribe response. If Edge Application Server is not authorized, the Edge Enabler Server provides a rejection response with cause information.

If the target UE and the 3GPP network support mobility between 5GC and EPC, the Edge Enabler Server monitors the availability of the user plane path management event notification from the 3GPP network by utilizing Nnef_APISupportCapability or Availability of service APIs event notifications provided by the CAPIF core function.

##### 8.6.3.2.2	Notify

Figure 8.6.3.2.2-2 illustrates the notify operation between the Edge Enabler Server and the Edge Application Server for continuous User plane path management event notifications.

Figure 8.6.3.2.2-1: User plane path management event API: Notify operation

1.	The Edge Enabler Server detects the user plane path management event of the UE (e.g. receiving User plane path management event notification for the UE from the 3GPP core network). The Edge Enabler Server may cache the detected User plane path management event notification locally with timestamp as the latest information of the UE. The Edge Enabler Server determines to notify the user plane path management event notification information (e.g., DNAI) to the Edge Application Servers which has subscribed for the user plane path management event.

2.	The Edge Enabler Server sends user plane path management event notification to the Edge Application Server. The Edge Enabler Server includes the user plane path management event notification information of the UE and optionally the timestamp. The timestamp can be included to indicate the age of the user plane path management event notification information. The Edge Enabler Server may only provide part of information included in the user plane path management event notification from 3GPP network (e.g. DNAI). If the Edge Enabler Server had provided indication of "AF acknowledgement to be expected" to 3GPP network as in clause 4.3 of 3GPP TS 23.502 [03], the Edge Enabler Server waits for acknowledgement from the EAS before it sends AF acknowledgement to the 3GPP core network.

##### 8.6.3.2.3	Subscription update

Figure 8.6.3.2.3-1 illustrates the subscription update operation between the Edge Application Server and the Edge Enabler Server for user plane path management event notifications.

Figure 8.6.3.2.3-1: User plane path management event API: Subscription update operation

1.	The Edge Application Server sends user plane path management event subscription update request to update an existing subscription. The subscription update request may include Subscription Type (Early and/or Late notification defined in clause 5.6.7 of 3GPP TS 23.501 [2]) and/or Indication of EAS Acknowledgement.

2.	The Edge Enabler Server checks if the Edge Application Server is authorized for the operation. If authorized, Edge Enabler Server checks if there exists a subscription with the 3GPP core network for the user plane path management event notifications corresponding to the updated information obtained in step 1 as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], which may be triggered by other Edge Application Server for the same UE.

a.	if a subscription with 3GPP core network does not exist corresponding to the updated information, then the Edge Enabler Server subscribes with the 3GPP core network (PCF, NEF or SCEF+NEF) for the user plane path management event notifications of the UE as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3] If the Edge Application Server provides Subscription Type and/or Indication of EES Acknowledgement, the Edge Enabler Server includes the type of subscription the indication of "AF acknowledgement to be expected" as information on AF subscription to corresponding SMF events within the AF Request;

b.	if a subscription with 3GPP core network exists corresponding to the updated information, then the Edge Enabler Server uses the locally cached user plane path management event notification information of the UE to respond to the Edge Application Server.

The Edge Enabler Server stores the updated subscription related to the Edge Application Server.

3.	The Edge Enabler Server responds with user plane path management event subscription response.

##### 8.6.3.2.4	Unsubscribe

Figure 8.6.3.2.4-1 illustrates the unsubscribe operation between the Edge Application Server and the Edge Enabler Server for user plane path management event notifications.

Figure 8.6.3.2.4-1: User plane path management event API: Unsubscribe operation

1.	The Edge Application Server sends user plane path management event unsubscribe request to the Edge Enabler Server to cancel the user plane path management event subscription.

2.	The Edge Enabler Server checks if another EAS requires to track the UE's user plane path change or not. If not, the Edge Enabler Server unsubscribes the user plane path management event notifications from the 3GPP core network for the corresponding UE.

3.	The Edge Enabler Server checks if the Edge Application Server is authorized for the operation. If authorized, the Edge Enabler Server removes the subscription related to the Edge Application Server and sends user plane path management event unsubscribe response to the Edge Application Server.

#### 8.6.3.3	Information flows

##### 8.6.3.3.1	User plane path management event subscribe request

Table 8.6.3.3.1-1 describes the information elements for a user plane path management event subscribe request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.3.3.1-1: User plane path management event subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the UE (i.e. GPSI or identity token) |
| Event ID(s) | M | Event ID(s) as specified in 3GPP TS 23.501 [2] |
| Event Report | M | Event Reporting Information as specified in 3GPP TS 23.502 [3] |
| Notification Target Address | M | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Type of subscription | O | Indicates Early and/or Late notification to inform if the notification needs to be received before and/or after UP path configuration. |
| Indication of EAS acknowledgement | O | This IE indicates the EES to include indication of "AF acknowledgement to be expected" within the AF request for subscribing UP path management events to 3GPP network and that the EAS will provide an acknowledgement as a response for the notifications of UP path management events to the EES. |
| Event Filter | O | Event filter as specified in 3GPP TS 23.501 [2] |

Editor's note:	The detailed applicable information in "Event Report" is FFS.

##### 8.6.3.3.2	User plane path management event subscribe response

Table 8.6.3.3.2-1 describes the information elements for a user plane path management event subscribe response from the Edge Enabler Server to the Edge Application Server.

Table 8.6.3.3.2-1: User plane path management event subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. If the Expiration time IE is not included, it indicates that the subscription never expires. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

##### 8.6.3.3.3	User plane path management event notification

Table 8.6.3.3.3-1 describes the information elements for a user plane path management event notification from the Edge Enabler Server to the Edge Application Server.

Table 8.6.3.3.3-1: User plane path management event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |
| Event report | M | Event reporting information as specified in clause 5.2.8.3.1 of 3GPP TS 23.502 [3] |

##### 8.6.3.3.4	User plane path management event subscription update request

Table 8.6.3.3.4-1 describes the information elements for a user plane path management event subscribe request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.3.3.4-1: User plane path management event subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | The identifier of the subscription to be updated. |
| Event ID(s) | O | Event ID(s) as specified in 3GPP TS 23.501 [2] |
| Event Report | O | Event Reporting Information as specified in 3GPP TS 23.502 [3] |
| Type of subscription | O | Indicates Early and/or Late notification to inform if the notification needs to be received before and/or after UP path configuration. |
| Indication of EAS acknowledgement | O | This IE indicates the EES to include indication of "AF acknowledgement to be expected" within the AF request for subscribing UP path management events to 3GPP network and that the EAS will provide an acknowledgement as a response for the notifications of UP path management events to the EES. |
| Event Filter | O | Event filter as specified in 3GPP TS 23.501 [2] |

##### 8.6.3.3.5	User plane path management event subscription update response

Table 8.6.3.3.5-1 describes the information elements for a user plane path management event subscription update response from the Edge Enabler Server to the Edge Application Server.

Table 8.6.3.3.5-1: User plane path management event subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription update request was successful. |
| > Expiration time (NOTE) | O | Indicates the expiration time of the updated subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure |
| NOTE: If the Expiry time IE is not included, it indicates that the subscription never expires. |  |  |

##### 8.6.3.3.6	User plane path management event unsubscribe request

Table 8.6.3.3.6-1 describes the information elements for a User plane path management event unsubscribe request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.3.3.6-1: User plane path management event unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |

##### 8.6.3.3.7	User plane path management event unsubscribe response

Table 8.6.3.3.7-1 describes the information elements for a User plane path management event unsubscribe response from the Edge Enabler Server to the Edge Application Server.

Table 8.6.3.3.7-1: User plane path management event unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the unsubscribe request was successful. |
| Failure response | O | Indicates that the unsubscribe request failed. |
| > Cause | O | Indicates the cause of unsubscribe request failure |

8.6.3.4	APIs

##### 8.6.3.4.1	General

Table 8.6.3.4.1-1 illustrates the API for user plane path management event.

Table 8.6.3.4.1-1: User plane path management API

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| EDGE3_User_plane_management_event API | Subscribe | Subscribe/Notify | Edge Application Server |
|  | Notify | Subscribe/Notify | Edge Application Server |
|  | Update | Subscribe/Notify | Edge Application Server |
|  | Unsubscribe | Subscribe/Notify | Edge Application Server |

##### 8.6.3.4.2	EDGE3_User_plane_management_event_subscribe operation

Service operation name: EDGE3_User_plane_management_event_subscribe

Description: The consumer subscribes to receive a user plane management event.

Inputs: See clause 8.6.3.3.1.

Outputs: See clause 8.6.3.3.2.

See clause 8.6.3.2.1 for details of usage of this operation.

##### 8.6.3.4.3	EDGE3_User_plane_management_event_notify operation

Service operation name: EDGE3_User_plane_management_event_notify

Description: The consumer is notified of an event by the Edge Enabler Server.

Inputs: See clause 8.6.3.3.3.

Outputs: None.

See clause 8.6.3.2.2 for details of usage of this operation.

##### 8.6.3.4.4	EDGE3_User_plane_management_event_update operation

Service operation name: EDGE3_User_plane_management_event_update

Description: The consumer updates an existing subscription for a user plane management event.

Inputs: See clause 8.6.2.3.4.

Outputs: See clause 8.6.2.3.5.

See clause 8.6.3.2.3 for details of usage of this operation.

##### 8.6.3.4.5	EDGE3_User_plane_management_event_unsubscribe operation

Service operation name: EDGE3_User_plane_management_event_unsubscribe

Description: The consumer unsubscribes for the previously subscribed events.

Inputs: See clause 8.6.3.3.6.

Outputs: See clause 8.6.3.3.7.

See clause 8.6.3.2.4 for details of usage of this operation.