3GPP TSG-SA WG6 Meeting #41-e	 S6-210356

e-meeting, 18th – 26th January 2021	(revision of S6-210165)

Source:	Tencent

Title:	Solution to Key issue #9: Support for UAS media application session monitoring and management

Spec:	3GPP TR 23.755 v0.11.0

Agenda item:	8.3

Document for:	Approval

Contact:	Shuai Zhao (Shuaiizhao AT Tencent DOT com)

1. Introduction

Following up on the Key Issue#9, with regards to the need to address UAS media session monitoring and management, this contribution proposes a potential solution so as to cover the application layers aspects of the KPI defined 7.1 of 3GPP TS 22.125.

2. Reason for Change

In most of the real-time media communications, the combination of SIP and SDP is used for session initialization and media parameter negotiation. Both protocols data are carried in a control plane prior to the media session. in the user plane.  SDP is mostly carried as a SIP payload which includes all media codec-related parameters agreed on by both parties.

Since SEAL offers SIP session management for verticals, it can be used for the UAS application layer monitoring and management of media sessions. With such a support, certain KPIs listed in Table 7.1-1 of Stage 1 requirement are fulfilled.

3. Proposal

It is proposed to include the proposed solution below into to 3GPP TR 23.755 v0.12.0.

* * * First Change * * * *

## 8.X	Solution #x – Support for UAS media application session management

## 8.x.1 	Solution description

In most real-time media communications, the combination of SIP and SDP is used for session initialization and media parameter negotiation. Both protocol data are carried in the control plane before real media traffic goes through the network.  SDP is mostly carried as a SIP payload which includes all media codec related parameters. The purpose of SDP is to convey information about media streams and provide sufficient information to enable joining and participating to a media session in a unicast scenario.

One of the important information carried in an SDP is the potential bandwidth usage for a particular media stream. For the use case of a UAV media-related payload, the knowledge of bandwidths that payload intends to use is important for resource allocation in the 3GPP network.

This solution illustrates the workflow regarding how the UAE server (acts as VAL server) requests SEAL NRM server to fulfill the session establishment requests with the following pre-conditions:

- A UAV tries to establish a SIP session between UAV media payload with UAV-C or USS/UTM using the 3GPP core network via the UAE server with the indication of resource requirements for the media session using SIP SDP.

Figure 8.x.1-1 illustrates scenarios in which a SIP session may be terminated immediately if a SEAL NRM server determines that there is not enough network bandwidth for a particular SDP description. Otherwise, SIP session may be established.

Figure 8.x.1-1: SIP session management based on network resource requirement

- The UAE server requests media session initialization using SDP in the SIP payload. The NRM server evaluates the request as specified in clause 14.3.3.2 of TS 23.434.

- The NRM server may choose to deny the SIP session request due to a lack of available network resources. Or in another case, the NRM server may determine the session resources can be granted from the SIP core by using clause 14.3.3.2 of TS 23.434.

### 8.x.2	Solution evaluation

This solution provides a viable technical workflow to manage media SIP sessions based on bandwidth indicated in SDP. However, 1) the current UAE-S does not support SDP interpretation. 2) the NRM Server does not have procedure to determine SIP session request based on network bandwidth requirement from the SDP passed from UAE-S.