3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210751

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0047 | rev | 0 | Current version: | 17.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location report timestamp support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL, UASAPP |  |  |  |  |  | Date: |  |  | 2021-04-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For certain VAL services, the location report timestamp plays an important role such as for search and rescure. The current specification does not provide such feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add location report timestamp in certain location response information flow |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing important timestamp for certain VAL servcies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.2.1	Location reporting configuration response

Table 9.3.2.1-1 describes the information flow from the location management server to the location management client for the location reporting configuration. This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.1-1: Location reporting configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| Requested location information | O (NOTE) | Identifies what location information is requested |
| Triggering criteria | O (NOTE) | Identifies when the location management client will send the location report |
| Minimum time between consecutive reports | O (NOTE) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| Report Timestamp | O (NOTE) | Indication of reporting timestamp in UTC |
| NOTE: If none of the information element is present, this represents a cancellation for location reporting. |  |  |

* * * Next Change * * * *

#### 9.3.2.2	Location information report

Table 9.3.2.2-1 describes the information flow from the location management client to the location management server for the location information reporting or from the location management server to the requesting location management client or VAL server to report location information.

Table 9.3.2.2-1: Location information report

| Information element | Status | Description |
| --- | --- | --- |
| Set of identities | M | Set of identities of the reporting VAL users or VAL UEs |
| Triggering event | M | Identity of the event that triggered the sending of the report |
| Location Information | M | Location information |
| Report timestamp | O | Indication of reporting timestamp in UTC |

* * * Next Change * * * *

#### 9.3.2.7	Location information notification

Table 9.3.2.7-1 describes the information flow from the location management server to the VAL server or the location management client.

Table 9.3.2.7-1: Location information notification

| Information element | Status | Description |
| --- | --- | --- |
| Identities list | M | List of the VAL users or VAL UEs whose location information needs to be notified |
| Identity | M | Identity of the VAL user or VAL UE subscribed to location of another VAL user or VAL UE (NOTE) |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information | M | Location information |
| Report timestamp | O | Indication of reporting timestamp in UTC |
| NOTE: This is only used for location management server sends location information notification to the VAL user or VAL UE who has subscribed the location. |  |  |

* * * Next Change * * * *