3GPP TSG-SA WG6 Meeting #41-e	S6-210911

e-meeting, 18th – 26th January 2021	(revision of S6-21xxxx)

Source:	Huawei, Hisilicon

Title:	Update key issue#3 on 5G Multicast service and 5G Broadcast service

Spec:	3GPP TR 23.783 v1.2.0

Agenda item:	8.1

Document for:	Approval

Contact:	Cuili – Ge <gecuili@huawei.com>

1. Introduction

This pCR is proposed to update the key issue #3 on 5G Multicast service and 5G Broadcast service.

2. Reason for Change

This pCR is specially highlighted the issues on how to select the 5G MBS services, i.e., 5G Multicast service and 5G broadcast service for MC service.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.783 v1.2.0.

* * * First Change * * * *

## 5.3	Key issue 3: Use of multicast

### 5.3.1	Description

Multicast/broadcast is a very critical feature for public safety related group communication. The mission critical services over LTE benefit from the use of multicast/broadcast. Multicast/Broadcast capabilities are expected to be important in 5GS for mission critical applications. The support of multicast/broadcast in 5GS is FFS. The lack of multicast/broadcast support in 5GS will almost certainly impact the ability to support mission critical services over 5GS.

This has impact to:

-	TS 23.280 subclause 5.2.1, 5.2.6, 5.2.7.1, 7.2, 7.3.1, 7.4.2.3.2, 7.5.3.2, 7.5.3.3, all of 10.7, 10.9.3.1, 10.11.4;

-	TS 23.379 subclause 5.7.4, 7.3.1, 7.4.2.3.2, 7.5.2.2, 7.5.2.7, 7.5.2.9, 7.5.2.10, 10.6.2.5.2.4, 10.9.1.6,  all of 10.10;

-	TS 23.281 subclause 5.5.4, 6.1.1, 6.2.2.3.2, 6.3.4B, 6.3.6, 6.3.7, all of 7.10; and

-	TS 23.282 subclause 5.4, 5.8.4, 5.12, 6.4.3.1.2, 6.4.4.1.4, 6.5.4.1.3, 6.6.4.1.3, 6.6.4.1.5, 6.7.4.1.3, all of 7.3, 7.5.2.1.11, 7.5.2.1.21, 7.5.2.7.2, 7.5.2.10.1, 7.5.2.10.2, 7.7.2.2.2.1, 7.7.2.3.2.1

The search terms to create the above list are: GCSE_LTE, MBMS, BM-SC, MB2-U, MB2-C, GC1, TMGI, TMGIs, MBSFN, ECGI, SAI, SAIs, TS 23.246, TS 23.468 and TS 29.468. Occurrences of these terms have been highlighted in copies of the TSs in annex A, annex B, annex C and annex D of this report.

#### 5.3.1.x Selection of 5G Multicast service or 5G Broadcast service for MC service

In 3GPP TS 23.247 [x], the configuration for MBS Session used by the AF to start the MBS Session towards 5GC which applies to both multicast and broadcast communications is defined in clause 7.1.1.  The MBS procedures for broadcast Session management are defined in clause 7.3.

The clause 4.2 specifies the certain phases during the data is delivered via multicast session and broadcast session.

From SA6 point of view, the following aspects needs to be considered when using 5G multicast and broadcast MBS session for MC service group communication:

-	When and how to configure the Multicast MBS session, send the service announcement, deliver the data over Multicast MBS session including switching from unicast to multicast, and release the Multicast MBS session for the group communication? Identify whether the existing mechanism for MC service over LTE MBMS can be reused with proper modification?

-	Whether the existing mechanism for MC service over LTE MBMS can be reused for the MC service over 5G Broadcast MBS session with proper modifications, e.g., term, reference update?

-	How to select the multicast or broadcast for a group communication?

5.3.2	Architectural requirements

Editor's Note: Capture agreements on architectural requirements for solving the key issue. This clause may be omitted if deemed unnecessary.