3GPP TSG-SA WG6 Meeting #42-e	S6-210401

e-meeting, 1st – 9th March 2021	(revision of S6-21xxxx)

Source:	BDBOS

Title:	Pseudo-CR on Scenario supporting multiple MC gateway UEs

Spec:	3GPP TR 23.700-79 V0.3.0

Agenda item:	8.5

Document for:	Approval

Contact:	Jürgen Rurainsky, juergen.rurainsky@bdbos.bmi.bund.de

1. Introduction

This paper ads text to support the scenario of multiple MC service users connected to the MC system through one or more MC gateway UEs.

2. Reason for Change

While the scenario of one MC gateway UE is already covered, the more public safety related scenario to support multiple MC service users connected to the MC system through one or more MC gateway UEs is not described.

Use case: 11 police cars, one emergency ambulance as well as one fire truck requested to a scene of a car accident related to a bank robbery. While the police cars securing the surrounding area and the police officers search for the suspects of the bank robbery, the emergency ambulance is helping the uninvolved victim of the car accident and the fire truck secures both cars. All first responder vehicles are equipped with MC gateway UEs and provide for the local forces the necessary connection to the dispatcher and other supporting agencies while using non-3GPP based devices.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-79 V0.3.0.

* * * First Change * * * *

# 4	Scenarios

## 4.1	MC gateway UE services and the relationship to MC service users/clients

### 4.1.1	General

The MC gateway UE service may provide the following necessary functions:

-	Allowing an MC service client to connect to a MC gateway UE.

-	Allowing an MC service client to choose between multiple MC GW UEs for the required MC service capability.

-	Support of MC service continuation while the MC service client changes the MC gateway UE association.

-	Ensure that the content of communications between the MC system and MC service client connected to an MC gateway UE is unaltered.

-	Ensure that the communication traffic attributes, e.g. priority and QoS, of an MC service client connected to an MC gateway UE remains independent from other MC service clients concurrently connected to the same MC gateway UE.

-	Independent MC service client identification, authorisation and service profile association using the gateway MC UE functionality.

-	Sharing of an MC gateway UE transport resources with multiple connected MC service clients.

-	Indication of potential MC gateway UE anomalies sent towards the connected MC service clients.

-	MC service client is serving an MC service user on a device which is connected to the MC gateway UE via non-3GPP transport.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>