3GPP TSG-SA WG6 Meeting #42-e	S6-210532

e-meeting, 1st – 9th March 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0042 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to LMS server APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL, eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The information elements for the procedure specified in clause 9.3.10is to be specified. Also the API definition for this procedure is required to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the information flows and the API definition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 may not be able to progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.2.x	Get UE(s) information request

Table 9.3.2.x-1 describes the information flow for a VAL server to get UE(s) information at the LM server.

Table 9.3.2.x-1: Get UE(s) information request

| Information element | Status | Description |
| --- | --- | --- |
| Location information | M | Location information around which the UE(s) information is requested. |
| Application defined proximity range information |  | M |  | Description of the range information over which the HD map dynamic information is required. |  |

#### 9.3.2.y	Get UE(s) information response

Table 9.3.2.y-1 describes the information flow for a LM server to respond with UE(s) information to the VAL server.

Table 9.3.2.y-1: Get UE(s) information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the VAE server in response to subscription request indicating success or failure |
| List of V2X UEs information (NOTE) | O | The information of the V2X UEs which were detected in the application defined proximity range. The list can be empty. |
| >V2X UE ID | M | The identifier of V2X UE |
| >Location information | M | Location information of V2X UE within the application defined proximity range |
| NOTE: This IE shall be included when the Result indicates success. |  |  |

* * * Next Change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_ObtainUesAtLocation | Get_UEs_Info | VAL server | Request/Response |

* * * Next Change * * * *

### 9.4.x	SS_ObtainUesAtLocation API

#### 9.4.x.1	General

API description: This API enables the VAL server to obtain UE(s) information in an application defined proximity range of a location from the location management server over LM-S.

#### 9.4.x.2	Obtain_UEs_Info operation

API operation name: Obtain_UEs_Info

Description: Request UEs information in an application defined proximity range of a location.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.x

Outputs: Refer subclause 9.3.2.y

See subclause 9.3.10 for the details of usage of this API operation.