3GPP TSG-SA WG6 Meeting #42-e	S6-210586

e-meeting, 1st – 9th March 2021	(revision of S6-210425)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0263 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MBMS delivery via MB2 interface for MCData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2021-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As described in section 5.4, it was agreed that there can be two mechanisms for delivery over MBMS: - MBMS delivery via MB2 interface: MB2 delivery, defined in 23.468, is also used for MCPTT and MCVideo, and is applicable to all data transfers using media plane. - MBMS download delivery over xMB interface: this mechanism, detailed in 26.346 and 26.348, uses MBMS User Service architecture and takes advantage of the FLUTE protocol for file delivery (FD). This mechansim is already specified in 23.282. This CR adds support for mechanism 1) for MCData transfers, only when the media plane is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update text to include MB2 delivery via M2 interface, including some updates to the use of dynamic MBMS bearer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data delivery using MBMS over MB2 would not be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 5.13 (new), 7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 4	Introduction

The MCData service suite provides a set of generic capabilities and specific services to enable one-to-one and group data communications between MCData users.

The MCData architecture utilises the common functional architecture to support mission critical services over LTE defined in 3GPP TS 23.280 [5] and aspects of the IMS architecture defined in 3GPP TS 23.228 [6], the Proximity-based Services (ProSe) architecture defined in 3GPP TS 23.303 [7], the Group Communication System Enablers for LTE (GCSE_LTE) architecture defined in 3GPP TS 23.468 [8] defining MBMS support via the MB2 interface, the MBMS User Service architecture defined in 3GPP TS 26.348 [19], the Security of the Mission Critical Service in 3GPP TS 33.180 [13] and the PS-PS access transfer procedures defined in 3GPP TS 23.237 [9] to enable support of the MCData service.

The MCData UE primarily obtains access to the MCData service via E-UTRAN, using the EPS architecture defined in 3GPP TS 23.401 [4]. Certain application functions of MCData service can be accessed using MCData UEs via non-3GPP access networks.

The MCData system provides the function to support interworking with LMR systems defined in 3GPP TS 23.283 [18].

* * * Next Change * * * *

## 5.13	MBMS delivery via MB2 interface

MBMS delivery via MB2 applies to MCData services that use media plane for user traffic delivery.

* * * Next Change * * * *

### 7.3.3	Use of dynamic MBMS bearer establishment

The MCData service shall support the procedure for using dynamic MBMS bearers as specified 3GPP TS 23.280 [5] with the following clarifications:

-	The MC service client is the MCData client;

-	The MC service server is the MCData server; and

-	The MC service ID is the MCData ID.

The MCData service shall use the MCData-6, MCData-SDS-1, MCData-SDS-3, MCData-FD-1, MCData-FD-3, MCData-DS-1 and MCData-DS-3 reference points for this procedure.

MCData may use dynamic MBMS bearer for the MCData features short data service, file distribution and data streaming. The MBMS bearer can be used by any group. Depending on the capacity of the MBMS bearer, the bearer can be used to broadcast one or more services in parallel.

Both the media packets as well as the transmission control messages to the receiving users may be sent on the MBMS bearer. Optionally a separate MBMS bearer could be used for the transmission control messages, due to different bearer characteristic requirements.

* * * End Changes * * * *