3GPP TSG-SA WG6 Meeting #42-e	S6-210677

e-meeting, 1st – 9th March 2021	(revision of S6-210532)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0042 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to LMS server APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL, eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The information elements for the procedure specified in clause 9.3.10is to be specified. Also the API definition for this procedure is required to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the information flows and the API definition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 may not be able to progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1, 9.3.2.x (new), 9.3.2.y (new), 9.4.1, 9.4.x (new), 9.4.x.1 (new), 9.4.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.3.1	General

Location information of VAL service user shall be provided by the location management client to the location management server. The location information reporting triggers are based on the location reporting configuration. Different type of location information can be provided.

Editor's note:	The security and privacy aspects related to VAL server obtaining UE's location information are to be addressed by SA3.

* * * Next Change * * * *

#### 9.3.2.x	Get UE(s) information request

Table 9.3.2.x-1 describes the information flow for a VAL server to get UE(s) information at the LM server.

Table 9.3.2.x-1: Get UE(s) information request

| Information element | Status | Description |
| --- | --- | --- |
| Location information | M | Location information around which the UE(s) information is requested. |
| Application defined proximity range information |  | M |  | Description of the range information over which the UE(s) information is required. |  |

#### 9.3.2.y	Get UE(s) information response

Table 9.3.2.y-1 describes the information flow for a LM server to respond with UE(s) information to the VAL server.

Table 9.3.2.y-1: Get UE(s) information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the VAE server in response to subscription request indicating success or failure |
| List of UEs information (NOTE) | O | The information of the UEs which were detected in the application defined proximity range. The list can be empty. |
| >UE ID | M | The identifier of UE |
| >Location information | M | Location information of UE within the application defined proximity range |
| NOTE: This IE shall be included when the Result indicates success. |  |  |

* * * Next Change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |

* * * Next Change * * * *

### 9.4.x	SS_LocationAreaInfoRetrieval API

#### 9.4.x.1	General

API description: This API enables the VAL server to obtain UE(s) information in an application defined proximity range of a location from the location management server over LM-S.

#### 9.4.x.2	Obtain_UEs_Info operation

API operation name: Obtain_UEs_Info

Description: Request UEs information in an application defined proximity range of a location.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.x

Outputs: Refer subclause 9.3.2.y

See subclause 9.3.10 for the details of usage of this API operation.