3GPP TSG-SA WG6 Meeting #42-e	S6-210477

e-meeting, 1st – 9th March 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0285 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of multicast usage from functional alias related information flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Response of functional alias related information flows from MC service server to MC service client are over unicast. Multicast is not applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the statement “This information flow is sent individually addressed on unicast or multicast” from the information flows |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.13.2.2, 10.13.2.2a, 10.13.2.4, 10.13.2.6, 10.13.2.7, 10.13.2.9, 10.13.2.10, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

#### 10.13.2.2	Active functional alias information user query response

Table 10.13.2.2-1 describes the information flow for the active functional alias information user query response from the MC service server to the MC service client.

Table 10.13.2.2-1: Active functional alias information user query response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user who performs the query. |
| MC service ID(s) | M | The identity of the MC service user(s) to be queried. |
| Query result | M | The active functional alias information retrieved from the MC service server for each of the MC service user(s) queried; or Null if no functional alias is active for the MC service user. |

#### 10.13.2.2a	Active functional alias information usage query response

Table 10.13.2.2a-1 describes the information flow for the active functional alias information usage query response from the MC service server to the MC service client.

Table 10.13.2.2a-1: Active functional alias information usage query response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user who performs the query. |
| Functional alias(es) | M | The functional alias(es) to be queried. |
| Query result | M | The list of MC service ID(s) of the MC service user(s) who have activated the functional alias being queried; or Null if this functional alias is not active for any user. |

#### 10.13.2.4	Functional alias activation response

Table 10.13.2.4-1 describes the information flow functional alias activation response from the MC service server to the MC service client.

Table 10.13.2.4-1: Functional alias activation response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the originator who triggers the functional alias activation request. |
| Functional alias list | M | A list of one or more functional aliases which the originator intends to activate. |
| Activation status per functional alias | M | Indicates the activation result for each functional alias in the list (activated, rejected, can be taken over). |

#### 10.13.2.6	Functional alias de-activation response

Table 10.13.2.6-1 describes the information flow functional alias de-activation response from the MC service server to the MC service client.

Table 10.13.2.6-1: Functional alias de-activation response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the originator who triggers the functional alias de-activation request. |
| Functional alias list | M | A list of one or more functional aliases which the originator intends to de-activate. |
| De-activation status per functional alias | M | Indicates the de-activation result for every functional alias in the list. |

#### 10.13.2.7	Functional alias status notification

Table 10.13.2.7-1 describes the information flow functional alias notification from the MC service server to the MC service client.

Table 10.13.2.7-1: Functional alias status notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the originator who triggers the functional alias activation, de-activation or take over request. |
| Functional alias list | M | A list of one or more functional aliases. |
| Operations | M | Activation, de-activation or take over status per functional alias. |

#### 10.13.2.9	Functional alias take over response

Table 10.13.2.9-1 describes the information flow functional alias take over response from the MC service server to the MC service client.

Table 10.13.2.9-1: Functional alias take over response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the originator who triggers the functional alias activation request. |
| Functional alias | M | A functional alias which the originator intends to take over. |
| Activation status per functional alias | M | Indicates the take over request result (accepted, rejected). |

#### 10.13.2.10	Functional alias revoke notification

Table 10.13.2.10-1 describes the information flow functional revoke notification from the MC service server to the MC service client.

Table 10.13.2.10-1: Functional alias revoke notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the originator who triggers the functional alias take over request. |
| Functional alias | M | The functional alias which is revoked. |

* * * * * * * END CHANGES * * * * * * *