3GPP TSG-SA WG6 Meeting #43	S6-211316

e-meeting, 24th May – 2nd June 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0064 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to usage of SEAL for V2X application layer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The clause 9.1 corresponds to usage of SEAL for V2X applications. New procedures and information flows which are utilized by V2X applications are to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the usage of LMS and GMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of applicable SEAL procedures for V2X applications will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.2, 9.1.1.3, 9.1.3.2, 9.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.1.1.2	Information flows

The following information flows of group management service of SEAL as specified in 3GPP TS 23.434 [6] are applicable for the V2X applications:

-	Group creation request specified in subclause 10.3.2.1;

-	Group creation response specified in subclause 10.3.2.2;

-	Group creation notification specified in subclause 10.3.2.3;

-	Group information query request specified in subclause 10.3.2.4;

-	Group information query response specified in subclause 10.3.2.5;

-	Group membership update request specified in subclause 10.3.2.6;

-	Group membership update response specified in subclause 10.3.2.7;

-	Group membership notification specified in subclause 10.3.2.8;

-	Group deletion request specified in subclause 10.3.2.9;

-	Group deletion response specified in subclause 10.3.2.10;

-	Group deletion notification specified in subclause 10.3.2.11;

-	Group information request specified in subclause 10.3.2.12;

-	Group information response specified in subclause 10.3.2.13;

-	Group information subscribe request specified in subclause 10.3.2.14;

-	Group information subscribe response specified in subclause 10.3.2.15;

-	Group information notify request specified in subclause 10.3.2.16;

-	Group information notify response specified in subclause 10.3.2.17;

-	Store group configuration request specified in subclause 10.3.2.18;

-	Store group configuration response specified in subclause 10.3.2.19;

-	Get group configuration request specified in subclause 10.3.2.20;

-	Get group configuration response specified in subclause 10.3.2.21;

-	Subscribe group configuration request specified in subclause 10.3.2.22;

-	Subscribe group configuration response specified in subclause 10.3.2.23;

-	Notify group configuration request specified in subclause 10.3.2.24;

-	Notify group configuration response specified in subclause 10.3.2.25;

-	Configure VAL group request specified in subclause 10.3.2.26;

-	Configure VAL group response specified in subclause 10.3.2.27;

-	Group announcement specified in subclause 10.3.2.28;

-	Group registration request specified in subclause 10.3.2.29;

-	Group registration response specified in subclause 10.3.2.30;

-	Identity list notification specified in subclause 10.3.2.31;

-	Group de-registration request specified in subclause 10.3.2.32;

-	Group de-registration response specified in subclause 10.3.2.33;

-	Location-based group creation request specified in clause 10.3.2.34;

The usage of the above information flows are clarified as below:

-	The identity list is the list of V2X UE IDs.

--	During group creation the identity list contains the list of V2X UE IDs that are part of the group to be created. If the group member list is empty, an empty group is created; and

-	The VAL service ID list is the list of V2X service IDs whose service communications are to be enabled on the group.

-	Table 9.1.1.2.1-1 describes the VAL service specific information corresponding to the V2X applications.

Table 9.1.1.2.1-1: V2X service specific information for group creation request

| Information element | Status | Description |
| --- | --- | --- |
| V2X group ID | O | The group ID to be used for the V2X group. If group ID is not provided then the group ID should be allocated by group management server. |
| Group join policy | O (NOTE) | The policy information to be enforced by the group management server for members joining the group. |
| Group leader ID | O (NOTE) | The identity information of the V2X user to be used for user controlled group join. |
| NOTE: If the information elements, identity list specified in 3GPP TS 23.434 [6] is empty and group leader ID is not included, then group join policy information element must be included. |  |  |

-	The VAL group ID is the V2X group ID.

-	The identity is the V2X UE ID;

-	The VAL server is the VAE server;

-	The VAL group configuration information is the V2X group configuration information;

-	The VAL group configuration data is the V2X group configuration information;

-	The VAL group information reference is the V2X group information reference; and

-	The VAL services requested is the V2X service IDs.

#### 9.1.1.3	Procedures

The following procedures of group management service of SEAL as specified in 3GPP TS 23.434 [6] are applicable for the V2X applications:

-	Group creation specified in subclause 10.3.3;

--	Group creation support is provided for the V2X applications (e.g. platooning and cooperative short distance grouping) to provide a dedicated group for the V2X UEs to communicate with each other. The group creation supports group without any group members to enable group member joining.

-	Group information query specified in subclause 10.3.4;

-	Group membership specified in subclause 10.3.5;

-	Group configuration management specified in subclause 10.3.6;

-	Location-based group creation specified in clause 10.3.7;

-	Group announcement and join specified in subclause 10.3.8;

-	Group member leave specified in clause 10.3.9;

-	Temporary groups specified in clause 10.3.10;

* * * Next Change * * * *

#### 9.1.3.2	Information flows

The following information flows of location management service of SEAL as specified in 3GPP TS 23.434 [6] are applicable for the V2X applications:

-	Location reporting configuration request specified in subclause 9.3.2.0;

-	Location reporting configuration response specified in subclause 9.3.2.1;

-	Location information report specified in subclause 9.3.2.2;

-	Location information request specified in subclause 9.3.2.3;

-	Location reporting trigger specified in subclause 9.3.2.4;

-	Location information subscription request specified in subclause 9.3.2.5;

-	Location information subscription response specified in subclause 9.3.2.6;

-	Location information notification specified in subclause 9.3.2.7;

-	Location reporting configuration cancel request specified in clause 9.3.2.8;

-	Get UE(s) information request specified in clause 9.3.2.9;

-	Get UE(s) information response specified in clause 9.3.2.10;

The usage of the above information flows are clarified as below:

-	The identity is the V2X UE ID or V2X group ID;

-	The set of identities is the V2X UE ID;

-	The identity list or identities list is the list of V2X UE IDs; and

-	The VAL server is the VAE server.

#### 9.1.3.3	Procedures

The following procedures of location management service of SEAL as specified in 3GPP TS 23.434 [6] are applicable for the V2X applications:

-	Event-triggered location reporting procedure specified in subclause 9.3.3;

-	On-demand location reporting procedure specified in subclause 9.3.4;

-	Client-triggered or VAL server-triggered location reporting procedure specified in clause 9.3.5;

-	Location reporting event triggers configuration cancel specified in subclause 9.3.6;

-	Location information subscription procedure specified in subclause 9.3.7;

-	Event-trigger location information notification procedure specified in subclause 9.3.8;

-	On-demand usage of location information procedure specified in subclause 9.3.9;

-	Obtaining UE(s) information at a location specified in clause 9.3.10;