3GPP TSG-SA6 Meeting #45-bis-e	S6-212221

# Online, , 11th Oct 2021 - 19th Oct 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0056 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Disposition Notification handling when LMR system temporarily disables Disposition Notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sepura Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCCI |  |  |  |  |  | Date: |  |  | 2021-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If a disposition notification for an LMR SDS multipoint message is requested (e.g. message read ¦ delivered) then an LMR system such as TETRA may disable forwarding this request to users on congestion grounds and notify the originator of the request that this has happened. Notification of this decision is not currently supported in LMR-3GPP interworking, leading to a FASMO issue when an originating MC client decides on time-out that a message with dispostion request has not been delivered and decides to re-send to a group. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional Disposition value is introduced which may be used by an IWF acting on behalf of users homed in the IWF in response to a Disposition Request, and forwarded within the MCData system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of FASMO and congestion caused by an MC client repeatedly sending SDS message unnecessarily |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.8.2.3, 10.8.2.6, 10.8.8.1, 10.8.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Mirror CR for Rel-17 in CR0057 / S6-212222 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.8.2.3	IWF MCData data disposition notification

Table 10.8.2.3-1 describes the information flow for the MCData data disposition notification sent from the IWF to the MCData server and from the MCData server to the IWF.

Table 10.8.2.3-1: IWF MCData data disposition notification

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user towards which the notification is sent |
| MCData ID | M | The identity of the MCData user sending notification |
| Conversation Identifier (see NOTE) | M | Identifies the conversation |
| Disposition association | M | Identity of the original MCData transaction |
| Disposition | M | Disposition which is delivered, read, delivered and read, or disposition prevented by system |
| NOTE: A reserved value of the Information Element shall be defined which indicates that the sender does not support this Information Element. |  |  |

* * * Next Change * * * *

#### 10.8.2.6	IWF MCData data disposition notification(s) (MCData server to IWF)

Table 10.8.2.6-1 describes the information flow for the MCData data disposition notification(s) sent from the MCData server to the IWF when the IWF is acting as proxy for MCData client(s).

Table 10.8.2.6-1: IWF MCData data disposition notification(s) (MCData server – IWF)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user towards which the notification is sent |
| MCData ID | M | The identity of the MCData user sending notification |
| Conversation Identifier | M | Identifies the conversation |
| Disposition association | M | Identity of the original MCData transaction |
| Disposition | M | Disposition which is delivered, read, delivered and read, or disposition prevented by system |

* * * Next Change * * * *

### 10.8.8	MCData user group SDS request to an MCData group including LMR users

#### 10.8.8.1	Signalling control plane

The procedure for an MCData user requesting to send a signalling control plane SDS to an MCData group that includes one or more LMR users is as specified in 3GPP TS 23.282 [6] subclause 7.4.2.5 for the group standalone short data service using the signalling control plane. In the case of implementation involving an IWF the difference is that one or more of the MCData clients 2 to n are located behind IWFs that have affiliated to the MCData group (see subclause 10.1.2 of the present document). The SDS is addressed to the MCData group ID. The IWF behaves as a peer MCData server. The IWF can also respond on behalf on a MCData client located behind the IWF to a disposition request with a disposition of 'disposition prevented by system' for forwarding to the originating MCData client.

#### 10.8.8.2	Media plane

The procedure for an MCData user requesting to send a media plane SDS to an MCData group that includes one or more LMR users is as specified in 3GPP TS 23.282 [6] subclause 7.4.2.6 for the group standalone short data service using the media plane. In the case of implementation involving an IWF the difference is that one or more of the MCData clients 2 to n can be located behind IWFs that have affilated to the MCData group (see subclause 10.1.2 of the present document). The SDS is addressed to the MCData group ID. The IWF behaves as a peer MCData server. The IWF can also respond on behalf on a MCData client located behind the IWF to a disposition request with a disposition of 'disposition prevented by system' for forwarding to the originating MCData client.

* * * End Change * * * *