3GPP TSG-SA WG6 Meeting #47-e	S6-220158

# e-meeting, 14th – 22nd February 2022	                                                 (revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0091 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete location retrieval in an area |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-02-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During the location based group formation, GMS knows the VAL service ID from the VAL server (received from the Group management client or VAL server). But when the GMS interacts with the LMS, it just receives a list of UEs within an area. Those UEs may support different VAL services so it is possible that incorrect VAL users or UEs (supporting different VAL service than the VAL server) are included into the group, it is needed to identify the supported VAL services for the retrieved UEs wihin an area. If LMS knows VAL service ID (e.g. due to Location information subscription procedure) LMS can include VAL service together with the retrieved list of UEs to the GMS. If GMS doesn’t receive VAL service ID for the UE, it can retrieve it from the CMS. In addition, in the location based UE group creation/update, it is unclear how the LMS can retrieve a list of UE identifiers. One possible option is to interact with 3GPP CN to get such information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add VAL service ID in Obtain UE list per location response message. Add a new CM service for retrieving VAL service ID associated with requested VAL UE or user ID. Add 3GPP CN interaction for LMS to retrieve the UE ID list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong UE may be grouped for un-related service. It is un-specified how to retrieve UE list in an area supporting the desired VAL services for VAL server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.12.4, 10.3.7, 11.3.2.x (new), 11.3.2.y (new), 11.3.x (new), 11.3.x.1 (new), 11.3.x.2 (new), 11.4.1, 11.4.x (new), 11.4.x.1 (new), 11.4.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.12.4	Location area monitoring notification procedure

Figure 9.3.12.4-1 illustrates the high level procedure of location area monitoring notification. The same procedure can be applied for location management client and other SEAL servers who have subscribe to the list of the UEs moving in or moving out of the specific location area.

Figure 9.3.12.4-1: Location are monitoring notification procedure

1.	One of the events occurs at the location management server as specified in the subscribe request. The location management server identifies the UEs which are moved into the area or moved out of the area based on their location data and time stamp of the location. The LMS may report the list of all UEs in the given location or UEs moved in and moved out. The monitoring capability for "Number of UEs present in a geographical area" provided by SCEF/NEF may be used by the LMS to retrieve all UE identifiers within the designated geographical area.

NOTE:	The reference UE and the application defined proximity range from the reference UE to be monitored can be expressed in a geographic area (e.g. circular shape) by the LMS after retrieving the location of the reference UE.

2.	The location management server sends a location area monitoring notification to the VAL server. In the notification message, the location management server includes the information as specified in Table 9.3.2.16-1.

* * * Next Change * * * *

### 10.3.7	Location-based group creation

Figure 10.3.7-1 below illustrates the location-based group creation.

Pre-conditions:

1.	The group management client, group management server, VAL server, location management server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator is not aware of the users' or UE identities which will be combined to form the VAL group.

Figure 10.3.7-1: Location-based group creation

1.	The group management client or the VAL server requests location-based group create operation to the group management server. The location criteria for determining the identities of the users or UEs to be combined shall be included in this message.

2.	The group management server requests the location management server for obtaining the users or UEs corresponding to the location information.

3.	The location management server composes the list of users or UEs within the requested location. The location management server may utilize the monitoring capability for "Number of UEs present in a geographical area" provided by SCEF/NEF to retrieve all UE identifiers within the designated geographical area.

4.	The group management server receives the composed list of users or UEs from the location management server. The location management server may include VAL service IDs associated with the list of users or UEs.

5.	During the group creation, the group management server creates and stores the information of the location-based group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s). If an external group identifier, identifying the member UEs of the VAL group at the 3GPP core network is available, then the external group ID is stored in the newly created VAL group’s configuration information. If VAL service ID list was received in step 1 as part of group formation criteria and not in step 4 for the composed list of users or UEs, the group management server shall retrieve the VAL service data from the configuration management server as described in clause 11.3.2.x and form the group with relevant users or UEs.

NOTE:	The exact policies are out of scope of the present document.

6.	The group management server provides a location-based group creation response to the group management client or the VAL server.

* * * Next Change * * * *

#### 11.3.2.x	Get VAL service request

Table 11.3.2.x-1 describes the information flow get VAL service request from the group management server to the configuration management server.

Table 11.3.2.x-1: Get VAL service request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the entity performing the request. |
| Identity list | O (NOTE) | The VAL user IDs or VAL UE IDs. |
| VAL service ID list | O (NOTE) | The requested VAL service IDs for the identity list. |
| NOTE: At least one of Identity list or VAL service ID list shall be included. With both Identity list and VAL service ID in the request, the response only contains common VAL services (as requested) for the requested VAL users or VAL UEs |  |  |

* * * Next Change * * * *

#### 11.3.2.y	Get VAL service response

Table 11.3.2.y-1 describes the information flow get VAL service response from the configuration management server to the group management server.

Table 11.3.2.y-1: Get VAL service response

| Information element | Status | Description |
| --- | --- | --- |
| A list of VAL service data | O (NOTE 1) | One or more VAL service data associated with the VAL user ID or VAL UE ID. |
| > Identity | M | The VAL user ID or VAL UE ID. |
| > VAL service IDs | M | The supported VAL service IDs. |
| > VAL service specific information | O (NOTE 2) | Placeholder for VAL service specific information. |
| Result | M | Indicates the success or failure for the operation. |
| NOTE 1: It is valid when the Result information element indicates success. NOTE 2: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

* * * Next Change * * * *

### 11.3.x	VAL service data

#### 11.3.x.1	General

The VAL service data related procedures are described in the following subclauses.

#### 11.3.x.2	Procedures

The procedure for consumer (e.g. group management server) obtaining the VAL service data is illustrated in figure 11.3.x.2-1.

Pre-conditions:

-	The consumer has the secure access to the configuration management server.

-	The configuration management server has VAL service data maintained for VAL UE or user.

Figure 11.3.x.2-1: Obtains the VAL service data for VAL UE(s) or VAL user(s)

1.	The consumer (e.g. group management server) sends a get VAL service request to the configuration management server for obtaining VAL service data. The request includes a list of VAL UE IDs or VAL user IDs and/or a VAL service ID list as service data retrieval filters.

2.	The configuration server retrieves the VAL service data for the requested VAL UEs or users or VAL services and sends get VAL service response to the consumer (e.g. group management server) with the discovered list of VAL service data.

* * * Next Change * * * *

### 11.4.1	General

Table 11.4.1-1 illustrates the SEAL APIs for configuration management.

Table 11.4.1-1: List of SEAL APIs for configuration management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_UserProfileRetrieval | Obtain_User_Profile | VAL server | Request /Response |
| SS_UserProfileEvent | Subscribe_User_Profile_Update | VAL server | Subscribe/Notify |
|  | Notify_User_Profile_Update | VAL server |  |
| SS_VALServiceData | Obtain_VAL_Service_Data | group management server | Request /Response |

* * * Next Change * * * *

### 11.4.x	SS_VALServiceData API

#### 11.4.x.1	General

API description: This API enables the group management server to communicate with the configuration management server for obtaining VAL service data over SEAL-X.

#### 11.4.x.2	Obtain_VAL_Service_Data operation

API operation name: Obtain_VAL_Service_Data

Description: Obtaining VAL service data.

Known Consumers: group management server.

Inputs: See subclause 11.3.2.x

Outputs: See subclause 11.3.2.y

See subclause 11.3.x.2 for the details of usage of this API operation.

* * * End of Changes * * * *