3GPP TSG-SA WG6 Meeting #47-e	S6-220441

e-meeting, 14th – 22nd February 2022	(revision of S6-220228, 391)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS23.289 | CR | 0035 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean up of switching between unicast and 5G MBS, between LTE eMBMS and 5G MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In this CR, the following aspects of swithing from/to 5G MBS and either unicat or LTE eMBMS are covered: - The editor note about potential MBSF based solution for EPS-5GMBS switching options should be removed from this release, since SA4 has no plan to introduce the related features. - The editor note about Service continuity between 5G MBS delivery and unicast delivery by full service mode should be removed, based on SA4 working plan. - Providing the description about how MCX client report the inter-system change event to the MCX service server and solve the corresponding Editors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Deleting the Editor note in 7.3.3.8.2.1 about Service continuity between 5G MBS delivery and unicast delivery by full service mode. - Deleting the Editor notes about enhancement or new solution for joint BMSC/MBSF in cluase 7.3.3.9. - Adding the description about knowledge of inter-system change at MCX service server in cluase 7.3.3.9 and deleting the editor notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The implementation of the specification for 5MBS will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.9.1, 7.3.3.9.2, 7.3.3.9.3, 7.3.3.9.4, 7.3.3.9.5, 7.x (new), 7.x.1 (new), 7.x.2 (new), 7.x.2.1 (new), 7.x.2.2 (new), 7.x.2.3 (new), 7.x.3 (new), 7.x.3.1 (new), 7.x.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************************************The first change****************************************

7.3.3.8.2.1	General

This solution provides the procedure which allows the MC service client to report the broadcast reception quality to the MC service server which is used to make the decision whether to use the unicast delivery to the MC service UE(s) which are suffering bad broadcast reception quality due to e.g., move out of the broadcast service area.

An MC service client monitors the broadcast MBS session to receive MC service media. Based on the received quality (e.g. radio level quality, RTP packet loss), the MC service client needs to inform the MC service server that the MC service client is able to receive the MC service media on the broadcast MBS session with sufficient quality or not.

This estimation of the broadcast reception quality may be dependent on for example the modulation and coding scheme (MCS) and measurements from the reference signals from the NG-RAN node(s), RTP packet loss, BLER of the received media.

**************************************The Second change*****************************************

##### 7.3.3.9.1	General

When working in transport only mode, the MC service server guides the MC service clients throughout the MC media transmission for switching between the LTE and 5G systems. For this purpose, the location management client sends location related information to the location management server, similar to the one defined in 3GPP TS 23.280 [3], which is triggered due to its location change – in this case due to Radio Access Technology (RAT) change, to inform the server hence the latter provides guidance related to how to receive the MC services after the location update.

The procedures cover both the deployment scenarios with/without MBSF/MBSTF. The procedures specify four inter-system switching related scenarios as follows:

1.	Inter-system switching from 5G MBS session to LTE eMBMS bearer, as in 7.3.3.9.2

2.	Inter-system switching from 5G MBS session to LTE unicast bearer, as in 7.3.3.9.3

3.	Inter-system switching from LTE eMBMS to 5G MBS session, as in 7.3.3.9.4

4.	Inter-system switching from LTE eMBMS to 5G unicast PDU session, as in 7.3.3.9.5

In all the inter-system switching related scenario described in 7.3.3.9.2, 7.3.3.9.3, 7.3.3.9.4 and 7.3.3.9.5, the functional entity that resides in 5GS may be NEF, or MBSF, or MB-SMF for session creation and together with PCF or PCC related interaction.

NOTE:	There will be a service interrupt when the MC service server performs path switch between 5G and LTE bearers or sessions.

##### 7.3.3.9.2	Inter-system switching from 5G MBS session to LTE eMBMS bearer

The procedure provided in figure 7.3.3.9.2-1 describes how the MC service server handles inter-system switching when the MC service UE switches from 5G to LTE network, where the MC service server is able to provide the MC services to the clients over eMBMS bearer(s).

Pre-conditions:

-	MC service clients are attached to the 5GS, registered and affiliated to the same MC service group X.

-	The MC services can be provided via both 5GS and EPS.

-	The MC service client(s) is within the eMBMS service area.

-	It is assumed that the MC service client(s) has not received the eMBMS service announcement while camping in 5GS.

Figure 7.3.3.9.2-1: Inter-system switching from 5G MBS session to LTE eMBMS bearer.

1.	An MC service group communication takes place, and the MC media is delivered over 5G MBS session (either broadcast or multicast session mode), which is associated to the MC service group X.

2.	The MC service UE performs handover to EPS.

3.	The MC service UE`s location information is provided via the location management client, triggered by RAT change, to the location management server, where the latter provides the location information to the MC service server.

4.	The MC service server analyses the location information and decides how to serve the client. If the MC service server decides to serve the client via eMBMS bearer, it may send an eMBMS bearer announcement as described in 3GPP TS 23.280 [3]. This step is optional as the service announcement related information could be sent in advance (implementation specific).

5.	If not already available, the MC service client stores the announced TMGI(s), service area, and any relevant information to the eMBMS, which is delivered via the bearer announcement. As a result, the MC service client starts monitoring the bearer.

6.	The MC service client sends an eMBMS listening status report to inform the server of its ability of receiving MC media over the specified bearer.

7.	The MC server sends the necessary information related to receiving the MC media in the form of the MapGroupToBearer.

8.	The MC service group communication takes place over EPS, and the MC media is transmitted over an eMBMS bearer.

##### 7.3.3.9.3	Inter-system switching from 5G MBS session to LTE unicast bearer

The procedure provided in figure 7.3.3.9.3-1 describes how the MC service server handles inter-system switching when the MC service UE switches from 5G to LTE network, where the MC service server is unable to provide the MC services to the client over eMBMS bearer.

Pre-conditions:

-	MC service clients are attached to the 5GS, registered and affiliated to the same MC service group X.

-	The MC services can be provided via both 5GS and EPS.

Figure 7.3.3.9.3-1: Inter-system switching from 5G MBS session to LTE unicast bearer.

1.	An MC service group communication takes place, and the MC media is delivered over 5G MBS session (either broadcast or multicast session mode), which is associated to the MC service group X.

2.	The MC service UE performs handover to EPS.

3.	The MC service UE`s location information is provided via the location management client, triggered by RAT change, to the location management server, where the latter provides the location information to the MC service server.

4.	The MC service server may interact with the EPC for providing the required media resources over the unicast bearer, if not already allocated.

5.	The MC service group communication takes place over EPS, and the MC media is transmitted over a unicast bearer.

##### 7.3.3.9.4	Inter-system switching from LTE eMBMS to 5G MBS session

The procedure provided in figure 7.3.3.9.4-1 describes how the MC service server handles inter-system switching when the MC service UE switches from LTE network to 5G, where the MC service server is able to provide the MC services to the client over 5G MBS sessions (either broadcast or multicast).

Pre-conditions:

-	MC service clients are attached to the EPC and affiliated to the same MC service group X.

-	The MC services can be provided via both 5GS and EPS.

-	The MC service client(s) is within the service area (if the session is limited to an area), where the MBS session is configured.

-	It is assumed that the MC service client(s) has not received the 5G MBS service announcement while camping in EPS.

Figure 7.3.3.9.4-1: Inter-system switching from LTE eMBMS bearer to 5G MBS sessions (either broadcast or multicast).

1.	An MC service group communication takes place, and the MC media is delivered over eMBMS bearer, which is associated to the MC service group X.

2.	The MC service UE performs handover to 5GS.

3.	The MC service UE`s location information is provided via the location management client, triggered by RAT change, to the location management server, where the latter provides the location information to the MC service server.

4.	The MC service server analyses the location information and decides how to serve the client. If the MC service server decides to serve the client via 5G MBS session, it may send a an MBS service announcement indicating information among others the session mode to serve the MC service client and the corresponding MBS session ID. This step is optional as the service announcement related information could be sent in advance (implementation specific).

5.	The MC service UE acts according to the MBS session mode provided to receive the DL media.

5a.	In case of multicast MBS sessions, the MC service UE performs a UE session join towards the 5GC indicating the MBS session ID to join. It may as well send a UE session join acknowledgement to the MC service server.

5b.	In case of broadcast MBS sessions, the MC service UE starts monitoring the broadcast MBS session.

6.	The MC service client sends an MBS listening status report to the server indicating its ability to receive media over the indicated MBS session.

7.	The MC service server sends a MapGroupToSessionStream over the MBS session providing the required stream information to receive the media related to the group communication.

8.	The MC service client processes the received information related to the MC media over the MBS session.

9.	The MC service group communication takes place over 5GS, and the MC media is delivered over the broadcast or multicast MBS session.

##### 7.3.3.9.5	Inter-system switching from LTE eMBMS to 5G unicast PDU session

The procedure provided in figure 7.3.3.9.5-1 describes how the MC service server handles inter-system switching when the MC service UE switches from LTE network to 5G, where the MC service server is able to provide the MC services to the client over 5G MBS sessions (either broadcast or multicast).

Pre-conditions:

-	MC service clients are attached to the EPC and affiliated to the same MC service group X.

-	The MC services can be provided via both 5GS and EPS.

Figure 7.3.3.9.5-1: Inter-system switching from LTE eMBMS bearer to 5G unicast PDU session.

1.	An MC service group communication takes place, and the MC media is delivered over eMBMS bearer, which is associated to the MC service group X.

2.	The MC service UE performs handover to 5GS.

3.	The MC service UE`s location information is provided via the location management client, triggered by RAT change, to the location management server, where the latter provides the location information to the MC service server.

4.	The MC service server may interact with the 5GC to request media resources (if not already allocated) with specific requirements over unicast PDU session, as it is able to serve the MC service client via unicast PDU session, as described in 3GPP TS 23.289 [22].

5.	The MC service group communication takes place over 5GS, and the MC media is delivered over unicast PDU session.

**************************************TheThird change*****************************************

## 7.x Enhancements for Location management over 5GS

### 7.X.1	General

In addition to the location management functions defined in 3GPP TS 23.280 [3], the location management system over 5GS shall support the following enhancement:

- Inter-system RAT changes event triggering and report as described in clause 7.3.3.9.1.

### 7.X.2	Information flows for location information

#### 7.X.2.1	General

In this clause, only those information flows which requires enhancement to 3GPP TS 23.280 [3] for MC services over 5G specific are included.

#### 7.X.2.2	Location reporting configuration

Table 7.x.2.2-1 describes the information flow from the location management server to the location management client for the location reporting configuration.

Compared to that is defined in 3GPP TS 23.280 [3], the following changes are made:

Adding a new NOTE 3 to clarify the usage of triggering event for inter-system RAT changes

Table 7.x.2.2-1: Location reporting configuration

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user to which the location reporting configuration is targeted. |
| Requested non-emergency location information | O (see NOTE 1) | Identifies what location information is requested, except for emergency or imminent peril calls or emergency alerts |
| Requested emergency location information | O (see NOTE 1) | Identifies what location information is requested, for emergency or imminent peril calls or emergency alerts |
| Triggering criteria in non- emergency cases | O (see NOTE 1 and NOTE 3 | Identifies when the location management client will send the location report in non-emergency cases |
| Handling criteria in not reporting location information cases | O | Identifies when the location management client will store location information (e.g. never, off-network, IOPS) |
| Triggering criteria in not reporting location information cases | O (see NOTE 2) | Identifies the causes when the location management client will generate location information |
| Minimum time between consecutive reports | O (see NOTE 1) | Defaults to 0 if absent and 0 for emergency calls, imminent peril calls and emergency alerts |
| Triggering criteria in emergency cases | O (see NOTE 1) | Identifies when the location management client will send the location report in emergency cases |
| NOTE 1: If none of the information elements is present, this represents a cancellation for location reporting based on Triggering criteria in emergency and non-emergency cases, if configured. NOTE 2: If not present, location information is generated based on Triggering criteria in emergency and non-emergency cases, if configured. NOTE3： Triggering criteria can indicate inter-RAT changes is required, |  |  |

#### 7.X.2.3	Location information report

Table 7.X.2.3-1 describes the information flow from the location management client to the location management server for the location information reporting.

Compared to that is defined in 3GPP TS 23.280 [3], the following changes are made:

Adding a new NOTE 5 to clarify the usage of triggering event for inter-system RAT changes

Table 7.X.2.3-1: Location information report (LMC – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service ID (see NOTE 4) | O | Identity of the requesting MC service user. |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the reporting MC service ID. |
| MC service UE label | O | Generic name of the reporting MC service UE |
| Triggering event (see NOTE 3 and NOTE5) | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 3: An on-demand request may be the triggering event. NOTE 4: In case of an on-demand request of an MC service user the MC service ID shall be provided. In case of an MC service server request or an event-triggered report, no MC service ID is provided. NOTE 5: Triggering event can be used for indicate the event of inter-system changes. |  |  |

Table 7.X.2.3-2 describes the information flow from the location management server to the MC service server for location information reporting.

Compared to that is defined in 3GPP TS 23.280 [3], the following changes are made:

Adding a new NOTE 4 to clarify the usage of triggering event for inter-system RAT changes

Table 7.X.2.3-2: Location information report (LMS – MC service server)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the MC service ID. |
| Triggering event (see NOTE 3 and NOTE 4) | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 3: An on-demand request may be the triggering event. NOTE 4: Triggering event may indicate requiring of location report due to inter-system RAT-type change |  |  |

### 7.x.3	Procedure

#### 7.x.3.1	General

In this clause, only the procedure which requires enhancement to 3GPP TS 23.280 [3] for MC services over 5G specific are included.

#### 7.x.3.2	Event-triggered location reporting procedure

NOTE 1: This procedure is valid for single MC system operation only.

The location management server provides location reporting configuration to the location management clients, indicating what information the location management server expects and what events will trigger the sending of this information to the location management server. The decision to report location information can be triggered at the location management client by different conditions. In addition to those defined in 3GPP TS 23.280 [3], the conditions could include inter-system RAT type changes. The location report can indicate the inter-system RAT changes event.

NOTE 2: How the location management client can identify or obtain information about the case that an inter-RAT change has occurred is implementation specific or out of the scope of this specification.

Similar to the usage of eMBMS bearer for transmitting of location reporting configuration defined in TS 23.280, the location reporting configuration message can also be sent over 5G MBS Session.

**************************************The End of changes*****************************************