3GPP TSG-SA WG6 Meeting #48-e	S6-220489

e-meeting, 5th – 14th April 2022

3GPP TSG-SA2 Meeting #149e	S2-2201517

Feb 14 – 25, 2022, Electronic meeting

Title:	Reply LS on MBS Service Area Identity and start procedure for broadcast service

Response to:	LS on MBS Service Area Identity and start procedure for broadcast service (S2-2200415/R3-221302)

Release:	Release 17

Work Item:	5MBS, NR_MBS-Core

Source:	SA2

To:	RAN3, RAN2, SA6

Cc:	SA4, SA5

Contact Person:

#### Name:	Xiaoyan Duan

Tel. Number:

E-mail Address:	duanxiaoyan@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 thanks RAN3 for their LS on MBS Service Area Identity and start procedure for broadcast service. SA2 discussed the LS and would like to provide the following feedback:

RAN3 has discussed the issue on MBS broadcast service continuity and identification, and would like to inform SA2 about our progress as follows: - To use the term “MBS Service Area ID” to replace the “MBS XXX ID” in the interim until SA2 comes up with final name; - It is configured via OAM at NG-RAN;

SA2 response

SA2 discussed and agreed to replace the "MBS XXX ID" with "MBS Frequency Selection Area (FSA) ID".

Regarding to the length of MBMS service area ID, RAN3 did not reach an agreement and would like to wait for the conclusion in SA2.

SA2 response

For the length of MBS FSA ID, SA2 considers the following two options:

Option 1: using 2 octets to align with MBMS SAI defined in 3GPP TS 23.003 and TS 36.331.

Option 2: using 3 octets for flexibility to allow further extension.

SA2 suggests 3-octet length but leaves the final decision to RAN2.

In the meanwhile, RAN3 also discussed on session start procedure for broadcast service and the view of majority is that indication of session start success or failure with cell accuracy is not needed. Instead, RAN3 is of the opinion that a success/failure on a per gNB basis may be sufficient and would like to know the views of SA2.

SA2 response

SA2 can accept that indication of session start success or failure is on a per gNB basis unless concerns are raised by SA6. SA2 would like to get feedback from SA6.

2. Actions:

To RAN WG3

ACTION: SA2 kindly requests RAN3 to take the above information into consideration.

To RAN WG2

ACTION: SA2 kindly requests RAN2 to confirm the length of MBS FSA ID suggested by SA2.

To SA WG6

ACTION: SA2 kindly requests SA6 to provide feedback on the indication of session start success or failure on a per gNB basis.

3. Date of Next TSG-SA WG2 Meetings:

TSG-SA2 Meeting #150E	April 6–12, 2022	E-meeting