3GPP TSG-SA WG6 Meeting #48-e	S6-220611

e-meeting, 5th – 14th April 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0048 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications related to multi carrier support for MBS session creation and announcement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-04-05 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In alignment with 3GPP TS 23.247, in case the MC service deployment supports multi carrier frequency, the frequency associated to a certain broadcast MBS session is provided to the MC service UE during MBS service announcement. Furthermore, the MC service server may also provide the MBS Frequency Selection Area ID (MBS FSA ID), which is the frequency associated to a certain broadcast area during the MBS service announcement in case of broadcast MBS service announcement. The MBS FSA ID may be provided by the MC service server during the MBS session creation and update procedures based on the business agreement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The MBS service announcement is updated to include two optional information elements: Frequency, and MBS FSA ID associated to broadcast MBS sessions if multi carrier deployment is supported. - In case of multi carrier deployment and based on the business agreement, the MC service server may provide the frequency(s) associated to a certain broadcast service area to the MB-SMF during MBS session creation procedure. - An EN related to how the MC service server is informed of the RAT capabilities of the MC service UE is removed, as it has already been addressed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some information and aspects related to multi carrier deployment is not clarified or specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.1 and 7.3.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Table 7.3.2.1-1: MBS service announcement for the use of MBS sessions

| Information element | Status | Description |
| --- | --- | --- |
| 5G MBS session information | M | Providing the MBS session related information if MCX server decides to use 5G MBS session to deliver MC service group communication data |
| >MBS session ID | M | The identity of the MBS session used to deliver MC service group communication data. It is either TMGI for broadcast MBS and multicast MBS sessions, or source specific IP multicast address for multicast MBS session |
| >MBS session mode | M | Indicate the service type of the MBS session, either a multicast MBS session or a broadcast MBS session |
| >MC service group ID | O | Indicate the MC service group ID associated to the MBS session |
| >MBS related SDP information | M | SDP related to application-level control signaling or media to be transmitted over the MBS session (e.g., codec, protocol ID, FEC information, IP address and ports) |
| >List of MBS Service Area information (see NOTE 5) | O | For the case of local MBS services, it indicates either multicast service area identifier(s) for multicast MBS session, or broadcast service area identifier(s) for broadcast MBS session |
| >MBS service announcement acknowledgement | O | Indicate if the MC service server requires an acknowledgement to the MBS service announcement |
| >Multicast MBS session related information (see NOTE 1) | O | Additional information to be used by the MC service client to join the multicast MBS session such as PLMN ID of the default PLMN service provider in case of source specific IP multicast address, DNN, and SNSSAI of the PDU session associated with the multicast MBS session |
| >MBS UE session join notification (see NOTE 2) | O | Indicate if the MC service server requires a notification from the MC service client once it has joined the multicast MBS session |
| >Monitoring state (see NOTE 3) | O | Indicate if the MC service client is required to actively monitor the broadcast MBS session quality and report it to the MC service server |
| >Frequency (see NOTE 3) | O | Identification of frequency associated with a broadcast MBS session, if multi carrier support is provided |
| >MBS Frequency Selection Area ID (MBS FSA ID) (see NOTE 3) | O | The frequency associated to a certain broadcast area, if multi carrier support is provided |
| eMBMS bearer information | O | Providing the 4G eMBMS bearer related information if MCX server decides to use 4G eMBMS additionally with 5G MBS session to deliver MC service group communication data |
| >TMGI (see NOTE 4) | M | TMGI information |
| >Alternative TMGI | O | A list of additional alternative TMGI may be included and used in roaming scenarios |
| >QCI | O | QCI information used by the ProSe UE-Network Relay to determine the ProSe Per-Packet Priority value to be applied for the multicast packets relayed to Remote UE over PC5 |
| >List of service area identifier | M | A list of service area identifier for the applicable MBMS broadcast area |
| >Frequency | O | Identification of frequency if multi carrier support is provided |
| >Unicast bearer status | O | An indication that the listening status of the LTE unicast bearer is requested |
| >eMBMS related SDP information | M | SDP with media and floor control information applicable to groups that can use this eMBMS bearer (e.g., codec, protocol id, FEC information) |
| >ROHC information | O | Indicate the usage of ROHC over the eMBMS bearer and provide the parameters of the ROHC channel to signal to the ROHC decoder |
| NOTE 1: Such information may be pre-configured in the MC service UE, or provided in any other implementation specific way NOTE 2: It is applicable for multicast MBS session NOTE 3: It is applicable for broadcast MBS session NOTE 4: TMGI for 4G eMBMS bearer can be the same or different with 5G MBS session ID. NOTE 5: Details of MBS service area information is defined in 3GPP TS 23.247 [15]. |  |  |

# * * * * Second change * * * *

##### 7.3.3.1.2	Procedure for pre-created MBS session and service announcement

Pre-conditions:

-	The MC service server has decided to use an MBS session for MC service group communications associated to a certain MC service group based on transport only mode.

-	The MC service server has performed MB-SMF discovery and selection either directly or indirectly via NEF/MBSF, unless the corresponding information is locally configured.

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same MC service group X.

-	The MC service server is aware whether to request the creation of the MBS service server with or without dynamic PCC rule.

Figure 7.3.3.1.2-1: Use of pre-created MBS session.

1.	The MC service server initiates an MBS session creation procedure towards the 5GC as described in 3GPP TS 23.247 [15]. The procedure starts once the MC service server initiates a TMGI allocation request (either directly to MB-SMF or indirectly via NEF). Upon the reception of the TMGI allocation response, the MC service server sends an MBS session creation request, including further information related to the MBS session, e.g., MBS session ID, MBS session mode and the QoS requirements if dynamic PCC rule is not considered. However, if dynamic PCC rule is considered, the MC service server defines these requirements at a later step, namely it sends an MBS authorization/policy create request towards PCF (either directly or to the NEF) indicating the QoS requirements.

NOTE 1:	In case of LTE eMBMS and 5G MBS co-existence, the MC service server may trigger the establishment of eMBMS bearers as described in 3GPP TS 23.280 [3] (or it may establish a unicast bearer) based on the RAT capabilities supported by the affiliated members in the MC service group X. If MBSF and BM-SC are co-located, TMGI and 4G eMBMS can be the same as the MBS session ID.

NOTE 2: For the case of multi carrier support for broadcast MBS sessions, the MC service server may indicate the frequencies within a broadcast MBS service area by providing the MBS FSA ID(s) to the MB-SMF or indirectly via NEF.

2.	The MC service server provides the MC service clients affiliated to MC service group X with the information related to the created MBS session via the MBS service announcement. As described in table 7.3.2.2-1, the MBS service announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type) and SDP information related to the MBS session under consideration.

NOTE 2:	The MC service server may send an MBS service announcement at an earlier step during the MBS session creation procedure towards the MC service clients once the MC group associated to the MBS session is known.

Optionally, the MC service server includes the information elements related to the established eMBMS bearer once the MC service server has determined the need, as indicated in table 7.3.2.2-1. The MC service clients which camp on LTE will subsequently react to the information elements related to the eMBMS bearer as described in 3GPP TS 23.280 [3].

3.	MC service clients store and process the received MBS session information.

4.	MC service clients may provide an MBS service announcement acknowledgment to the MC service server to indicate the reception of the corresponding MBS service announcement.

5.	Based on the MBS session mode; either multicast or broadcast, the following actions take place.

5a.	For multicast MBS sessions, MC service clients initiate a UE session join request towards the 5GC using the information provided via the MBS service announcement. Hence, upon the first successful UE session join request, the multicast is then established, and the radio resources are reserved if the session is in an active state. The established session can either be in active or inactive state as indicated in 3GPP TS 23.247 [15]. The MC service clients sends a UE session join notification towards the server.

NOTE 3:	For broadcast MBS sessions, the session is established as part of the session creation procedures as described in 3GPP TS 23.247 [15], hence the network resources are reserved both in 5GC and NG-RAN.

5b.	For broadcast MBS sessions, the MC service clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS service announcement information, MC service clients report the monitoring state back to the MC service server.

NOTE 4:		It is implementation specific whether the broadcast reception quality level is determined per MBS session, per media stream or per MBS QoS flow level via e.g., measurements of radio level signaling such as the reference signals from the NG-RAN node(s), packet loss.

6.	The MC service clients provide a notification related to the announced session (multicast or broadcast session) in the form on an MBS listening status report.

7.	An MC service group communication setup takes place as specified in TS 23.379 [6], TS 23.281 [4], or TS 23.282 [5]. The MC service server determines to use the pre-created MBS session for this group communication.

# * * * * End of changes * * * *