3GPP TSG-SA WG6 Meeting #48-e	S6-220974

05th – 14th April 2022, Online	(revision of S6-220806r01)

# Source:	vivo

# Title:	New KI on service continuity

# Document for:	Approval

Agenda Item:	9.13

1. Introduction

One of the PINAPP features is, the UE can have a direct communication with PINAPP into a PIN, without have any routing from 5GS.

But, there are the situations that may have a service disruption or relocation:

-	When UE moves out of a PIN, the direct communication between two PIN applications may be influenced and changed (for example, via PEGC);

-	When the PIN application is in the status of moving, for example, moving from one PIN to another, the communication connection between two PIN applications should be maintained.

How to guarantee the service continuity should be studied at the TR phase.

2. Proposal

It is proposed to agree the following text in 3GPP TR 23.700-78 v 0.1.0.

# * * * * First change * * * *

4	Key issues

4.1	Key issue #a: Service continuity

As indicated in clause 6.38 of TS 22.261[x], the following requirements that describes the service continuity will be studied in this TR phase and reflect in this key issues:

-	The 5G system shall be able to minimize service disruption when a PIN Element changes the communication path from one PIN Element (e.g. PIN Element with Gateway Capability) to another PIN Element or operator provided mobile access. The communication path between PIN Elements may include licensed and unlicensed spectrum as well as 3GPP and non-3GPP access.

The PIN application is the application deployed on PIN elements (for example, the UE) in PIN. One of the PIN application features is, the PIN application in UE can have a direct communication with other PIN application into a PIN, without have any routing from 5GS.

But, there are the situations that may have a service disruption or relocation:

-	When UE moves out of a PIN or the terminal PIN application is in the status of moving, the original direct communication between two PIN applications may be influenced and changed (for example, the direct communication between two PIN applications may be routed via PEGC);

-	Usually if a PIN application on a UE is consuming service from an application server and due to UE mobility, the PIN application on the UE should continue to receive services from the application server.

It is required to study the following:

-	How to the support the service continuity when the PIN application in UE is in the status of moving?

-	How to the support the service continuity when UE moves out of a PIN and keep the communication towards the PIN application?

NOTE 1:	Some of the service continuity work has relationship to SA2 work and depends on the SA2’s feedback later.

4.2	Key issue #x: <Title>

This clause describes the key issue with a suitable title. Please provide a high-level description of the key issue along with a list of open issues.

* * * *   Next change   * * * *

9	Overall evaluation

This clause provides a summary of architecture enhancements and solution evaluations.

9.1	Architecture enhancements

9.2	Key issue evaluations

9.2.1	General

All the key issues, solutions and architecture enhancements specified in this technical report are listed in Table 9.2.1-1.

Table 9.2.1-1 provides a mapping of the key issues to the related solutions. It also indicates whether the solution requires enhancement to the Release-17 architecture and lists the dependencies on other working groups.

Table 9.2.1-1 Key issue and solutions

| Key issues (evaluation clause reference) | Solution | Architectural enhancement (clause reference) | Enhancements required | Dependency on other working groups |
| --- | --- | --- | --- | --- |
| KI #a Service continuity |  |  |  |  |
| KI#1: <title> | Solution #x: <title> | 6.x | Architecture / None | <WG> |
|  | Solution #y: <title> | 6.y | Architecture / None | <WG> |

9.2.2	Evaluation of key issue #a: Service continuity

9.2.x	Evaluation of key issue #x

This clause provides an overall evaluation of all the solutions defined for Key Issue #x.

* * * *   End of changes   * * * *