33GPP TSG-SA WG6 Meeting #49-e	S6-221103

e-meeting, 16th – 22nd May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0330 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Migration procedure during and ongoing private communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The introduced CR provide a generic procedure to be followed by an MC service client which has detected the need to migrate to another MC system during an ongoing private communication, e.g., MCPTT private call. The procedure includes both scenarios whether the MC service client migrates to its primary MC system or a partner MC system. Via this procedure the ongoing private communication is suspended due to migration of one private communication participant, the other participant is informed, and the communication is resumed once the migration is over. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A guidance of the behaviour of an MC service client in the form of procedure to be followed when a client has detected the need to migrate to another MC system during an ongoing private communication. - The suspention and resume messages are considered to be notifications/acknowlegment instead of request/response (where the latter is mentioned in TR 23.700-90) since the suspension is not requested nor it depend on the other participant. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The ongoing private communication will drop due to one participant has migrated to another MC system. - Private call participant will not be aware of the reason the private commincation has dropped. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.16.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 10.16.2.X Migration during an ongling private communication

##### 10.16.2.X.1 General

This subclause provides a genric guidance on the behaviour an MC service client follows to perform migration during an ongoing private communication, e.g., MCPTT private call. Once the MC service client detects the need to migrate during an ongoing private communication, it may initate preparations and UE configuration which facilitate migration as mentioned in clause 10.1.1.2. The described procedure is applicable to the scenarios whether an MC service client is migrating into its primary MC system or a partner MC system.

##### 10.16.2.X.2 Information flows

###### 10.16.2.X.2.1 Private call suspend notification

Tables 10.16.2.X.2.1-1, 10.16.2.X.2.1-2, and 10.16.2.X.2.1-3 describe the information flow during a private call suspend notification, which is sent from the migrating MC client to MC service server, from one MC service server to the other, and from MC service server to the participate MC client, respectively.

Table 10.16.2.X.2.1-1: Private call suspend notification information elements (from migrating MC client to MC service server)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the migrating client during a private call, e.g., MCPTT ID. |
| MC service ID | M | The MC service ID of the participant MC service client. |
| Suspension justification | M | Indicate that the reason of suspension is due to migration, and the call will be suspended until the migration procedure is finalized. |

Table 10.16.2.X.2.1-2: Private call suspend notification information elements (from one MC service server to the other)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the migrating client during a private call, e.g., MCPTT ID. |
| MC service ID | M | The MC service ID of the participant MC service client. |
| Suspension justification | M | Indicate that the reason of suspension is due to migration, and the call will be suspended until the migration procedure is finalized. |

Table 10.16.2.X.2.1-3: Private call suspend notification information elements (from MC service server to participant MC client)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the migrating client during a private call, e.g., MCPTT ID. |
| MC service ID | M | The MC service ID of the participant MC service client. |
| Suspension justification | M | Indicate that the reason of suspension is due to migration, and the call will be suspended until the migration procedure is finalized. |

###### 10.16.2.X.2.2 Private call suspend acknowledgement

Tables 10.16.2.X.2.2-1, 10.16.2.X.2.2-2, and 10.16.2.X.2.2-3 describe the information flow during a private call suspend acknowledgement, which is sent from the participant MC service client to the MC service server, from one MC service server to the other, and from MC service server to the migrating MC service client, respectively.

Table 10.16.2.X.2.2-1: Private call suspend acknowledgement information elements (from participant MC service client to MC service server)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the participating MC service client which has received private call suspend notification. |

Table 10.16.2.X.2.2-2: Private call suspend acknowledgement information elements (from one MC service server to the other)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the migrating MC service client initiating the private call suspend notification. |

Table 10.16.2.X.2.2-3: Private call suspend acknowledgement information elements (from the MC service server to the migrating MC service client)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the migrating MC service client initiating the private call suspend notification. |

###### 10.16.2.X.2.3 Private call resume notification

Tables 10.16.2.X.2.3-1, 10.16.2.X.2.3-2, and 10.16.2.X.2.3-3 describe the information flow during a private call resume notification, which is sent from the already migrated MC client to MC service server, from one MC service server to the other, and from MC service server to the participate MC client, respectively.

Table 10.16.2.X.2.3-1: Private call resume notification information elements (from the migrated MC client to MC service server)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the already migrated MC service client. |
| MC service ID | M | The MC service ID of the participating MC service client. |
| SDP offer | O | Media parameters of the migrated MC service client |
| Implicit floor request | O | An indication whether the migrated user is also requesting the floor. |
| Location information | O | Location of the migrated MC client. |
| Requested priority | O | Application priority level requested for this call. |

Table 10.16.2.X.2.3-2: Private call resume notification information elements (from one MC service server to the other)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the already migrated MC service client. |
| MC service ID | M | The MC service ID of the participating MC service client. |
| SDP offer | O | Media parameters of the migrated MC service client |
| Implicit floor request | O | An indication whether the migrated user is also requesting the floor. |
| Location information | O | Location of the migrated MC client. |
| Requested priority | O | Application priority level requested for this call. |

Table 10.16.2.X.2.3-3: Private call resume notification information elements (from the MC service server to the participating MC service client)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the already migrated MC service client. |
| MC service ID | M | The MC service ID of the participating MC service client. |
| SDP offer | O | Media parameters of the migrated MC service client |
| Implicit floor request | O | An indication whether the migrated user is also requesting the floor. |

###### 10.16.2.X.2.4 Private call resume acknowledgement

Tables 10.16.2.X.2.4-1, 10.16.2.X.2.4-2, and 10.16.2.X.2.4-3 describe the information flow during a private call resume acknowledgement, which is sent from the participant MC service client to the MC service server, from one MC service server to the other, and from MC service server to the already migrated MC service client, respectively.

Table 10.16.2.X.2.4-1: Private call resume acknowledgement information elements (from participant MC client to MC service server)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the participating MC service client. |
| MC service ID | O | The MC service ID of the migrated MC service client. |
| SDP answer | M | The offered media parameters are confirmed |

Table 10.16.2.X.2.4-2: Private call resume acknowledgement information elements (from one MC service server to the other)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the participating MC service client. |
| MC service ID | O | The MC service ID of the migrated MC service client. |
| SDP answer | M | The offered media parameters are confirmed |

Table 10.16.2.X.2.4-3: Private call resume acknowledgement information elements (from MC service server to the migrated MC service client)

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the participating MC service client. |
| MC service ID | O | The MC service ID of the migrated MC service client. |
| SDP answer | M | The offered media parameters are confirmed |

##### 10.16.2.X.3 Procedure

Figure 10.16.2.X.3-1 represents a generic procedure to be followed when migration is needed to be done during an ongoing private communication, e.g., private MCPTT call.

Figure 10.16.2.X.3-1: Migration during ongoing private communication.

- A private call takes place between MC service client 1 and MC service client 2, where the former is connected to MC system A and the latter to MC system B.

- The MC service client 1 detects the need to migration into MC system C and user 1 is notified to be prepared for possible service interruption. Furthermore, upon detection, MC service client 1 performs initial steps to be considered prior migration, e.g., UE configuration, authorization, and selection of user profile that permits migration, as described in clause 10.1.1.2.

- MC service client 1 sends a private call suspend notification to MC system A.

- The private call suspend notification is then to be sent from MC system A to MC system B.

- The private call suspend notification is then sent from MC system B to MC service client 2 in order to put the call on hold during the migration process. Hence, MC user 2 is informed that the call is to be on hold due to migration.

- MC client 2 may send a private call suspend acknowledgment to MC system B.

- MC system B may then send the acknowledgement to MC system A.

- MC system A may then send the acknowledgement to MC client 1 to acknowledge the reception of private call suspension.

NOTE 1: Steps 6-8 are optional.

- Migration takes place and MC system C retrieves MC service client 1 profile:

9a. if MC system C is the primary MC system, retrieval of MC service client 1 profile takes place according to clause 10.1.4.3.1.

9b. if MC system C is a partner MC system, retrieval of MC service client 1 profile takes place according to clause 10.1.4.3.2.

- As part of the migration procedure, authentication and signal plane migration takes place according to clause 10.1.6.2.

- Once the migration is finalized and connection is established, MC service client 1 which is now connected to MC system C sends a private call resume notification MC system C.

- The private call resume notification is to then be sent to MC system B.

- The private call resume notification is then to be sent to MC client 2 to change the status of the private call from on hold to active.

- MC service client 2 may send a private call resume acknowledgment to MC system B.

- MC system B may then send the acknowledgement to MC system C.

- MC system C may then send the acknowledgement to MC client 1 to acknowledge the reception of private call resume notification.

NOTE 2: steps 14-16 are optional.

- The private call continues between MC service client 1 and MC service client 2, where the former is connected to MC system C and the latter to MC system B.

# * * * * End of changes * * * *