3GPP TSG-SA WG6 Meeting #51-e	S6-222694

e-meeting, 10th – 19th October 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0070 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | bulk registration of Non-MSGin5G UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is proposes bulk registration procedure of Non-MSGin5G UEs. This CR also removes the EN in clause 8.2.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | bulk registration procedure of Non-MSGin5G UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | bulk registration procedure of Non-MSGin5G UEs is not supported by MSGin5G service. The EN in clause 8.2.4 cannot be removed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4, 8.2.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*****************Change 1************************

8.2.4	Non-MSGin5G UE De-registration

The Message Gateway performs de-registration with the MSGin5G Server on behalf of the Non-MSGin5G UEs, in order to terminate services from the MSGin5G Server.

NOTE:	After the procedure is completed, the Message Gateway may communicate the result to the requesting Non-MSGin5G UE and the procedure is out of scope of this document.

The procedure assumes that the Message Gateway is responsible for initiating the de-registration from the MSGin5G Server on behalf of the Non-MSGin5G UE. The signaling flow for Non-MSGin5G UE de-registration is illustrated in figure 8.2.4-1.

Pre-conditions:

1.	The Message Gateway successfully performed registration with the MSGin5G Server on behalf of the Non-MSGin5G UE.

Figure 8.2.4-1: Non-MSGin5G UE de-registration

1.	The Message Gateway determines to de-register the Non-MSGin5G UE with the MSGin5G Server.

2.	The Message Gateway sends a Non-MSGin5G UE de-registration request to the MSGin5G Server that includes the UE Service ID associated with the Non-MSGin5G UE, as shown in Table 8.2.4-1.

Table 8.2.4-1: Non-MSGin5G UE de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the Non-MSGin5G UE. |

3.	Upon receiving the request, the MSGin5G Server may initiate authentication procedures with the Message Gateway on behalf of the Non-MSGin5G Client. The MSGin5G Server deletes any applicable UE Service ID and associated MSGin5G Client Profile information that it has stored.

NOTE:	The authentication procedures in step 3 are built on top of the transport layer mechanism specified in Annex Y.2 of 3GPP TS 33.501 [16].

4.	The MSGin5G Server replies with a Non-MSGin5G UE de-registration response as shown in table 8.2.4-2.

Table 8.2.4-2: Non-MSGin5G UE De-registration response

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the Non-MSGin5G UE. |
| De-registration result | M | Indication if the de-registration is success or failure |
| Failure Cause | O | The reason for failure |

*****************Change 2************************

8.2.X	Non-MSGin5G UE bulk registration and bulk de-registration

Non-MSGin5G UEs (i.e., Legacy 3GPP UEs or Non-3GPP UEs) are connected to the MSGin5G Server through a Message Gateway. The Message Gateway may decide to use bulk registration procedure specified in this clause based on service policy. The procedure for Non-MSGin5G UE bulk registration is illustrated in figure 8.2.x-1.

Pre-conditions:

1.	The Message Gateway has been pre-configured with the MSGin5G Server address.

2.	The Message Gateway has been configured with the necessary information as specified in clause 8.1.3, i.e. UE Service ID and credentials on behalf of Non- MSGin5G UEs, and may also been configured with Non-MSGin5G UEs’ Profile if available, to enable authentication and Non- MSGin5G UEs registration at the Message Server.

3.	A secured connection has been established between the Message Gateway and the MSGin5G Server.

Figure 8.2.x-1: Non-MSGin5G UE registration

1.	The Message Gateway checks whether the Non-MSGin5G UE registrations can be bulked. If the Non-MSGin5G UE registrations cannot be bulked, the Message Gateway shall use the procedure specified in clause 8.2.3 and the rest steps in this clause shall not be used.

NOTE 1:	The conditions of whether the Non-MSGin5G UE registrations can be bulked are implementation specific and out of the scope of the current specification.

2.	The Message Gateway sends a Non-MSGin5G UE bulk registration request to the MSGin5G Server. The Information Elements specified in table 8.2.x-1 are included in the MSGin5G UE bulk registration request.

Table 8.2.x-1: MSGin5G UE bulk registration request

| Information element | Status | Description |
| --- | --- | --- |
| Bulk registration indicator | M | Indicates that this request is a Non-MSGin5G UE bulk registration request. |
| Number of individual MSGin5G UE registration requests | M | Indicates total number of Non-MSGin5G UE registration requests which are bulked in this Non-MSGin5G UE bulk registration request |
| List of individual Non-MSGin5G UE information | M | Each element in this list contains information as specified in Table 8.2.3-1. |

3.	Upon receiving the Non-MSGin5G UE bulk registration request, the MSGin5G Server splits the Non-MSGin5G UE bulk registration request, executes the authentication procedures with the Message Gateway on behalf of the Non-MSGin5G Client and authorises the Non-MSGin5G UE to receive the MSGin5G Service individually as specified in clause 8.2.3. If the registration is successful, the MSGin5G Server stores every UE Service ID and associated Non-MSGin5G UE Profile information of bulk registered Non-MSGin5G UE respectively.

Editor's note 1:	Security aspects of bulk registration and de-registration depend on SA3 and is FFS

4.	The MSGin5G Server includes all corresponding MSGin5G UE registration responses in a Non-MSGin5G UE bulk registration response and sends it to Message Gateway. The Information Elements specified in table 8.2.x-2 are included in the Non-MSGin5G UE bulk registration response.

Table 8.2.x-2: MSGin5G UE bulk registration response

| Information element | Status | Description |
| --- | --- | --- |
| Bulk registration response indicator | M | Indicates that this response is a Non-MSGin5G UE bulk registration response. |
| Number of individual Non-MSGin5G UE registration responses | M | Indicates total number of Non-MSGin5G UE registration responses which are bulked in this MSGin5G UE bulk registration response |
| List of individual Non-MSGin5G UE registration response | M | Each element in this list contains information as specified in Table 8.2.1-4. |