3GPP TSG-SA WG6 Meeting #51-e	S6-222711

e-meeting, 10th – 19th October 2022	(revision of S6-22xxxx)

Title:	LS on application-level authorization scope for northbound API invocation

Response to:

Release:	Release 18

Work Item:	Study on application enablement aspects for subscriber-aware northbound API access (FS_SNAAPP)

Source:	3GPP TSG SA WG6

To:	3GPP TSG CT WG3

Cc:	3GPP TSG SA WG3

Contact person:	Yuji Suzuki

yuuji.suzuki.rm@nttdocomo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Draft TR 23.700-95 v1.6.0

1	Overall description

SA6 has been studying in TR 23.700-95 the enhancements to the existing CAPIF functional model so that the CAPIF can support the subscriber-aware northbound API access (SNA), a scenario in which the northbound API invocation requires the API invoker obtaining the user’s (i.e. resource owner’s) consent before being allowed to access the resource.

In the SA6 study, it is assumed that the API invoker receives an access token for the API invocation, and that the resources to be accessed need to be identified by the "scope" of the access token. Considering the use cases described in Annex A of TR 23.700-95, the API-level ("can invoke this API"), user-level ("can invoke an API affecting this user, e.g. QoS"), and application-level ("can invoke an API for this application") granularity is required.

In solution #X, SA6 specifies that the API, user, and application information is to be included in the scope, but how to map application-level scope to network resources is expected to be defined in CT3. For example, considering the use case described in Annex A.1 of TR 23.700-95, ideally the API invoker may change the QoS of the resource owner only for a particular game. However, if the scope is defined only for the API-level or the user-level granularity, the game provider's server may send request to change the QoS of the same user for different applications (e.g., voice call), and the CAPIF, which decides whether to permit the API invocation based on the resource owner's authorization information, can allow such a QoS change.

SA6 would like CT3 to study how to map application-level scope to network resources in Release 18 stage 3.

2	Actions

To CT3

ACTION: 	SA6 asks CT3 to take the above request into account.

3	Dates of next TSG SA WG 6 meetings

SA6#52-e              14th November – 18th November 2022 	meeting

SA6#52-bis-e        16th January – 25th January 2023 	meeting