3GPP TSG-SA6 Meeting #51-e	S6-222768

# Online, , 10th Oct 2022 - 19th Oct 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0132 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New procedure and information flows for EAS Service APIs enablement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, Uangel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In FS_eEDGEAPP study, the enablement of Service APIs exposed by EAS was studied as Key Issue #2 in TR 23.700-98; and concluded to support for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs within the EDGEAPP architecture by exploiting CAPIF as specified in clause 7.8 of TR 23.700-98. The details are specified in TR 23.700-98 as follows: - architectural requirements for EAS Service APIs as specified in the clause 5.2 of TR 23.700-98 - architectural assumptions, operations, and information flows as specified in the clause 7.8 of TR 23.700-98 - new service KPI IE for EAS Service API as specified in the clause 7.8.2.3 of TR 23.700-98 |  |  |  |  |  |  |  |  |
| Summary of change: |  | New procedure and information flows are added to support for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs within the EDGEAPP architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Exposing EAS Service APIs within the EDGEAPP architecture is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | S6-222767 refers to the new clause (8.x) proposed in S6-222768. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE

8.x	Exposure of EAS Service APIs

8.x.1	General

The edge enabler layer supports for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs. In order to fulfil the architectural requirements specified in the clause 5.2.5, the edge enabler layer exploits CAPIF as specified in 3GPP TS 23.222 [6] to support publication/discovery, and change subscription of EAS Service APIs with providing the following functionalities:

-	An EAS may act as an API provider by supporting API provider domain functions (i.e., API exposing function, API publishing function, and API management function)

-	An EAS may act as an API invoker

-	An EES acts as a CAPIF provider by supporting CAPIF core function (CCF)

Note:

-	EES provides support for the logging and audit of EAS API invocation as specified in the clause 8.19 and 8.22 of 3GPP TS 23.222 [6].

-	EES controls the service API access as specified in clause 8.12 of 3GPP TS 23.222 [6].

8.x.2	Procedure

8.x.2.1	General

The essential operations regarding EAS Service APIs complying with CAPIF are as follows:

-	An EAS (acting as API provider) may publish its EAS Service APIs to EES (acting as CAPIF provider) and EES may further publish the EAS service APIs to remote EES via CCF interconnection.

-	An EAS (acting as API invoker) may discover EAS Service APIs from EES (acting as CAPIF provider) and EAS Service APIs from remote EES via CCF interconnection

-	An EAS (acting as API invoker) may subscribe to be notified of dynamic information or availability of EAS Service APIs from EES (acting as CAPIF provider)

-	As EAS1(acting as API invoker) discovers EAS2 (acting as API provider) from EES, it can invoke the Service APIs directly from EAS2

8.x.2.2	Publication, discovery, and subscription of EAS Service APIs

The Figure 8.x.2.2-1 depicts the publication, discovery, and subscription steps for EAS Service API enablement using the CAPIF operations as shown in Annex A of TS 23.222 [6].

Pre-conditions:

1.	The EAS #A-1 and EAS #A-2 have completed the EAS registration with the EES #A.

Figure 8.x.2.2-1: Publication/discovery/invocation operations for EAS Service API enablement

1.	The EAS #A-2 (AMF) registers its API provider domain functions to the EES #A (CCF) via CAPIF-5.

2.	The EAS #A-2 (APF) publishes its exposing Service API(s) to the EES #A (CCF) via CAPIF-4.

3.	The EAS #A-1 (API invoker) performs onboarding process with the EES #A (CCF) via CAPIF-1.

4.	The EAS #A-1 (API invoker) discovers from the EES #A (CCF) a Service API required to run via CAPIF-1.

5.	The EAS #A-1 (API invoker) invokes the Service API provided by EAS #A-2 (AEF) via CAPIF-2 as discovered from the EES #A (CCF).

6. 	The EAS #A-1 (API invoker) subscribes to notifications of any updates of target Service APIs on the EES #A (CCF) via CAPIF-1.

7.	The EES #A (CCF) and EES #B (CCF) inter-operate with each other via CAPIF-6 for interconnection operations for publication and discovery of Service APIs managed by each EES.

As per the step 7, the Figure 8.x.2.2-2 depicts the distributed deployment of EESs (CCFs) as shown in clause 7.4 of TS 23.222 [6].

Figure 8.x.2.2-2 Distributed EES (CCF) deployment for API invocation across EES

The essential operations regarding distributed EES deployment and across EES API invocation complying with CAPIF are as follows

-	In the distributed deployment, EES#A (acting as CCF#A) and EES#B (acting as CCF#B) interact with CCF#C via CAPIF-6 reference point. The CCF#C assumes the role of a centralized repository of the service APIs in the PLMN trust domain. (e.g., Service APIs exposed by EAS #A-2 and EAS #B in the Figure 8.x.2.2-2)

-	The EES#A and EES#B publishes the service API(which are published by EAS #A-2 and EAS #B respectively) provided by its connected API exposing function(s) to CCF#C, and obtains the service API information provided by other EES from centralized CCF#C

-	EAS#A-1 (acting as API invoker) is able to discover and invoke the service APIs provided by EAS#B connected to CCF#B

END OF CHANGES