3GPP TSG-SA WG6 Meeting #51-e	S6-222806

e-meeting, 10th – 19th October 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0132 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of redundant transmission for URLLC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S6 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently the 5G network designs the AF influence the URSP to allow the AF to configure the URLLC redundant transmission for URLLC. How the SEAL can faciliate the application to better utlize this feature is not specified yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new procedure to enable the VAL server to configure the URLLC redundant transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Verticals cannot configure the URLLC redundant transmission via the SEAL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /********************* First Change *********************/

### 14.3.x	URLLC redundant transmission configuration procedures

#### 14.3.x.1	General

The procedures related to the 5G URLLC redundant transmission configuration which utilizes the application guidance for URSP determination to allow the VAL server to provide the application layer input to impact the URSP determination.

#### 14.3.x.2	URLLC redundant transmission configuration procedure

URLLC redundant transmission configuration procedure is used by the VAL server to provide the configuration of URLLC redundant transmission to the network to impact the URSP determination.

Figure 14.3.x.2-1: URLLC redundant transmission configuration procedure

- The VAL server sends URLLC redundant transmission configuration request to the NRM server to provide the application information. The request includes Service Description, Service Parameters, a specific UE, or a group of UE(s) or any UE that the AF request may be associated with, subscription to the outcome.

- The NRM server initiates the application guidance for URSP determination procedure towards the 5GC as described in 3GPP TS 23.502 [11].

- The NRM server returns the URLLC redundant transmission configuration response to the VAL server.

# /********************* First Change *********************/

#### 14.3.2.xx	URLLC redundant transmission configuration request

Table 14.3.2.xx-1 describes the information flow for URLLC redundant transmission configuration request from the VAL server to the NRM server.

Table 14.3.2.xx-1: URLLC redundant transmission configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Service Description | M | The Service Description as defined in 3GPP TS 23.502 [11], cl. 4.15.6.10 and 3GPP TS 23.503 [19]. |
| Service Parameters | M | The Service Parameters as defined in 3GPP TS 23.502 [11], cl. 4.15.6.10 and 3GPP TS 23.503 [19]. |
| List of VAL UE IDs | O (NOTE) | List of VAL UEs whose configuration is associated with |
| List of VAL UE IDs | O (NOTE) | List of VAL UEs whose configuration is associated with |
| Any UE indication | O (NOTE) | Any UE association with the Service Description. |
| Notification URL | O | The URL to receive the event notification about the outcome of the UE Policies delivery due to application guidance for URSP determination. |
| NOTE: At least one of those IEs shall be present. |  |  |

#### 14.3.2.yy	URLLC redundant transmission configuration response

Table 14.3.2.yy-1 describes the information flow for URLLC redundant transmission configuration response NRM server to the VAL server.

Table 14.3.2.yy-1: URLLC redundant transmission configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the |

# /********************* End of Changes *********************/