3GPP TSG-SA WG6 Meeting #51-e	S6-222881

e-meeting, 10th – 19th October 2022	(revision of S6-222645)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0029 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additions to functional entities on support for multi-USS deployments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-10-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Additions to functional entities due to support for multi-USS deployments are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The additions to functional entities due to support for multi-USS deployments are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The additions to functional entities due to support for multi-USS deployments are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.4, 5.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***   First change   ***

### 5.3.4	UAE client

The UAE client supports interactions with the UAS application specific client(s).

The UAE client provides the client side UAS application layer support functions as below:

-	receiving and storing C2 operation mode configurations;

-	selecting primary and secondary C2 communication modes based on the configurations;

-	switching of C2 communication in emergency scenarios;

-	supporting UAV application message communication handling;

-	providing the UAE server with the Multi-USS capability;

-	receiving and storing Multi-USS policies; and

-	based on Multi-USS policies, switching of UAS application specific server in emergency scenarios.

### 5.3.5	UAE server

If CAPIF is supported, the UAE server acts as CAPIF's API exposing function to provide service APIs to the UAS application specific server (e.g. USS/UTM) or another UAE server as specified in 3GPP TS 23.222 [3], or acts as CAPIF's API invoker to consume the service APIs provided by another UAE server.

The UAE server provides the server side UAS application layer support functions as below:

-	performing group based QoS management for the UAS (i.e. pair of UAV and UAV-C) by using SEAL APIs.

-	receiving C2 operation mode configuration from UAS application specific servers (e.g. USS/UTM) and further configuring the UAS UEs (i.e. UAV, UAV-C);

-	triggering C2 communication mode switching with the UAS UEs;

-	receiving and storing the selected C2 communication modes from the UAS UEs;

-	monitoring the real-time status of UAS UEs by using SEAL APIs;

-	supporting UAV application message communications between UAVs; and

-	receiving Multi-USS policies from UAS application specific servers and further configuring the UAS UEs (i.e. UAV).

***   End of change   ***