3GPP TSG-SA WG6 Meeting #51-e	S6-222884

e-meeting, 10th – 19th October 2022	(revision of S6-222865, 222362, 222059, 221871, 221520)

# Source:		InterDigital

# Title:		New Solution: Support for DAA

Spec:		3GPP TR 23.700-55

# Agenda Item:	9.9

# Document for:	Approval

Contact:		Atle Monrad (atle.monrad@interdigital.com)

1. Introduction

A standardised mechanism for support of the UAE-layer for DAA is proposed to secure that 3GPP has means that the UAVs has functionality within the UAE layer to support the application for detect and avoid of potential interfering objects. It is proposed to add a new solution for the UAE-layer support of DAA in 3GPP TR 23.700-55 v 1.0.0.

* * * *   Start of changes   * * * *

## 7.6	Solution #4: UAE-layer support for DAA

### 7.6.1	Architecture enhancements

None.

### 7.6.2	Solution description

#### 7.6.2.1	General

This solution aims to address the gaps identified in Key Issue #4 "Support of detect and avoid services and applications".

The solution covers registration of the UAE clients DAA capability to the USS and assistance by the UAE layer for provision of DAA policies from the UAS application specific server to the UAE client and the UAE server.

The functions of the UAS application specific server and the UAS application specific client are out of scope of the solution.

#### 7.6.2.2	Registration of DAA capabilities

Pre-conditions:

-	The UAE client has discovered the UAE server and is aware of the address of the UAE server (e.g., FDQN).

NOTE:	How the UAE client is provisioned with the UAE server information is outside the scope of the current document.

-	The UAV has already been assigned with the UAV ID.

Figure 7.6.2.2-1: Registration of DAA capabilities of UAE client and UAE server

1.	The UAE client sends a registration request to the UAE server. The UAE client includes an indication of its DAA capability.

2.	The UAE server performs authentication and authorization check (e.g., based on pre-provisioned security information or by interacting with UAS application specific server.

3.	The UAE server sends a registration response to the UAE client indicating success or failure of the registration for DAA support.

#### 7.6.2.3	Provision of DAA capabilities

##### 7.6.2.3.1	DAA management

Figure 7.6.2.3.1-1 illustrates the DAA management procedure where the UAE server receives an application request for managing the DAA configuration parameters from the UAS application specific server.

Pre-condition:

-	The UAV has received its UAS ID from the UAS application specific server.

-	The UAV has performed the UAS UE registration procedure.

Figure 7.6.2.3.1-1: DAA management procedure

1.	The UAS application specific server sends to the UAE server a DAA management request with the DAA configuration parameters. The request includes the UAV (UAE client) identification information and the DAA configuration parameters. The UAE server stores the DAA configuration parameters corresponding to the UAV ID. The configuration includes a DAA deconflicting policy that provides parameters/rules on how the UAE layer can assist with respect to detect and avoid critical situations. For example, the deconflicting policy can include rules for UAV communication with the UASS and/or other UAVs or other actions (e.g., change path or altitude) in a situation where the UAS application specific client does not seem to act. The DAA deconflicting policy can also consider whether the UAV is in-coverage and out-of-coverage of the network and whether the flight path conflict can be resolved locally by the UAS application.

NOTE:	The complete list of parameters for the deconflicting policy will be specified during the normative work.

2.	The UAE server sends to the UAS application specific server a DAA management response with a positive or negative acknowledgement of the request.

3.	UAE server executes the DAA configuration according to clause 7.6.2.3.2.

4.	After successful execution of DAA configuration, the UAE server notifies the UAS application specific server with DAA management complete based on the configured capabilities of the UAE client and the UAE server.

##### 7.6.2.3.2	DAA configuration

Figure 7.6.2.3.2-1 illustrates the DAA configuration procedure. This procedure enables the UAE server to configure the UAE client with DAA configuration parameters, based on a request from UAS application specific server.

Pre-conditions:

1.	The UAS UEs are connected to 5GS and authenticated and authorized by UAS application specific server as specified in clause 5.2 of 3GPP TS 23.256 [4].

2.	UAE server has established a UAE session with the respective UAE clients as the UAE clients are successfully registered to the UAE server.

3.	UAE server has performed the initiation of DAA management capabilities as in clause 7.6.2.2.2.

Figure 7.6.2.3.2-1: DAA configuration procedure

1.	The UAE server sends a DAA configuration request to the UAE client. The UAE client receives a DAA configuration request from the UAE server that includes the DAA configuration parameters.

NOTE 1:	Details in case of e.g. removal of DAA configuration will be specified during the normative work in stage 2 and stage 3.

2.	The UAE client stores or removes the DAA configuration parameters as per the information received in step 1. The UAE client provides the application layer with the DAA configuration.

NOTE 2:	Communication between the UAE client and the application layer is not standardised.

3.	The UAE client sends a DAA configuration response to the UAE server.

#### 7.6.2.4	UAE-layer assisted change of flight path due to DAA

The Figures 7.6.2.4-1 and 7.6.2.4-2, illustrates the procedure with UAE-layer assisted change of client-initiated flight path due to DAA. Figure 7.6.2.4-1 shows how a UAS application specific client initiate a DAA procedure.

Pre-conditions:

1.	UAE client and UAE server have indicated DAA support.

2.	UAS application specific server has provided DAA configuration parameters to the UAE client.

Figure 7.6.2.4-1: client-initiated UAE layer assisted change of flight path due to DAA

1.	When the UAS application, based on the DAA configuration parameters detects UAVs or objects in proximity, the UAS application will provide the UAE client with information related to change of flight path.

NOTE 1:	The service logic used for DAA by the UAS application and the communication between the UAE client and the application layer is not standardised.

The UAE client sends a DAA client information to the UAE server indicating a detected or resolved flight path conflict with one or more UAVs in proximity. The DAA client information can include updated trajectory with a cause of DAA.

If the UAE client considers an emergency situation (e.g., due to lack of response from the UAS application specific server), the UAE client shall inform the application layer.

2.	The UAE server sends the DAA client information to the UAS application specific server.

NOTE 2:	The need to coordinate SEAL services for flight path monitoring and notification (e.g., interaction with flight path deviation detection) to fulfil support for DAA can be discussed and decided as part of the normative work.

3.	The UAS application specific server reply with a DAA client information acknowledge to the UAE server. The UAS application specific server includes information about how to resolve the situation, i.e. accept the resolution or inform the client how to act.

Coordination with Real-Time UAV connection status monitoring and location reporting is performed by the UAE server to secure that the UAE-layer is not reporting deviations due to deliberate modification in flight path due to DAA as irregular events, see 3GPP TS 23.255 [3] clause 7.5 and 3GPP TS 23.434[5], clause 9.3.

4.	The UAE server acknowledges the DAA client information acknowledge to the UAE client, and the UAE client provides the application layer with the information from the UAS application specific server.

NOTE 3:	The communication between the UAE client and the application layer is not standardised.

Figure 7.6.2.4-2 shows how a UAS application specific server communicate with a client to initiate a change of communication path based on how this or other clients move.

Pre-conditions:

1.	UAE client and UAE server have indicated DAA support.

2.	UAS application specific server has provided DAA configuration parameters to the UAE client.

3.	The application specific server has received information from clients or network functions that a cause that DAA is necessary. The service logic used for DAA by the UAS application specific server is not standardised.

Figure 7.6.2.4-2: server-initiated UAE-layer assisted change flight path due to DAA

1.	The UAS application specific server sends a DAA server information to the UAE server indicating a detected flight path conflict. The request includes the UAV identification and new/corrected flight path information. The UAE server verifies that the request is authorized as described above before sending the request for change of flight path to the UAE client.

2.	The UAE client receives a DAA server information from the UAE server.

3.	The UAE client informs the UAS application specific client about change of flight path.

NOTE 4:	The communication between the UAE client and the application layer is not standardised.

4.	The UAE client sends to the UAE server a DAA flight path update response to indicate a change of flight path.

Coordination with Real-Time UAV connection status monitoring and location reporting is performed by the UAE server to secure that the UAE-layer is not reporting deviations due to deliberate modification in flight path due to DAA as irregular events, see 3GPP TS 23.255 [3] clause 7.5 and 3GPP TS 23.434 [5], clause 9.3.

5.	The UAE server sends a DAA flightpath response to confirm a change of flight path/DAA conflict resolution to the UAS application specific server.

### 7.6.3	Solution evaluation

This clause provides an evaluation of this solution.

* * * *   Next change   * * * *

### 9.2.1	General

All the key issues, solutions and architecture enhancements specified in this technical report are listed in Table 9.2.1-1.

Table 9.2.1-1 provides a mapping of the key issues to the related solutions. It also indicates whether the solution requires enhancement to the Release-17 architecture and lists the dependencies on other working groups.

Table 9.2.1-1 Key issue and solutions

| Key issues (evaluation clause reference) | Solution | Architectural enhancement (clause reference) | Enhancements required | Dependency on other working groups |
| --- | --- | --- | --- | --- |
| KI #1 Direct communication between UAVs |  |  |  | SA2 |
| KI #2: Support for multi-USS deployments | Solution #1: Change of USS during flight NOTE | 7.3 | None | None |
|  | Solution #2: Support for USS re-mapping for a UAS | 7.4 | None | None |
| KI 3: Coordination between Uu and PC5 for direct UAV-to-UAV or UAV-to-UAV-C communication | Solution #3: Support for C2 direct mode feasibility reporting | 7.5 | None | SA2 |
| KI 4: Support of detect and avoid services and applications | Solution #4: UAE-layer support for DAA | 7.6 | None | None |
| NOTE: Change of DN/EDN to avoid disruption while in flight due to change of USS is not covered by this solution. |  |  |  |  |

* * * *   End of changes   * * * *