3GPP TSG-SA WG6 Meeting #51-e	S6-222978

e-meeting, 10th-19th October 2022	(revision of S6-222671 rev 3)

Source:	Convida Wireless LLC

Title:	AC Association-aware solution update

Spec:	3GPP TR 23.700-98 v.1.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

1. Introduction

The proposal in S6-222671 is a revision of S6-222383_rev2, as discussed at SA6 #50.

Compared to S6-222383_rev2 the changes in S6-222671  are:

- Corrections and editorials.

- Clarifications in clause 7.27.2.2.1.2 on the meaning and use of dynamic grouping

- Adding solution evaluation text in 7.27.3

- Adding a Background clause after this one, containing a limited number of notes from previous discussions. Please refer to previous versions of this contribution for more such clarifications.

S6-222671 rev1 includes the following changes (highlighted in blue in the proposed text):

- clauses 7.27.0, 7.27.1 and 7.27.2.8 are updated to clarify the solution when CAAR is implemented as a function of ECS.

- clause 7.27.3 (Evaluation) is updated to clarify how KI #17 open issues are addressed by this solution

- clause 7.27.3 (Evaluation) is also updated to clarify the architectural impact differences between CAAR implementation as a separate architectural entity vs. as a ECS function.

S6-222671 rev2 includes the following changes (highlighted in blue in the proposed text):

- Deletion in 7.27.2.2.1.2 of text on AC Association ID assignment.

- Undid a number of updates in the AAP for simplification. Instead of “common EAS characteristics” IE, only one of 2 component IEs (Common EAS aggregate Service KPIs) has been maintained. The text where the old IE was referenced has been updated.

- Added assumptions in clause 7.27.0 modeled by those in Solutions #28 and #31.

S6-222671 rev3 includes

- Removal of some of the changes in rev 1 and 2 (no track marks left), overall simplifications.

- Updates to the assumptions

- Corrections to AAP

S6-222978 includes

- Removal in 7.27.0 of the sentence “CAAR functionality can be implemented as part of ECS functionality or within a new, standalone entity.”

- Removal of highlights.

2. Background

Notes/responses from previous discussions:

- Addition of AC Association (AA) ID list to the EDN configuration IE in 7.27.2.3 should be removed.

Response: without providing AA ID info in the EDN configuration, common EES selection is very difficult. Currently this addition applies only to option (ii) anyway. If the idea is to have the ECS determine a single EES to be provided to the EEC in the response, this is problematic on many fronts:

  - An EEC service provisioning request should be able to serve more than one AC at a time. Without understanding which EES is suitable as common EES for which AC association, the procedure needs to be repeated to serve multiple ACs if at least one is in an association. Alternatively, when the provisioning request includes more than one AC that has AAP (AA Profile), the response is ambiguous.

  - The restriction of having only one suitable EES acting as common EES per EDN or deployment is unnecessary and so far, unfounded.

  - ACs and EECs have additional info that can be used to determine the best EES from a list – therefore they should be provided with a list if available.

- Why is the List of common EAS characteristics needed in AAP – can’t the discovery filters be used for common EAS determination?

Response: AAP is pre-provisioned at the AC; therefore, it is the only way to ensure that the members of the association use common criteria. In addition, the filter criteria should be used to determine regular EASs for the AC. In many cases connecting to common EAS is optional.

- How are the “common UE characteristics” and “common AC characteristics” IEs used?

Response: The two lists have been clarified with more text added in the AAP table and in clause 7.27.2.2.1.2.

In general, “common AC characteristics” is used for dynamic grouping. When the ACs have been pre-grouped, the grouping relies on a common “UE group ID” which is pre-provisioned to all the members.

- Why is “AC association ID” made optional in the Profile?

Response: AC association ID is being used as a key in the global database of AC associations, therefore it needs to be unique in the system. I think only if it is assigned by EEL, we can ensure this

- Why is supporting dynamic grouping necessary?

Response: Supporting dynamic grouping is the only significant functionality that can be provided by EEL for this feature. While support of pre-determined grouping is important for coverage of a broad range of use cases, pre-determined grouping relies more on non-SA6 functionality than on EEL.

3. Reason for Change

Provide further details for solution #27.

4. Proposal

It is proposed to agree to this solution #27 update to 3GPP TR 23.700-98.

* * * First Change * * * *

## 7.27	Solution #27: Enabling AC Association Aware services by selecting common EASs

### 7.27.0	General

This clause proposes solutions for KI#17 based on two distinct assumptions for achieving a common EES  (which  is necessary before determining a common EAS). The options corresponding to the two assumptions are termed “assumed common EES” and “with CAAR”. CAAR functionality and deployment options are described as part of this solution.

These two alternatives can be summarized as follows:

- Determine common EAS based on the assumption that common EES has been achieved/ pre-provisioned (option termed “assumed common EES”).

  - NOTE 1: The requirements and underlying assumptions for this option are to be detailed in the normative phase.

- Determine common EAS based on the assumption that ACs to be associated are mobile and may span multiple EDNs. This option uses a new  functionality for maintaining information on AC Association serviced per EES (i.e., option ”with CAAR” functionality), in order to optimize common EES discovery.

The following descriptions apply to both  alternatives for determining Common EES, unless sub-clause titles or clarifying NOTEs specify otherwise.

This solution also relies upon the following assumptions:

- The architecture and cardinality rules from TS 23.558 [2] v17.5.0 clause 6 apply.

- The ACID definition from TS 23.558 [2] v17.5.0 clause 7.2.5 applies, allowing ACs with the same or different ACIDs (which belong to applications) to discover and use a common EAS.

- ACs to be associated may span multiple EES service areas and/or EDNs and require continuity of common EAS support for all ACR scenarios in Rel-17.

- Common EAS information (i.e., AC Association Profile or AAP) can be stored in the AC profile and can be uniquely identified by the AC association Profile ID (AAP ID). The AAP can include criteria for determining the ACs to be associated and criteria for determining a common EAS. How the AAP is provided to ACs is out of scope.

- Criteria for determining ACs to be associated can include:

  - A single, common identifier provisioned (via AAP) after the necessary grouping criteria has been applied (out of scope of this specification) to all ACs in scope.

  - One or more filter criteria provisioned (via AAP) after the necessary grouping criteria has been determined (out of scope of this specification) to all ACs in scope. The filter criteria are applied by EES to determine the ACs to be grouped.

- AAP information can be stored in the EEL, including by EESs and CAAR, as applicable. Mechanisms for updating AAP information stored in the EEL and for resolving conflicts, if necessary, are addressed as part of the normative phase.

- Procedures for coordination of the ACRs for AC association members are to be addressed in the normative phase.

- Synchronization of CAAR query and update procedures is to be addressed in the normative phase.

- How to ensure AAP ID uniqueness within the system is to be addressed in the normative phase.

- Whether the usecases and assumptions underlying the solution need to be further restricted, and how, is to be addressed in the normative phase.

### 7.27.1	Architecture enhancements

As described in clause 7.27.0, multiple options for the implementation of this solution are provided. Option (ii) relies on the introduction of new functionality termed Central AC Association Repository (CAAR). CAAR functionality can be implemented by ECS or as an independent CAAR server.

This clause introduces architectural enhancements based on the assumption that the SP deploys an independent server which hosts the CAAR functionality, and which communicates with multiple ECSs and each of the EESs serving Common EAS groups.

NOTE 1: The architectural enhancement introduced in this clause is optional. It can be used for the option “with CAAR” (ii) (as described in clause 7.27.0). CAAR functionality can be integrated with  ECS.

Figure 7.27.1-1: EDGEAPP architecture enhanced with Central AC Association Repository

The EDGE-X reference point is introduced to support EES interaction with a Central AC Association Repository (CAAR). The EES can store, update, and remove information about the AC associations it provides services to via EDGE-X reference point.

The EDGE-Y reference point is introduced to support ECS interaction with CAAR. The ECS can query the AC association information via EDGE-Y reference point.

NOTE 2: It is to be determined in the normative phase whether CAAR and Binding Server from solution #30 can be merged.

### 7.27.2	Solution description

#### 7.27.2.1	General

This solution addresses key issue #17 on discovery of a common EAS in clause 4.17.

The solution describes enhancements to several edge procedures that enable ACs/EECs of different UEs to share AC association information with the EEL. This AC association is formed such that the services for the associated ACs are optimized by selecting a common EAS.

NOTE:	In the following descriptions within this solution, the term "Associated ACs" is used to denote a set of ACs on different UEs for which the association described above has been configured via an AC association Profile. How the AC association Profile is determined is out of scope of the current specification.

#### 7.27.2.2	New Information Elements

##### 7.27.2.2.1	AC Association Profile

###### 7.27.2.2.1.1	Description

The AC association Profile (AAP) is proposed as a new IE in TS 23.558:

Table 7.27.2.2.1.1-1: AC association Profile

| Information element | Status | Description |
| --- | --- | --- |
| Association ID (NOTE 1) | M | Identifier of the Association |
| AC association type | M | Choice of dynamic grouping (multi-user, multi-session, etc.), pre-grouped or none (default) |
| List of UE filter criteria |  | O |  | Information for filtering the UEs with associated ACs. |  |
| > UE group ID | O | If present, it indicates a 3GPP Group ID pre-provisioned (e.g., as External Group ID) to the UEs with ACs in the associated group/ association. |
| > UEs service area | O | If provided, it indicates the Service area for determining other UEs in the association. The UE location is described in clause 7.3.2. The optional additional EAS selection criteria describe criteria for the EAS selection (e.g., "same latency for all" or "lowest latency for the own UE location"). |
| List of associated ACs characteristics | O | Information for determining the associated ACs. |
| > common ACID | O | If provided, all ACs in the association have the same AC ID as indicated |
| > common ACs Type | O | If provided, all ACs in the association have the same AC Type as indicated |
| > common ACs Schedule | O | If provided, all ACs in the association have the same AC Schedule as indicated |
| List of Common EAS aggregate Service KPIs | O | Service characteristics provided by the common EAS, detailed in Table 8.2.5-1. The characteristics are described to meet the requirements for the AC association. |

NOTE 1: It is to be determined in the normative phase whether and how this IE can also include an Association ID type, format or numbering (e.g. system-wide unique numbering, etc.)

NOTE: The list of IEs in Table 7.27.2.2.1.1 -1 and whether AC association information should be provided in a new AC association Profile IE, or via existing IEs, is to be determined in the normative phase.

###### 7.27.2.2.1.2	Determining grouping based on AC Association type

AC Association Profiles (AAPs) are assumed to be provisioned using procedures out of scope of the current document.

- AC associations of type pre-grouped are those for which the application of grouping criteria is done out-of-band (i.e., via an external group ID). Each of the association members is provided with an AAP which includes AC Association type (i.e., pre-grouped), UE group ID (same for all the ACs and equal to the external group ID) and List of Common EAS aggregate Service KPIs. This allows pre-grouped associations to be used for a variety of usecases. The AC grouping is then performed at EEL based only on this, limited AAP information.

- AC associations of type dynamic grouping are those for which the AAP provides characteristics/ criteria of the individual ACs (or corresponding UEs) within the association, with the AC grouping being performed at EEL based on the entire AAP information.

The "List of associated AC characteristics" and the “UE filter criteria” IEs are used as characteristics for filtering the ACs in the dynamic association.

Therefore, for AC associations of type dynamic grouping, the possibilities for EEL-based grouping are:

- Based on a matching associated ACs characteristics

- Based on UE service area

- Based on UE group ID (e.g., external Group ID)

- Combinations of the above

NOTE 1: Dynamic grouping which is based solely on UE group ID (i.e., option 3 above) is logically equivalent with the use of a pre-grouped AC Association.

For all AC association types, if the “List of Common EAS aggregate Service KPIs” IEs is present in the AAP, it is used to determine the EAS(s) suitable to act as common EAS for the association, in addition to the EAS discovery filters. If the IE is not present in the AAP, common EAS determination relies upon other parameters provided, e.g., EAS discovery filters.

AC Association ID is a unique identifier of an association within the EEL deployment or the system. The ID is necessary for uniquely identifying AC associations in the absence of the full AAP information.

#### 7.27.2.3	Enhancements to existing Information Elements

The enhancements captured (marked with bold text) in tables within this clause are proposed to Information Element tables in 3GPP TS 23.558 [2]. Enhancements to requests, responses and notifications are described in the following clauses via procedural descriptions.

Table 7.27.2.3-1: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g., V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g., time windows) |
| > EAS Geographical Service Area | O | Location(s) (e.g., geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area | O | Topological area (e.g., cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > Service permission level | O | Required level of service permissions e.g., trial, gold-class |
| > Service feature(s) | O | Required service features e.g., single vs. multi-player gaming service |
| AC association profile | O | Describes an association between ACs and an association between ACs and a common EAS. |
| NOTE 1: Either "List of AC characteristics", "List of EAS characteristics" or AC association profile shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

NOTE 1: The enhancement to EEC Context shown in table 7.27.2.3-2 is necessary for the option “with CAAR” (ii) as described in clause 7.27.0. It is to be determined in the normative phase whether this functionality is necessary for the “assumed common EES” (i) as described in clause 7.27.0.

Table 7.27.2.3-2: EEC Context

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| EEC Context ID | M | Identifier assigned to the EEC Context |
| Source EES Endpoint | M | The endpoint address (e.g., URI, IP address) of the EES that provided EEC context ID. |
| UE Identifier | O | The identifier of the hosting UE (i.e., GPSI or identity token) |
| List of EDGE-1 subscriptions | O | List of subscriptions IDs for capability exposure to the EEC ID (NOTE 1). |
| UE location | O | Latest UE location of the UE hosting the EEC which was available at the EES. |
| List of AC Profiles | O | Information about the ACs as described in Table 8.2.2-1. |
| List of AC association profiles | O | List of all AC associations applicable to the EEC |
| List of Service Session Contexts | O | List of associated Service Session Context IEs. Each Service Session Context includes information maintained by the EES for the services (involving UE related resources) received from an EAS registered to the EES. |
| > Service Session Context | M | Service Session Context is described in Table 8.2.8-2 (NOTE 2) |
| NOTE 1: The corresponding EDGE-1 subscription information may include 3GPP CN subscription information such as subscription correlation ID NOTE 2: Whether the Service Session Context IE needs to contain AC Association ID as optional element, is to be determined in the normative phase. |  |  |

NOTE 2: The enhancement to EDN configuration information shown in table 7.27.2.3-3 is necessary for the option “with CAAR” (ii) as described in clause 7.27.0., only.

The IE enhancement is used to provide EECs in the service provisioning phase with information about which EESs already act as common EESs serving AC associations, and which are candidates. This enables the EEC to determine which EES to choose if common EAS association services are needed. Note that the service provisioning response can also be limited by ECS policies to include only the EDNs/EESs which provide such services. The information about served AC Associations per EES is provided by EESs to ECS using CAAR functionality.

Table 7.27.2.3-3: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| EDN connection information (NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| > DNN/APN | M | Data Network Name/Access Point Name |
| > S-NSSAI | O | Network Slice information |
| > EDN Topological Service Area | O | The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| List of EESs | M | List of EESs of the EDN. |
| > EESID | M | The identifier of the EES |
| > EES Endpoint | M | The endpoint address (e.g., URI, IP address) of the EES |
| > EASIDs (NOTE 2) | O | List of EASIDs registered with the EES. |
| > EES Provider identifier | O | The identifier of the EES Provider (such as ECSP) |
| > EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| > EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| > List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| > EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| > EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| >List of association IDs (NOTE 3) | O | List of association IDs of the AC associations the EES is serving |
| Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g., no EAS information or a subset of EAS information related to the EES). NOTE 3: Whether the list needs to contain the entire AC association Profile, or the IDs are sufficient, is to be determined in the normative phase. |  |  |

#### 7.27.2.4	Enhancements to Service Provisioning for determining common EES

##### 7.27.2.4.1	Using the “assumed common EES” option (option i)

If the “assumed common EES” option is used, procedures for determining common EES are not necessary.

##### 7.27.2.4.2	Determining common EES with CAAR (option ii)

During the service provisioning procedure defined in 3GPP TS 23.558 [2], the presence of an AC association Profile in the request triggers the use of the enhancements detailed in this clause.

The service provisioning response is used to provide information about which EESs already act as Common EESs, i.e., providing services to AC associations, as well as candidates. In this option, the EES information is provided to EECs via the EDN Configuration Information in the service provisioning response, and it can be obtained by ECS using CAAR queries.

Enhancements to 3GPP TS 23.558 clause 8.3.3 Service Provisioning

Steps:

- The service provisioning request (3GPP TS 23.558 [2] Table 8.3.3.3.2-1) from EEC to ECS is enhanced to include an AC association Profile.

- Upon receiving the request, the ECS uses the AC association Profile information (e.g., association IDs) to query the (EES ID, AC association ID list) tuples stored by CAAR to determine whether an EES is already serving the AC association(s). The ECS uses the AC association ID(s) to query the CARR using the procedure in 7.27.2.8.

- If the processing of the request with AC association Profile information was successful, the ECS responds to the EEC with a service provisioning response. The ECS uses the EES(s) obtained from the step 2 CAAR query to update the enhanced EDN configuration information (Table 7.27.2.3-4) with EES(s) already serving the AC association and sends it in the provisioning response.

#### 7.27.2.5	Enhancements to  EAS Discovery for determining common EAS

During the EAS discovery procedure defined in 3GPP TS 23.558 [2], if the EEC sends an EAS discovery request to an EES including AC association Profile it triggers the use of the enhancements detailed in this clause. Based on the AC association Profile, the EES determines an EAS discovery result that includes a common EAS for the AC association.

Enhancements to 3GPP TS 23.558 [2] clause 8.5.2.2 EAS Discovery (request-response model)

Additional pre-condition:

- An AC association Profile has been provided to the EEC.

Steps:

The EEC sends an EAS discovery request to the EES. The EAS discovery request includes an AC association Profiles. Included in the AC association Profile is information regarding an association between AC(s) on this UE and other ACs indicating that the use of a common EAS is required.

2.	Upon receiving the request, the EES determines whether a common EAS is available to provide services to the associated ACs that meet the criteria specified in the AC association Profile (i.e., List of Common EAS aggregate Service KPIs).

#### 3.	If the processing of the request was successful, the EES responds to the EEC with an EAS discovery response which includes information about the discovered common EAS able to provide services to the associated ACs.

#### 7.27.2.6	Enhancements to ACR

During the detection, decision-making and execution phases of the ACR procedures defined in 3GPP TS 23.558 [2], an AC association Profile can be used to identify ACs requiring a common EAS. This information can be used in the ACR procedures to coordinate the transitioning of an AC to a common EAS or transitioning a group of already associated ACs from a common S-EAS to a common T-EAS.

NOTE 1: Coordination of the ACRs for AC association members is to be addressed in the normative phase.

Enhancements to 3GPP TS 23.558[2] clause 8.8.3.2	Discover T-EAS procedure

Steps:

- The S-EES checks if a registered or cached T-EAS satisfies the AC association Profile (e.g., Common EAS aggregate Service KPIs ) stored in the EEC context and can serve as a common EAS for the associated ACs.

- The EAS discovery filter (Table 7.27.2.3-1) within the EAS discovery request issued from the S-EES to the T-EES includes AC association Profile information (e.g., Common EAS aggregate Service KPIs).

- The T-EES discovers the T-EAS(s) utilizing the Common EAS aggregate Service KPIs of the AC association profile, such that a common EAS able to serve the associated ACs is provided in the response.

Enhancements to 3GPP TS 23.558 clause 8.8.3.3	Retrieve T-EES procedure

NOTE 2: In the “assumed common EES” (i) option, T-EES equals S-EES, so no enhancements are required for 3GPP TS 23.558 clause 8.8.3.3.

Steps:

1.	The T-EES retrieve request (3GPP TS 23.558 [2] Table 8.8.4.6-1) from the S-EES to the ECS is enhanced to optionally include AC association Profile.

2.	The ECS interacts with the 3GPP core network to retrieve the UE locations.

The ECS determines a common T-EES for the AC association.

NOTE 3: The ECS uses AC association Profile, UE location, as well as CAAR query as detailed in clause 7.27.2.Z to make this determination.

In addition, if an “associated UE group ID” is provided in the AC association Profile, the ECS can interact with the 3GPP core network to determine the number of UEs present in candidate EES(s) service area(s). Alternatively, the ECS uses “Associated UEs service area” in the AC association Profile to determine whether the candidate EES service area meets the needs of the AC association. The ECS uses the associated UE group ID and UE locations to determine whether T-EESs are candidates for serving the AC association.

Enhancements to 3GPP TS 23.558 clause 8.8.3.4	ACR launching procedure

Steps:

- The ACR request message (3GPP TS 23.558 [2] Table 8.8.4.4-1) sent to the EES by EEC is enhanced to optionally include an AC association ID.

- If the request in step 1 is for ACR initiation and if the EAS notification indication in ACR initiation data is provided in the step 1 request and the EAS has subscribed to receive such notification, the EES includes the association ID of the AC association in the notification.

The notified EAS uses the AC association ID during the ACR status update procedure and stores it with the Application context maintained. The content of the application content maintained by EAS is out of scope of the current document.

Enhancements to 3GPP TS 23.558 [2] clause 8.8.3.8	ACR status update procedure

Steps:

- The EAS sends the ACR status update request message (3GPP TS 23.558 [2] Table 8.8.4.19-1) to the S-EES that is enhanced to include an association ID of the AC association for which the ACR has been performed.

NOTE 4: S-EAS obtains the association ID via pre-provisioning, from an ACR management event notification or directly from the AC and stores it in the Application Context. T-EAS obtains the association ID via pre-provisioning or via ACT.

- If ACR is successful, the receiving EES uses the association ID information to update the information about the AC associations served.

In the option “with CAAR” (ii) the EES update is to the information available at CAAR. The S-EES removes its corresponding (association ID, EES ID) tuple from CAAR. The T-EES adds its corresponding (association ID, EES ID) tuple to CAAR.

NOTE 5: In the “assumed common EES” (i) option, no enhancement to step 2 is required.

#### 7.27.2.7	Other procedural enhancements

Enhancements to 3GPP TS 23.558[2] clause 8.9.1.1	EEC Context handling at EEC registration

If the EEC registration request does not include a previously assigned EEC Context ID value, the receiver EES creates an EEC Context. The receiver EES assigns an EEC context ID and sets the source EES Endpoint to its own Endpoint. The EEC ID, UE Identifier and List of AC association profiles are set based on the corresponding registration request parameters.

Enhancements to 3GPP TS 23.558[2] clause 8.9.1.5	Other EEC Context handling

When the EES determines that a registered EAS is providing services to an AC which is part of an AC association Profile, the EES updates the corresponding Service Session context with the Association ID. Conversely, when the EES determines that a Service Session is no longer used within an AC association, the EES removes the Association ID from the corresponding Service Session context.

In the option “with CAAR” (ii) the EES also updates the information about served AC Associations available at CAAR accordingly. During ACR, the S-EES removes its corresponding (association ID, EES ID) tuple from CAAR. The T-EES adds its corresponding (association ID, EES ID) tuple to CAAR.

#### 7.27.2.8	New procedures

##### 7.27.2.8.1	New procedures for option ii

###### 7.27.2.8.1.1	General

The new procedures described in clauses 7.27.2.8.1.2 and 7.27.2.8.1.3 are necessary for option (ii)

##### The CAAR function is that of maintaining information on AC Association serviced per EES and can also be integrated in ECS functionality.

##### 7.27.2.8.1.2	EES Update of AC Associations with CAAR (option ii)

The EES procedure for updating AC Association information available at CAAR uses an update request in which the following information is provided to the CAAR: EES ID, list of association IDs of the associations being served, registrar ECS information including list of EDNs. This information is updated by EES when any of this information changes or is about to change (e.g., in advance of EES de-registration).

##### 7.27.2.8.1.3	ECS Query of AC Associations with CAAR (option ii)

The ECS procedure for querying the AC Association information at CAAR uses the association IDs received (e.g., in a service provisioning request) and its own ECS information. The response provides a list of its registered EESs which serve the AC Association Profiles IDs being queried for.

NOTE: Whether and how CAAR queries are used to direct EECs to a different ECS for the purpose of receiving services consistent with the AC Association is to be addressed in the context of enabling inter-ECSP deployments.

### 7.27.3	Solution evaluation

This solution addresses Key Issue #17, discovery of a common EAS, while providing the following functionality addressing the first two open issues of KI #17:

- Introduces specific, clarifying, and consistent terminology for use in the common EAS scenario, e.g., “AC association”, “pre-grouped” (AC Association), “dynamic grouping” (AC Association), etc.

- Harmonizes support for usecases in which determining a common EES is not necessary (i.e., “assumed common EES” option) with support for usecases in which determining a common EES is necessary (i.e., option “with CAAR”) as described in clause 7.27.0.

- Provides support for various types of AC associations, i.e., “pre-grouped” and “dynamic grouping”, as described in clause 7.27.2.2.1.2. as well as details on determining grouping (in clause 7.27.2.2.1.2)

- Describes (in clause 7.27.2.8) new procedures for interaction with CAAR

- Specifies the impact of introducing this functionality on the following existing procedures: Service provisioning, EAS discovery and EEC context handling.

Solution #27 provides the following functionality addressing the last open issue of KI #17:

- Description of  ACR-related procedures. Specifically, impacts on all the following ACR-related procedures are detailed: Discover T-EAS, Retrieve T-EES, ACR launching, ACR status update.

- Coordination of ACR operations is partly addressed in clause 7.27.2.6 by indicating the information which can be used to address this issue. The clause includes a NOTE clarifying that based on this information, the procedures for coordination of the ACRs for AC association members are to be addressed in the normative phase.

This solution can impact Rel-17 architecture though through the introduction of CAAR as a new entity.

Editor’s Note: Because the CAAR query and CAAR update procedures in clause 7.27.2.8.1 are not synchronized, a race condition can happen (i.e., ACs in association may get different common EAS). How to address that issue is FFS.