3GPP TSG-SA WG6 Meeting #51-e	S6-222979

e-meeting, 10th-19th October 2022	(revision of S6-222672)

Source:	Convida Wireless LLC

Title:	Solution #15 update to address EN

Spec:	3GPP TR 23.700-98 v.1.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

1. Introduction

This is a  proposal to resolve the final EN in Solution #15 by clarifying that registered EEC may provide EAS selection in the EEC registration update.

2. Reason for Change

Provide further details for solution #15 and resolve theEN.

3. Proposal

It is proposed to agree to this solution #15 update to 3GPP TR 23.700-98.

* * * First Change * * * *

## 7.15	Solution #15: Initial EAS selection declaration

### 7.15.1	Architecture enhancements

None.

### 7.15.2	Solution description

This solution addresses KI#8 and KI#14. In this solution, the EES can know the selected EAS and the EES is enabled to trigger the EAS traffic influence after initial EAS is being determined.

Figure 7.15.2-1: Initial EAS selection declaration

In Figure 7.15.2-1 step 1 and 2, the EEC performs the start-up procedures for initial service provisioning and EAS discovery. EEC may send EAS discovery to multiple EESs. If registration is required by an EES, EEC registers into the EES before EAS discovery.

In step 3, the EEC (or AC and EEC) selects the initial EAS from the discovered EAS candidates.

In step 4, the EEC sends Selected EAS declaration request with AC ID, EAS ID, EAS endpoint and UE ID to the selected EES (which is determined based on the selected EAS).

The EES, in step 5:

-	may apply the EAS traffic influence with the N6 routing information of the EAS in the 3GPP Core Network, based on application KPIs and if the EAS traffic influence was not done before.

NOTE 1:	EES can also influence the EAS traffic in advance.

The EEC is then responded by the selected EES with success/failure of the request in step 6.

NOTE 2:	It is up to the AC to decide when to connect to the selected EAS (either immediately or wait for a while) once the AC knows the selected EAS.

NOTE 3:	The AC is not depicted in above figure for simplicity and solution to address interaction between AC and EEC is related to KI#4 for step 1 to 3.

Table 7.15.2-1 describes information elements for the selected EAS announcement request sent from the EEC to the serving EES.

Table 7.15.2-1: Selected EAS announcement request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | The identifier of the UE. |
| AC ID | O | The Application Client ID. |
| Security credentials | M | Security credentials. |
| Selected EAS ID | M | Selected EAS identifier. |
| Selected EAS Endpoint | M | Endpoint of the selected EAS. |

Currently, the EES can be aware of ongoing AC-EAS session if EAS uses EDGE-3 exposure APIs with UE IP address as input. The APIs are Eees_UEIdentifier and Eees_SessionWithQoS.

NOTE 4:	SSC creation requires EEC Context existence. It can be determined in the normative phase whether the timing for SSC creation depends on when Eees_UEIdentifier or Eees_SessionWithQoS is invoked. It can also be possible that the SSC is not created at all during the entire AC-EAS communication period.

As an alternative to sending the Selected EAS announcement request, registered EECs can provide an EEC registration update providing or updating the Selected EAS Endpoint IE, as introduced by solution #39.

### 7.15.3	Solution evaluation

This solution addresses KI#14. It allows the EES to be aware of the selected EAS in initial service start so that the EES can control application traffic influence for the initial application traffic. This solution also addresses KI#8 about how the EES is aware of the application session information. It is a viable solution.

* * * Next Change * * * *

## 10.2	Key issue evaluations

### 10.2.0	General

All the key issues and solutions specified in this technical report are listed in Table 10.2.0-1.

The table provides a mapping of the key issues to the related solutions. It also lists the dependencies on other working groups.

Table 10.2.0-1 Key issue and solutions

| Key issues (evaluation clause reference) | Solution | Solution (clause reference) |  |  |  | Dependency on other working groups |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Key issue #1: Enhanced notification service to the EEC | Solution #1: Service provisioning via push notification |  | 7.1 |  |  |  |  |
|  | Solution #3: Service provisioning triggering via SMS over NAS |  | 7.3 |  |  |  |  |
|  | Solution #20: Propagation of EEL notifications to EEC using Edge Notification Server |  | 7.20 |  |  |  |  |
| Key issue #2: Enablement of Service APIs exposed by EAS | Solution #8: EAS Service API enablement using CAPIF |  | 7.8 |  |  |  |  |
|  | Solution #11: A deployment option for alignment with ETSI MEC using CAPIF |  | 7.11 |  |  |  |  |
| Key issue #3: Enhancements to service continuity planning | Solution #6: ACR update in service continuity planning |  | 7.6 |  |  |  |  |
|  | Solution #7: EES monitors UE mobility for service continuity planning |  | 7.7 |  |  |  |  |
|  | Solution #12: Service continuity planning allowance |  | 7.12 |  |  |  |  |
|  | Solution #21: Prediction expiration time for service continuity planning enhancement |  | 7.21 |  |  |  |  |
|  | Solution #37: ACR request trigger timing |  | 7.37 |  |  |  |  |
| Key issue #4: EDGE-5 | Solution #22: Support simultaneous EAS connectivity in ACR |  | 7.22 |  |  |  | SA3 |
|  | Solution #34: EDGE-5 APIs |  | 7.34 |  |  |  |  |
| Key issue #5: Alignment of EDGEAPP and ETSI MEC | Solution #11: A deployment option for alignment with ETSI MEC using CAPIF |  | 7.11 |  |  |  | SA5 |
|  | Solution #36: Alignment of EDGEAPP and ETSI MEC |  | 7.36 |  | SA5 |  |  |
| Key issue #6: Edge services support across ECSPs | Solution #4: ECS discovery through serving ECS to support edge services across ECSPs |  | 7.4 |  |  |  |  |
|  | Solution #5: ECS enhancement to discover EESs via other ECSs to support edge services across ECSPs |  | 7.5 |  |  |  |  |
|  | Solution #13: Update ECS configuration information |  | 7.13 |  |  |  | SA2 |
| Key issue #7: Application traffic filter exposure | Solution #2: Traffic filter support for EDGE-3 API addressing application traffic detection |  | 7.2 |  |  |  |  |
| Key issue #8: EAS selection synchronization | Solution #15: Initial EAS selection declaration Solution #39: EAS selection synchronization at registration |  | 7.15 7.39 |  |  |  |  |
| Key issue #9: Enhancement of dynamic EAS instantiation triggering | Solution #32: Dynamic EAS instantiation triggering and notification |  | 7.32 |  |  |  | SA5 |
|  | Solution #33: Support for EEC Discovery of EAS(es) before instantiation |  | 7.33 |  |  |  | SA5 |
|  | Solution #40: EAS instantiation status provisioned by ECS |  | 7.xx |  |  |  | SA5 |
| Key issue #10: Support for roaming UEs | Solution #4: ECS discovery through serving ECS to support edge services across ECSPs |  | 7.4 |  |  |  | SA3 |
|  | Solution #5: ECS enhancement to discover EESs via other ECSs to support edge services across ECSPs |  | 7.5 |  |  |  | SA3 |
|  | Solution #13: Update ECS configuration information |  | 7.13 |  |  |  | SA2 |
|  | Solution #14: V-ECS Discovery via the H-ECS |  | 7.14 |  |  |  | SA3 |
| Key issue #11: ACR between EAS and Cloud Application Server | Solution #24: ACR between CAS and EAS |  | 7.24 |  |  |  |  |
|  | Solution #25: ACR between EAS and Cloud Application Server |  | 7.25 |  |  |  |  |
| Key issue #12: EEL service differentiation | Solution #12: Service continuity planning allowance |  | 7.12 |  |  |  |  |
|  | Solution #16: EAS discovery for different users |  | 7.16 |  |  |  |  |
| Key issue #13: Edge enabler layer support for EAS synchronization |  |  |  |  |  |  |  |
| Key issue #14: Application traffic influence for initially selected EAS | Solution #9: Application traffic influence trigger from EAS |  | 7.9 |  |  |  |  |
|  | Solution #15: Initial EAS selection declaration |  | 7.15 |  |  |  |  |
|  | Solution #17: Traffic influence for initial EAS discovery |  | 7.17 |  |  |  |  |
| Key issue #15: Support of constrained devices for Edge | Solution #10: low power mode support |  | 7.10 |  |  |  |  |
|  | Solution #18: Constraint device in EDGEAPP |  | 7.18 |  |  |  |  |
| Key issue #16: support of NAT deployed within the edge data network | Solution #23: UE identification with NAT |  | 7.23 |  |  |  | SA2, SA3 |
| Key issue #17: Discovery of a common EAS | Solution #27: Enabling AC Association Aware services by selecting common EASs |  | 7.27 |  |  |  |  |
|  | Solution #28: Common EAS discovery using EAS selection information |  | 7.28 |  |  |  |  |
|  | Solution #29: Discovery of a common EAS |  | 7.29 |  |  |  |  |
|  | Solution #30: Common EAS selection |  | 7.30 |  |  |  |  |
|  | Solution #31: Discover common EAS |  | 7.31 |  |  |  |  |
| Key issue #18: Linkage between EASs | Solution #26: Bundled EASs |  | 7.26 |  |  |  |  |
| Key issue #19: ACR scenario combination | Solution #19: EES determines the selected ACR scenario |  | 7.19 |  |  |  |  |
|  | Solution #35: EEC selected ACR scenarios |  | 7.35 |  |  |  |  |
| Key issue #20: Method of supporting federated EAS service |  |  |  |  |  |  |  |
| Key issue #21: Simultaneously EAS connectivity in ACR | Solution #22: Support simultaneous EAS connectivity in ACR |  | 7.22 |  |  |  |  |
| Key issue #22: EAS discovery in Edge Node sharing scenario |  |  |  |  |  |  |  |

* * * Next Change * * * *

### 10.2.8	Key issue #8: EAS selection synchronization

Solution #39 addresses KI#8 about how to enable the EES to leverage pre-existent EAS information at the EEC in order to enable service session communications efficiently for IoT devices. The solution proposes EAS selection request indicator to be sent in EEC registration request (for constrained device) to request the EES for EAS selection support. If the indication is present, the EES includes discovered EAS list along with EAS profile in the response.

Solution #15 allows the EES to be informed of the EAS selected by EEC when the initial EAS services start, so that the EAS selection information is synchronized between the EEC and EES. This solution provides the option for registered EECs to use the existing EEC registration update to provide this information. It also provides the option, for any EEC, to use a new proposed API to provide this information to the EES.

Solutions #39 and #15 are complementary and are both viable.

* * * Next Change * * * *

### 11.2.3	Solution conclusions

The study concludes with following solution considerations for the normative work:

1.	Following individual solutions, corresponding to the key issues, will be considered as candidate solutions:

i.	for Key issue #1 (Enhanced notification service to the EEC):

a.	Solution #1: Service provisioning via push notification

-	SEAL Notification Service will be specified in TS 23.434 to support the Solution #1

-	The usage of SEAL Notification Service in TS 23.558 will be captured to Solution #1

b.	Solution #3: Service provisioning triggering via SMS over NAS

-	The usage of SMS over NAS to trigger service provisioning procedure will be specified in TS 23.558 as per Solution #3.

c.	Solution #20: Propagation of EEL notifications to EEC using Edge Notification Server

-	SEAL Notification Service will be specified in TS 23.434 to support the Solution #20

-	The usage of SEAL Notification Service in TS 23.558 will be captured to Solution #20

ii.	for Key issue #2 (Enablement of Service APIs exposed by EAS):

a.	Solution #8: EAS Service API enablement using CAPIF

iii.	for Key issue #3 (Enhancements to service continuity planning):

a.	Solution #6: ACR update in service continuity planning

b.	Solution #7: EES monitors UE mobility for service continuity planning

c.	Solution #12: Service continuity planning allowance

d.	Solution #21: Prediction expiration time for service continuity planning enhancement

e.	Solution#37: ACR request trigger timing

iv.	for Key issue #4 (EDGE-5):

a.	Solution #34 (EDGE-5 APIs)

v.	for Key issue #5 (Alignment of EDGEAPP and ETSI MEC):

a.	Solution #x (<<title>>)

vi.	for Key issue #6 (Edge services support across ECSPs):

a.	Solution #x (<<title>>)

vii.	for Key issue #7 (Application traffic filter exposure):

a.	Solution #2: Traffic filter support for EDGE-3 API addressing application traffic detection

viii.	for Key issue #8 (EAS selection synchronization):

- Solution #39 (EAS selection synchronization at registration)

- Solution #15 (Initial EAS selection)

ix.	for Key issue #9 (Enhancement of dynamic EAS instantiation triggering):

a.	Solution #x (<<title>>)

x.	for Key issue #10 (Support for roaming UEs):

a.	Solution #x (<<title>>)

xi.	for Key issue #11 (ACR between EAS and Cloud Application Server):

a.	Solution #x (<<title>>)

xii.	for Key issue #12 (EEL service differentiation):

a.	Solution #12 (Service continuity planning permission)

xiii.	for Key issue #13 (Edge enabler layer support for EAS synchronization):

a.	Solution #x (<<title>>)

xiv.	for Key issue #14 (Application traffic influence for initially selected EAS):

a.	Solution #9 (Application traffic influence trigger from EAS)

b.	Solution #15 (Initial EAS selection declaration)

c.	Solution #17 (Traffic influence for initial EAS discovery)

xv.	for Key issue #15 (Support of constrained devices for Edge):

a.	Solution #x (<<title>>)

xvi.	for Key issue #16 (Support of NAT deployed within the edge data network):

a.	Solution #23 (UE identification with NAT)

xvii.	for Key issue #17 (Discovery of a common EAS):

a.	Solution #x (<<title>>)

xviii.	for Key issue #18 (Linkage between EASs):

a.	Solution #26 (Bundled EASs)

xix.	for Key issue #19 (ACR scenario combination), see also clause 10.2.19:

a.	The principle of Solution #19, #35, #38 that the EEL will offer support for utilizing a combination of ACR scenario(s) will be followed.

b.	The principle of Solution #35 that the EAS selection entity performs selection of the ACR scenario combination will be followed; the EEC will select zero ACR scenario or a single ACR scenario or multi-ACR scenarios in the ACR scenario list according to the EEL participants service continuity capabilities and AC requirements.

c.	The principle of Solution #35 that the ACR scenario list is communicated to the EES via the selected EAS announcement request will be followed, the EAS announcement request will be enhanced with the ACR scenario list.

d.	The principle of Solution #19 and #35 that the ACR scenario list is communicated to the EAS via the ACR selection notification and that the EAS will subscribe to such notification will be followed; the ACR selection notification will provide the selected ACR scenario list.

e.	The principle of Solution #19 and #35 that each ACR decision-making entity (e.g. EEC/EES/EAS) will use the ACR scenario list to decide if ACR detection needs to be performed will be followed; a gating condition that the ACR scenario is present in the ACR selected scenario list will be added to the ACR detection phase of every ACR scenario.

f.	The principle of Solution #38 that ACR execution will be coordinated after ACR detection happens will be followed; the ACR management event notification and the ACR information notification will be enhanced with information about start of ACR execution.

xx.	for Key issue #20 (Method of supporting federated EAS service):

a.	Solution #x (<<title>>)

xxi.	for Key issue #21 (Simultaneously EAS connectivity in ACR):

a.	Solution #22: Support simultaneous EAS connectivity in ACR

xxii.	for Key issue #22 (EAS discovery in Edge Node sharing scenario):

a.	Solution #x (<<title>>)

2.	Individual solutions, not listed under bullet 1 may be adopted in technical specification with appropriate enhancements.