3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230146(Merged Draft)

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

Source:	Ericsson

Title:	SEALDD server relocation

Spec:	3GPP TS 23.433 v1.0.0

Agenda item:	8.14

Document for:	Approval

Contact:	Wenliang Xu (wenliang.xu@ericsson.com)

1. Introduction

This pCR updates SEALDD TS with TR sol#10.

2. Reason for Change

The TR solution#10 is added in the TS.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.433.

* * * First Change * * * *

## 9.X	SEALDD server relocation

### 9.X.1	General

The SEALDD server may be relocated due to UE mobility or load re-balance.

If SEALDD server is adapted to EDGEAPP as an EAS, the EAS relocation procedure can be used to support SEALDD server relocation for both UE mobility and SEALDD server load re-balance. If SEALDD is not adapted to EDGEAPP, for UE mobility, the SEALDD client can discover (e.g. using DNS) a new SEALDD server in the target area and establish a new SEALDD communication channel including the old SEALDD communication channel information. For load re-balance, the SEALDD server can discover (e.g. using DNS) an equivalent SEALDD server and communicate with the new SEALDD server.

Based on existing service continuity mechanism supported by 3GPP core network (e.g. BP/ULCL), during SEALDD server relocation with UPF change, the new UPF takes care of the existing unfinished application traffic flow towards the old VAL server and inter-UPF tunnel is used to forward the traffic. For new application traffic flow which may have UE's new IP address as source IP address, the new SEALDD server sends it directly to the new VAL server.

For SEALDD server relocation, the inter-SEALDD server communication via SEALDD-E reference point is needed, which transfers the SEALDD context from the old SEALDD server to the new SEALDD server.

The following procedures detail the EDGEAPP ACT part between old SEALDD server (i.e. S-EAS) and new SEALDD server (i.e. T-EAS) as described in 3GPP TS 23.558 [8], clause 8.8.2.2 to clause 8.8.2.6. Also, a high-level flow is provided to show the scenario used in EDN.

NOTE:	The way to provide SEALDD server endpoint can be via VAL server or via EES, and discovery of the new SEALDD server can utilize procedure described in clause 9.4. The ACT procedure in the VAL server is also executed according to EDGEAPP ACR scenario but its detail is out of scope of SA6.

Editor's note:	The context transfer scope (service level and/or UE level) of SEALDD service subscription depends on the API details of the service subscription over SEALDD-S reference point, which is FFS.

9.X.2	Procedures

#### 9.X.2.1	SEALDD context transfer

Figure 9.X.2.1-1 describes SEALDD context transfer procedure in pull (step 1a and 2a) or push (step 1b and 2b) operation.

Pre-condition:

1.	For pull operation, the old SEALDD server endpoint is available in the new SEALDD server

2.	For push operation, the new SEALDD server endpoint is available in the old SEALDD server.

Figure 9.X.2.1-1: SEALDD context transfer

The transferred SEALDD context includes the service subscription information created upon VAL server's interaction for requesting SEALDD transmission service (e.g. step 1 and 2 in Figure 9.2.2.1-1), and also the SEALDD client and SEALDD server communication tunnel management information (e.g. UE IP address) which is created upon SEALDD client's interaction for requesting data transmission (e.g. step 5 and 6 in Figure 9.2.2.1-1).

Editor's note:	Other detailed information in SEALDD context and how to use it are FFS.

#### 9.x.2.2	SEALDD relocation with IP replacement

Figure 9.x.2.2-1 illustrate the procedure for the SEALDD relocation with IP replacement.

Pre-conditions:

1.	The VAL server discovers and selects the SEALDD server by CAPIF functions.

Figure 9.x.2.1-1: SEALDD relocation with IP replacement

1.	The regular data transmission connection is established according to clause 9.2.2.2.

2.	The source SEALDD server (i.e., SEALDD server #1) detects the user plane change or UE mobility, and perform the target SEALDD server discovery and selection procedure, as specified in clause 9.4, and the target SEALDD server should support seamless transport layer (e.g. TCP/TLS/QUIC) service continuity capability.

3.	The source SEALDD server sends a Sdd_Relocation request to the selected target SEALDD server. The request includes the source VAL server information, the SEALDD client information, the SEALDD flow ID, and may include the IP replacement indicator.

4.	Upon receiving the request, the target SEALDD server performs an authorization check. If authorization is successful, the target SEALDD server sends a response to the source SEALDD server, with the traffic descriptor of target SEALDD server (i.e., address/port) for the SEALDD client.

5.	The SEALDD context is transferred between SEALDD servers, as described in clause 9.x.2.1. Additional transport layer context (e.g. TCP/TLS/QUIC) is transferred from the source SEALDD server to target SEALDD server.

NOTE: The application context between VAL servers may be relocated during or after step 5.

6.	The source SEALDD server request 5GC to perform IP replacement procedure, as defined in clause 6.3.3 of 3GPP 23.548 [x]. The request includes the traffic descriptor of source SEALDD server (i.e., address/port), the traffic descriptor of target SEALDD server (i.e., address/port) and/or the target DNAI.

By triggering 5GC to perform the IP replacement procedure from the source SEALDD server, the UE or SEALDD client is not aware of the change of SEALDD server.

#### 9.X.2.3	SEALDD relocation in EDN

Pre-conditions:

1.	VAL Server 1 and 2 are adapted to the EDGEAPP as EAS.

2.	VAL Server 1 and 2 register its associated SEALDD server in EES as described in clause 9.4.3.2.

Figure 9.x.2.2-1: SEALDD relocation in EDN

1.	An application client on a UE, acting as a VAL client, establishes a SEALDD flow over SEALDD-UU to send application data to VAL Server 1. SEALDD client and SEALD server 1 maintain SEALDD flow information (e.g., SEALDD flow ID, VAL IDs/addresses, VAL requirements).

2.	Application data is sent via the SEALDD flow between the VAL Client and VAL Server 1.

3.	The UE moves and generates a mobility event in the 5GC.

4.	The UE's mobility event triggers the execution of an Application Context Relocation (ACR) procedure as described in 3GPP TS 23.558 [8]; or VAL server 1 triggers ACR due to load re-balancing reason. Any of the ACR scenarios detailed in 3GPP TS 23.558 [8] clauses 8.8.2.2-8.8.2.6 may occur. In this step, the first three phases of the ACR procedure are performed, up to ACT. In this step VAL Server 1 acts as EAS 1 and VAL Server 2 as EAS 2, therefore participating in corresponding signaling. VAL server 2 has been selected as SEALDD-enabled server meeting the ACR criteria to be the target EAS.

5.	Before triggering ACT, VAL Server 1 sends a SEALDD notification of ACR event to SEALDD Server 1.

6.	In response to the SEALDD notification of ACR event, SEALDD server 1 transfers the SEALDD context to the SEALDD server 2 which serves VAL Server 2 as described in clause 9.x.2.1, with push operation.

Editor's note:	Use of pull operation in a DD server relocation scenario is FFS.

7.	SEALDD server 1 applies the functionality specified in 3GPP TS 23.502 [7] clause 5.2.6.7 for AF traffic influence, providing the N6 routing information for the SEALDD client and SEALDD server 2. If the SEALDD server 2 supports transportation layer service continuity, the SEALDD server 1 additionally include SEALDD IP replacement information (i.e. SEALDD server 1 endpoint and SEALDD server 2 endpoint) in the AF traffic influence. Since the UE is not aware of SEALDD server change, the new SEALDD traffic (due to new VAL traffic sent by VAL client) is sent by UPF towards the new SEALDD server, this handling in UPF is agnostic to the SEALDD server 2.

8.	If the SEALDD server 2 has no transporation layer service continuity support, a notification is sent to SEALDD client to update the connection info, e.g. update the allocated IP address and port for SEALDD-Uu communication. Then the SEALDD client acknowledges the received SEALDD Connection info update notification. After SEALDD client is aware of the new SEALDD-Uu IP address and port, it starts to send new SEALDD traffic (received from the VAL client) over the new connection. The new SEALDD server maps the received SEALDD traffic to the application traffic according to the SEALDD traffic descriptor and VAL service ID. The new SEALDD server sends the recovered application traffic to new VAL server. The downlink application traffic sent from the new VAL server to VAL client is processed similarly.

9.	SEALDD Server 1 notifies VAL Server 1 of the completion of the SEALDD flow transfer.

10.	VAL Server 1 (acting as EAS1) and VAL Server 2 (acting as EAS 2) execute the Application Context Transfer (ACT) procedure step corresponding to the pending ACR scenario (3GPP TS 23.558 [8] clauses 8.8.2.2-8.8.2.6), which is out the 3GPP scope.

11.	The post-ACR clean-up phase is executed, as described in the corresponding ACR scenario (3GPP TS 23.558[10] clauses 8.8.2.2-8.8.2.6).

12.	The application data from the VAL Client is sent via the SEALDD flow (with SEALDD server 2) to VAL Server 2.

NOTE:	The SEALDD-UU client endpoint of the SEALDD flow in this step is maintained the same as in step 1.

9.X.3	Information flows

9.x.3.1	SEALDD context push request

Table 9.x.3.1-1 describes the information flow from the old SEALDD server to the new SEALDD server to push the SEALDD context.

Table 9.x.3.1-1: SEALDD context push request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identifies the requestor (i.e. old SEALDD server). |
| SEALDD-Uu Context | M | Identifies the context related to SEALDD-Uu connection. |
| SEALDD-S Context | M | Identifies the context related to SEALDD-S subscription. |
| Transportation layer context | O | Identifies the context related to Transportation layer. |

#### 9.x.3.2	SEALDD context push response

Table 9.x.3.2-1 describes the information flow from the new SEALDD server to the old SEALDD server for responding to the SEALDD context push request.

Table 9.x.3.2-1: SEALDD relocation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |

#### 9.x.3.3	SEALDD context pull request

Table 9.x.3.3-1 describes the information flow from the new SEALDD server to the old SEALDD server to pull the SEALDD context.

Table 9.x.3.3-1: SEALDD context pull request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identifies the requestor (i.e. new SEALDD server). |

#### 9.x.3.4	SEALDD context pull response

Table 9.x.3.4-1 describes the information flow from the new SEALDD server to the old SEALDD server for responding to the SEALDD context pull request.

Table 9.x.3.4-1: SEALDD context pull response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-Uu Context | M | Identifies the context related to SEALDD-Uu connection. |
| SEALDD-S Context | M | Identifies the context related to SEALDD-S subscription. |
| Transportation layer context | O | Identifies the context related to Transportation layer. |

#### 9.x.3.5	SEALDD enabled relocation request

Table 9.x.3.5-1 describes the information flow from the source SEALDD server to the target SEALDD server for requesting the SEALDD relocation service.

Table 9.x.3.5-1: SEALDD relocation request

| Information element | Status | Description |
| --- | --- | --- |
| source VAL server information | M (NOTE) | Identify of the source VAL server |
| SEALDD client information | M | Identify of the SEALDD client |
| SEALDD flow ID | O | Identity of the SEALDD flow. |
| IP replacement indicator | O | Indicate the IP replacement operation to the target SEALDD server |
| NOTE: The source VAL server info is used by target SEALDD server to discover and select the target VAL server. |  |  |

#### 9.x.3.6	SEALDD enabled relocation response

Table 9.x.3.6-1 describes the information flow from the target SEALDD server to the source SEALDD server for responding to the relocation request.

Table 9.x.3.6-1: SEALDD relocation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| Traffic descriptor of target SEALDD server | O | The traffic descriptor of target SEALDD server (i.e., address/port) for the SEALDD client |

### 9.X.4	APIs

9.x.4.1	General

Table 9.x.4.1-1 illustrates the APIs exposed by SEALDD server for UE’s service continuity.

Table 9.x.4.1-1: List of SEALDD server APIs for UE’s service continuity

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_DDContext | Push Request | Request/Response | SEALDD server |
|  | Pull Request | Request/Response | SEALDD server |
| Sdd_Relocation | Request | Request/Response | SEALDD server |

#### 9.x.4.2	Sdd_DDContext_Push Request operation

API operation name: Sdd_DDContext_Push Request

Description: The consumer requests to push  SEALDD context.

Inputs: See clause 9.x.3.1.

Outputs: See clause 9.x.3.2

See clause 9.x.2.1 for details of usage of this operation.

#### 9.x.4.3	Sdd_DDContext_Pull Request operation

API operation name: Sdd_DDContext_Pull Request

Description: The consumer requests to pull SEALDD context.

Inputs: See clause 9.x.3.3.

Outputs: See clause 9.x.3.4

See clause 9.x.2.1 for details of usage of this operation.

#### 9.x.4.4	Sdd_ Relocation Request operation

API operation name: Sdd_ Relocation Request

Description: The consumer requests for one time for SEALDD relocation service.

Inputs: See clause 9.x.3.5.

Outputs: See clause 9.x.3.6

See clause 9.x.2.2 for details of usage of this operation.

* * * End of Changes * * * *