3GPP TSG-SA WG6 Meeting #52-Bis-e meeting	S6-230117

e-meeting, 11th – 20th January 2023

Source:	vivo

Title:	Identities in PIN

Spec:	3GPP TS 23.542 v0.0.0

Agenda item:	8.19

Document for:	Approval

Contact:	Huazhang Lv, huazhang.lv@vivo.com

1. Introduction

Add the the identities of functional entities in PIN to section 7.3 of TS. The identities include PIN ID, PIN cliend ID and etc.

2. Reason for Change

This paper is to add the identities of functional entities in PIN.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.542 V0.0.0

* * * Start of Change 1 * * *

### 7.2.2	PIN ID

The PIN ID is a unique value in PLMN that identifies the PIN.

### 7.2.3	PIN server ID

The PIN server ID is a unique value in PLMN domain that identifies the PIN server.

### 7.2.4	PIN client ID

The PIN client ID is a globally unique value that identifies the PIN client.

The PIN client ID is used for general identities for PINE, PEMC and PEGC. For PEMC and PEGC, the PIN client ID may also be PEMC ID and PEGC ID.

### 7.2.5	Application Client ID (ACID)

The ACID identifies the client side of a particular application, for e.g. SA6Video viewer, SA6MsgClient etc. For example, all SA6MsgClient clients will share the same ACID.

In case that the UE is running mobile OS, the ACID is a pair of OSId and OSAppId.

### 7.2.6	UE ID

The UE ID uniquely identifies a particular UE within a PLMN domain. Following identities are examples that can be used:

a)	GPSI, as defined in 3GPP TS 23.501 [02].

NOTE:	To protect privacy of the user, MSISDN can be used as GPSI only after obtaining user's consent.

* * * End of Change * * *