3GPP TSG-SA WG6 Meeting #53-e	S6-230626

Athens, Greece 27th February – 3rd March 2023	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0106 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding descriptions of new functional entities and reference points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPP |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New functional entities and reference points have been added to support RNAA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Subclauses are added to describe new functional entities and reference points. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS lacks the descriptions of the new functional entities and reference points and remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.X (new), 6.3.Y (new), 6.4.X (new), 6.4.Y (new), 6.4.Z (new), 6.4.W (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.3.X	Authorization function

The authorization function consists of the following capabilities:

-	Receiving the authorization grant from the resourse owner client;

-	Receiving the authorization grant revocation request from the resource owner;

-	Providing the API invoker with the authorization information which is needed to invoke service APIs; and

-	Verifying the authorization information.

6.3.Y	Resource owner client

The resource owner client consists of the following capabilities:

-	Providing an authorization grant for service API invocations; and

-	Revoking an authorization grant for sevice API invocations.

* * * Next Change * * * *

6.4.X	Reference point CAPIF-8 (between the resource owner client and the authorization function)

The CAPIF-8 reference point, which exists between the resource owner client and the authorization function, is used for obtaining and revoking the authorization of API invocation.

The CAPIF-8 reference point supports:

-	Providing authorization for service API invocations; and

-	Revoking authorization for sevice API invocations.

NOTE:	The security aspects of CAPIF-8 will be specified by SA3.

Editor's note:	Reference to the appropriate SA3 specification is needed.

6.4.Y	Reference point CAPIF-9 (between the API exposing function and the authorization function)

The CAPIF-9 reference point, which exists between the API exposing function and the authorization function, is used for verifying the authorization information.

The CAPIF-9 reference point supports:

-	Verifying the authorization information upon the service API invocation.

NOTE:	The security aspects of CAPIF-9 will be specified by SA3.

Editor's note:	Reference to the appropriate SA3 specification is needed.

6.4.Z	Reference point CAPIF-10 (between the API invoker and the authorization function)

The CAPIF-10 reference point, which exists between the API invoker and the authorization function, is used for the API invoker within the PLMN trust domain to obtain the authorization information needed for API invocation.

The CAPIF-10 reference point supports:

-	Providing the API invoker with the authorization information.

NOTE:	The security aspects of CAPIF-10 will be specified by SA3.

Editor's note:	Reference to the appropriate SA3 specification is needed.

6.4.W	Reference point CAPIF-10e (between the API invoker and the authorization function)

The CAPIF-10e reference point, which exists between the API invoker and the authorization function, is used for the API invoker outside the PLMN trust domain to obtain the authorization information needed for API invocation.

The CAPIF-10e reference point supports:

-	Providing the API invoker with the authorization information.

NOTE:	The security aspects of CAPIF-10e will be specified by SA3.

Editor's note:	Reference to the appropriate SA3 specification is needed.