3GPP TSG-SA WG6 Meeting #55	S6-232001

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231938)

Title:	Reply LS on the reuse of EVEX as specified in TS 26.531

Response to:	LS S6-231664 on Reply LS on the reuse of EVEX as specified in TS 26.531 from SA4

Release:	Rel-18

Work Item:	ADAES

Source:	3GPP TSG SA WG6

To:	3GPP SA4

Cc:	3GPP SA2, 3GPP CT3

Contact person:	Sapan Shah

sapan.shah@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA6 thanks SA4 for their rely LS on possible reuse of the SA4-defined EVEX framework and also for providing agreed CR 0005 for TS 26.531.

Regarding “SA4 kindly invites SA6 to explain their requirements in greater detail to SA4”, SA6 would like to provide following details about the requirements.

For,

2. The UE Application shall be able to send/push collected data on-demand (i.e. without need for setting up session) to ASP through a request/response mechanism.

Consider a use case, where application client is already configured to collect required data (could be application specific way over R7 interface or client can one time fetch the configuration from DCAF over R2 interface). While using the service, the user/application client is having bad experience, and the application client needs to send collected data to the application server to take further course of action.

The requirements here are as follows:

- The client should have one time provisioning of configuration to know which performance data to collect while sending the report.

- The client should send data report at any time whenever client/user is experiencing degradation of service.

- For sporadic or infrequent data reports, the client should not be required to setup and terminate a Data Reporting Session (as specified in TS 26.532) every time it requires to share the data report(s).

For,

1. The ASP shall be able to retrieve/pull collected data on-demand (i.e. without need for setting up session) from the UE Application through a request/response mechanism.

SA4 reply indicates (trimmed):

“SA4 believes that this requirement is partially met by the EVEX framework in Release 17.”

“However, note that the above mechanism does not trigger a particular UE Application to send data on demand to the ASP, in which case the SA6 requirement is not met by the EVEX framework in Release 17.”

SA6 would like to indicate SA4 that SA6 requires a mechanism to trigger particular UE to send data on demand to the ASP.

Consider a use case, where an application server received service experience report from a UE-1, and in order to identify the problem, the application server selects few other UEs from the same location as UE-1 and requests to provide service experience report at that time immediately. This is used mainly for analytics performed in ADAES which is a SEAL service. Please note selection of UEs by application server is not within scope of SA6.

The requirements here are as follows:

- The application server/ASP should be able to pull service experience report from multiple UEs at given time.

Apart from providing above clarifications, SA6 would also like to ask following clarifications regarding EVEX:

- What exactly is a ‘session' in context of EVEX? Is it a session-oriented transport channel, or just a configuration context or something else?

- When data reporting session as specified in 26.531/26.532 is created and terminated by the DDCC?

- What do we mean by terminating a session? And why is it required?

- What are the impacts of session creation and termination on UE and network resources?

- Can application client trigger DDCC to send an immediate report?

# 2	Actions

To SA4

ACTION:

SA6 asks SA4 to consider above clarifications and provide required enhancements to EVEX framework as defined in TS 26.53 and TS 26.532 in Rel-18. SA6 also asks SA4 to provide answers to the above questions.

# 3	Dates of next TSG SA WG 6 meetings

SA6#56                 21st August – 25th August 2023 	Gothenburg, Sweden

SA6#57                 9th October – 13th October 2023 	Xiamen, China